3GPP TSG RAN WG2 Meeting #129	R2- 2501097

Athens, Greece, 17th - 21st Feb. 2025

Agenda Item:	8.8.5

Source:	ETRI

Title:		Discussion on the impact of regenerative payload

Document for:	Discussion and Decision

1	Introduction

As seen in the recently endorsed RAN3 CR [1],

16.14.4 Switchover 16.14.4.1 Definitions A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for an specific NTN payload. The feeder link switchover is a Transport Network Layer procedure. Service link switch refers to a change of the serving NTN payload. Both hard and soft feeder link switchover are supported in transparent or regenerative NTN payloads.

support of feeder link switch in transparent or regenerative payloads has been clearly specified in TS 38.300. The serving cell is not changed after feeder link switch in regenerative payload, which is different from feeder link switch in transparent payload. This different effect of regenerative payload lead to impacts on the time-based measurement initiation for feeder link switch and the description of t-Service. In this paper, we discuss the impacts and propose clarifications for them.

2	Discussion

2.1	Impacts of regenerative payload on the feeder link switch

Feeder link switch is simply a change of the transport link between a satellite and an NTN gateway and thus it is a Transport Network Layer procedure. In transparent payload, the feeder link switch always results in a change of the serving cell. However, in regenerative payload, the serving cell is not changed after the feeder link switch either for a quasi-Earth-fixed cell or an Earth-moving cell.

In regenerative payload case, the serving cell is not changed after the feeder link switch.

Noting Observation 1, we can find a significant impact of regenerative payload on the existing time-based measurement initiation for the feeder link change. As illustrated and discussed in [2], the existing time-based measurement initiation is designed for a UE to have enough time in finding the best cell because the serving cell abruptly stops serving the area covered by it at t-Service. However, as observed, the serving cell does not stop serving the area after feeder link switch in regenerative payload. Consequently, in regenerative payload case, time-based measurement initiation for feeder link switch is not necessary.

In regenerative payload, time-based measurement initiation for feeder link switch is not necessary.

Based on Observation 2, impacts of regenerative payload on the existing specifications have been investigated as follows.

In TS 38.300 [Sec. 16.14.3.3, 3],

The time-based measurement initiation may be applicable for the feeder link switchover case for cell re-selection.

The text was added in Rel-18 and thus it implicitly assumed transparent payload. Considering Observation 2, the text needs to be modified to explicitly refer to transparent payload.

The description of the time-based measurement initiation for the feeder link switch in TS 38.300 needs to be modified more clearly, i.e., it only applies for transparent payload.

Considering Observation 3, we propose the following:

RAN2 clarify that the time-based measurement initiation for the feeder link switch may be applicable for the feeder link switch only in case of transparent payload.

In TS 38.331 [Sec. 6.3.1, 4],

t-Service Indicates the time information on when a cell provided via NTN is going to stop serving the area it is currently covering. This field applies for both service link switches in NTN quasi-Earth fixed cell and feeder link switches for both NTN quasi-Earth fixed and Earth-moving cell.

In [5], clarification of t-Service is proposed for the feeder link switch in regenerative payload. We support the proposal. Considering Observation 1, the first sentence which defines t-Service contradicts the second sentence in case of regenerative payload. This is because a cell provided via NTN does not stop serving the area after feeder link switch occurring at t-Service in regenerative payload. Consequently, the second sentence should be modified.

In view of regenerative payload, the description of t-Service in TS 38.331 is contradictory to its definition; it should be modified.

To resolve this contradiction, we propose the following:

RAN2 clarify that t-Service applies for both service link switch regardless of the payload type and feeder link switch only in case of transparent payload.

3	Conclusion

In this paper, the following observations are made:

Observation 1	In regenerative payload case, the serving cell is not changed after the feeder link switch.

Observation 2	In regenerative payload, time-based measurement initiation for feeder link switch is not necessary.

Observation 3	The description of the time-based measurement initiation for the feeder link switch in TS 38.300 needs to be modified more clearly, i.e., it only applies for transparent payload.

Observation 4	In view of regenerative payload, the description of t-Service in TS 38.331 is contradictory to its definition; it should be modified.

In this paper, we propose the followings:

Proposal 1	RAN2 clarify that the time-based measurement initiation for the feeder link switch may be applicable for the feeder link switch only in case of transparent payload.

Proposal 2	RAN2 clarify that t-Service applies for both service link switch regardless of the payload type and feeder link switch only in case of transparent payload.

References

R3-247934, (BL CR to 38.300) Support for Regenerative Payload and MBS broadcast in NR NTN, Ericsson, Thales, Deutsche Telekom, Nokia, ESA, CATT, ZTE, Sateliot, Huawei, Dish Networks, Echostar, Eutelsat Group, Xiaomi, Samsung, CMCC, LG Electronics, NEC, Lenovo, ETRI, RAN3 #126, Nov. 2024.

R2-2409071, Discussion on support of regenerative payload, ETRI, RAN2 #127bis, Oct. 2024.

TS 38.300, V18.4.0, Dec. 2024.

TS 38.331, V18.4.0, Dec. 2024.

R2-2410649, Discussion on regenerative payload, Fujitsu, RAN2 #128, Nov. 2024.