3GPP TSG-RAN WG2 Meeting #129		R2-2501118

Athens, Greece, 17th – 21st February, 2025

Source:	vivo

Title:		Discussion on topology and intermediate relay UE (re)selection

Agenda Item:		8.13.2

Document for:	Discussion and Decision

# Introduction

In RAN2 #127 and #128 meeting, issues related to Relay discovery and (re)selection was discussed and some initial agreements were reached, mainly for general principles and topologies. A bunch of proposals covered by the following email about discovery and relay selection were not handled due to lack of time.

[Post127][401][Relay] Multi-hop relay discovery and (re)selection (LG)

In this contribution, we would like to further discuss the issues which are not covered in above post-email discussion and are affected by SA2 progress, such as:

- Cases of topologies to be supported based on current progress;

- The relay (re)selection at intermediate relay UE;

# Discussion

## 2.1 Intermediate relay UE reselection

In previous email discussion, the following proposal was given based on the questionnaire provided[1]:

[EASY] Proposal 8: Any relay (re)selection triggering condition is not specified for intermediate Relay UE.

It is because in that time, some companies thought that the relay (re)selection may be only performed by the multi-hop U2N remote UE to minimize the spec impact.

However, according to newest agreed SA2 CR[2], the relay (re)selection at intermediate relay UE is already supported:

##### 6.4.3.8.3 Relay reselection and mobility support <omitted…> The relay reselection procedure can be used by a 5G ProSe Intermediate UE-to-Network Relay, taking the role of the 5G ProSe Remote UE in the procedure, when its connection with a parent Relay (either a 5G ProSe Intermediate UE-to-Network Relay or a 5G ProSe UE-to-Network Relay) is lost or the link quality drops below a configured threshold. If the 5G ProSe Intermediate UE-to-Network Relay can not complete the reselection, e.g. it is not able to discover the 5G ProSe UE-to-Network Relay, or the 5G ProSe Intermediate UE-to-Network Relay determines to not perform the reselection procedure, it may send re-selection indication and optionally with a cause value to 5G ProSe Remote UE via the established multi-hop relay path. After receving the re-selection indication the 5G ProSe Remote UE may trigger the reselection of the relays as described above. During the transmition of re-selection indication to 5G ProSe Remote UE, after receving the re-selection indication a 5G ProSe Intermediate UE-to-Network Relay in the path based on configuration may determine to relay the re-selection indication to 5G ProSe Remote UE, or perform relay reselection by itself as described above. Intermediate U2N Relay0 Intermediate U2N Relay1 Intermediate U2N Relay2 Intermediate U2N Relay3 Remote UE1 U2N Relay1 U2N Relay2 Figure 6.4.3.8.3-2: Relay reselection for 5G ProSe Remote UE with 5G ProSe Multi-hop UE-to-Network Relays with Root Relay change

Therefore, we understand we need to further discuss the relay (re)selection conditions at intermediate relay UE in RAN2.

Observation 1: SA2 agreed that the relay reselection procedure can be used by a 5G ProSe Intermediate UE-to-Network Relay when its connection with a parent Relay is lost or the link quality drops below a configured threshold.

Proposal 1: RAN2 confirms that relay (re)selection triggering condition should be specified for intermediate Relay UE.

For relay selection and reselection, some conditions are already on the table.

For relay selection[1]:

[(Re)selection triggering conditions] [EASY] Proposal 6a: The following relay selection triggering conditions for Remote UE are supported for the multi-hop relay operation. [Relay selection triggering conditions for Remote UE] q) If the Remote UE has no serving cell; j) If the Remote UE does not have a selected intermediate Relay UE; [ESAY] Proposal 6b: The following relay re-selection triggering conditions for Remote UE are supported for the multi-hop relay operation at least when there is only one intermediate Relay UE. [Relay re-selection triggering conditions for Remote UE] q) If the SL-RSRP of the currently selection intermediate Relay UE is below a configured threshold; r) If the SD-RSRP of the currently selected intermediate Relay UE is below a configured threshold; s) If the upper layer indicates not to use the currently selected intermediate Relay UE; t) If the upper layer of the selected intermediate Relay UE requests the release of the PC5-RRC connection; u) If the RLF is detected on the PC5-RRC connection with the current intermediate Relay UE; v) If the Remote UE receives a notification message from the intermediate Relay UE caused by one of the following: h-1) if intermediate Relay UE detects PC5 RLF between intermediate Relay UE and last Relay UE (or serving intermediate Relay UE) h-2) if intermediate Relay UE receives RRCReconfiguration message for HO (if the intermediate Relay UE is in RRC_CONNECTED) h-3) if intermediate Relay UE performs cell reselection h-4) if intermediate Relay UE fails Uu RRC connection establishment/Resume via last Relay UE h-5) if intermediate Relay UE receives PC5-RRC connection release between intermediate Relay UE and last Relay UE (or serving intermediate Relay UE) [EASY] Proposal 6C: Following condition can be additionally considered for relay selection - If direct Uu signal strength of current serving cell of the Remote UE is below a configured signal strength threshold. [EASY] Proposal 6D: The following solution is not considered in this release: - The last Relay UE generates notification message when the Uu RSRP is decreased under a configured threshold [EASY] Proposal 6E: The following solution is not considered in this release: - The intermediate Relay UE generates notification message when the PC5 RSRP is decreased under a configured threshold. [EASY] Proposal 6F: in addition to proposal 6b, the following condition can be considered for generating notification message. - h-6) if the intermediate Relay UE receives PC5-S connection release between the intermediate Relay UE and last Relay UE (or serving intermediate Relay UE)

For relay reselection[1]:

[Relay Triggering conditions for intermediate Relay UE] - If the intermediate Relay UE has no serving cell; - If the intermediate Relay UE does not have a selected last (/serving intermediate) Relay UE; - If the SL-RSRP of the currently selected last (/serving intermediate) Relay UE is below a configured threshold; - If the SD-RSRP of the currently selected last (/serving intermediate) Relay UE is below a configured threshold; - If the upper layer indicates not to use the currently selected last (/serving intermediate) Relay UE; - If the upper layer of the selected last (/serving intermediate) Relay UE requests the release of the PC5-RRC connection; - If the RLF is detected on the PC5-RRC connection with the current last (/serving intermediate) Relay UE; - If the intermediate Relay UE receives a notification message from the last (/serving intermediate) Relay UE caused by one of the following: h-1) if last Relay UE detects Uu RLF h-2) if serving intermediate Relay UE detects PC5 RLF h-3) if last (/serving intermediate) Relay UE receives RRCReconfiguration message for HO h-4) if last (/serving intermediate) Relay UE performs cell reselection h-5) if last (/serving intermediate) Relay UE fails Uu RRC connection establishment/Resume h-6) if last (/serving intermediate) Relay UE receives Uu connection release between gNB and last (/serving intermediate) Relay UE h-7) if serving intermediate Relay UE receives PC5-RRC connection release between last (/the other serving intermediate) Relay UE and the serving intermediate Relay UE

In this phase, we understand the conditions should first be discussed/agreed for remote UE, then we can further discuss the intermediate relay UE, but to make a progress we think at least some of the conditions for intermediate relay UE can also be discussed concurrently, e.g. the ones related to link quality between intermediate relay UE and its parent relay UE, to be aligned with SA2 CR:

- when its connection with a parent Relay (either a 5G ProSe Intermediate UE-to-Network Relay or a 5G ProSe UE-to-Network Relay) is lost or the link quality drops below a configured threshold.

So, we understand the listed c) to h) can be modified to be supported. Therefore, we propose:

Proposal 2: Support following Relay (re)selection conditions for intermediate Relay UE:

- If the SL-RSRP of the currently selected last (/serving intermediate) Relay UE is below a configured threshold;

- If the SD-RSRP of the currently selected last (/serving intermediate) Relay UE is below a configured threshold;

- If the upper layer indicates not to use the currently selected last (/serving intermediate) Relay UE;

- If the upper layer of the selected last (/serving intermediate) Relay UE requests the release of the PC5-RRC connection;

- If the RLF is detected on the PC5-RRC connection with the current last (/serving intermediate) Relay UE;

- If the intermediate Relay UE receives a notification message from the last (/serving intermediate) Relay UE.

According to SA2 CR, ‘During the transmition of re-selection indication to 5G ProSe Remote UE, after receving the re-selection indication a 5G ProSe Intermediate UE-to-Network Relay in the path based on configuration may determine to relay the re-selection indication to 5G ProSe Remote UE, or perform relay reselection by itself as described above’. To minimize the spec impact, we think we can leave it to intermediate relay UE implementation whether to perform relay (re)selection itself or transfer the re-selection indication to child node.

Proposal 3: When Relay (re)selection conditions are fulfilled, it is up to intermediate Relay UE implementation whether to do relay (re)selection or send the relay (re)selection indication to its child node.

## 2.2 Cross path topologies and Uu threshold

For topologies, some agreements reached so far are as follows:

Agreements:

The following cases are supported for L2 multihop relay:

- One last Relay UE can have two connections with one intermediate Relay UE and one Remote UE (the intermediate Relay UE and Remote UE are physically different UE).

- Two physically different Remote UE(s) can have each indirect path via the same intermediate Relay UE and the same last Relay UE.

FFS if the last relay UE can use the same L2ID for both of the connections in either case.

Cases with two indirect paths to the gNB for the same remote UE are not supported.

An Intermediate Relay UE can serve multiple multi-hop indirect paths of different Remote UEs.

For multi-hop structure, the only topology supported so far is the tree-like topology as in Figure 1:

Figure 1. Tree-like multi-hop U2N relay topology

Except Figure 1, we understand another topology which companies are interested is the following cross path topology[1]:

(Figure 2.5B-3)

It should be noticed that SA2 sent a reply LS to us[3], indicating that intermediate relay needs to establish PC5 link with the relay from which the discovery announcement message is received before sending/forwarding the discovery announcement message.

Regarding the following question about Model A discovery from RAN2, SA2 would like to provide the answer as below. - Question from RAN2: Whether an intermediate relay needs to establish PC5 link with the relay from which the discovery announcement message is received before sending/forwarding the discovery announcement message. - Answer from SA2: Yes. To clarify behaviour of Intermediate UE-to-Network Relay related to Model A discovery including the above aspect, SA2 agreed the attached CR. SA2 also discussed behaviour of Intermediate UE-to-Network Relay related to Model B discovery that Intermediate UE-to-Network Relay is not required to setup PC5 link with the relay from which the discovery response message is received before forwarding the discovery response message.

So in our understanding, this cross-path topology is not supported as the intermediate relay UE will only have one parent relay with a layer-2 link been established between them. We suggest to exclude this case.

Proposal 4: Cross-path topology (i.e. (physically) different remote UE selects each (physically) different last relay UE via one (physically same) intermediate relay UE) is not supported.

Then for the tree-like topology in Figure.1, for the following FFS:

FFS if the last relay UE can use the same L2ID for both of the connections in either case.

We understand the last relay UE can only use one L2 ID for both connections. The L2 ID(s) identify the PC5-RRC connections, so different L2 ID means different node from RAN2’s perspective, so different L2 ID will make the tree-like topology same as the cross-path topology from RAN2’s perspective.

Proposal 5: Last relay UE can only use the same L2 ID for both connections in tree-like multi-hop U2N relay topology as in Figure.1.

Another FFS we would also like to briefly discuss it the following one in RAN2 #128[4]:

Agreements:

A lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE.

If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE (i.e., minimise number of hops when possible).  FFS if operation as an intermediate relay UE towards a different cell would be supported.

The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE.  If the upper bound is not configured, there is no threshold, but this does not override the previous agreement.  FFS if there would be additional restrictions in the case of no upper bound.

In our understanding, the possible cases for upper bound configuration for intermediate relay UE are as follows:

Figure 2. Different cases of threshold configuration for intermediate relay UE and last relay UE

According to the agreements, the UE should be last relay whenever possible, so that if the Uu RSRP is in the Zone A1, the UE cannot be intermediate relay UE anyway. Moreover, it is not reasonable that the upper bound for intermediate relay UE is configured higher than that of last relay UE (zone B1). So the reasonable configuration is to configure the upper bound for intermediate relay UE in Zone B2. When the upper bound is not configured, then the actual upper bound for intermediate relay UE should be the lower bound for last relay UE, according to our previous agreement. We can make it clear in the agreement.

Proposal 6: If no upper bound of Uu RSRP for the UE to operate as an intermediate relay UE is configured, the UE will instead use the lower bound of Uu RSRP for the UE to operate a last relay UE to be the one to operate as an intermediate relay UE.

# Conclusion

This contribution mainly discussed the intermediate relay UE (re)selection conditions, the topologies and the Uu threshold configuration for intermediate relay UE. The contribution concludes with:

Observation 1: SA2 agreed that the relay reselection procedure can be used by a 5G ProSe Intermediate UE-to-Network Relay when its connection with a parent Relay is lost or the link quality drops below a configured threshold.

Intermediate relay UE (re)reselection

Proposal 1: RAN2 confirms that relay (re)selection triggering condition should be specified for intermediate Relay UE.

Proposal 2: Support following Relay (re)selection conditions for intermediate Relay UE:

- If the SL-RSRP of the currently selected last (/serving intermediate) Relay UE is below a configured threshold;

- If the SD-RSRP of the currently selected last (/serving intermediate) Relay UE is below a configured threshold;

- If the upper layer indicates not to use the currently selected last (/serving intermediate) Relay UE;

- If the upper layer of the selected last (/serving intermediate) Relay UE requests the release of the PC5-RRC connection;

- If the RLF is detected on the PC5-RRC connection with the current last (/serving intermediate) Relay UE;

- If the intermediate Relay UE receives a notification message from the last (/serving intermediate) Relay UE.

Proposal 3: When Relay (re)selection conditions are fulfilled, it is up to intermediate Relay UE implementation whether to do relay (re)selection or send the relay (re)selection indication to its child node.

Topology

Proposal 4: Cross-path topology (i.e. (physically) different remote UE selects each (physically) different last relay UE via one (physically same) intermediate relay UE) is not supported.

Proposal 5: Last relay UE can only use the same L2 ID for both connections in tree-like multi-hop U2N relay topology as in Figure.1.

Uu threshold for intermediate relay UE

Proposal 6: If no upper bound of Uu RSRP for the UE to operate as an intermediate relay UE is configured, the UE will instead use the lower bound of Uu RSRP for the UE to operate a last relay UE to be the one to operate as an intermediate relay UE.

# References

- R2-2410981, Report of [POST127][401][Relay] MH relay discovery and (re)selection, LG Electronics Inc;

- S2-2501292, Clarification on Multi-hop Relay Reselection, Huawei, HiSilicon, China Telecom, 3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e, E-meeting, Jan 20 – 24, 2025;

- S2-2501334, Reply LS on relay discovery announcement;

- RAN2 #128 chair notes;