3GPP TSG RAN WG2 Meeting #129                          	   R2-2501162

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.5.2

Source:	Qualcomm Incorporated

Title:	Discussion on On-demand SSB SCell Operation

WID/SID:	Netw_Energy_NR_enh-Core – Release 19

Document for:	Discussion

# Introduction

The objectives of on-demand SSB SCell are outlined with the follows [1].

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this contribution, we discuss on-demand SSB transmission scenarios and on-demand SSB cases for on-demand SSB Scell operations and the potential RAN2 specification work.

# Discussion

## L3 measurement for Case #2

At RAN1 #116, the following aggreements were made on the following use cases and scenarios for further study [2].

Agreement Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for Agreement For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios. - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321) - This does not preclude SCell for which activation is completed - FFS: The case where SCell activation is completed FFS: Application timing between NW triggering message and on demand SSB transmission

On-demand SSB may be cell defining (i.e., SSB with associated SIB1) or non-cell defining (i.e., SSB without associated SIB1). At RAN1#116-bis, the following agreement was made for always-on SSB (Case #2) and no always-on SSB (Case #1).

Agreement - For a cell supporting on-demand SSB SCell operation, - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not. - For on-demand SSB on the cell, down select between the following alternatives - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not. - Alt-2: On-demand SSB is limited to non-cell-defining SSB. - FFS: Further limitations to on-demand SSB

At RAN1#118, the following agreement was made for always-on SSB (Case #2) and no always-on SSB (Case #1) [5] with non-cell-defining SSB that is not on synchronization raster.

Agreement For a cell supporting on-demand SSB SCell operation, at least the following is supported - On-demand SSB on the cell is not located on synchronization raster. - On-demand SSB on the cell is non-cell-defining SSB FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

However, RAN1 could not conclude if cell-defining on-demand SSB on sync-raster is supported.

At RAN1#118bis, the following agreement was made for always-on SSB (Case #2) and on-demand SSB [7].

Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases. - Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB - Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain

Additionally, RAN4 LS to RAN1 (cc: RAN2) [8] listed the following questions for Case #2 on-demand SSB.

In RAN4#112bis meeting, RAN4 discussed the impacts on RRM requirements for on-demand SSB in Obj. 1 and SSB adaptation in Obj. 3. During the discussion, RAN4 identified the following questions. [RAN4 Questions]: Obj. 1 related to on-demand SSB SCell operation_ Based on the discussion, RAN4 identified that the relation between always-on SSB and on-demand SSB (i.e., Case#2 as agreed in RAN1) has impacts on UE’s behavior and requirements in SCell. RAN4 would like RAN1 to clarify at least the following aspects: - Q1: What is the relation in terms of periodicity between always-on SSB and OD-SSB? - Q2: What is the relation in terms of time location between always-on SSB and OD-SSB? - Q3: What is the relation in terms of frequency location between the always-on SSB and OD-SSB? - Q4: What is the spatial relation between the always-on SSB and OD-SSB? Obj.3 related to SSB adaptation in time domain For the SSB adaption in the objective of adaptation of common signal/channel transmits, RAN4 would also like RAN1 to clarify the relation for SSB adaptation at least for following aspects. - Q1: What is the relation in terms of time location before and after SSB adaptation? - Q2: What is the relation in terms of frequency location before and after SSB adaptation? - Q3: What is the spatial relation before and after SSB adaptation?

Based on the agreements (made so far) on Case #2 with always-on SSB and on-demand-SSB, we can have the following combinations:

Combination 1: Always-on SSB is CD-SSB on sync raster and OD-SSB is NCD-SSB not allocated on sync raster, as shown in Figure 1.

With always-on SSB and on-demand SSB allocated at different frequency allocations (e.g., always-on SSB is on sync rater and on-demand is off sync raster), a UE has to track always-on SSB on sync raster and on-demand SSB off sync raster. This complicate UE’s behavior for monitoring and measuring the always-on SSB and on-demand SSB.

Additionally, with carrier aggregation, multiple SCells may have different frequency allocations respectively for always-on SSB and on-demand SSB, which further complicate UE’s monitoring and measuring the always-on SSB and on-demand SSB on different SCells.

Figure 1. Always-on SSB and OD-SSB at different frequencies

Observation 1. A UE has to track always-on SSB and on-demand SSB at two separate frequencies, which adds complexity in tracking SSBs and contradicts with the legacy specs in which the SSB configured for a particular L1 procedure is always on the same frequency.

Observation 2. With carrier aggregation, the complexity for a UE to track SSBs is further increased if multiple SCells deploy such combination.

Combination 2: Always-on SSB is NCD-SSB not allocated on sync raster and OD-SSB is NCD-SSB not allocated on sync raster, as shown in Figure 2.

Figure 2 Always-on SSB and OD-SSB at same frequency

At RAN1 #119, the following agreements were made to respond RAN4’s question [11].

Agreement Response to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?) of Obj.1: - The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. - The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB. - RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB. Further update to be made based on RAN1#119 progress.

Agreement Response to Q2 (What is the relation in terms of time location between always-on SSB and OD-SSB?) of Obj.1: - RAN1 understands the time location of OD-SSB in Q2 refers to the time location of possible OD-SSB burst. - RAN1 is still discussing the relation in terms of time location between always-on SSB and OD-SSB

Agreement Response to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?) of Obj.1: - The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB.

Agreement Down-select at least one of the following alternatives. - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - Alt 2: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB. - Alt 3: Do not support the case where always-on SSB is CD-SSB on a synchronization raster. Down-select at least one of the following alternatives. - Alt A: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB can be same or different from the frequency location of always-on SSB, subject to its configuration. - Alt B: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB. - Alt C: Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster.

Agreement Response to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?) of Obj.1: - SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

Observation 3. RAN1 has agreed Combination 2 (always-on SSB is NCD-SSB and off sync raster), where on-demand SSB is allocated at the same frequency with always-on SSB and on-demand SSB is with a periodicity of SSB periodicities on the SSB transmission grid.

With the agreed Combination 2, the same transmit power may be applied to both always-on SSB and on-demand SSB, which simplifies UE’s monitoring and measurement filtering.

Observation 4. Same transmit power may be applied to both always-on SSB transmission and OD-SSB transmission.

At RAN2 #127, the following agreement was made for L3 measurement on on-demand SSB [6].

Agreements: 4. Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1). 5. For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated. 6. RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements. 7. RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

At RAN2 #127bis, the following options were discussed for the L3 measurement of Case #2, and no conclusion had made.

L3 RRM Framework - Option1: Based on different measurement configuration when OD-SSB is transmitted - Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted

RAN4 informs RAN2 with the agreement and question in the LS to RAN2 (cc: RAN1) [9].

RAN4#112bis extensively discussed impact on RRM requirements from SSB adaptation. The following agreements were reached after discussion: Agreements: - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption: - L1-RSRP for serving cell - L1-SINR - BFD - CBD - SCell activation - TCI state switching - NR intra-frequency L3 measurement - NR inter-frequency L3 measurement However, L3 measurement requirement in RAN4 is defined based on SMTC configuration rather than SSB configuration in legacy. RAN4 would like to ask RAN2 that whether there is any SMTC impact due to SSB adaptation.

For Case #2 (always-on SSB), L3 measurement is already supported with SMTC configured per always-on SSB. In our view, the on-demand SSB of Case #2 is more for SCell time/frequency synchronization, L1 measurements and SCell activation, instead of L3 measurement.

Observation 5. For Case #2 (always-on SSB), L3 measurement is already supported with always-on SSB.

As shown in Figure 3, with both always-on SSB and OD-SSB are NCD-SSB and transmitted on the same frequency location, network may configure SMTC period with the periodicity of the always-on SSB and SMTC duration based on the decision on whether on-demand SSB is included.

Furthermore, since same transmit power is applied to both always-on SSB and on-demand SSB, the combination of the measurements may be simplified. Thus, it can be up to UE implementation for how to filter and combine the measurements of always-on SSB and on-demand SSB.

Figure 3 SMTC for Case #2 (always-on SSB)

Observation 6: For Case #2 (always-on SSB), with both always-on SSB and OD-SSB as NCD-SSB and transmitted on the same frequency location, network may configure an SMTC with consideration of OD-SSB.

Observation 7. For Case #2 (always-on SSB), the SMTC configured for always-on SSB can be used for both always-on and OD-SSB, which should be network’s decision.

Observation 8. For Case #2 (always-on SSB), it is up to UE implementation for how to filter and combine the measurements of always-on SSB and on-demand SSB.

For Case #2 L3 measurements, based on the above observations, we make the following proposals.

Proposal 1. RAN2 agrees that no more specification work on L3 measurements is needed for Case #2 (always-on SSB).

Since on-demand SSB is transmitted in burst, which may not be observed consistently for neighboring cell monitoring, on-demand SSB based L3 measurement should not be supported for periodic neighboring cell measurement and report.

Proposal 2. On-demand SSB based L3 measurement and report are not supported for neighboring cell monitoring.

## MAC CE for On-demand SSB transmission indication

On-demand SSB is transmitted when it is triggered, e.g., for SCell time/frequency synchronization, L1/L3 measurements, SCell selection or activation, etc.

The following options were agreed at RAN1 #116-bis [3] for on-demand SSB signaling from gNB.

Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. Timing for on-demand SSB transmission (e.g., when the triggered SSB starts and ends) will be separately discussed. Agreement For a cell supporting on-demand SSB SCell operation, further study the following options. - Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication. - Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided. - FFS: Details of the signaling - Other options are not precluded. FFS: Details on On-demand SSB transmission indication

Additionally, the following agreement on gNB’s indication of on-demand SSB transmission was agreed at RAN1 #117 [4].

Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. FFS: Scenarios where the above signalings are applicable

Furthermore, the following agreement on deactivation of on-demand SSB transmission was agreed at RAN1 #118bis [7].

Agreement For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - Option 3: Configuration/indication of the duration of on-demand SSB transmission window - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI - FFS: Each option is applicable to which Cases or Scenarios FFS: Details related to each of the above options

Figure 4: On-demand SSB Cases and Scenarios

As shown above in Figure 4, to support on-demand SSB, UE may expect on-demand SSB transmissions at least for the following on-demand SSB transmission scenarios for both on-demand SSB cases.

- On-demand SSB transmission Scenarios 2: before UE receives SCell activation command.

- On-demand SSB transmission Scenario 2A: when UE receives SCell activation command.

At RAN1 #119, the following agreements were made for on-demand SSB deactivation [11].

Agreement For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 FFS: Whether the value of N can be implicitly determined using a timer

Based on RAN1’s agreements, on-demand SSB transmission may be indicated and deactivated explicitly by MAC CE.

Observation 8. On-demand SSB transmission may be indicated and deactivated explicitly by MAC CE.

Observation 9. To support on-demand SSB transmission indication and deactivation via MAC CE, a unified MAC CE design should be supported.

At RAN2 #127, the following agreements were made for indication of on-demand SSB transmission [6].

Agreement Scenarios on OD-SSB: 1. RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B. OD-SSB transmission indication: 2. RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration. 3. New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

At RAN2 #127bis, more agreements were made for indication of on-demand SSB transmission [10].

Agreements 1. No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required. 2. Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A. 3. NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

Based on RAN1 and RAN2 agreements, as well as the observations, the MAC-CE for indication and deactivation of on-demand SSB transmission may include at least the following fields.

- Field 1: IDs or bitmap for Scells or Scell groups

- Field 2: 1-bit for indication and deactivation for each Scell

- Field 2: a periodicity of the OD-SSB, e.g., one of the candidate OD-SSB periodicity values configured in RRC

  - Option 1: One OD-SSB periodicity for one Scell

  - Option 2: One OD-SSB periodicity for a group of Scells, e.g., a Scell group is configured with the same candidate OD-SSB periodicity values such as Group 1 or Group 2 shown below.

    - Group 1 = {Cell 0, Cell 1} with candidate OD-SSB periodicity values = {5, 20, 40, 80} ms

    - Group 2 = {Cell 2, Cell 3} with candidate OD-SSB periodicity values = {5, 10, 20, 40, 80, 160} ms

  - The size of the field indicating the OD-SSB periodicity may be based on the number of configured candidate OD-SSB periodicity values for an Scell or an Scell group or the maximum number of configured candidate OD-SSB periodicity values for all Scells or Scell groups.

Based on above analysis, we propose the follows.

Proposal 3: Support a least the following fields for the MAC-CE indicating and deactivating OD-SSB transmission:

- Field 1: IDs or bitmap for Scells or Scell groups

- Field 2: one bit for indication and deactivation for each Scell

- Field 3: one of the candidate OD-SSB periodicity values configured in RRC

  - Option 1: One OD-SSB periodicity for one Scell

  - Option 2: One OD-SSB periodicity for a group of Scells

# Conclusion

In this contribution, we discussed on-demand SSB transmission scenarios and on-demand SSB cases for on-demand SSB Scell operations and potential RAN2 specification work and concluded with the following observations and proposals.

L3 measurement for Case #2

Combination 1: Always-on SSB is CD-SSB on sync raster and OD-SSB is NCD-SSB not allocated on sync raster.

Observation 1. A UE has to track always-on SSB and on-demand SSB at two separate frequencies, which adds complexity in tracking SSBs and contradicts with the legacy specs in which the SSB configured for a particular L1 procedure is always on the same frequency.

Observation 2. With carrier aggregation, the complexity for a UE to track SSBs is further increased if multiple SCells deploy such combination.

Combination 2: Always-on SSB is NCD-SSB not allocated on sync raster and OD-SSB is NCD-SSB not allocated on sync raster.

Observation 3. RAN1 has agreed Combination 2 (always-on SSB is NCD-SSB and off sync raster), where on-demand SSB is allocated at the same frequency with always-on SSB and on-demand SSB is with a periodicity of SSB periodicities on the SSB transmission grid.

Observation 4. Same transmit power may be applied to both always-on SSB transmission and OD-SSB transmission.

Observation 5. For Case #2 (always-on SSB), L3 measurement is already supported with always-on SSB.

Observation 6: For Case #2 (always-on SSB), with both always-on SSB and OD-SSB as NCD-SSB and transmitted on the same frequency location, network may configure an SMTC with consideration of OD-SSB.

Observation 7. For Case #2 (always-on SSB), the SMTC configured for always-on SSB can be used for both always-on and OD-SSB, which should be network’s decision.

Observation 8. For Case #2 (always-on SSB), it is up to UE implementation for how to filter and combine the measurements of always-on SSB and on-demand SSB.

Proposal 1. RAN2 agrees that no more specification work on L3 measurements is needed for Case #2 (always-on SSB).

Proposal 2. On-demand SSB based L3 measurement and report are not supported for neighboring cell monitoring.

MAC CE for On-demand SSB transmission indication

Observation 8. On-demand SSB transmission may be indicated and deactivated explicitly by MAC CE.

Observation 9. To support on-demand SSB transmission indication and deactivation via MAC CE, a unified MAC CE design should be supported.

Proposal 3: Support a least the following fields for the MAC-CE indicating and deactivating OD-SSB transmission:

- Field 1: IDs or bitmap for Scells or Scell groups

- Field 2: one bit for indication and deactivation for each Scell

- Field 3: one of the candidate OD-SSB periodicity values configured in RRC

  - Option 1: One OD-SSB periodicity for one Scell

  - Option 2: One OD-SSB periodicity for a group of Scells

# References

[1] RP-242354 “Revised WID: Enhancements of network energy savings for NR”, RAN Meeting #105, Melbourne, Australia, Sep 9-12, 2024.

[2] “Chair notes RAN1#116 eom0” RAN1 #116	, Athens, Greece, February 26th – March 1st, 2024.

[3] “Chair notes RAN1#116bis eom0” RAN1 #116bis	, Changsha, China, April 15th – 19th, 2024

[4] “Chair notes RAN1#117 eom0” RAN1 #117, Fukuoka, Japan, May 20th – 24th, 2024, 2024

[5] “Chair notes RAN1#118 eom0” RAN1 #118, Maastricht, NL, August 19th – 23rd, 2024

[6] R2-2407571 “Report from session on V2X/SL, R18/19 MOB, and R19 NES” RAN2 #127, Maastricht, NL, August 19th – 23rd, 2024

[7] “Chair notes RAN1#118bis eom0” RAN1 #118bis, Hefei, China, October 14th – 18th, 2024

[8] R4-2416913 “LS on SSB relation in On-demand SSB and SSB adaptation on SCell”, Hefei, China, October 14th – 18th, 2024

[9] R4-2416911 “LS on SSB adaptation”, Hefei, China, October 14th – 18th, 2024

[10] “Report from session on V2X/SL, R18/19 MOB, and R19 NES” RAN2 #127bis, Hefei, China, October 14th – 18th, 2024

[11] “Chair notes RAN1#119 eom0”, RAN1 #119, Orlando, US, November 18th – 22nd, 2024