3GPP TSG-RAN WG2 Meeting #129	R2-2501776

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	 8.12.3

Source:	 Huawei, HiSilicon

Title:	 Enhancements for UE-initiated/event-driven beam management

Document for:  Discussion and Decision

1	Introduction

For Rel-19 MIMO, according to the WID as shown below [1] and recent RAN1 inputs [5], objectives relevant to UE-initiated/event-driven beam management (UEIBM) and asymmetric DL/UL TRP deployment are identified to have significant RAN2 impacts in terms of RRC configuration and MAC procedure, which are outlined in the WID.

RP-242394, Revised Work Item: NR MIMO Phase 5 The detailed objectives are as follows: RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

In this contribution, we present detailed analysis and provide our considerations on how RAN2 can start working on UEIBM, based on RAN1 agreements so far.

2	UE Initiated/Event-driven Beam Management

In order to reduce the periodic CSI reporting overhead and latency, event-based CSI reporting was agreed by RAN1 for Rel-19 MIMO WI i.e., the UE reports CSI only when the event is triggered. This includes specification of the event triggering determination procedure and of the reporting procedure after the event is triggered. In the following, we provide our considerations on the trigger event for event-driven beam management, beam reporting and UL transmission. Then, we consider how to model this in the specifications.

2.1	Event triggering

2.1.1	Application of Time window length and M

The following agreement was made for event triggering in RAN1:

Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification. Agreement On UE-initiated/event-driven beam reporting, regarding trigger events, the following working assumption in RAN1#118bis is confirmed: On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. - Q is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2.

According to the RAN1 agreements, an event-2 instance occurs when the L1-RSRP of the new beam is a threshold value greater than the current beam. This threshold is configured by the network. Then, there are two cases:

- Case-1: upon occurrence of an event-2 instance, the UE initiates the reporting procedure.

- Case-2 (subject to a UE capability): when M event-2 instances for the same beam occur within a time window, the UE initiates the reporting procedure.

Based on the RAN1 agreements of Case-2, RRC configures the threshold, time window length and M. According to the RAN1 agreements, Event-1 and Event-7 are both supported for the UE-initiated/event-driven beam reporting. It is likely that this event evaluation procedure can also be applied to Event-1 and Event-7.

Observation 1: RRC should configure a threshold, time window length and M for the event-triggering determination procedure of the UEIBM.

Agreement Regarding the triggering event determination for Event 2, the event instance(s) counting is per new beam. Further study candidate condition(s) of resetting the counting including whether resetting is needed.

UE should maintain a timer and counter to operate during the event triggering determination procedure. According to the recent RAN1 agreement, the event instance(s) counting(counter) is per new beam for Event-2 and Event-7. Therefore, the timer should also be maintained for each new beam. In contrast, for Event-1, there is only a single timer and counter operating within the event-triggering determination procedure.

Proposal 1: For the Event-1 triggering determination procedure, the UE maintains a counter and a timer for current beam. For the Event-2 and Event-7 triggering determination procedure, the UE maintains a counter and a timer for each new beam.

Currently, BF and RLF are declared upon receiving a certain number of instances of an event from lower layers, which is similar to the RAN1 agreement for Case-2 above. However, these procedures have some differences:

- In the BF detection procedure, the timer is (re)started upon the reception of an instance, and the counter is incremented. Upon timer expiry, the UE resets the counter to 0. When the counter reaches the configured number of event instances, the UE considers beam failure to be detected.

- In the RLF procedure, the N310 “out-of-sync” instances need to be consecutive, then a timer is started, which can by stopped if N311 consecutive “in-sync” instances are received. At the point of timer expiration, RLF is declared.

The RAN1 agreement does not consider consecutive instances, but a number of instances in a time window. A time window of e.g., 300ms with 3 instances, requires recording the time of up to 2 instances, then when a third one occurs, if the first one occurred less than 300ms ago, the event is triggered, otherwise the first instance is discarded. If the number of instances to trigger an event is more like 30, the UE implementation impact is significantly higher.

Even though it does not strictly match with the RAN1 agreement, an alternative method could be like the method used for BF i.e., the UE uses a single timer for an event and at every instance, the timer is (re)started and the counter is reset if the timer expires. If the timer is 10ms and instances are occurring more frequently than every 10ms, the effect is similar to a 300ms window with 30 instances.

As the number of event instances is not decided yet, RAN2 can send a LS to RAN1, asking for the expected range for the count of such instances. Then, whether the BFD approach can be determined.

Proposal 2: For each event, at each even instance, the timer is (re)started and the counter is incremented. If the timer expires, the counter is reset to 0 (like for BF detection). Besides, RAN2 can ask RAN1 the expected range for the count of the event instances in a LS. Additionally, the technical proposal is also shown in Annex.

2.2	Specification aspects

2.2.1	MAC Layer

RAN1 is discussing in which specification to capture the UEIBM. For the BFR procedure, two components are used for beam failure detection, BFI_COUNTER and beamFailureDetectionTimer. Once beam failure is detected, the UE can perform BFR via random access or MAC CE with beam failure and candidate beam information. These procedures are captured in MAC.

UEIBM contains the event evaluation and reporting procedure, the event evaluation is also based on a counter and a timer. The whole procedure is quite similar to beam failure detection and BFR so it makes sense to also capture it in MAC. Companies may argue that RAN1 decided that the first PUCCH channel is a new UCI type for both Mode-A and Mode-B, which is L1 signalling. However, during the last RAN2 meeting, a significant number of companies had consensus that, whatever the first PUCCH channel is, it can be a notification from PHY to MAC similar to PRACH. So technically, we don't see any blocking point and prefer a clean modelling for BFR, UEIBM procedures.

Proposal 3: Capture the event evaluation and reporting (using timer and counter) for UEIBM in the MAC specification.

There are two possible approaches for event evaluation modelling:

-	Approach 1: L1 provides the RSRP to MAC layer, and MAC layer check whether condition is met based on the threshold, timer and counter.

-	Approach 2: The L1 checks whether RSRP condition for an event instance is met based on the threshold and sends an event instance to upper layer if condition is met. MAC checks whether event is met based on counter and timer for event instance.

For approach 1, if the comparison occurs in the PHY layer, the PHY layer sends the instance to the higher layers. Alternatively, for approach 2, if conducted in the higher layers, the PHY layer sends the beam quality metrics, such as L1-RSRP, to the higher layers, allowing them to perform the comparison based on beam quality. Both methods are possible and the final decision could up to RAN1.

Proposal 4: It’s up to RAN1 to discuss which approach is used. Following this, a LS should be sent to inform RAN1.

- Approach 1: L1 provides the RSRP to MAC layer, and MAC layer check whether condition is met based on the threshold, timer and counter.

- Approach 2: The L1 checks whether RSRP condition for an event instance is met based on the threshold and sends an event instance to upper layer if condition is met. MAC checks whether event is met based on counter and timer for event instance.

2.2.1	RRC configurations

The parameters described above are part of the recent RAN1 Liaison Statement to RAN2 (R1-2410758), which focuses on the RRC impacts for Rel-19 MIMO UEIBM. Below is a specific description of the new parameters:

- The parameter newBeamResourceSetEvent2-r19 defines the new beam resource set for Event-2.

- The parameter eventType-r19 indicates the event type used for UEI beam reporting.

- The parameter valueOfQ-r19 is used to specify the value of Q for Event-7.

- The parameter configuredResourceForSecondChannelOfModeB-r19 informs Type-1 CG PUSCH resource for second channel in mode-B.

- The parameter eventThresholdEvent1-r19 sets a threshold value for trigger event detection regarding Event-1.

- The parameter firstPUCCHResourceConfig-UEIBR-r19 configures the periodic PUCCH resource for first PUCCH for both mode-A and mode-B.

For Rel-19 MIMO, event-triggered reporting is introduced, along with corresponding configurations. The parameters above are proposed to be under the IE CSI-ReportConfig. It is up to RAN2 to decide how to capture these parameters in RRC.

Observation 2: RAN1 has introduced some new RRC parameters: newBeamResourceSetEvent2-r19, eventType-r19, valueOfQ-r19, configuredResourceForSecondChannelOfModeB-r19, eventThresholdEvent1-r19, firstPUCCHResourceConfig-UEIBR-r19.

There is an existing parameter CSI-ResourceConfig, which is used to configure the new beam resource set for periodic, semi-persistent, and aperiodic CSI reporting, andhas the same function as newBeamResourceSetEvent2-r19. Therefore, we propose to reuse the existing IE for beam resource sets instead of introducing a new one.

Proposal 5: Reuse the existing CSI-ResourceConfig IE for the beam resource set instead of introducing a new newBeamResourceSetEvent2-r19.

2.3	Beam reporting and UL transmission

For MIMO, the UE sends the L1 report to the network using UCI. Two modes (Mode-A and Mode-B) have been agreed by RAN1 for the beam reporting procedure.

•	Mode-A: beam report is transmitted in a UCI in dynamically allocated UL resource. One advantage of Mode-A is that the  scheduling flexibility, which can help to avoid collision issue on the second channels. Although the minor drawback is that the dynamic resource allocation of step-2 in Mode-A could introduce some latency, the allocated resources are really used. Therefore, the Mode-A was agreed by RAN1 as a fundamental feature.

•	Mode-B: beam report is transmitted in a UCI in pre-configured UL resources. This mode significantly reduces the complexity for the network as the network does not need to monitor whether UEI beam report is carried in the pre-configured resource at all times. Additionally, Mode-B offers better resource efficiency, as the pre-configured resources can be shared among UEs.

2.3.1	First UL channel Transmission

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, Alt-2 is supported for both mode-A and mode-B: the first PUCCH channel is a new UCI type. - Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration. - It is RAN1’s understanding that the RRC parameter is NOT associated with SchedulingRequestId. - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. - Reuse multiplexing/dropping rule(s) of SR as baseline - FFS: overlapping with SR/LRR or PUSCH Note: Further details on first PUCCH retransmission (if supported) for mode A and mode B will be continue to be separately discussed in RAN1.

RAN1 has decided that the first PUCCH channel is a new UCI type for both Mode-A and Mode-B, excluding the option of using the first PUCCH as a dedicated SR. This channel is associated with a periodic PUCCH resource configuration but is not linked to the SchedulingRequestId. The first PUCCH channel transmission can be notified from PHY, which is similar to PRACH, as we described in section 2.2.1, and the details can be further discussed based on further RAN1 discussion.

Observation 3: The first PUCCH channel configuration is a new UCI type with RRC parameter firstPUCCHResourceConfig-UEIBR for both mode-A and mode-B. This configuration does not serve as a Scheduling Request.

2.3.2	Second UL channel Transmission

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for regarding Mode-B, the pre-configured resource(s) for the second channel in Step-2 is at least type 1 CG-PUSCH. - FFS: PUCCH as the second channel - FFS: Whether the PUSCH can be with UL data

Beam reporting for Mode A and B is carried in UCI on the second UL channel. RAN1 agreed to support using PUSCH for the second UL channel is PUSCH, and the possibility of using PUCCH FFS in mode-B. The design of the second UL channel needs to consider the scenario where a skipping mechanism may be applied.

Observation 4: The second UL channel could be PUSCH for both mode-A and mode-B, using PUCCH in mode-B is FFS.

In the legacy skipping mechanism with CSI reporting in Rel-15, when there is an aperiodic CSI report to be transmitted on PUSCH, the UL grant cannot be skipped even though there is no available data in the MAC layer, since CSI multiplexing in the physical layer requires a MAC PDU to be generated. Semi-persistent CSI reporting on PUSCH is generated by the PHY layer and cannot be multiplexed with UL data. This procedure is invisible to the MAC layer, so it does not involve UL skipping mechanism.

TS 38.214 clause 5.1.3 CSI reporting on PUSCH can be multiplexed with uplink data on PUSCH except that semi-persistent CSI reporting on PUSCH activated by a DCI format is not expected to be multiplexed with uplink data on the PUSCH. CSI reporting on PUSCH can also be performed without any multiplexing with uplink data from the UE.

For periodic and semi-periodic CSI reporting on PUCCH, the UL grant is skipped if there is no available data i.e., CSI cannot be multiplexed on PUSCH in case PUCCH and PUSCH overlapping in time domain, and if there is some available data, the CSI is multiplexed on PUSCH without PUCCH transmission, those two cases cannot be distinguished by network.

In this case, the network has to attempt decoding CSI on both PUSCH and PUCCH. In Rel-16, an enhancement was introduced for periodic and semi-periodic CSI reporting to reduce network complexity. Specifically, the UL grant cannot be skipped if CSI is to be multiplexed on this PUSCH transmission even there is no available data, i.e. network can always assume the CSI is always transmitted on PUSCH in case PUCCH and PUSCH overlapping.

TS 38.321 clause 5.4.3.1.3 The MAC entity shall: 1> if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity. 1> else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity.

Observation 5: UL grant cannot be skipped for periodic and semi-periodic CSI multiplexed on PUSCH, nor for aperiodic CSI reporting on PUSCH.

In Rel-19, a new CSI is introduced, i.e. the UE-initiated/event-driven beam reporting. How UL skipping to work with this new CSI needs to be studied regarding mode-A and mode-B based beam reporting.

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, at least Option-3 is supported: - Option-1: The same Type-1 CG PUSCH can carry UL-SCH, any other UCI, and the beam report. - Option-2: The Type-1 CG PUSCH is a dedicated type-1 CG PUSCH for carrying the beam report - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can NOT carry any other UCI. - Option-3: The Type-1 CG PUSCH is a type-1 CG PUSCH for carrying the beam report - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can carry any other UCI. FFS: whether Type-1 CG PUSCH can be transmitted if the pre-configured Type-1 CG PUSCH does NOT carry the beam report

Based on the above agreements, RAN2 should send a LS to RAN1 to ask whether the UEI beam reporting is visible to the MAC layer. If it is not, discussion of the UL skipping mechanism is unnecessary; however, if it is, RAN2 should consider the potential impact on the MAC layer.

Proposal 6: Send a LS to RAN1 to ask whether the UEI beam reporting is visible to the MAC layer.

The UE may be configured with DRX (Discontinuous Reception). DRX operation determines whether the MAC entity monitors the PDCCH or not. When DRX is configured, the UE does not need to continuously monitor the PDCCH, it only needs to wake up during the DRX active time to monitor the PDCCH.

In mode-A beam reporting procedure of UEIBM, the UE transmits the first PUCCH to request resources for PUSCH. Subsequently, the network sends the response via the PDCCH, which indicates the allocated PUSCH resources for beam reporting.

TS 38.321 clause 5.7 When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: …… - a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or

Similarly to the SR procedure, if DRX is configured, the UE must remain awake after transmitting the first PUCCH, until the DL scheduling information for resource allocation is received.

In mode-B beam reporting procedure of UEIBM, the second UL channel transmission is CG PUSCH, which is not subject to PDCCH monitoring, and thus we think there is no impact.

Proposal 7: In mode-A of UEIBM, the UE should remain in DRX active time to receive DCI for the second UL transmission.

Proposal 8: In mode-B of UEIBM, the second UL transmission is not subject to DRX active time.

# 3	Conclusion

In this paper, we have addressed the objective regarding initial analysis on RAN2 impact for MIMO. There are the following observations and proposals, and the draft LS to RAN1 and technical proposal is attached in Annex.

Event triggering

Observation 1: RRC should configure a threshold, time window length and M for the event-triggering determination procedure of the UEIBM.

Proposal 1: For the Event-1 triggering determination procedure, the UE maintains a counter and a timer for current beam. For the Event-2 and Event-7 triggering determination procedure, the UE maintains a counter and a timer for each new beam.

Proposal 2: For each event, at each even instance, the timer is (re)started and the counter is incremented. If the timer expires, the counter is reset to 0 (like for BF detection). Besides, RAN2 can ask RAN1 the expected range for the count of the event instances in a LS. Additionally, the technical proposal is also shown in Annex.

Specification aspects

Proposal 3: Capture the event evaluation and reporting (using timer and counter) for UEIBM in the MAC specification.

Proposal 4: It’s up to RAN1 to discuss which approach is used. Following this, a LS should be sent to inform RAN1.

- Approach 1: L1 provides the RSRP to MAC layer, and MAC layer check whether condition is met based on the threshold, timer and counter.

- Approach 2: The L1 checks whether RSRP condition for an event instance is met based on the threshold and sends an event instance to upper layer if condition is met. MAC checks whether event is met based on counter and timer for event instance.

Observation 2: RAN1 has introduced some new RRC parameters: newBeamResourceSetEvent2-r19, eventType-r19, valueOfQ-r19, configuredResourceForSecondChannelOfModeB-r19, eventThresholdEvent1-r19, firstPUCCHResourceConfig-UEIBR-r19.

Proposal 5: Reuse the existing CSI-ResourceConfig IE for the beam resource set instead of introducing a new newBeamResourceSetEvent2-r19.

Beam reporting and UL transmission

Observation 3: The first PUCCH channel configuration, it is a new UCI type with RRC parameter firstPUCCHResourceConfig-UEIBR for both mode-A and mode-B. This configuration does not serve as a Scheduling Request.

Observation 4: The second UL channel could be PUSCH for both mode-A and mode-B, using PUCCH in mode-B in FFS.

Observation 5: UL grant cannot be skipped for periodic and semi-periodic CSI multiplexed on PUSCH, nor for aperiodic CSI reporting on PUSCH.

Proposal 6: Send a LS to RAN1 to ask whether the UEI beam reporting for mode-B is visible to the MAC layer.

Proposal 7: In mode-A of UEIBM, the UE should remain in DRX active time to receive DCI for the second UL transmission.

Proposal 8: In mode-B of UEIBM, the second UL transmission is not subject to DRX active time.

# 4	Reference

- RP-242394, WID revision: NR MIMO Phase 5;

- RAN1 #118 chairman notes;

- RAN1 #118bis chairman notes;

- RAN1 #119 chairman notes;

- TS 3GPP 38.331 V18.3.0;

- TS 3GPP 38.321 V18.3.0.

# 5	Annex – LS

3GPP TSG-RAN WG2 Meeting #129	R2-25xxxxx

Athens, Greece, Feb. 17th – 21st, 2025

Title:	(draft) LS on Rel-19 UEIBM

Response to:	-

Release:	Rel-19

Work Items:	NR_MIMO_Ph5-Core

Source:	RAN WG2, Huawei, HiSilicon

To:	RAN WG1

CC:	-

Contact Person:

Name:					TBD

E-mail Address:		TBD

Attachment:	               -

1. Overall Description:

RAN2 would like to ask the following questions:

1. For UEI beam reporting, RAN2 noticed the RAN1 agreement on Type-1 CG PUSCH can NOT carry UL-SCH. And RAN2 would like to ask RAN1 whether the Mode-A and Mode B grant for event driven beam report is visible to MAC layer, i.e. whether or not to generate an empty MAC PDU delivered to PHY layer.

Question 1: whether the Mode-A and Mode B grant for event driven beam report is visible to MAC layer, i.e. whether or not to generate an empty MAC PDU delivered to PHY layer?

2. RAN2 discussed two options on how to model the specification on UEIBM and would like to ask RAN1.

Question 2: which option is used for event triggering determination.

- Option-1: L1 provides the RSRP to MAC layer, and MAC layer check whether condition is met based on the threshold, timer and counter.

- Option-2: The L1 checks whether RSRP condition for an event instance is met based on the threshold and sends an event instance to MAC layer if condition is met. MAC checks whether event is met based on counter and timer for event instance.

2. Actions:

To RAN WG1

ACTION: RAN2 respectfully asks RAN1 to consider above information for Rel-19 MIMO and to provide answers to the above questions.

3. Date of Next RAN WG2 Meetings:

TSG-RAN WG2 Meeting #129-bis			April 07 to 11, 2025			China, CN

TSG-RAN WG2 Meeting #130				May 19 to 23, 2025				Malta, MT

# 6	Annex – TP

5	MAC procedures

5.XY	 	UE Initiated/Event-driven Beam Management

The MAC entity may be configured by RRC per CSI-ResourceConfig with a UE Initiated/Event-driven Beam Management procedure, for indicating the serving gNB whether the SSB/CSI-RS satisfied the corresponding events. The procedure of the event triggering determination is detected by counting event instance indication from the lower layers to the MAC entity.

RRC configures the following parameters in the CSI-ReportConfig for the UE Initiated/Event-driven Beam Management:

-	EventInstanceMaxCount for the UEIBM (per event);

-	EventDetectionTimer for the UEIBM (per event);

The following UE variables are used for the event-triggering determination procedure:

-	UEIBM_COUNTER (per new beam in the associated CSI-ResourceConfig): counter for event instance indication which is initially set to 0.

For the event triggering determination procedure, the MAC entity shall for each new beam configured for UE Initiated/Event-driven Beam Management:

1>	if the CSI-ReportConfig is set to eventType and eventType is set to Event1, or Event2, or Event7:

2>	if an event instance indication for a new beam in the associated CSI-ResourceConfig has been received from lower layers:

3>	start or restart the EventDetectionTimer of the new beam;

3>	increment UEIBM_COUNTER of the new beam by 1;

3>	if UEIBM_COUNTER of the new beam >= EventInstanceMaxCount:

4>	inform the lower layer to trigger a UEI beam reporting procedure for this CSI-ReportConfig;

1>	if the EventDetectionTimer expires; or

1>	if EventDetectionTimer, EventInstanceMaxCount, or any of the reference signals used for UEIBM is reconfigured by upper layers:

2>	set UEIBM_COUNTER to 0.

1>	if the lower layer indicates that the UEI beam report is successfully sent:

2>	set UEIBM_COUNTER to 0;

2>	stop the EventDetectionTimer, if configured;

2>	consider the event triggering determination procedure successfully completed.