3GPP TSG-RAN WG2 Meeting #129	R2-2501177

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	 8.12.2

Source:	 Huawei, HiSilicon

Title:	 Discussion on Asymmetric DL sTRP/UL mTRP

Document for:  Discussion and Decision

1	Introduction

For Rel-19 MIMO, objectives relevant to asymmetric DL/UL TRP deployment are identified to have significant RAN2 impacts. Progress on the MAC CE definition approach for this topic have been made in RAN1 and RAN2 meetings. In the last meeting, RAN2 agreements were as follows:

Agreement - New MAC CE is introduced for PL offset update for asymmetric DL sTRP/UL mTRP. This new MAC CE is identified by new eLCID. - Absolute value of PL offset is indicated in the new MAC CE. For the offset value, the value range is [-12, 60] dB and the step size is 4dB. - In the MAC CE, PL offset value can be updated for any configured TCI states with RRC configured PL offset, i.e., not limited to the activated TCI states.

Also, RAN1 has send a LS by asking RAN2 to check whether RAN1 agreements would cause any MAC impact.[1]

In this contribution, we present detailed analysis on RAN2 work on asymmetric DL sTRP/UL mTRP based on RAN1 and RAN2 agreements so far, and provide our considerations on the RAN1 LS.

2	Asymmetric DL sTRP/UL mTRP

In Multiple Transmit/Receive Point (multi-TRP) operation, a serving cell can schedule the UE from two TRPs, providing better coverage, reliability and/or data rates for PDSCH, PDCCH, PUSCH, and PUCCH. In pre-Rel-19 mTRP scenarios, each TRP has downlink and may have uplink, as shown in Figure 1 below. In Rel-19, a new scenario, asymmetric DL sTRP/UL mTRP is considered, in which one TRP has downlink and uplink while the other TRP only has uplink, as shown in Figure 2.

Figure 1. pre-Rel-19 multi-TRP scenario

Figure2. Rel-19 Asymmetric DL sTRP/UL mTRP scenario

2.1	MAC CE Format

2.1.1	New MAC CE for PL offset update

In pre-Rel-19 mTRP scenarios, each TRP can transmit DL RS and the UE can measure the path loss directly based on each DL RS from the TRP. In Rel-19, a new scenario is considered, in which one TRP does not transmit any DL RS. For the TRP with DL RS transmission, the UE calculates the pathloss (PL) based on the DL RS. However, for the UL-only TRP without DL RS transmission, a new mechanism is needed e.g., the pathloss is calculated using the PL to the TRP with DL transmission and a PL offset is applied. RAN1 agreed that different PL offset values can be configured for different TCI states of a TRP, and RAN2 agreed to use a new MAC CE for PL offset update.

According to the RAN2 agreements, the updated PL offsets are not limited to the activated TCI States, which means that the maximum number of updated PL offsets in the MAC CE is not linked to the maximum number of activated TCI States. If there is no restriction on the maximum number of updated PL offsets and a bitmap is used to indicate the activation/deactivation status of the concerned joint/UL TCI state, the bitmap should be 64/128 bits and the MAC CE length is fixed, which leads to significant overhead. If the maximum number of updated PL offsets is indicated by RRC or fixed in the specification and a bitmap is used to indicate the activation/deactivation status of the joint/UL TCI state, the bitmap length should be the maximum number of updated PL offsets and the TCI State ID field should be included in the MAC CE. However, due to the randomness of channel, the number of PL offsets that need to be updated each time depends on the UE location and thus is not unpredictable. Therefore, it is preferable that the new MAC CE for PL offset update has a variable size and includes a flexible number of PL offsets. The exact number of updated PL offsets can be calculated via the L field of MAC CE subheader or indicated explicitly in the MAC CE.

Proposal 1: The new MAC CE for PL offset update includes a flexible number of PL offsets.

Proposal 2: The new MAC CE for PL offset update includes the impacted TCI State ID(s).

Agreement For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

Based on the RAN1 agreement, both joint TCI mode and separate TCI mode are supported for PL offset update. In our view, one single MAC CE would be sufficient for both modes (joint and separate). The TCI mode (joint or separate) of each MAC CE can be distinguished by the unifiedTCI-StateType of the Serving Cell indicated by the Serving Cell ID field. In separate TCI mode, only the UL TCI state ID is associated with a PL offset, thus the PL offset update MAC CE only contains UL TCI State ID without DL TCI state ID. In joint TCI mode, the joint TCI state ID is included. Therefore, joint/separate TCI mode can share one common MAC CE format for updating PL offset. The MAC CE format shown in Figure 3 can be adopted as the baseline.

Proposal 3: One common MAC CE format for PL offset update can be used for joint and separate TCI mode.

Figure 3. New MAC CE for PL offset update

### 2.1.2	MAC CE for TCI State activation/deactivation

For the asymmetric DL sTRP/UL mTRP deployment scenario, RAN1 agreed to reuse the Rel-18 unified TCI framework:

Agreement - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

The R18 legacy Enhanced TCI state activation/deactivation MAC CE format is shown in Figure 4.

Figure 4. Rel-18 Enhanced TCI state activation/deactivation MAC CE format

According to TS 38321[2], the Rel-18 activation/deactivation MAC CE can support the {two DL TCI states + two UL TCI states} configuration, which covers the combination of {one DL TCI state + up to two UL TCI states} in RAN1 agreement. Therefore, the legacy MAC CE format can be reused in the asymmetric DL sTRP/UL mTRP scenario. Obviously, there will be no impact on the joint TCI mode. However, for the separate TCI mode, since there is no DL TCI State for the UL-only TRP, the corresponding Fi,j field should set to be 0, and the UE should ignore this Fi,j field.

Proposal 4: Reuse the Rel-18 TCI State activation/deactivation MAC CE for joint/separate modes for the asymmetric mTRP scenario.

Proposal 5: When the TCI activation/deactivation MAC CE for separate TCI mode is used in the asymmetric mTRP scenario, the Fi,j field for the TCI state of the UL-only TRP is set to 0 and the UE ignores this Fi,j field.

### 2.1.3	MAC CE for Power Headroom Triggering

According to TS 38.213[3], PH is calculated by physical layer and indicated to the MAC layer. Then, the MAC layer includes it in the PHR MAC CE when UL resource are available after PHR is triggered. In Rel-19, RAN1 made the following agreement on PH calculation for asymmetric DL sTRP/UL mTRP scenarios:

Agreement For the asymmetric DL sTRP/UL mTRP scenarios, support to include PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH

Based on the above agreement, RAN2 can assume that the physical layer indicates the calculated PH for asymmetric DL sTRP/UL mTRP scenarios to the MAC layer, and the MAC layer includes it in the existing mTRP PHR MAC CE, i.e. no new mTRP PHR MAC CE is introduced.

Proposal 6: No new mTRP PHR MAC CE is introduced i.e., the existing mTRP PHR MAC CEs can be used.

2.2	UE Behaviour

Based on the above agreement, RAN2 should introduce in RRC a PL offset for each joint and UL TCI state and a new MAC CE with eLCID for updating the PL offset associated with TCI states after RRC configuration. The calculation of PL for multi-TRPs is shown in Figure 5.

Figure 5. PL calculation in asymmeteric mTRP scenario

### 2.2.1	RRC Reconfiguration

RRC supports delta signalling, in which the UE is required to keep fields that are absent. For example, if a number of TCI-UL-State were previously configured in BWP-UplinkDedicated for some UL BWP, and the network performs a reconfiguration of some DL parameters, or of some other UL BWP, the UE keeps the existing TCI-UL-State configured in this UL BWP.

If the PL offset is included in TCI-UL-State, after the reconfiguration, does the UE use the PL offsets previously configured by RRC, or does it use the path loss offsets modified by MAC CEs received until the reconfiguration?

Figure 6. PL offset selection in RRC reconfiguration scenario

Proposal 7: Ask RAN1 whether, after an RRC reconfiguration in which the path loss offset values are neither released nor modified by RRC, the UE continues using the values modified by MAC CEs, or reverts to the values configured by RRC.

### 2.2.2	BWP Switching

Another scenario is BWP switching. For example, the UE is operating in BWP#1 and the PL offset values are modified by a MAC CE. Then, the BWP is switched to BWP#2. Now if the UE is switched back to BWP#1 from BWP#2, does the UE uses the values that were previously used on BWP#1 i.e., as modified by MAC CEs when the UE was previously operating on BWP#1 or does the UE uses the values configured by RRC for BWP#1?

Figure 6. PL offset selection in BWP switching scenario

Proposal 8: Ask RAN1 whether, after BWP switching, the UE reverts to the RRC-configured path loss offset values for the new BWP or uses the last value that were used on this BWP i.e., including modifications by MAC CEs.

2.3	Discussion on RAN1 LS

Agreement Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

RAN1 sent an LS to ask whether the above agreements would cause any MAC impact.[1]

The 2TA mechanism in the MAC layer is implemented as follows:

- the TI field in the MAC RAR is used to indicate one of the two TAGs to which the Timing Advance Command is applied (tag-Id or tag2-Id)

- the TI field in the Absolute Timing Advance Command MAC CE is used to indicate one of the two TAGs to which the Timing Advance Command is applied (tag-Id or tag2-Id)

- the TAG ID field in the Timing Advance Command MAC CE is used to indicate the TAG Identity of the addressed TAG.

We now consider the MAC impacts on the above 2TA agreements are.

Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature.

- Since coresetPoolIndex is a RRC parameter added in Rel-18 to identify multi-TRP transmission, this new feature will cause no MAC impact.

One downlink reference timing is supported and applied to both TAGs.

- The application of one downlink reference timing is a UE behaviour on physical layer, and will cause no MAC impact.

One single n-TimingAdvanceoffset is configured and applied to both TAGs.

- The application of one single n-TimingAdvanceoffset (RRC parameter) is a UE behaviour on physical layer, and will cause no MAC impact.

Any of the TCI states can be associated with any one of the two TAGs.

- According to TS 38331[4], the tag-Id-ptr refers to the TAG of the serving cell where the TCI state is applied, the value n0 means that the TCI state is associated with the TAG indicated by tag-Id, value n1 means that the TCI state is associated with the TAG indicated by tag2-Id. Therefore, this feature is supported in the existing specification and has no MAC impact.

The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA.

- According to TS 38321[2], the existing MAC RAR format and mechanism can be fully reused in the asymmetric mTRP scenario without any MAC impact.

The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA.

- According to TS 38321[2], the existing MAC CE format and mechanism can be fully reused in the asymmetric mTRP scenario without any MAC impact.

Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA.

- The report of the UE capability of “Overlapping UL transmission reduction” should be introduced in RRC, which has no MAC impact.

Proposal 9: RAN2 assumes no MAC impact to support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios.

# 3	Conclusion

From the analysis on RAN2 impacts for MIMO, we made the following proposals:

Proposal 1: The new MAC CE for PL offset update includes flexible number of PL offsets.

Proposal 2: The new MAC CE for PL offset update includes the impacted TCI State ID(s).

Proposal 3: One common MAC CE format for PL offset update can be used for joint and /separate TCI mode.

Proposal 4: Reuse the Rel-18 TCI State activation/deactivation MAC CE for joint/separate modes for the asymmetric mTRP scenario.

Proposal 5: When the TCI activation/deactivation MAC CE for separate TCI mode is used in the asymmetric mTRP scenario, the Fi,j field for the TCI state of the UL-only TRP is set to 0 and the UE ignores this Fi,j field.

Proposal 6: No new mTRP PHR MAC CE is introduced i.e., the existing mTRP PHR MAC CEs can be used.

Proposal 7: Ask RAN1 whether, after an RRC reconfiguration in which the path loss offset values are neither released nor modified by RRC, the UE continues using the values modified by MAC CEs, or reverts to the values configured by RRC.

Proposal 8: Ask RAN1 whether, after BWP switching, the UE reverts to the RRC-configured path loss offset values for the new BWP or uses the last value that were used on this BWP i.e., including modifications by MAC CEs.

Proposal 9: RAN2 assumes no MAC impact to support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios.

# 4	Reference

- R1-2410758 LS to RAN2 on RRC and MAC impacts for Rel-19 NR MIMO Ph5

- TS 3GPP 38.321 V18.2.0

- TS 3GPP 38.213 V18.4.0

- TS 3GPP 38.331 V18.3.0