3GPP TSG-RAN-WG2 Meeting #129	R2-2501195

Athens, GR,17th– 21st Feb, 2025

Agenda item	:		8.6.4

Source	: 	Sharp

Title	: 	Discussion on issues for supporting conditional LTM

Document for	:	Discussion and Decision

Introduction

Revised WID [1] for enhancement mobility was approved and RAN2 started discussion on the WI. For conditional LTM, RAN2 starts discussion from this meeting;

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In the last meeting, we agreed below,

- The triggering condition of conditional LTM can be based on L3 measurement.

- CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

- The L1 execution condition of a candidate cell is associated to only one triggering event.

- For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

- To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:
- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.
- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

- For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

In this paper, we discuss some issued related to conditional LTM.

Discussion

Early sync phase

Early UL sync

In the previous meeting, RAN2 agreed that PDCCH order early TA acquisition and UE based TA measurement mechanism are supported to early UL synchronization. In addition, source gNB doesn’t need to send PDCCH order to UE if UE performs RA procedure for early synchronization based on the LTM configuration itself (e.g., UE selects preamble index/SSB index for candidate cells based on RRC configuration), i.e. RRC-based RA procedure for early synchronization. The main difference between RRC-based RA procedure and PDCCH order-based RA procedure is whether UE needs to receive PDCCH order or not. RRC-based RA procedure can benefit from low latency because the UE does not need to wait for the PDCCH order and performs early UL synchronization according to the LTM candidate configuration. Therefore, RRC-based RA procedure for early synchronization should be considered.

Proposal 1. RRC-based early TA acquisition mechanism for early synchronization can be considered for conditional LTM.

New MAC CE including the TA

In the previous meeting, RAN2 made agreements related to TA acquisition as above:

・The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly

to the UE). This agreement will be included in the LS to RAN1/3/4.

・The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the

TA value when UE switches to that candidate cell during CLTM.

In our understanding, new MAC CE should include TA value and candidate configuration Id for each candidate cell.

Based on these information, the UE can know the TA value and which candidate cell is associted with this TA value.

Proposal 2: New MAC CE include following information for each candidate cell:

- TA value

- Candidate configuration Id

TA maintenance

In the current draft MAC CR, new TAT for early synchronization is introduced as an ltm-Candidate-TimeAlignmentTimer. According to the description, the new TAT controls how long the MAC entity considers the CLTM candidate cells to be uplink time aligned. Here, we think RAN2 agreed that candidate cell TA is maintained by a new timer. And NW may indicate two independent TA values for two candidate cells. Then, UE should maintain multiple new TATs according to the new MAC CE.

Proposal 3. New TAT for early synchronization should be maintained per candidate configuration.

And the new TAT should indicate how long the TA value for the candidate cell is valid. This purpose is same as legacy TAT. So, the UE should apply the new TAT value, if running, as a TA Timer for the target cell upon UE executes conditional LTM.

Proposal 4. When UE executes CLTM, UE should apply a value of the new TAT for the target configuration to TA timer.

MAC CE rapporteur suggested to discuss whether the new TAT should be stopped upon MAC reset due to LTM cell switch execution. Depending on the cell or the beam quality, CLTM to another cell may be required again immediately after the LTM cell switching. In that case, TAT values for candidate cells other than the serving cell might be reused. It should improve robustness of mobility. Therefore, New TAT for candidate cells should not be stopped upon LTM cell switch execution.

Proposal 5. New TAT for candidate cells should not be stopped upon LTM cell switch execution

Evaluation phase

When does UE start the evaluation?

In Rel-18 LTM, UE starts L1 measurement and reporting when L1 measurement is configured in RRC reconfiguration. Then, gNB determines whether to trigger LTM cell switch for the UE based on the L1 measurement reports. In case of CHO, when UE is configured with CHO configuration, UE starts evaluating event condition of conditional reconfiguration. Therefore, in the same way, UE should start conditional LTM event evaluation upon UE is configured with C-LTM configuration.

Proposal 6. UE starts conditional LTM event evaluation once it is configured with C-LTM configuration.

Evaluation mechanism

Similarly, the event evaluation mechanism for C-LTM should be aligned with the event triggered L3 measurement report, just as the mechanism for CHO aligns with the event triggered L1 measurement report. This alignment simplifies the specification and reduces workload.

Proposal 7. The event evaluation mechanism for C-LTM should be the same as that for event-triggered L1 measurement reporting.

L1 and L3 conditions

In the current CR, there is the FFS point as below:

Editor’s Note: FFS whether L1 and L3 execution conditions can be configured simultaneously or not.

In the previous meeting, it was agreed that L3 condition is used to evaluate in CLTM as bellow:

The triggering condition of conditional LTM can be based on L3 measurement.

In the measurement model, L1 measurement results can be obtained instantly and may fluctuate with the instantaneous channel status; in contrast, L3 measurement results minimize the fluctuation by the L3-filtering and take more time for the UE to obtain. If the UE executes cell switching based on L1 conditions, the UE can perform cell switching instantly. On the other hand, if the UE executes cell switching based on L3 conditions, the UE can perform cell switching steadily. Simultaneous L1 and L3 execution conditions can allow UE to perform cell switching based on L1 and L3 conditions to obtain the advantage of both. As a result, for the network to flexibly decide which conditions to configure, we propose that the three types of conditions can be configured for each beam: 1) L1 conditions only, 2) L3 conditions only, and 3) L1+L3 condition combination. However, since L1 conditions are evaluated at the MAC layer, whereas the L3 conditions are evaluated at the RRC layer, how does the MAC layer and the RRC layer interact with each other to trigger the CLTM execution should be further specified. Additionally, for the configuration of conditions, it should be further discussed whether the condition is configured on a per-cell basis or a per-beam basis. If the condition is configured on a per-cell basis, all the candidate beams in the candidate cell apply the same condition. This simplifies the configuration and reduces the signalling overhead. On the other hand, if the condition is configured on a per-beam basis, each candidate beam has its own conditions, which provides the flexibility for the network.

Proposal 8. L1 and L3 conditions can be configured simultaneously.

Proposal 9. How does the MAC layer and the RRC layer interact with each other to trigger the CLTM execution should be further specified.

Proposal 10. Specify that the conditions are configured on a per-cell basis or on a per-beam basis.

Event for C-LTM

In the current CR, there is the FFS point as below:

Editor’s Note: FFS whether we re-use events specified for the event-triggered L1 measurements or we specify events specifically for CLTM.

In previous discussion about event triggered measurement, it was agreed that some events are supported as below:

Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

・Event LTM2: Beam of serving cell becomes worse than absolute threshold;

・Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

・Event LTM4: Beam of candidate cell becomes better than absolute threshold;

・Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

We can use these events for event triggered L1 measurement can be reused for CLTM. According to WID[1], Rel-19 CLTM does not support DC scenario. Therefore, Event LTM3 and Event LTM5 should be reused for CLTM.

Proposal 11: Event LTM3 and Event LTM5 are reused for CLTM.

- CondEvent LTM3: Beam of candidate cell becomes better than absolute threshold

- CondEvent LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2

Number of beams needed to fulfil condition

In the last meeting, how to apply beams to C-LTM event evaluation was discussed. There are two alternatives to evaluate the beams for C-LTM

- Alt 1. When at least one beam fulfils the associated execution condition, UE triggers the LTM cell switch.

- Alt 2. When N of beams fulfils the associated execution condition, UE triggers the LTM cell switch. N is configured by the network.

If RAN2 support Alt2, network has more flexibility to control CLTM cell switch. Therefore, we prefer Alt2.

Proposal 12. When N of beams fulfils the associated execution condition, UE triggers the LTM cell switch. N is configured by the network.

Layer Interactions

In the current MAC CR, MAC entity selects one candidate configuration if any beams of candidate cells satisfy the L1 condition. And the MAC indicates it to the RRC layer. And, in the current RRC CR, RRC entity selects one candidate configuration if there are multiple candidate cells. It is redundant and wasteful behaviour from UE perspective. To simplify the UE behaviour, (1) MAC entity indicates all beams/candidate configurations which satisfy the L1 condition to the RRC layer (if L1 condition evaluation is configured), (2) RRC layer selects one target configuration and indicates it to the MAC entity, (3) MAC entity performs LTM related procedure for the indicated cell.

Proposal 13. MAC should indicate all candidate configurations which entry condition is fulfilled to RRC. RRC should always indicate target configuration ID to MAC layer.

Execution phase

Upon LTM cell switch execution, UE can switch the serving cell with/without random access procedure. For RACH-based LTM cell switch, there are two type of RACH procedure, i.e. CFRA and CBRA. In Rel-18 LTM, source DU determines to trigger LTM cell switch and notifies it to target DU if source DU and target DU are not the same DUs. Hence, target gNB (target DU) can prepare dedicated resource for the UE when CFRA is needed. Using CFRA is useful for reducing communication interruption period. On the other hand, if CFRA is used for C-LTM, all of candidate cells need to reserve CFRA resources for the UE for the entire duration that C-LTM is configured. However, in case of CHO, both of CBRA and CFRA are supported. It means that the network can tolerate some waste of CFRA resources. So, in the same way, both CBRA and CFRA should be supported for conditional LTM.

Observation 1. In case of CHO, both of CBRA and CFRA are supported.

Proposal 14. Both CBRA and CFRA should be supported for conditional LTM.

Furthermore, there is an issue on how to handle UL grant to complete LTM cell switch procedure. In Rel-18 LTM, an UL grant is provided by configured grant or dynamic grant.

In case of CG-based RACH-less cell switch, UE is configured with configured grant to be used for target cell by RRC signalling, and the UE sends complete message by using a beam which is indicated in the LTM CSC MAC CE. Target cell will prepare UL resource after source DU indicates the UE’s switch to the target DU. In case of C-LTM, if this mechanism is applied to UE, whose cell switch timing is unknown, the candidate cells must constantly reserve UL resources for the UE.

In case of DG-based RACH-less cell switch, UE monitors PDCCH on the target cell and sends complete message upon receiving PDCCH for DG. Then, the target cell will prepare UL resource after source DU indicates the UE’s switch to the target DU. Similarly, in the case of DG-based, target cell will prepare UL resource and send PDCCH to the UE after source DU indicates the UE’s switch to the target DU. In case of C-LTM, if this mechanism is applied to UE, whose cell switch timing is unknown, the candidate cells must constantly reserve UL resources for the UE.

In both cases configured grant and dynamic grant, it is not reasonable to consistently reserve UL grant for the UE, whose cell switch timing is unknown.

Observation 2. To perform RACH-less LTM cell switch, gNB needs to provide UL grant so as UE to send an RRCReconfigurationComplete message.

Observation 3. It is not reasonable to continue providing Dynamic/Configured grant for the UE, whose cell switch timing is unknown.

The key point is that target cell (DU) knows when UE switches the serving cell. If target cell (DU) knows when UE switch the serving cell, the target can reserve the UL grant depending on when the UE switches the serving cell (as mentioned above). Here, we propose that the UE notifies execution of conditional LTM cell switch to source cell after deciding to execute the switch and before executing the switch.

Figure 1. Notification of C-LTM cell switch

When the event conditions are met, the UE can perform RACH-less LTM cell switch if the UE is DL/UL synchronized with target cell. UE needs UL grant towards the source for the notification, but it seems no problem based on the consideration of LTM use case i.e. LTM is the mobility mechanism of the UE where communication occurs continuously and it is desirable to have little interruption.

Proposal 15. UE notifies that UE will perform LTM cell switch to the source DU just before the LTM cell switch, source DU notifies it to target DU, and target DU ensures UE has UL grant.

In legacy CHO/CPAC, the UE continues evaluation even after the entry conditions are met, and if the leaving condition is met, the UE doesn’t perform CHO/CPAC for that cell. In L1 event triggered measurement report, RAN2 agreed to support the leaving condition mechanism. And the NW should cancel to send an LTM CSC MAC CE if the NW receives L1/L3 MR which is triggered by the leaving condition. So, as with CHO/CPAC, if leaving condition for the target cell is satisfied, UE should cancel ongoing CLTM cell switch execution for the cell.

Proposal 16. If leaving condition for the target cell is satisfied, UE should consider the target cell as a cell which doesn’t satisfy the condition for cell switching.

In the previous meeting, RAN2 agreed that the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration. In other words, the TCI states activation/deactivation for conditional LTM will be provided by the Candidate Cell TCI States Activation/Deactivation MAC CE. However, for conditional LTM, UE performs cell switch based on some configured conditions, so the cell switch command is not needed and the TCI activation/indication in CSC is not provided accordingly. If the UE can determine which TCI state for the target cell among the TCI states activated by the Candidate Cell TCI States Activation/Deactivation MAC CE by itself, an additional signalling for TCI activation/indication from gNB will not be required. Moreover, the UE will check whether the corresponding beam quality meets the configured conditions while determining which TCI state is to be applied. Thus, the UE can determine a TCI state to be applied upon C-LTM cell switch execution for the target cell among the TCI states activated by the Candidate Cell TCI States Activation/Deactivation MAC CE in conditional LTM.

Proposal 17. UE autonomously determines a TCI state to be applied upon CLTM cell switch execution for a target cell among the TCI states activated by the Candidate Cell TCI States Activation/Deactivation MAC CE.

Configuration

Furthermore, legacy LTM configuration is not maintained as UE variables. On the other hand, configurations for conditional reconfiguration are managed as UE variable. If RAN2 follows CHO method, RAN2 needs to introduce a new UE variable for conditional LTM. If RAN2 introduce a new UE variable, UE/gNB needs to manage the entries for conditional LTM separately from the entries for the normal LTM, the interactions between the entries for conditional LTM and the entries for the normal LTM will be complicated and CSI measurement configuration will become more complicated. Therefore, the entries for conditional LTM are managed in ltm-Config which maintains Rel-18 LTM configuration.

Proposal 18. The entries for conditional LTM is managed in ltm-Config which maintains Rel-18 LTM configuration.

Subsequent C-LTM

The intention of Rel-18 subsequent LTM is to avoid frequent RRC configuration upon cell switch. The definition of subsequent LTM in Rel-18 is “Subsequent LTM is done by repeating the early synchronization, LTM cell switch execution, and LTM cell switch completion steps without releasing other LTM candidate configurations after each LTM cell switch completion” in [2]. Since Rel-18 only supports the subsequent intra-CU LTM and now Rel-19 subsequent C-LTM focuses only on intra-CU cases, we propose that the definition concept of Rel-18 subsequent LTM can be reused for Rel-19 subsequent intra-CU C-LTM with the revision as “Subsequent intra-CU C-LTM is done by repeating the early synchronization, LTM cell switch execution, and LTM cell switch completion steps without releasing other C-LTM candidate configurations after each C-LTM cell switch completion”.

Observation 4. The definition concept of Rel-18 subsequent LTM can be reused for Rel-19 subsequent intra-CU C-LTM since Rel-18 LTM focuses on intra-CU cases.

Proposal 19. The definition of Rel-19 subsequent intra-CU C-LTM is “Subsequent intra-CU C-LTM is done by repeating the early synchronization, C-LTM cell switch execution, and C-LTM cell switch completion steps without releasing other C-LTM candidate configurations after each C-LTM cell switch completion.”

Failure Recovery

In Rel-18 LTM, LTM fast recovery procedure is supported. When UE detects the LTM cell switch failure or RLF, UE performs cell selection, and if the selected cell is candidate cell for LTM, UE performs LTM cell switch procedure to the cell. This mechanism can be reused for CLTM.

Proposal 20. CLTM fast recovery is supported.

Coexistence of Rel18 LTM and CLTM

According to the WID, the purpose of supporting R19 intra-CU CLTM is improvement of robustness. Thus, UE is expected to perform CLTM when the network fails to instruct LTM cell switching at the appropriate time. Therefore, CLTM configuration should be same to R18/19 LTM configuration excluding execution conditions. If so, LTM configuration and CLTM configuration should be able to use for non-conditional LTM triggered by the LTM CSC MAC CE.

Proposal 21. A candidate configuration for CLTM can be used for a candidate configuration for non-conditional LTM

In the current RRC CR, CR Rapp suggest to re-use the current ltm-Config and ltm-CandidateConfig for CLTM by including execution conditions in the ltm-Config and ltm-CandidateConfig. If CLTM configuration can be used non-conditional LTM, it is reasonable that NW configures UE with eight LTM candidate configurations for MN LTM instead of configuring CLTM and LTM separately from the perspective of signalling overhead

Proposal 22. Not support to configure 8 candidate configurations only for CLTM, separate from the (non-conditional) LTM configuration

Conclusion

Proposal 1. RRC-based early TA acquisition mechanism for early synchronization can be considered for conditional LTM.

Proposal 2: New MAC CE include following information for each candidate cell:

-	TA value

-	Candidate configuration Id

Proposal 3. New TAT for early synchronization should be maintained per candidate configuration.

Proposal 4. When UE executes CLTM, UE should apply a value of the new TAT for the target configuration to TA timer.

Proposal 5. New TAT for candidate cells should not be stopped upon LTM cell switch execution

Proposal 6. UE starts conditional LTM event evaluation once it is configured with C-LTM configuration .

Proposal 7. The event evaluation mechanism for C-LTM should be the same as that for event-triggered L1 measurement reporting.

Proposal 8. L1 and L3 conditions can be configured simultaneously.

Proposal 9. How does the MAC layer and the RRC layer interact with each other to trigger the CLTM execution should be further specified.

Proposal 10. Specify that the conditions are configured on a per-cell basis or on a per-beam basis.

Proposal 11: Event LTM3 and Event LTM5 are reused for CLTM.

-	CondEvent LTM3: Beam of candidate cell becomes better than absolute threshold

-	CondEvent LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2

Proposal 12. When N of beams fulfils the associated execution condition, UE triggers the LTM cell switch. N is configured by the network.

Proposal 13. MAC should indicate all candidate configurations which entry condition is fulfilled to RRC. RRC should always indicate target configuration ID to MAC layer.

Proposal 14. Both CBRA and CFRA should be supported for conditional LTM.

Proposal 15. UE notifies that UE will perform LTM cell switch to the source DU just before the LTM cell switch, source DU notifies it to target DU, and target DU ensures UE has UL grant.

Proposal 16. If leaving condition for the target cell is satisfied, UE should consider the target cell as a cell which doesn’t satisfy the condition for cell switching.

Proposal 17. UE autonomously determines a TCI state to be applied upon CLTM cell switch execution for a target cell among the TCI states activated by the Candidate Cell TCI States Activation/Deactivation MAC CE.

Proposal 18. The entries for conditional LTM is managed in ltm-Config which maintains Rel-18 LTM configuration.

Proposal 19. The definition of Rel-19 subsequent intra-CU C-LTM is “Subsequent intra-CU C-LTM is done by repeating the early synchronization, C-LTM cell switch execution, and C-LTM cell switch completion steps without releasing other C-LTM candidate configurations after each C-LTM cell switch completion.”

Proposal 20. CLTM fast recovery is supported.

Proposal 21. A candidate configuration for CLTM can be used for a candidate configuration for non-conditional LTM

Proposal 22. Not support to configure 8 candidate configurations only for CLTM, separate from the (non-conditional) LTM configuration

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- TS 38.300 V18.1.0 (2024-05), 5G; NR; NR and NG-RAN Overall description; Stage-2