3GPP TSG-RAN WG2 Meeting #129		R2-2501213

Athens, Greece, Feb 17 – 21, 2025

Agenda Item:	8.10.2.2

Source: 	LG Electronics

Title:  	MRO for CHO and Associated CPAC

Document for:	Discussion

Introduction

In this contribution, we focus on the Rel-18 DCCA (i.e. CHO+CPAC) mobility MRO based on the last agreements.

Discussion

Failure logging

We have summarized the agreements made in RAN2 so far regarding the failure logging function related to CHO+CPAC as follows:

RAN2#126 - RAN2 to enhance the RLF report with additional information regarding the state of the two execution conditions. We see later if we also can enhance the SCGFailureInformation report. RAN2#127 - UE includes following information in RLF report: - b. Time information regarding condition fulfilment for CHO with candidate SCGs. Details are FFS. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled. - c. Measurement results of PCells and PSCells. RAN2#127bis - UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. Details FFS. - Include the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report. Details FFS. RAN2#128 - RAN2 understands that current agreements is that the UE shall logs (in RLF report, SHR report and SCGFailureInformation) time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell). We don’t intend to do further or more granular enhancements. - Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and SCGFailureInformation also. We will check what the spec impact of this is, e.g. something in the spec today may already make the UE log this.

According to the above agreements, when the UE with the current CHO+CPAC configuration performs RLF-report logging upon failure, the UE logs the time gap between the satisfaction of the CHO and the associated CPA/CPC execution conditions and logs which execution condition was satisfied first.

However, since the UE does not log the actual time when the first execution condition was satisfied, the network cannot accurately predict the satisfaction times of the first and second execution conditions even if the network receives this time gap information. Without knowing this actual time, the network may not clearly identify the cause when the UE fails to perform mobility at the expected time for CHO+CPAC, and thus cannot appropriately update the execution condition configuration.

Also, including this information in the SCG failure information can be helpful for the network by the same reason above. Thus, we propose that the UE should log the satisfaction time of the first execution condition when logging the RLF report and SCG failure information.

Proposal 1. For CHO with associated CPA/CPC, the UE should include the satisfaction time of the first execution condition in the MCG RLF-report or SCG failure information as part of the failure information.

Near failure logging

Based on a flow chart which we analysed CHO+CPAC flow shown in section 5.1, this flow chart shows logging cases that can occur after the UE is configured with CHO+CPC. According to the flow chart, we can see that in all cases where CHO+CPC logging is required, except for clear success case, the CHO+CPC information can be reported to the network via an RLF report, SCG failure information or SHR (Successful Handover Report).

In other words, although PSCell change can be performed together, CPC is an additional condition based on CHO and the configuration is the same as CHO+SCG, so there is no need to consider SPR (Successful PSCell change or addition Report) for new logging information of the CHO+CPC.

Proposal 2: For CHO with associated CPA/CPC, the UE reports information for the near failure case to the network via SHR not SPR.

According to the previous agreements of RAN2#128 meeting, the UE performs logging of the SHR with the same contents as logging of the RLF-report. That is, the UE does not log the actual time when the first execution condition was satisfied (as we address in 2.1). We think the same issue is here as like the RLF-report that the network may not clearly identify the cause of the near failure.

Thus, we propose the following for the SHR logging to resolve the same issue above:

Proposal 3. For CHO with associated CPA/CPC, the UE should include the satisfaction time of the first execution condition in the SHR as part of the near failure information.

Conclusion

It is proposed to discuss and decide on the following proposals:

Proposal 1. For CHO with associated CPA/CPC, the UE should include the satisfaction time of the first execution condition in the MCG RLF-report or SCG failure information as part of the failure information.

Proposal 2: For CHO with associated CPA/CPC, the UE reports information for the near failure case to the network via SHR not SPR.

Proposal 3. For CHO with associated CPA/CPC, the UE should include the satisfaction time of the first execution condition in the SHR as part of the near failure information.

References

- 3GPP TS 38.331 V18.2.0 (2024-06)

Annexes

CHO+CPC Flow Chart

SHR information

SuccessHO-Report-r17 ::= SEQUENCE { sourceCellInfo-r17 SEQUENCE { sourcePCellId-r17 CGI-Info-Logging-r16, sourceCellMeas-r17 MeasResultSuccessHONR-r17 OPTIONAL, rlf-InSourceDAPS-r17 ENUMERATED {true} OPTIONAL}, targetCellInfo-r17 SEQUENCE { targetPCellId-r17 CGI-Info-Logging-r16, targetCellMeas-r17 MeasResultSuccessHONR-r17 OPTIONAL}, measResultNeighCells-r17 SEQUENCE { measResultListNR-r17 MeasResultList2NR-r16 OPTIONAL, measResultListEUTRA-r17 MeasResultList2EUTRA-r16 OPTIONAL } OPTIONAL, locationInfo-r17 LocationInfo-r16 OPTIONAL, timeSinceCHO-Reconfig-r17 TimeSinceCHO-Reconfig-r17 OPTIONAL, shr-Cause-r17 SHR-Cause-r17 OPTIONAL, ra-InformationCommon-r17 RA-InformationCommon-r16 OPTIONAL, upInterruptionTimeAtHO-r17 UPInterruptionTimeAtHO-r17 OPTIONAL, c-RNTI-r17 RNTI-Value OPTIONAL, ..., [[ eutraTargetCellInfo-r18 SEQUENCE { targetPCellId-r18 CHOICE { cellGlobalId-r18 CGI-Info-Logging-r16, pci-arfcn-r18 PCI-ARFCN-EUTRA-r16}, targetCellMeas-r18 MeasQuantityResultsEUTRA OPTIONAL } OPTIONAL, measResultServCell-RSSI-r18 RSSI-Range-r16 OPTIONAL, measResultNeighFreqList-RSSI-r18 MeasResultNeighFreqList-RSSI-r18 OPTIONAL, eutra-C-RNTI-r18 EUTRA-C-RNTI OPTIONAL, timeSinceSHR-r18 TimeSinceSHR-r18 OPTIONAL ]]}

RLF Report information

RLF-Report-r16 ::= CHOICE { nr-RLF-Report-r16 SEQUENCE { measResultLastServCell-r16 MeasResultRLFNR-r16, measResultNeighCells-r16 SEQUENCE { measResultListNR-r16 MeasResultList2NR-r16 OPTIONAL, measResultListEUTRA-r16 MeasResultList2EUTRA-r16 OPTIONAL } OPTIONAL, c-RNTI-r16 RNTI-Value, previousPCellId-r16 CHOICE { nrPreviousCell-r16 CGI-Info-Logging-r16, eutraPreviousCell-r16 CGI-InfoEUTRALogging } OPTIONAL, failedPCellId-r16 CHOICE { nrFailedPCellId-r16 CHOICE { cellGlobalId-r16 CGI-Info-Logging-r16, pci-arfcn-r16 PCI-ARFCN-NR-r1 }, eutraFailedPCellId-r16 CHOICE { cellGlobalId-r16 CGI-InfoEUTRALogging, pci-arfcn-r16 PCI-ARFCN-EUTRA-r16}}, reconnectCellId-r16 CHOICE { nrReconnectCellId-r16 CGI-Info-Logging-r16, eutraReconnectCellId-r16 CGI-InfoEUTRALogging } OPTIONAL, timeUntilReconnection-r16 TimeUntilReconnection-r16 OPTIONAL, reestablishmentCellId-r16 CGI-Info-Logging-r16 OPTIONAL, timeConnFailure-r16 INTEGER (0..1023) OPTIONAL, timeSinceFailure-r16 TimeSinceFailure-r16, connectionFailureType-r16 ENUMERATED {rlf, hof}, rlf-Cause-r16 ENUMERATED {t310-Expiry, randomAccessProblem, rlc-MaxNumRetx, beamFailureRecoveryFailure, lbtFailure-r16, bh-rlfRecoveryFailure, t312-expiry-r17, spare1}, locationInfo-r16 LocationInfo-r16 OPTIONAL, noSuitableCellFound-r16 ENUMERATED {true} OPTIONAL, ra-InformationCommon-r16 RA-InformationCommon-r16 OPTIONAL, ..., [[ csi-rsRLMConfigBitmap-v1650 BIT STRING (SIZE (96)) OPTIONAL ]], [[ lastHO-Type-r17 ENUMERATED {cho, daps, spare2, spare1} OPTIONAL, timeConnSourceDAPS-Failure-r17 TimeConnSourceDAPS-Failure-r17 OPTIONAL, timeSinceCHO-Reconfig-r17 TimeSinceCHO-Reconfig-r17 OPTIONAL, choCellId-r17 CHOICE { cellGlobalId-r17 CGI-Info-Logging-r16, pci-arfcn-r17 PCI-ARFCN-NR-r16 } OPTIONAL, choCandidateCellList-r17 ChoCandidateCellList-r17 OPTIONAL ]], [[ pSCellId-r18 CHOICE { cellGlobalId-r18 CGI-Info-Logging-r16, pci-arfcn-r18 PCI-ARFCN-NR-r16 } OPTIONAL, mcgRecoveryFailureCause-r18 ENUMERATED {t316-Expiry, scgDeactivated, spare2, spare1} OPTIONAL, scgFailureCause-r18 ENUMERATED {t310-Expiry, randomAccessProblem, rlc-MaxNumRetx, synchReconfigFailureSCG, scg-ReconfigFailure, srb3-IntegrityFailure, scg-lbtFailure-r16, beamFailureRecoveryFailure-r16, t312-Expiry-r16, bh-RLF-r16, beamFailure-r17, spare3, spare2, spare1 } OPTIONAL, elapsedTimeSCGFailure-r18 ElapsedTimeSCGFailure-r18 OPTIONAL, voiceFallbackHO-r18 ENUMERATED {true} OPTIONAL, measResultLastServCell-RSSI-r18 RSSI-Range-r16 OPTIONAL, measResultNeighFreqList-RSSI-r18 MeasResultNeighFreqList-RSSI-r18 OPTIONAL, bwp-Info-r18 AttemptedBWP-Info-r18 OPTIONAL, elapsedTimeT316-r18 ElapsedTimeT316-r18 OPTIONAL ]] }, eutra-RLF-Report-r16 SEQUENCE { failedPCellId-EUTRA CGI-InfoEUTRALogging, measResult-RLF-Report-EUTRA-r16 OCTET STRING, ..., [[ measResult-RLF-Report-EUTRA-v1690 OCTET STRING OPTIONAL ]] } }