3GPP TSG-RAN WG2 Meeting #129	R2-2501225

Athens, Greece, Feb. 17th – 21st, 2025

Source:        ZTE Corporation

Title:           Consideration on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx

Agenda item:   8.19

Document for:  Discussion and Decision

- Introduction

In the last meeting, a LS[1] was sent from RAN4 on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx. In this LS, a 2UL FDD-FDD band combination (i.e. CA_n5-n8) with partially overlapped frequency range was discussed and RAN4 agreed that 2UL/2DL CA_n5-n8 is supported using non-concurrent n5 DL and n8 UL.

1. Overall Description: RAN4 has discussed the UE RF requirements for 2UL/2DL inter-band FDD-FDD CA_n5-n8. It is agreed that 2UL/2DL CA_n5-n8 is supported using non-concurrent n5 DL and n8 UL. Because n8 UL frequency range of CA_n5-n8 partially overlaps with n5 DL frequency range, there is no concurrent operation between n8 UL transmitting and n5 DL receiving, which is a special FDD-FDD inter-band CA case. Figure below conceptually illustrates the intended operation of this UL CA combination where the operation will be alternating between the two status under the RRC configuration (UL CA_n5-n8 and DL CA_n5-n8). Usually for FDD-FDD interband CA, a band pair nA+nB can support simultaneous (nAUL, nBDL) and (nBUL, nADL) by default. For the specific band pair n5+n8, the capabilities for simultaneous (n5UL, n8DL) and (n8UL, n5DL) can be different, currently simultaneous (n5UL, n8DL) is supported but simultaneous (n8UL, n5DL) is not. RAN4 kindly asks RAN2: - To make necessary modifications in RAN2 specification and/or to add UE capability, if needed, to enable the described functionality. - If there’s a capability defined, it’s mandatory for the corresponding band combinations when indicating UL CA support. - Comparing to the TDD-TDD/TDD-FDD CA non-simultaneous Rx/Tx feature, it’s encouraged to double check whether there is an issue for this FDD-FDD CA case, and report back to RAN4 if those are found. NOTE: This is intended only for special case when UL and DL from different FDD bands partially overlap. This is not intended for FDD bands which do not partially overlap. NOTE: The described functionality is targeted also for cases when higher order CA configuration (e.g CA_n5A-n8A-nXXA) supports 2UL in n5 and n8. Currently there is only one band combination, i.e., n5+n8 has such an issue, however, RAN4 appreciate if RAN2 may consider a generic future-proof solution in case more similar NR FDD-FDD inter-band CA band combinations where UL and DL from different FDD bands partially overlap are introduced in RAN4. 2. Actions: To RAN WG2: RAN4 respectively asks RAN2 to consider the above information. and to make necessary modifications into RAN2 specification and to define UE capability if necessary.

In this paper, we will analyse whether it would have any RAN2 impact.

- Discussion

In this chapter, we first give a short description on the current inter-band CA capabilities, then analyse whether any necessary RAN2 modifications are needed.

2.1 RAN2 Existing Inter-band CA Simultaneous Rx/Tx Capabilities

From the TDD/FDD aspect, there are 3 Inter-band CA band combination types:

- Type 1: TDD-TDD Inter-band CA

- Type 2: TDD-FDD Inter-band CA

- Type 3: FDD-FDD inter-band CA

For the first 2 types, the UEs can have different implementations, which means some UEs can support simultaneous Rx/Tx but some UEs can not. Thus, the simultaneousRxTxInterBandCA/ simultaneousRxTxInterBandCAPerBandPairs was introduced to indicate different UE implementations.

| simultaneousRxTxInterBandCA Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band NR CA. If this field is included in ca-ParametersNR-ForDC, it indicates the UE supports simultaneous transmission and reception between any UL/DL band pair within a cell group and across MCG and SCG in TDD-TDD and TDD-FDD inter-band NR-DC. It is mandatory for certain TDD-FDD and TDD-TDD band combinations defined in TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4]. This capability does not apply to the following components within TDD-TDD and TDD-FDD inter-band NR-CA or NR-DC combinations: - Intra-band NR-CA or NR-DC component - Inter-band NR-CA or NR-DC component where the frequency range of one TDD band is a subset of the frequency range of the other NR TDD band (as specified in TS 38.101-1 [2]). | BC | CY | N/A | N/A |
| --- | --- | --- | --- | --- |
| simultaneousRxTxInterBandCAPerBandPair Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band NR CA for each band pair in the band combination. Encoded as a bitmap with size L * (L – 1) / 2, and bit N (leftmost bit is indexed as bit 0) is set to "1" if the UE supports simultaneous transmission and reception for band pair (x, y), where L is the number of band entries in the band combination, x and y are the indices of the band entry in the band combination (the first band entry is indexed as 0), x < y, and N = x*(2*L – x – 1)/2 + y – x – 1. If this field is included in ca-ParametersNR-ForDC, each bit of this field indicates whether the UE supports simultaneous transmission and reception between each band pair, within a cell group and across MCG and SCG in TDD-TDD and TDD-FDD inter-band NR-DC. The UE does not include this field if the UE supports simultaneous transmission and reception for all applicable band pairs in the band combination (in which case simultaneousRxTxInterBandCA is included) or does not support for any band pair in the band combination. It is mandatory for certain band pairs as specified in TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4]. The UE shall consistently set the bits which correspond to the same band pair. | BC | CY | N/A | N/A |

For the Type 3 (i.e. FDD-FDD inter-band CA), as described in the LS, usually for FDD-FDD interband CA, a band pair nA+nB can support simultaneous (nAUL, nBDL) and (nBUL, nADL) by default, which means for the FDD-FDD inter-band CA without frequency range overlap, simultaneous Rx/Tx was supported by default and thus no capability was defined.

Observation 1: For the legacy FDD-FDD inter-band CA cases (i.e. FDD-FDD inter-band CA without frequency range overlap), simultaneous Rx/Tx was supported by default and thus no capability was defined.

2.2 FDD-FDD inter-band CA with Frequency Range Overlap

In this chapter, we’d like to discuss whether a new capability/some RAN2 spec modification would be needed for the FDD-FDD inter-band CA with frequency range overlap case.

Issue 1: Whether a new capability is needed

Normally, whether a new capability is needed depends on whether the UEs can have the different implementations. However, as agreed in RAN4, the “2UL/2DL CA_n5-n8 is supported using non-concurrent n5 DL and n8 UL”, which means that the UE would have the same implementation, e.g. simultaneous (n5UL, n8DL) is supported but simultaneous (n8UL, n5DL) is not.

Observation 2: For the UE that support the 2UL/2DL CA_n5-n8, UE would have the same implementation, i.e. simultaneous (n5UL, n8DL) is supported but simultaneous (n8UL, n5DL) is not.

Furthermore, as clarified in the LS [1] “currently there is only one band combination, i.e., n5+n8 has such an issue”, and according to the RAN4’s submission in the last meeting, there was also a simple solution to this issue, e.g. add a note to include RAN4’s agreement in the RAN4 spec. Thus at this stage, there is no need to add a new capability. The new capability is needed only when some UEs have different implementations.

Proposal 1: No need to introduce new simultaneousRxTx capability for the 2UL/2DL CA_n5-n8 case, for the UE support the 2UL/2DL CA_n5-n8 band combination, the UE should support simultaneous (n5UL, n8DL) but not support simultaneous (n8UL, n5DL) by default.

Issue 2: Whether some other RAN2 spec modification is needed

In the current specification, the simultaneousRxTxInterBandCA/ simultaneousRxTxInterBandCAPerBandPairs were defined for the TDD-TDD and FDD-TDD inter-band CA case, thus, the FDD-FDD with or without frequency range overlap would have no impact for these two capabilities.

Proposal 2: No spec modification on the simultaneousRxTxInterBandCA/ simultaneousRxTxInterBandCAPerBandPairs is needed.

If the proposal1/2 were agreed, a reply LS can be sent back to RAN4 to include these 2 proposals.

Proposal 3: If the proposal1/2 were agreed, a reply LS can be sent back to RAN4 to include these two proposals.

- Conclusion and proposals

With the above analysis, we have the following observations and proposals:

Observation 1: For the legacy FDD-FDD inter-band CA cases (i.e. FDD-FDD inter-band CA without frequency range overlap), simultaneous Rx/Tx was supported by default and thus no capability was defined.

Observation 2: For the UE that supports the 2UL/2DL CA_n5-n8, the UE would have the same implementation, e.g. simultaneous (n5UL, n8DL) is supported but simultaneous (n8UL, n5DL) is not.

Proposal 1: No need to introduce new simultaneousRxTx capability for the 2UL/2DL CA_n5-n8 case, for the UE support the 2UL/2DL CA_n5-n8 band combination, the UE should support simultaneous (n5UL, n8DL) but not support simultaneous (n8UL, n5DL) by default.

Proposal 2: No spec modification on the simultaneousRxTxInterBandCA/ simultaneousRxTxInterBandCAPerBandPairs is needed.

Proposal 3: If the proposal1/2 were agreed, a reply LS can be sent back to RAN4 to include these two proposals.

- Reference

- R4-2420410 LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx RAN4