3GPP TSG-RAN WG2 Meeting # 129	R2-2501233

# Athens, Greece, 17th – 21st February 2025

Agenda item:	8.6.4

Source:	Google

Title:	Discussion on L1 conditions for conditional LTM

Document for:	Discussion & Decision

- Introduction

In this paper, we will discuss how to configure L1 conditions for conditional LTM.

- Discussion

The exection condition for conditional LTM is defined as below (excerpted from the running RRC CR).

ASN.1 structure of the exectuion condition for conditional LTM

As shown above, the ExecutionConditions-r19 includes the LTM-CandidateId-r18, and either includes either the l1-Conditions-r19 or l3-Coditoins-r19. The l1-Condittions-r19 is a LTM-CSI-ReportConfigId-r18. It is up to the gNB to decide to configure either the L1 conditions or L3 conditions for the UE. In case the gNB is a disaggregated gNB (i.e., a CU and a DU), the CU is responsible for assigning the LTM-CandidateId-r18. In the case of L3 conditions, it is no doubt that the CU is responsible for configuring the L3 conditions. However, it is not clear whether the CU or the DU configures the L1 conditions.

As specified in 38.331, LTM-CSI-ReportConfigId-r18 is included in the LTM-CSI-ReportConfig which is included in the CellGroupConfig for the requested LTM candidate cell or the serving cell. Based on TS 38.473 (as shown below), the DU transmits provides the LTM-CSI-ReportConfig to the CU. The CU then transparently transmits the CellGroupConfig to the UE as specified in TS 38.473.

### 8.3.1 UE Context Setup <unrelated part omitted> … the CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8]. <unrelated part omitted> If the CSI Resource Configuration is contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration(s) in the CellGroupConfig IE for the requested LTM candidate cell. ### 8.3.4 UE Context Modification (gNB-CU initiated) <unrelated part omitted> The CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8]. <unrelated part omitted> If the CSI Resource Configuration IE is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message and the SpCell ID IE is also included, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration in the CellGroupConfig IE for the requested LTM candidate cell identified by the SpCell ID IE. If the CSI Resource Configuration IE is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message while the SpCell ID IE is absent, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration in the CellGroupConfig IE for the serving cell.

If the CU decides to configure the L1 conditions, then the CU is required to decode the CellGroupConfig IE for the serving cell to retrieve ltm-CSI-ReportConfigToAddModList-r18. The CU retrives one or multiple LTM-CSI-ReportConfigId-r18 IEs from the ltm-CSI-ReportConfigToAddModList-r18. If the ltm-CSI-ReportConfigToAddModList-r18 includes multiple LTM-CSI-ReportConfig-r18 IEs, the CU selects a LTM-CSI-ReportConfigId-r18 IE from the LTM-CSI-ReportConfig-r18.

Alternatively, the DU generates the L1 conditions and sends the L1 conditions to the CU. The CU then transparently signals the L1 conditions to the UE. There are two options for this alternative.

Option 1: 	The DU deccides to configure the conditional LTM for the UE during the existing LTM candidate cell preparation. The CU transparently forwards the L1 conditions to the UE.

Option 2: 	The CU requests the DU to provide L1 conditions for the conditional LTM and the DU provides the L1 conditions to the CU.

Based on the above, we provide the following proposal:

Proposal 1: 	RAN2 should discuss and decide which node (i.e., either the CU or the DU) configures the L1 conditions.

Proposal 2: 	If RAN2 agrees that the DU generates the L1 conditions and provides to the CU, RAN2 sends the agreement in a LS to RAN3.

If RAN2 agrees that the DU provides the L1 conditions (i.e., LTM-CSI-ReportConfigId-r18) to the CU, “OCTET STRING” should be used for l1-Conditions-r19 as shown below. Thus, the CU can transparently include the L1 conditions in the ExecutionConditions-r19. This simplifies the CU implemenetation.

ExecutionConditions-r19 ::= SEQUENCE { ltm-CandidateId-r19 LTM-CandidateId-r18, executionCondition-r19 CHOICE { l1-Conditions-r19 OCTET STRING (LTM-CSI-ReportConfigId-r18), l3-Conditions-r19 SEQUENCE (SIZE (1..2)) OF MeasId }

Proposal 3: 	If RAN2 agrees that the DU generates the L1 conditions, “OCTET STRING” should be used for l1-Conditions-r19.

# Conclusion

RAN2 is kindly to discuss and agree the following proposals.

Proposal 1: 	RAN2 should discuss and decide which node (i.e., either the CU or the DU) configures the L1 conditions.

Proposal 2: 	If RAN2 agrees that the DU generates the L1 conditions and provides to the CU, RAN2 sends the agreement in a LS to RAN3.

Proposal 3: 	If RAN2 agrees that the DU generates the L1 conditions, “OCTET STRING” should be used for l1-Conditions-r19.

# Reference

[1] TS 38.331 v18.4.0

[2] TS 38.473, v18.4.0