3GPP TSG-RAN2 Meeting #129	R2- 2501242

# Athens, Greece, Feb 17th – 21st, 2025

Agenda item:		8.7.4.1 (NR_XR_Ph3)

Source:	LG Electronics Inc.

Title: 	Discussion on LCP enhancement for XR

Document for:	Discussion and Decision

1.	Introduction

According to WID of Rel-19 XR[1], it is agreed to define additional Logical channel priority handling based on the delay/deadline information.

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

In RAN2#128 meeting [2], some details using additional LCP are agreed, which is to use additional LCH priority with LCH priority-adjusted data for both the first round and the second round of the LCP procedure as a baseline, but fallback to default priority at second round is supported with optional capability.

Agreements on LCP prioritization - As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. - As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

This document discusses remaining issues on LCP enhancement for XR.

2.	Discussion

In RAN2#128 meeting, several companies suggested to adjust Bj/PRB when the additional priority is applied for a LCH, in order to ensure that all the LCH priority-adjusted data in the LCH can be included in the uplink grant, before including the data in other LCHs.

However, adjusting Bj/PRB for LCH with LCH priority-adjusted data does not ensure that all the LCH priority-adjusted data in the LCH can be included in the uplink grant as well, since there could be LCH priority-adjusted data in another logical channel. For example, if there is LCH priority-adjusted data in LCH1 and LCH2, if the PBR/Bj is adjusted to higher value in order to ensure the transmission of LCH priority-adjusted data in LCH1, non LCH priority-adjusted data in LCH1 can be included prior to the LCH priority-adjusted data buffered in LCH2, even though the logical channel priority of LCH2 is adjusted.

In addition, if the PBR/Bj is adjusted for LCH with LCH priority-adjusted data, it would also cause an issue on fairness of other LCH with lower priority regardless of whether the LCH includes LCH priority-adjusted data or not. In RAN2#128 meeting[2], due to strong concerns on fairness issue, it is agreed that UE can support to fallback from additional priority to default priority during the 2nd round of LCP procedure. Given the 1st round of current LCP procedure with Bj and PBR is defined in order to ensure the fairness to all logical channels, the same principle should be kept in the 1st round even though the logical channel has LCH priority-adjusted data.

Therefore, there is no strong benefit to modify PBR or Bj for LCH with LCH priority-adjusted data, as long as it doesn’t ensure the transmission of LCH priority-adjusted data in all cases.

Proposal 1. There is no need to modify PBR or Bj for LCH with LCH priority-adjusted data.

In RAN2#127bis meeting [3], it is agreed to define additional delay threshold to determine LCH-priority adjusted data

Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold.

On the other hand, during the RAN2#125bis meeting, some companies suggested to consider the PDU set importance for delay-aware LCP for XR. In our understanding, low importance data should not be prioritized in congestion scenario, since the transmission of low importance data is not essential although it has less remaining time due to reduced PDCP discard timer value. Therefore, when the PSI-based discard is activated, low importance data should not be considered as LCH priority-adjusted data.

Proposal 2. When the PSI-based discard is activated, low importance data should not be considered as LCH priority-adjusted data.

Meanwhile, the logical channel priority is used not only in logical channel prioritization, but also in other procedure e.g., for intra-UE prioritization and BSR triggering procedure.

For intra-UE prioritization, in RAN2#127 meeting [4], it was remained as FFS on whether the additional priority for LCH with LCH priority-adjusted can be applied as well.

FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)

In NR Rel-16, the intra-UE prioritization mechanism is introduced to support tighter latency and reliability. In the Rel-16 intra-UE prioritization, when the two UL grants are overlapped, the priority of each UL grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU. In other words, logical channel priority is also considered when the UE determines which UL grant should be prioritized.

However, the Rel-16 intra-UE prioritization does not consider the remaining time of data, and it may have trouble to satisfy strict delay requirement for XR service because one UL grant which is used for data from the highest priority of logical channel may be always prioritized over any other UL grant. For example, when two UL grants are given for XR service and one of UL grants has the highest priority logical channel with the data having long remaining time till the PDCP discard timer expiry but another UL grant has the relatively lower priority with the data having short remaining time, the Rel-16 intra-UE prioritization prioritizes the UL grant having highest priority logical channel with the data having long remaining time. However, from XR service perspective, another UL grant for data with short remaining time should be prioritized in this case because it is not urgent to transmit the data with long remaining time in the highest priority logical channel and it can be transmitted later due to sufficient remaining time. In our understanding, it is a similar issue as in delay-aware LCP, since it is defined in order to ensure the transmission of data with short remaining time.

Therefore, given that delay-aware LCP is defined in order to ensure the transmission of LCH priority-adjusted data, the additional LCH priority should also be applied to the intra-UE prioritization as well, in order to ensure the transmission of LCH priority-adjusted data when the two UL grants are overlapped.

Proposal 3. The additional priority for LCH with LCH priority-adjusted data should also be applied on intra-UE prioritization.

For BSR triggering procedure, the logical channel priority is also used to determine whether to trigger new Regular BSR. However, there is no essential need to consider additional priority for BSR triggering procedure, since it is not directly related to transmission of LCH priority-adjusted data. In addition, in Rel-18 XR, DSR procedure is already defined to report the buffer status of urgent data with short remaining time (i.e., delay-critical data). Therefore, even though the buffered data becomes LCH priority-adjusted data or new LCH priority-adjusted data is arrived for an LCH, there is no need to further consider additional priority for BSR triggering procedure, since the buffer status of urgent data would be reported using existing DSR procedure anyway.

Proposal 4. No need to apply additional priority for BSR triggering procedure.

3.	Conclusion

In this contribution, it is discussed for the LCP enhancement for XR. This document includes following proposals.

Proposal 1. There is no need to modify PBR or Bj for LCH with LCH priority-adjusted data.

Proposal 2. When the PSI-based discard is activated, low importance data should not be considered as LCH priority-adjusted data.

Proposal 3. The additional priority for LCH with LCH priority-adjusted data should also be applied on intra-UE prioritization.

Proposal 4. No need to apply additional priority for BSR triggering procedure.

4.	Reference

[1] RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3

[2] Report of 3GPP TSG RAN WG2 meeting #128, Orlando, USA, 18 - 22 November, 2024

[3] Draft Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China, 14 - 18 October, 2024

[4] Draft Report of 3GPP TSG RAN WG2 meeting #127 Maastricht, Netherlands, 19 - 23 August, 2024