3GPP TSG-RAN WG2 Meeting #129	R2-2501250

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:		8.2.4

Source:		Futurewei

Title:		Discussions on A-IoT data transmission and other aspects

Document for:		Discussion and Decision

# Introduction

In the WID [1] of Rel-19 Ambient IoT (A-IoT) WI, the RAN2-specific scope with respect to A-IoT data transmission and other general aspects is described as the following:

    - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R.

    - Only MAC layer is included

In this contribution, we discuss various aspects of A-IoT MAC PDU/signaling format, data retransmission for failure handling, segmentation, and Message Size/Command Type information.

# Discussions

## On overall A-IoT MAC PDU/signaling format

According to the latest TR 23.700-13 [2], an A-IoT device supports, from the bottom up, A-IoT AS layers, A-IoT NAS layer, and A-IoT Data layer, as shown in Figures 1 and 2 below. AIoT Device NAS protocol is supported between the AIoT Device and the AIOTF. The AIoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response. Although the detailed design is to be finalized by other WGs, it is a reasonable expectation that when a Command Request contains a Write command, the information (AIoT Data) to be written to the target device(s) is encapsulated in the Command Request, and when a Command Request contains a Read command, the information (AIoT Data) read from the target device is encapsulated in a Command Response sent by the target device in response to the Command Request.

Figure 1 (same as Figure 8.1.2.2-3: Example Protocol Stack between AIOTF and AIoT Device for Topology 1 (Direct Path) in [2])

Figure 2 (same as Figure 8.1.2.3-3: Example Protocol Between AIOTF and AIoT Device for Topology 1 (indirect Path via AMF) in [2])

According to TR 38.769 [3], the A-IoT AS layers include A-IoT PHY layer and A-IoT MAC layer. Therefore, an A-IoT NAS PDU enters (for transmission) and leaves (for reception) an A-IoT MAC entity as an A-IoT MAC SDU.

Observation 1. Although the detailed design is to be finalized by other WGs, it is a reasonable expectation that when a Command Request contains a Write command, the information (AIoT Data) to be written to the target device(s) is encapsulated in the Command Request, and when a Command Request contains a Read command, the information (AIoT Data) read from the target device is encapsulated in a Command Response sent by the target device in response to the Command Request.

Observation 2. An A-IoT NAS PDU, which may contain a Command Request for the R2D direction, or an Inventory Response or a Command Response for the D2R direction, enters and leaves an A-IoT MAC entity as an A-IoT MAC SDU.

Based on the above observations, all A-IoT MAC PDUs can be categorized into two types: one that contains an A-IoT MAC SDU and one that does not. Examples of A-IoT MAC PDUs that do not contain any A-IoT MAC SDU include Msg1 sent in 3-step CBRA and Msg2, because they contain only the random ID, which is an AS information. Examples of A-IoT MAC PDUs containing an A-IoT MAC SDU include Msg3 (containing an Inventory Response) and subsequent R2D data transmissions (containing a Command Request) and D2R data transmissions (containing a Command Response). An A-IoT paging message may or may not include an A-IoT MAC SDU, depending on whether the A-IoT paging message further encapsulates a Command Request.

No matter containing an A-IoT MAC SDU or not, an A-IoT MAC PDU may contain one or more AS control information, e.g., the D2R resource scheduling information in an A-IoT paging message or Msg2, the random ID in Msg1 and Msg2, the segmentation indication in a D2R data transmission, etc. In addition, it has been discussed in SA2 that information in the A-IoT paging message for identifying the devices to respond is sent to devices not as an A-IoT NAS PDU. Instead, the reader needs to process and retrieve that information from the Inventory Request, which is received from the AIOTF, and present the information in the A-IoT paging message as AS information. We propose that each of these AS control information be carried in MAC CEs so that, as a general format, an A-IoT MAC PDU either contains one or more A-IoT MAC CEs without any A-IoT MAC SDU or contains one A-IoT MAC SDU and zero or more A-IoT MAC CEs.

To differentiate between A-IoT MAC SDU and A-IoT MAC CE, we suggest introducing the concept of A-IoT MAC element, which can be an A-IoT MAC data element (DE) or A-IoT MAC CE, where an A-IoT MAC DE carries an A-IoT MAC SDU and an A-IoT MAC CE does not. Both A-IoT MAC DE and A-IoT MAC CE begins with a D/C (data/control) field, which indicates whether the A-IoT MAC element is an A-IoT MAC DE or an A-IoT MAC CE.

Proposal 1. AS control information is carried in a form of MAC CE so that, as a general format, an A-IoT MAC PDU either contains one or more A-IoT MAC CEs without any A-IoT MAC SDU or contains one A-IoT MAC SDU and zero or more A-IoT MAC CEs.

Proposal 2. RAN2 introduce the concept of A-IoT MAC element, which can be an A-IoT MAC data element (DE) or A-IoT MAC CE, where an A-IoT MAC DE carries an A-IoT MAC SDU and an A-IoT MAC CE does not. Both A-IoT MAC DE and A-IoT MAC CE begins with a D/C (data/control) field, which indicates whether the A-IoT MAC element is an A-IoT MAC DE or an A-IoT MAC CE.

There can be two options for a general format of A-IoT MAC DEs, as illustrated and described below:

- Option 1. An A-IoT MAC DE consists of the D/C field and a Data field (for carrying the A-IoT MAC SDU), as shown in Figure 3, if we mandate the A-IoT MAC DE, when present, to always be the last A-IoT MAC element in the A-IoT MAC PDU.

Figure 3. A-IoT MAC DE Format Option 1

- Option 2. An A-IoT MAC DE consists of the D/C field, a Length field, and the Data field, as shown in Figure 4, if we do not mandate the A-IoT MAC DE, when present, to always be the last A-IoT MAC element in the A-IoT MAC PDU. The Length field (a fixed-length field) indicates the length of the Data field to assist the recipient in parsing the A-IoT MAC DE properly.

Figure 4. A-IoT MAC DE Format Option 2

There can be two options for a general format of A-IoT MAC CEs, as illustrated and described below:

- Option A. An A-IoT MAC CE consists of the D/C field, a Type field, an optional Length field, and optionally one or more type-dependent information fields as the payload, as shown in Figure 5. The Type field, which is a fixed-length field, identifies the purpose of the A-IoT MAC CE and is used for determining the presence of the Length field and of the type-dependent information fields, if any, as to be specified in the stage-3 MAC specification. In Option A, there can be two types of A-IoT MAC CEs: fixed-length A-IoT MAC CEs and variable-length A-IoT MAC CEs. The lengths of all fixed-length A-IoT MAC CEs are to be specified in the stage-3 MAC specification and hence can be determined by the recipient based on the Type field without a need for the Length field. In all variable-length A-IoT MAC CEs, the Length field is present to indicate the total length of all type-dependent information fields that are present. The length information, whether indicated implicitly or explicitly, is important for the recipient to parse the A-IoT MAC CE properly.

Figure 5. A-IoT MAC CE General Format Option A

- Option B. An A-IoT MAC CE consists of the D/C field, the Length field, the Type field, and optionally one or more type-dependent information fields as the payload, as shown in Figure 6. In Option B, the Length field is mandatorily present after the D/C field and before the Type field to indicate the total length of all remaining field(s) in the A-IoT MAC CE, so that even if a device does not support certain type of A-IoT MAC CE, e.g., the A-IoT MAC CE is optionally supported or is introduced in a later release, the device still knows how to skip the A-IoT MAC CE to parse the remaining portion of the A-IoT MAC PDU properly. Therefore, Option B provides forward compatibility with a price that the Length field is always present.

Figure 6. A-IoT MAC CE General Format in Option B

The Length fields in A-IoT MAC DEs and A-IoT MAC CEs do not have to use the same unit. For example, the Length field in A-IoT MAC CEs can indicate the total length of all remaining field(s) in the A-IoT MAC CE in bits while the Length field in A-IoT MAC DEs can indicate the length of the Data field in octet. The details of the units can be finalized after all types of A-IoT MAC CEs and their contents are identified.

Proposal 3. RAN2 design a format of A-IoT MAC DE for carrying A-IoT MAC SDU in the A-IoT MAC PDU. Options 1 and 2, as described in Figures 3 and 4, respectively, can be considered for down-selection.

Proposal 4. RAN2 design a general format of A-IoT MAC CEs for carrying various types of AS control information in the A-IoT MAC PDU. Options A and B, as described in Figures 5 and 6, respectively, can be considered for down-selection.

## On data retransmission for failure handling

RAN2 has decided, during Rel-18 SI phase, that (ACK/NACK-based) RLC-like retransmission is not supported for A-IoT. Meanwhile, after failing to receive an expected D2R response from a recognized device, the reader is allowed to send another R2D trigger message including the same Command Request to cause the device to retransmit the Command Response, as devices always follow the command from the reader. However, if we do not mandate such behaviors (i.e., immediately re-sending the same Command Request after a failure) on all readers, and when there is no further R2D data transmission needed to the device, there is still ambiguity on the device side as to whether its previous D2R data transmission has been received successfully or not (and whether it needs to re-access or not) if the device does not receive anything back from the reader.

Due to the clock drifting issue at the devices, it is expected that many subsequent paging (or similar R2D trigger) messages are needed, in addition to the initial paging message, to complete an inventory operation. To address the ambiguity issue mentioned above, we propose that after all transmissions triggered by a current paging (or similar R2D trigger) message are complete, whether successfully or unsuccessfully, the reader sends a subsequent paging (or similar R2D trigger) message with the same paging ID. This subsequent paging message can include the 16-bit random ID(s) of the device(s), if any, from which the Msg1 has been received but the Msg3 or subsequent D2R data transmission, which was expected during the previous paging cycle, has not been received successfully by the reader. In response to seeing their respective 16-bit random IDs being listed, these devices can make a re-access attempt either during this subsequent paging cycle or a later paging cycle triggered with the same paging ID. Devices, which had transmitted during the previous paging cycle and the 16-bit random ID of which is not indicated in this subsequent paging (or similar R2D trigger) message considers its previous D2R transmission to the reader has been successful and therefore will not respond to any subsequent paging (or similar R2D trigger) message with the same paging ID from the reader. The subsequent paging (or similar R2D trigger) message may include information to trigger additional devices, which were unable to participate in the previous paging cycles of the same paging ID, to participate in the current paging cycle.

Proposal 5. A subsequent paging (or similar R2D trigger) message with a same paging ID can include the 16-bit random ID(s) of the device(s), if any, from which the Msg1 has been received but the Msg3 or subsequent D2R data transmission, which was expected during the previous paging cycle, has not been received successfully by the reader. In response, these devices can make a re-access attempt either during this subsequent paging cycle or a later paging cycle triggered with the same paging ID.

Proposal 6. Devices, which had transmitted during the previous paging cycle and the 16-bit random ID of which is not indicated in this subsequent paging (or similar R2D trigger) message considers its previous D2R transmission to the reader has been successful and therefore will not respond to any subsequent paging (or similar R2D trigger) message with the same paging ID from the reader.

## On Segmentation

According to the WID [1], segmentation is supported at least in D2R. However, we propose that in the D2R direction, segmentation is not supported on Msg1. The reader shall always schedule enough D2R resource for a device to send its Msg1 in one shot.  And if a device cannot transmit its Msg1 in one shot due to lack of energy, it should not attempt to send its Msg1 in the first place.

We also prefer that segmentation is not supported on Msg3. If Msg3 can be segmented, it would require RAN2 to design a different R2D trigger (other than Msg2) for triggering the device to send the remaining segment(s) of its Msg3, complicating the random access procedures and the R2D trigger message design. Therefore, the reader shall also always schedule enough D2R resource for a device to send its Msg3 in one shot. This requires that the reader knows the size of the Inventory Response to be received in Msg3. An Inventory Response from the device should include the device’s upper layer Device ID, which may have a variable length unknown to the reader, unless explicitly indicated to the reader by the AIOTF. Fortunately, according to the latest TR 23.700-13 [2], AIOTF has the visibility to the type and structure of the Device ID being used for paging the devices. Therefore, the AIOTF should be able to indicate, to the reader, the size of the expected Inventory Response from the device, even if the size information of the expected D2R response is not provided by the AF. And if a device cannot transmit its Msg3 in one shot due to lack of energy, it should not attempt to send its Msg3 in the first place, because it will take a long time (up to tens of seconds) for the device to be re-charged sufficiently, by which time the random ID of the device is no longer valid such that sending the remaining segment of its previous Inventory Response becomes meaningless. The device may simply attempt a re-access after it is re-charged sufficiently.

Proposal 7. In the D2R direction, segmentation is not supported on Msg1 nor Msg3.

We further propose that segmentation is not supported by A-IoT MAC layer for all R2D transmissions to avoid requiring A-IoT devices to support reassembly at the A-IoT MAC layer. However, if at the A-IoT NAS layer, the AIOTF is able to divide a Command Request of writing a large data into multiple Command Requests of writing smaller data with the memory address(es) of each smaller data being provided in the respective smaller Command Requests, the reader will transmit the multiple smaller Command Requests sequentially and the device will place the multiple received (smaller) data into its memory based on the memory addresses being provided. Whether such A-IoT NAS layer based segmentation for the R2D direction can be supported or not is up to SA2 to decide.

Proposal 8. Segmentation is not supported by A-IoT MAC layer for all R2D transmissions.

If Proposal 8 is agreed, RAN2 can send an LS to SA2 informing the decision and asking whether A-IoT NAS layer based segmentation with memory addresses provided in the Write commands can be supported by AIOTF for Write commands when the data to be written into the device is too large.

Proposal 9. If Proposal 8 is agreed, RAN2 send an LS to SA2 informing the decision and asking whether A-IoT NAS layer based segmentation with memory addresses provided in the Write commands can be supported by AIOTF for Write commands when the data to be written into the device is too large.

## On Message Size/Command Type information

In their reply LS [4], SA2 provided the following answer:

Q2: Can there be a case where a D2R response is not expected for a R2D message for command service type? If so, can the CN indicate to the reader whether a message is expected in D2R direction in response to an R2D message for command service type (e.g., either explicitly or implicitly by indicating the response message size if available)?

SA2 answer: No, a D2R response is always expected for a R2D message for command service type (e.g., the service data response for 'Read' service operation, and the operation result indication response for 'Write' or 'Disable' service operation).

In their LS [5], SA2 provided the following answer:

Q1: Can the CN provide, to the reader, an estimate of the expected size of the following D2R message(s) in response to the service request?

SA2 answer: If provided by AF, AIOTF provides the reader with the approximate D2R message size based on AF request. Whether the CN can determine the approximate D2R message size if not provided by AF is under discussion in SA2.

A-IoT devices are not as versatile as the traditional UEs in that they are made most likely for a specific application. It is reasonable to expect that when the AF initiates an A-IoT service request with a Read command, the AF knows the size of the information to be read from the device. Therefore, the expected D2R message size for Read command can be provided to the reader.

Even if the AF does not provide the expected D2R response size information to the AIOTF (or the AIOTF does not do so to the reader), for a Write or Disable command, the size of the information indicating the operation result (i.e., the Command Response) from the device may be very limited. In this case, in addition to knowing the service type is command, if the reader further knows the exact command type (i.e., Read vs. Write vs. Disable), the reader may be able to estimate the D2R resource needed for the Command Response with a better accuracy than having no information at all. Therefore, RAN2 can further ask SA2 whether information about the command type can be provided from the AIOTF to the reader in case that information about the approximate D2R message size is not provided by the AF and the AIOTF cannot determine the approximate D2R message size by itself.

Proposal 10. RAN2 further asks SA2 whether information about the command type can be provided from the AIOTF to the reader in case that information about the approximate D2R message size is not provided by the AF and the AIOTF cannot determine the approximate D2R message size by itself. Without the approximate D2R message size information, the reader may use the command type information to estimate the D2R resource needed for the Command Response with a better accuracy than having no information at all.

# Conclusions

We propose the following:

Proposal 1. AS control information is carried in a form of MAC CE so that, as a general format, an A-IoT MAC PDU either contains one or more A-IoT MAC CEs without any A-IoT MAC SDU or contains one A-IoT MAC SDU and zero or more A-IoT MAC CEs.

Proposal 2. RAN2 introduce the concept of A-IoT MAC element, which can be an A-IoT MAC data element (DE) or A-IoT MAC CE, where an A-IoT MAC DE carries an A-IoT MAC SDU and an A-IoT MAC CE does not. Both A-IoT MAC DE and A-IoT MAC CE begins with a D/C (data/control) field, which indicates whether the A-IoT MAC element is an A-IoT MAC DE or an A-IoT MAC CE.

Proposal 3. RAN2 design a format of A-IoT MAC DE for carrying A-IoT MAC SDU in the A-IoT MAC PDU. Options 1 and 2, as described in Figures 3 and 4, respectively, can be considered for down-selection.

Proposal 4. RAN2 design a general format of A-IoT MAC CEs for carrying various types of AS control information in the A-IoT MAC PDU. Options A and B, as described in Figures 5 and 6, respectively, can be considered for down-selection.

Proposal 5. A subsequent paging (or similar R2D trigger) message with a same paging ID can include the 16-bit random ID(s) of the device(s), if any, from which the Msg1 has been received but the Msg3 or subsequent D2R data transmission, which was expected during the previous paging cycle, has not been received successfully by the reader. In response, these devices can make a re-access attempt either during this subsequent paging cycle or a later paging cycle triggered with the same paging ID.

Proposal 6. Devices, which had transmitted during the previous paging cycle and the 16-bit random ID of which is not indicated in this subsequent paging (or similar R2D trigger) message considers its previous D2R transmission to the reader has been successful and therefore will not respond to any subsequent paging (or similar R2D trigger) message with the same paging ID from the reader.

Proposal 7. In the D2R direction, segmentation is not supported on Msg1 nor Msg3.

Proposal 8. Segmentation is not supported by A-IoT MAC layer for all R2D transmissions.

Proposal 9. If Proposal 8 is agreed, RAN2 send an LS to SA2 informing the decision and asking whether A-IoT NAS layer based segmentation with memory addresses provided in the Write commands can be supported by AIOTF for Write commands when the data to be written into the device is too large.

Proposal 10. RAN2 further asks SA2 whether information about the command type can be provided from the AIOTF to the reader in case that information about the approximate D2R message size is not provided by the AF and the AIOTF cannot determine the approximate D2R message size by itself. Without the approximate D2R message size information, the reader may use the command type information to estimate the D2R resource needed for the Command Response with a better accuracy than having no information at all.

# References

RP-243326, A-IoT WID, RAN1 Vice-chair (Huawei).

TR 23.700-13 V1.3.0.

TR 38.769 V19.0.0.

R2-2500063_S2-2501241, Reply LS on Assistance Information from CN to the Reader, SA2.

R2-2500052_S2-2413035, LS on A-IoT Conclusions in SA WG2, SA2.