3GPP TSG-RAN WG2 Meeting #128	R2-2501257

Athens, Greece, Feb. 17th – 21st, 2025

Source:                Qualcomm Incorporated

Title:	MRO enhancement for SON and MDT

Agenda Item:	8.10.4

Document for:	Discussion and Decision

# Introduction & Background

This contribution discusses NTN SON RAN2 issues based on RAN3 LS [1].

# Discussion

RAN3 sends LS[1]  to RAN2 regarding SON/MDT for NTN and ask RAN2 to consider the following two issues.

---------------------------------------------------------------------------------------------------------------------------------------

- Regarding SON/MDT for NTN:

On MRO for NTN, RAN3 agreed that UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not.

On MDT for NTN, RAN3 assumes that the geographical area and/or the mapped cell ID can be used as the Area Scope for the logged MDT for NTN over NGAP. Further, RAN3 would like to confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for Area Scope of logged MDT over Uu for NTN, to help RAN3 decide whether to specify geographical area and/or mapped cell ID over NGAP.

---------------------------------------------------------------------------------------------------------------------------------------

For the first RAN3 agreement, RAN3 asks RAN2 to decide to reuse the existing IE or not for that UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”.

CondTriggerConfig-r16 ::=        SEQUENCE {

condEventId                      CHOICE {

condEventA3                      SEQUENCE {

a3-Offset                        MeasTriggerQuantityOffset,

hysteresis                       Hysteresis,

timeToTrigger                    TimeToTrigger

},

condEventA5                      SEQUENCE {

a5-Threshold1                    MeasTriggerQuantity,

a5-Threshold2                    MeasTriggerQuantity,

hysteresis                       Hysteresis,

timeToTrigger                    TimeToTrigger

},

...,

condEventA4-r17                  SEQUENCE {

a4-Threshold-r17                 MeasTriggerQuantity,

hysteresis-r17                   Hysteresis,

timeToTrigger-r17                TimeToTrigger

},

condEventD1-r17                  SEQUENCE {

distanceThreshFromReference1-r17 INTEGER(0.. 65525),

distanceThreshFromReference2-r17 INTEGER(0.. 65525),

referenceLocation1-r17           ReferenceLocation-r17,

referenceLocation2-r17           ReferenceLocation-r17,

hysteresisLocation-r17           HysteresisLocation-r17,

timeToTrigger-r17                TimeToTrigger

},

condEventT1-r17                  SEQUENCE {

t1-Threshold-r17                 INTEGER (0..549755813887),

duration-r17                     INTEGER (1..6000)

},

condEventD2-r18                  SEQUENCE {

distanceThreshFromReference1-r18 INTEGER(0.. 65535),

distanceThreshFromReference2-r18 INTEGER(0.. 65535),

hysteresisLocation-r18           HysteresisLocation-r17,

timeToTrigger-r18                TimeToTrigger

}

MeasResultNR ::=                        SEQUENCE {

physCellId                              PhysCellId                                                                  OPTIONAL,

measResult                              SEQUENCE {

cellResults                             SEQUENCE{

resultsSSB-Cell                         MeasQuantityResults                                                 OPTIONAL,

resultsCSI-RS-Cell                      MeasQuantityResults                                                 OPTIONAL

},

rsIndexResults                          SEQUENCE{

resultsSSB-Indexes                      ResultsPerSSB-IndexList                                             OPTIONAL,

resultsCSI-RS-Indexes                   ResultsPerCSI-RS-IndexList                                          OPTIONAL

}                                                                                                               OPTIONAL

},

...,

[[

cgi-Info                                CGI-InfoNR                                                                  OPTIONAL

]],

[[

choCandidate-r17                        ENUMERATED {true}                                                           OPTIONAL,

choConfig-r17                           SEQUENCE (SIZE (1..2)) OF CondTriggerConfig-r16                             OPTIONAL,

triggeredEvent-r17                      SEQUENCE {

timeBetweenEvents-r17                   TimeBetweenEvent-r17                                                    OPTIONAL,

firstTriggeredEvent-r17                 ENUMERATED {condFirstEvent, condSecondEvent}                            OPTIONAL

}                                                                                                               OPTIONAL

]],

[[

firstEntering-r18                       ENUMERATED {true}                                                           OPTIONAL

]]

}

Currently, condition trigger configuration already includes location, time, measurement based events as follow highlighted. And UE will indicate which trigger event is the first triggered event, as highlighted. Then based on existing IEs, gNB should know which CHO trigger condition is fulfilled. Then existing IE should be enough to reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”.

- Existing IEs for CHO should be enough to reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”.

For the second RAN3 agreement, currently MBS NTN is being discussed following agreements, the MBS service area can be presented by geographical area.

Agreements:

1.	The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle.

2.	The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon.

3.	The IntendedServiceArea is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed)

4.	A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s).

5.	RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates.

6.	Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way)

7.	The legacy SIB modification procedure is applied to update the intended service area information in the new SIB.

Since NTN MDT area So it should be assumed that the geographic area defined for MBS NTN can be used for MDT for NTN. But this needs to be further confirmed if any changes for MBS NTN. The MBS NTN area is defined for each MBS service, and the geographic area for logged MDT in case of NTN could be for non-MBS service, and can be different from MBS NTN area. Therefore, a separate geographic area should be introduced for NTN MDT.

- RAN2 assumes a separate parameters with the signaling structure agreed for encoding geographical area in case of MBS NTN should be introduced for geographical area based logged MDT for NTN. RAN2 should discuss whether to support reference location/radius or polygon-based or both.

# Conclusion

- Existing IEs for CHO should be enough to reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”.

- RAN2 assumes a separate parameters with the signaling structure agreed for encoding geographical area in case of MBS NTN should be introduced for geographical area based logged MDT for NTN. RAN2 should discuss whether to support reference location/radius or polygon-based or both.

# Reference

[1] R2-2500026	LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT (R3-247908; contact: Samsung)	RAN3