3GPP TSG-RAN WG2 Meeting #129	R2-2501275

Athens, Greece, 17th – 21st February, 2025

Agenda item:	8.9.4

Source:	Samsung

Title:	Impact of introducing PWS broadcasting for NB-IoT

WID/SID:	IoT_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

In RAN#106 the latest WID for IoT NTN Phase 3 is provided [1]:

The aim of this WI is enhancement of IoT-NTN with the following objectives: - Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - Specify necessary enhancements for full eNB as regenerative payload e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4] - Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In this contribution, we provide some discussions related to PWS broadcasting for NB-IoT.

# Discussion

- RAN2#127bis agreements on PWS:

Agreements:

1.	RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS

2.	The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes.

3.	For NB-IoT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/ skip some unnecessary fields):

- SystemInformationBlockType10-NB for primary ETWS notification;

- SystemInformationBlockType11-NB for secondary ETWS notification;

- SystemInformationBlockType12-NB for CMAS notification.

4.	Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB):

- etws-Indication;

- cmas-Indication.

5.	Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (can come back to clarify further what immediately means)

6.	Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases)

- RAN2#128 agreements on PWS:

Agreements:

1.	We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements.

2.	We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed

3.	Add the following PWS indication in Paging-NB:

- etws-Indication;

- cmas-Indication.

## Connected mode indication

- In last meeting, an LS was sent to RAN1 regarding feasibility of UE monitoring paging for PWS signaling in RRC Connected for NB-IoT. This is expected to be treated in RAN1#120, but it could be useful for RAN2 to still discuss different options if RAN1 does not think that it is feasible to introduce monitoring for Paging in connected mode. One issue with monitoring for Paging in connected mode is that it increases complexity of UEs.

- The key of monitoring Paging for PWS signalling is to be an efficient manner to reach all UEs in a cell with a single type of signal. In normal LTE, the UE also monitors for P-RNTI in connected mode. In eMTC, the UE however monitors a separate RNTI, the SI-RNTI. For eMTC, UE complexity is introduced for the sake of efficient network signalling, which is however not as efficient as only monitoring P-RNTI.

- The other key part of Paging for PWS signalling is that it is just a single-bit indicator which indicates that PWS signalling is taking place and that the UE shall acquire the PWS signalling. If instead network-signalling efficiency is traded off for UE simplicity, then a dedicated indication to UEs in connected mode may be used. For instance if a new MAC CE is used to indicate PWS signalling, then the added UE complexity of a network indicating PWS is very small. The tradeoff is that the network has to indicate this to every UE that is in connected mode. This is likely a reasonable tradeoff for NB-IoT as it introduces the likeliness of the feature being implemented and thus used.

Proposal 1: Depending RAN1 reply, RAN2 to consider dedicated indication of PWS broadcasting to NB-IoT UE in connected mode, for instance via a new MAC CE.

## Idle mode operation for PWS broadcasting

- In eMTC and normal LTE, there are different types categories of cells that the UE may camp on. These are acceptable cells, suitable cells, barred cells and reserved cells. In most cases, a UE may only camp on acceptable and suitable cells. Suitable cell criteria can be simplified as 1) the cell being part of a PLMN the UE is allowed to use, 2) that cell is not barred and 3) cell selection criteria are fulfilled. The acceptable cell criteria is that the 1) cell is not barred and 2) it fulfills the cell selection criteria. The acceptable cell criteria is introduced to allow a UE to camp on a cell which is not part of its PLMNs, in order to perform emergency calls and to receive PWS broadcasting.

Observation 1: In eMTC and normal LTE, a UE can camp on an acceptable cell to receive PWS broadcasting.

- For NB-IoT, acceptable cell category were never introduced. The reason for this is that an NB-IoT UE was not able to initiate emergency calls and until this release, was not able to receive PWS broadcasting. This means that there was no use of a UE camping on a cell being a part of PLMN the UE is not allowed to use. This also means that the UE can only camp on cells which are part of PLMNs the UE is allowed to use. With PWS broadcasting being introduced to NB-IoT, there may be a need to introduce the ability of an NB-IoT UE to camp on a cell only for the purpose of receiving PWS broadcasting. One such very important use case, is to allow for UEs with NB-IoT NTN chipsets to camp on an NB-IoT NTN cell in emergency cases to receive important ETWS or CMAS signalling. Otherwise, some UEs will not be allowed to camp on the cells and thus be unable to receive the PWS signalling.

Observation 2: With current NB-IoT idle mode procedures, some users will not be able to receive PWS signalling.

Proposal 2: A UE shall be able to camp on an NB-IoT NTN cell for the purpose of receiving emergency broadcast, without the cell being a part of UEs PLMNs.

Proposal 3: RAN2 to discuss introducing acceptable cell category for NB-IoT, or other solutions.

# Conclusion

In this contribution, we discussed issues related to PWS signaling for NB-IoT.

Observation 1: In eMTC and normal LTE, a UE can camp on an acceptable cell to receive PWS broadcasting.

Observation 2: With current NB-IoT idle mode procedures, some UEs will not be able to receive PWS signalling.

Proposal 1: Depending RAN1 reply, RAN2 to consider dedicated indication of PWS broadcasting to NB-IoT UE in connected mode, for instance via a new MAC CE.

Proposal 2: A UE shall be able to camp on an NB-IoT NTN cell for the purpose of receiving emergency broadcast, without the cell being a part of UEs PLMNs.

Proposal 3: RAN2 to discuss introducing acceptable cell category for NB-IoT, or other solutions.

# Reference

RP-243278, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, Mediatek, RAN#106, Madrid, Spain, December 2024.

3GPP TS 23.401 “General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access”, Release 19, V19.0.0, September 2024.