3GPP TSG-RAN WG2 Meeting #128

Athens, Greece, 17 – 21 February 2025	R2-2501283

Agenda item:	8.7.4.1

Source:	Rakuten Mobile

Title:	Discussion on remaining issues related to LCP enhancements

WID/SID:	NR_XR_Ph3

Document for:	Discussion / Decision

# Introduction

In previous RAN2 discussions, the application of additional priority for LCH has been actively discussed, leading to

agreements that additional LCH priority should be applied in both the first and second rounds of the LCP procedure.

RAN127: Agreements 1. As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. 2. FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation. 3. Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. 4. We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold. RAN128: Agreements 1. As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. 2. As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

However, several open issues remain regarding:

- Configuration details for additional priority handling.

- Impact of Bj (bucket size) and PBR (Prioritized Bit Rate) on LCP performance.

- Interaction with intra-UE prioritization mechanisms, including uplink grant allocation, retransmissions, and Scheduling Request (SR) prioritization.

Logical Channel Prioritization (LCP) has been extensively discussed in RAN2, with consensus on the application of additional LCH priority across both the first and second rounds of the LCP procedure, alongside an optional fallback capability for UEs to return to default priority if needed​ RP-242517 SR for XR WI 2. However, while these agreements set a foundation, they leave many details unresolved, particularly in terms of how additional priority should be configured, applied, and dynamically adjusted. Without standardized configuration parameters, we risk inconsistent vendor implementations, unpredictable scheduling behavior, and fairness concerns across different network deployments.

Operators require fine-grained control over LCP priority behavior, ensuring that additional priority is applied in a way that aligns with QoS objectives while maintaining scheduling fairness. The ability to configure priority persistence, manage interactions with scheduling constraints such as Bj and PBR, and define network-level policies for LCP adjustments is essential for ensuring interoperability across implementations.

This contribution presents a structured configuration framework that addresses these gaps by defining explicit parameters, ensuring that additional priority handling remains predictable, scalable, and adaptable to real-world deployment scenarios.

# Discussions

## Configuration Details for Additional Priority Handling in

In previous RAN2 discussions, companies have raised concerns regarding how additional LCH priority should be applied in practice. While the agreement to support additional priority in both rounds of LCP provide a baseline behavior, the lack of specific configuration parameters means that different vendors may implement this in different ways, leading to interoperability challenges.

One major concern is whether additional priority should persist indefinitely or be dynamically adapted based on network conditions. Some companies argue that strict persistence ensures consistency, while others advocate for a dynamic approach, where priority is re-evaluated based on buffer status, remaining delay budget, or competing traffic demands. If left undefined, we risk situations where certain UEs aggressively retain additional priority, potentially leading to LCH starvation for other services.

Another area of debate is who controls priority adaptation—the UE or the network. Some believe UE-driven adjustments allow for localized optimizations, reducing signaling overhead, while others stress that gNB-controlled priority scaling ensures network-wide consistency. There is also the question of interactions with existing scheduling constraints, such as PBR and Bj. Additional priority may allow a UE to send more data than originally allocated, which could impact scheduling fairness if not properly managed.

From an implementation perspective, it is clear that a uniform approach is needed—one that allows the network to configure additional priority handling explicitly, ensuring consistent behavior across all UEs while retaining enough flexibility to adapt to different deployment conditions.Observation 1: The absence of standardized configuration parameters for additional LCH priority handling could lead to inconsistent implementations across vendors, making scheduling behavior unpredictable and difficult to optimize for network operators.

- Introduce explicit configuration parameters to define how additional priority is applied, persisted, or revoked across different scheduling conditions.

- Ensure that these parameters are configurable via RRC signaling, allowing networks to dynamically control additional LCH priority handling based on real-time traffic demands.

## Priority Adaptation and Persistence Considerations

While the agreement to apply additional priority across both LCP rounds provides a clear baseline, it does not specify when and how priority should be revoked or adjusted dynamically. This lack of clarity has raised concerns among companies regarding fairness and scheduling predictability. Some companies argue that if an LCH retains additional priority beyond what is strictly necessary, other traffic flows may be delayed unnecessarily, leading to unintended resource monopolization.

A possible solution is to introduce a time-based threshold, where priority is gradually reduced after a certain duration, allowing other LCHs to compete for resources. Another approach is to tie priority retention to buffer status, where priority persists only if delay-critical data remains in the buffer. Without such controls, we risk situations where some UEs retain priority for too long, while others drop priority prematurely, affecting QoS consistency across different devices.

An additional complexity arises in scenarios where multiple LCHs have additional priority. Some companies suggest a rotational fairness mechanism, where priority shifts between competing LCHs over multiple scheduling intervals, preventing one LCH from monopolizing uplink resources indefinitely.

- Without clear rules on priority persistence, some UEs may retain additional priority for longer than necessary, while others may drop it prematurely, leading to inconsistent QoS behavior across devices.

- Introduce a priorityPersistenceMode parameter to define whether priority is STATIC (always retained) or DYNAMIC (adjusted based on network conditions).

- Define a timeRemainingThreshold parameter to determine when additional priority should expire, ensuring that LCHs do not retain priority longer than needed.

- Implement a fairness rotation mechanism for cases where multiple LCHs have additional priority, ensuring equitable scheduling access.

## Impact on the existing LCP procedure

During previous RAN2 discussions, concerns were raised regarding how additional LCH priority might impact Bj and PBR constraints, particularly in networks with high congestion or competing traffic flows. One major concern is that applying additional priority could override Bj or PBR limits, leading to unintended resource monopolization by certain LCHs. This could cause best-effort or lower-priority traffic to be deprioritized indefinitely, creating fairness issues across UEs.

Another issue is that BSR-based scheduling may not accurately reflect the impact of additional priority adjustments. Today, BSR reports buffer availability to the gNB, but additional priority can alter how much data is actually transmitted, potentially misaligning grant allocation decisions. If the network expects a certain data volume to be transmitted based on BSR, but additional priority allows more data to be scheduled than expected, this can create inefficiencies in grant assignment and overall resource utilization.

Some companies argue that PBR and Bj should remain strictly enforced, ensuring that additional priority does not introduce uncontrolled scheduling deviations. Others suggest that PBR constraints should be relaxed dynamically for delay-sensitive LCHs, allowing for short-term priority boosts while still maintaining long-term fairness. There is also debate on whether Bj should be extended dynamically for priority-extended LCHs, ensuring that priority-extended packets are not prematurely discarded due to Bj expiration.

Given these challenges, a structured approach is needed to ensure that additional priority handling does not conflict with Bj, PBR, or BSR-based scheduling.

- Additional priority handling introduces potential conflicts with Bj and PBR limits, potentially allowing certain LCHs to exceed their originally allocated resources, which could impact fairness and lower-priority traffic scheduling.

- Introduce a priorityBypassPBR parameter that allows networks to define whether additional priority can bypass PBR constraints under certain conditions.

- Implement a priorityBufferExtensionFactor that enables Bj to extend dynamically for LCHs with additional priority, ensuring delay-critical data is not prematurely discarded.

## Ensuring Compatibility Between Additional Priority and   PBR/Bj Constraints

PBR and Bj are critical mechanisms in uplink scheduling that regulate transmission rate limits and buffer retention policies, ensuring fair and efficient resource distribution among logical channels (LCHs). The introduction of additional priority handling raises concerns about how these mechanisms should interact with priority-extended LCHs, particularly in scenarios where multiple high-priority LCHs compete for scheduling opportunities.

PBR enforces rate-based transmission limits, preventing any single LCH from dominating uplink resource allocation. Some companies advocate for strict PBR enforcement, ensuring that additional priority does not override existing rate controls. Others argue that delay-sensitive LCHs should be allowed temporary PBR bypassing, ensuring that time-sensitive data is not constrained by rigid rate limits. A potential compromise is to introduce a dynamic scaling factor, where priority-extended LCHs can temporarily exceed PBR limits within a defined threshold, balancing fairness and scheduling efficiency.

Similarly, Bj defines how long buffered data remains eligible for transmission before being discarded. When additional priority is applied, certain LCHs may transmit data later than originally scheduled, increasing the risk that Bj expires before transmission occurs, leading to unexpected data loss. Some companies propose dynamically adjusting Bj for LCHs with additional priority, allowing priority-extended data to remain in the buffer longer. Others argue that Bj should remain fixed, ensuring that additional priority does not disrupt buffer management policies.

A further concern is Bj exhaustion in networks with multiple priority-extended LCHs. If too many LCHs receive extended Bj, this could reduce scheduling efficiency, making it difficult for lower-priority LCHs to compete for resources, ultimately impacting overall uplink performance.

- If additional priority is applied without considering PBR, certain LCHs may consume more uplink resources than originally intended, leading to potential scheduling fairness issues.

- Define a PBRScalingFactor that allows temporary PBR increases for priority-extended LCHs, ensuring that delay-sensitive traffic is not unnecessarily restricted.

- Without adjustments, additional priority handling may cause Bj expiration issues, leading to premature data loss despite the LCH being prioritized for transmission.

- Introduce a BjScalingFactor that dynamically extends Bj for LCHs with additional priority, ensuring that priority-extended data is not discarded before it can be scheduled.

- Allow networks to define a maximum Bj extension limit, preventing Bj from extending indefinitely and ensuring scheduling fairness across multiple LCHs.

## Ensuring Fairness Between High-Priority and Lower-Priority LCHs

While additional priority is intended to enhance scheduling for delay-sensitive traffic, it is equally important to prevent it from completely starving lower-priority LCHs. Some companies express concerns that once an LCH receives additional priority, it may be scheduled indefinitely, while other traffic—such as uplink retransmissions, lower-priority QoS flows, and best-effort data—may be delayed indefinitely.

One way to mitigate this issue is by introducing a fairness threshold, which would limit how many consecutive TTIs an LCH can retain additional priority before allowing lower-priority LCHs to be scheduled. This would ensure that even when additional priority is applied, lower-priority traffic is not completely blocked from uplink scheduling.

Another approach is to introduce a traffic-awareness mechanism, where the UE automatically scales down additional priority when lower-priority LCHs have accumulated a certain amount of backlogged data. This would prevent situations where best-effort traffic remains unscheduled for extended periods, ensuring better overall scheduling balance.

- Additional priority handling must include fairness controls to ensure that lower-priority LCHs are not indefinitely deprioritized, preventing scheduling imbalance within the UE.

- Introduce a fairness threshold that limits the number of consecutive TTIs an LCH can retain additional priority before other LCHs are scheduled.

- Implement a traffic-awareness mechanism that scales down additional priority when lower-priority LCHs accumulate significant buffered data, preventing best-effort traffic from being completely blocked.

# Conclusions

This paper provides our views on whether LCH priority

Configuration Details for Additional Priority Handling in

- Introduce explicit configuration parameters to define how additional priority is applied, persisted, or revoked across different scheduling conditions.

- Ensure that these parameters are configurable via RRC signaling, allowing networks to dynamically control additional LCH priority handling based on real-time traffic demands.

## Priority Adaptation and Persistence Considerations

- Without clear rules on priority persistence, some UEs may retain additional priority for longer than necessary, while others may drop it prematurely, leading to inconsistent QoS behavior across devices.

- Introduce a priorityPersistenceMode parameter to define whether priority is STATIC (always retained) or DYNAMIC (adjusted based on network conditions).

- Define a timeRemainingThreshold parameter to determine when additional priority should expire, ensuring that LCHs do not retain priority longer than needed.

- Implement a fairness rotation mechanism for cases where multiple LCHs have additional priority, ensuring equitable scheduling access.

## Impact on the existing LCP procedure

- Additional priority handling introduces potential conflicts with Bj and PBR limits, potentially allowing certain LCHs to exceed their originally allocated resources, which could impact fairness and lower-priority traffic scheduling.

- Introduce a priorityBypassPBR parameter that allows networks to define whether additional priority can bypass PBR constraints under certain conditions.

- Implement a priorityBufferExtensionFactor that enables Bj to extend dynamically for LCHs with additional priority, ensuring delay-critical data is not prematurely discarded.

Ensuring Compatibility Between Additional Priority and   PBR/Bj Constraints

- If additional priority is applied without considering PBR, certain LCHs may consume more uplink resources than originally intended, leading to potential scheduling fairness issues.

- Introduce a BSR Alignment Mechanism, ensuring that BSR accurately reflects additional priority-based Bj/PBR changes.

- Define a PBRScalingFactor that allows temporary PBR increases for priority-extended LCHs, ensuring that delay-sensitive traffic is not unnecessarily restricted.

- Without adjustments, additional priority handling may cause Bj expiration issues, leading to premature data loss despite the LCH being prioritized for transmission.

- Introduce a BjScalingFactor that dynamically extends Bj for LCHs with additional priority, ensuring that priority-extended data is not discarded before it can be scheduled.

- Allow networks to define a maximum Bj extension limit, preventing Bj from extending indefinitely and ensuring scheduling fairness across multiple LCHs.

Ensuring Fairness Between High-Priority and Lower-Priority LCHs

- Additional priority handling must include fairness controls to ensure that lower-priority LCHs are not indefinitely deprioritized, preventing scheduling imbalance within the UE.

- Introduce a fairness threshold that limits the number of consecutive TTIs an LCH can retain additional priority before other LCHs are scheduled.

- Implement a traffic-awareness mechanism that scales down additional priority when lower-priority LCHs accumulate significant buffered data, preventing best-effort traffic from being completely blocked.

# References