3GPP TSG-RAN WG2 #129	R2-2501286

Athens, Greece, 17th – 21st February, 2025

Agenda Item:	8.1.3

Source:	Ericsson

Title:	NW-side data collection for beam management and positioning

Document for:	Discussion

# 1	Introduction

The discussion on the topic of NW-side data collection has progressed during the last RAN2 meetings. The discussion has focused on different technical aspects, such as when the UE should start/stop the NW-side data collection, how the UE should report the logged data, etc.

# 2	Discussion on NW-side data collection for beam management

In the following, we further develop the expected technical impacts related to this topic, taking into account the agreements reached in the previous meetings and the FFS.

## 2.1	Configuration aspects and content of the data collection report

In RAN2#127, it was agreed that when the network wants to perform NW-side data collection, it will transmit to the UE a measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. For the case of beam management, the NW may configure the UE with a specific set of CSI-RS/SSB resources specifically configured for training purposes. On such resources, the UE will then start performing layer-1 measurements and log the measurement results and any possible useful information associated to it. 
Going into more details on the necessary configuration, we know that from the point of view of the gNB, this set of CSI-RS/SSB resources used for training purposes may correspond to the set A and set B. However, unlike the case of UE-side data collection, the UE does not need to know that a certain resource set is a Set A or B, because that will be left to the NW implementation, and the UE just has to perform the same type of measurements in those sets of resources. The UE only needs to know that the purpose of this set of resources is to perform measurements for the purpose of NW-side data collection, rather than for conventional operations. 
Hence, from the configuration point of view, the radio resource configuration for the purpose of NW-side data collection consists of a list of logging configurations (e.g. each logging configuration can be associated to a periodic or event-based logging conditions), each associated to a CSI resource configuration (i.e. to one CSI-ResourceConfig IE) comprising one or more CSI-RS/SSB resource sets in which the UE performs the radio measurements.

RAN2 assumes that the radio resource configuration for the purpose of NW-side data collection consists of a list of logging configurations each associated to a CSI resource configuration (i.e. to a CSI-ResourceConfig IE) on which the UE shall perform radio measurements and the related logging.

Related to the content of the data collection report, in RAN2#127-bis, it was agreed that “at least L1-RSRPs and/or beam-IDs needs to be collected by UE”. In our view, along with such information, also the timestamp of the measurement might be beneficial, especially for the case of event-based measurements. The timestamp of a logged measurement is already included in the logged MDT report, as a tool to assist the network in retrieving NW operating conditions at the moment in which the measurement was taken by the UE. In the same way, in the context of AIML, a timestamp would allow the network to correlate a given UE radio measurement with the NW conditions/configurations experienced at the network, which is critical to ensure a proper AIML model setup.

Along with the L1-RSRP and beams IDs, the UE includes the timestamp related to the point in time in which a reported measurement was taken.

### 2.1.1	Inputs to RAN1

The necessary signalling to configure the UE to perform the NW-side data collection will be obviously captured in the RRC ASN.1. However, the procedures related to how the UE should perform the L1 measurements and the storing/logging of such measurements should be captured in RAN1 specifications. This is because, in legacy all the procedures on how the UE performs the channel measurements on configured CSI-RS/SSB resources and transmits the related reports are captured in TS 38.214. Hence, it should be up to RAN1 to evaluate the impact on their specifications. RRC specification will only be in charge of the signalling, and on the procedures to transmit via RRC the logged measurements.

L1 procedures to perform radio measurements and logging for the NW-side data collection purpose are not captured in RAN2 specification, i.e. RAN1 to evaluate impact in TS 38.214. RAN2 will be only in charge of designing the configuration signalling and the procedures for transmitting via RRC the logged measurements.

Since all the above assumptions captured in Proposal 1, Proposal 2, Proposal 3 are of RAN1 interest, we suggest RAN2 informing RAN1 on that.

RAN2 to inform RAN1 about any agreements related to Proposal 1, Proposal 2, Proposal 3.

## 2.2	When the UE should perform/stop the radio measurements for NW-side data collection

In RAN2#127-bis, it was agreed that both periodic logging and event-triggered data logging will be supported.

For the case of periodic logging, in our view, this means that the UE will start the data logging upon receiving the data collection configuration from the network and when the corresponding periodic CSI resources are received for the first time.

The UE starts the data collection upon receiving a L3 logging configuration for NW-side data collection for periodic logging and when the corresponding periodic CSI resources are received for the first time.

Related to the logging periodicity, we believe that it should be controllable by the network, in order to reduce the amount of data to be logged and the impact on the UE. For example, the CSI-RS/SSB periodicity could be 80ms as per the CSI resource configuration, but for periodic logging for NW-side data collection, it might be enough to just perform measurements (and hence logging) every 640ms. And for example, if a certain event is triggered, the periodicity of logging can be increased to e.g. 160ms, in order to provide more granular results to the network.

The logging periodicity/interval for both periodic and event-triggered data collection is configured in the L3 logging configuration.

For the stopping of data collection, it was agreed in RAN2#127 that the data collection can be stopped once the UE faces issues such as becoming power limited. However, in general, the data collection, once started, can continue for a certain configurable time (as in logged MDT), or it can continue until certain events are fulfilled, for the case of event-triggered data collection. For periodic data collection, network can de-configure the UE to stop collecting measurement results.

Besides the case of memory limitation issues (already agreed in RAN2#127), the UE may stop the event triggered NW-side data collection procedure when:

  - data collection has been done for a certain configurable logging duration

  - the event that triggered the data collection is no longer fulfilled.

Related to the event-triggered data collection, the ongoing email discussion encompasses two different options; namely, 1) L3 based event, 2) L1 beam measurement-based event. In L3 based event, the discussion revolves around defining a L3 event similar to A1/A2 event upon which the UE would start data collection. However, it should be noted that A1/A2 are cell level events that are calculated by the UE using all available beams in the cell according to the RRM framework. This type of event can be useful; however, it should be noted that the benefit of event-based mechanisms is to allow the UE to start measuring certain beams only when the UE is in certain regions of the cell. For example, the network may configure the UE to start performing measurements on certain narrow beams (CSI-RSs) when the UE is in the coverage of one or more wide beams (SSBs), i.e. when an event related to the coverage status of such wide beams is fulfilled. Hence, designing an event based on the whole cell level measurements may not be always useful to reduce the overhead at the UE, because the UE may then need to start measuring a much larger set of narrow beams unnecessarily.

The benefit of event-based data collection is to allow the UE to start performing measurements on certain beams (e.g. narrow beams) only when the UE is in certain regions (e.g. under the coverage of one or more wide beams), thereby limiting the UE power consumption. Events based on cell level measurements are not useful to address this objective.

Additionally, it is in general preferrable to design events that can be easily used as input during the inference. For the BM use case, the inference is performed at the DU, because all the L1 measurements are received via UCI at the DU. However, if we define events based on cell level measurements, this means that RRM measurement reports received by the CU should be considered for the BM inference at the DU. This would complicate the handling of the inference at the gNB.

If we define events for the data logging exclusively based on cell level measurements, it means that during the inference at the gNB, the cell level measurement reports (RRM reports) received by the CU should be taken into account for the BM inference at the DU.

Given the above considerations, we are fine to specify events based on cell level measurements. However, events based on L1-RSRP should be considered because that better reflects what the DU “sees” during the inference, and also it further limits the UE overhead (as outlined above). To limit the fluctuations of the L1-RSRP, L3 filtering parameters, such as the TimeToTrigger, can be used.

RAN2 to support the following events for the logging of NW-side data:

  - L3 serving cell measurements becoming worse/better than a threshold

  - Filtered L1 measurements associated to one or more CSI-RS/SSB resources of serving cell becomes worse/better than threshold.

## 2.3	Design of the UEInformationResponse/Request framework for AIML

In RAN2#127-bis, it was agreed that the UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection, and that an availability indication will be used by the UE to signal the availability of collected data. 
The first issue to discuss is which SRB to use for the transmission of the UEInformationRequest/UEInformationResponse. The UEInformationRequest is already mapped to SRB1, so in our view SRB1 can be reused. The UEInformationResponse is instead mapped to SRB1 or SRB2 (when logged measurement information is included).

In legacy, the UEInformationRequest is mapped to SRB1, and the UEInformationResponse is mapped to SRB1 or SRB2 (when logged measurement information is included).

In meeting RAN2#127-bis, it was proposed that for the UEInformationResponse the SRB4 could be used. However, in our view this proposal is not very well motivated given that SRB4 is today used for QoE which however uses a completely different data transmission mechanisms based on RRC segmentation, and not based on the “on-demand” framework, that instead RAN2 has decided to adopt for the reporting of AIML collected data. On the other hand, SRB2 is used for the logged MDT that, as said, resembles very much the AIML use cases, both on the signalling mechanism (both based on “on-demand”) and on the prioritization (both logged MDT and AIML-related collected data can be assumed to have the same priority). 
Hence, our preference is to reuse the SRB2, or a new SRB that can be assumed to be used for all the AIML possible use cases from now on.

The UEInformationRequest requesting AIML data is transmitted via SRB1 (as any other data request contained in such message), and the UEInformationResponse is transmitted via SRB2, or via a new defined SRB (to be used for all AIML use cases).

### 2.3.1	Signalling of availability indication

Another FFS captured during the last meeting is the content of the UEInformationRequest/UEInformationResponse messages and the details of the availability indication. 
We first note that the UEInformationRequest/UEInformationResponse framework based on availability indication, is used also for retrieving SON and MDT-related reports. Hence, it is important to distinguish the signalling of the AIML-related use cases from the legacy use cases. This means that the availability indication should at least contain a reference to the type of data that are available for transmission, e.g. “NW-side collected data”, and in turn, the existing UEInformationRequest should be extended to include at least an indicator indicating that the gNB is requesting “NW-side collected data”.

The availability indication transmitted by the UE should contain at least a reference to the type of data that are available for transmission (e.g. “AIML NW-side collected data”), and the UEInformationRequest should contain at least a reference to the type of data that are requested by the gNB (e.g. “AIML NW-side collected data”).

It was agreed that UE indicates to the network upon low power state and when the memory is low. In legacy, the data availability indication is transmitted in any RRC complete message, such as RRCResumeComplete, RRCReconfigurationComplete, RRCReestablishmentComplete message. However, the classical SON/MDT reports are designed to be transmitted in gNBs different than the gNB in which the measurement was configured (for the case of logged MDT), or different from the gNB in which a certain event (such as RLF) occurred (for the case of SON reports, such RLF-Reports, RA-Reports, etc). For the BM use cases however we have not agreed yet on whether inter-gNB signalling for the transmission of AIML collected data is supported. Hence, as baseline, RAN2 needs to assume that the collected data should be at least transmitted to the gNB that configured the AIML NW-side data collection. This means that we cannot limit the transmission of the availability indication exclusively to the RRC complete messages that may be transmitted just sporadically when connected to a certain gNB. For this reason, we consider that as baseline the availability indication should be transmitted via UEAssistanceInformation. For example, upon starting the data collection when a logging event is fulfilled the UE can transmit the UEAssistanceInformation as soon as data becomes available for transmission.

As baseline, the data availability indication is transmitted at least in the UEAssistanceInformation message when there are data available for transmission. Whether it can be transmitted also in RRC complete messages (as in SON/MDT) depends on the handling of collected data during mobility.

In RAN2#127, it was agreed that no standardized RRC segmentation procedure is needed, as for the logged MDT measurements. The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded, and then transmit this selected list of measurements in the UEInformationResponse. In particular, as in logged MDT, the UEInformationResponse can include the logged entries in chronological order, and optionally the availability indication related to further data to transmit still in the memory. This framework is already present in the context of SON/MDT, and it can just be adopted for this new use case.

As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT.

## 2.4	UE memory capabilities

In RAN2#127, we have agreed that the UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size. Before discussing the minimum value of the memory size, we propose waiting for more RAN1 inputs related to the data that the UE could log and report to the gNB, so that RAN2 can do a more thorough analysis of such minimum requirement. Additionally, in our view if the UE supports more memory than this minimum memory size, that should be indicated by the UE. 
In our view, the memory allocated for the data collection purposes can be shared for the UE- and NW-side data collection. This is to simplify the capability reporting and UE implementation, so that the network by receiving this capability from the UE can understand that UE- and NW-side data collection is supported.

The UE memory for data collection is shared for NW- and UE-side data collection.

Define in a UE capability some different minimum memory values that a UE can support for the logging of collected data. RAN2 to wait RAN1 inputs on the type of data to be logged by the UE, before discussing the right values.

## 2.5 Handling of collected data during mobility

Several contributions submitted to the last RAN2#127-bis meeting addressed the handling of AIML collected data during mobility. The discussion so far as mainly focused on aspects related to how to configure/start/stop the data collection, and how to report the collected data. It is the natural next step to start discussing what happens at the data collected but not yet transmitted, when certain mobility-related events occur, e.g. a handover is executed, or an RLF/HOF is detected, or an RRC state transition is performed by the UE. In our view, there are mainly two alternatives:

- The UE discards/clears the collected data when performing a mobility procedure

- The UE retains the collected data, and possibly transmit them to a different node than the node configuring the data collection (e.g. the target gNB of the HO, or the gNB to which the UE reconnects/reestablishes after an RLF)

Needless to say that the option 2 above is certainly preferrable from a technical point of view, because it allows the source gNB to retrieve all the collected data, even when mobility events take place before the UE manages to transmit all the collected data. However, the standardization effort of the option 2 should be also assessed in relationship to the time left for this Rel.19 WI. 
For example, if option 2 is pursued, RAN2 should first discuss whether the data retaining should be supported in all mobility cases or only in some cases, e.g. the retaining of the data may be supported only in case of HO, whereas in case of RLF the UE may discard all the data. Then, it should be discussed how the UE is informed on whether it is allowed or not allowed to transmit the AIML collected data to another gNB, e.g the target gNB of the HO. 
Finally, since the node retrieving the data should transmit the collected data back to the source gNB, some work on the Xn interface might be needed, and as such RAN3 involvement would be needed.

Given the above considerations, we propose for simplicity to limit the scope of this discussion for Rel.19.

For Rel.19, as baseline, RAN2 assumes that the UE discards/clears the AIML collected data at mobility events (e.g. handover to a different gNB than the source gNB, RLF/HOF, RRC state transitions).

# 3	Discussion on NW-side data collection for positioning

Network-side data collection for positioning occurs in the following network-based positioning methods:

- Case 3a: NG-RAN node assisted/LMF-based positioning with gNB side model (direct positioning)

- Case 3b: NG-RAN node assisted/LMF-based positioning with LMF-side model (assisted positioning)

Case 3a and Case 3b both are based on UL measurements performed at the gNB and share similar flows as the legacy UL-TDOA positioning method. As such the impact is primarily in RAN3 domain NRPPa.

The main impact in terms of RAN2 is the RRC configuration for SRS which can follow the legacy mechanism. However, one additional aspect to consider is that for AI/ML model both measurements and label are needed. For the training, the necessary information is the gNB measurements (RTOA) and the label/ground truth is UE location or true RTOA. The LMF would have to configure the reporting of UE location coinciding the SRS transmission periodicity. The gNB for the case 3a can also obtain the UE location using RRC; that is there would be no requirement of having an LPP session. This is already possible in legacy as below for MDT.

#### –	CommonLocationInfo

The IE CommonLocationInfo is used to transfer detailed location information available at the UE to correlate measurements and UE position information.

CommonLocationInfo information element

-- ASN1START

-- TAG-COMMONLOCATIONINFO-START

CommonLocationInfo-r16 ::= SEQUENCE {

gnss-TOD-msec-r16          OCTET STRING     OPTIONAL,

locationTimestamp-r16      OCTET STRING     OPTIONAL,

locationCoordinate-r16     OCTET STRING     OPTIONAL,

locationError-r16          OCTET STRING     OPTIONAL,

locationSource-r16         OCTET STRING     OPTIONAL,

velocityEstimate-r16       OCTET STRING     OPTIONAL

}

-- TAG-COMMONLOCATIONINFO-STOP

-- ASN1STOP

CommonLocationInfo field descriptions

gnss-TOD-msec Parameter type gnss-TOD-msec defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationTimeStamp Parameter type DisplacementTimeStamp defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationCoordinate Parameter type LocationCoordinates defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationError Parameter LocationError defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationSource Parameter LocationSource defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

velocityEstimate Parameter type Velocity defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

UE sending ground truth UE location coordinate to gNB via RRC is more efficient compared to sending it first to LMF which further forwarding to gNB, especially when UE is requested to send updated location coordinate when moving.

Case 3a/3b NW side data collection should be driven by RAN3 as it has primarily NRPPa impact. The main impact on RAN2 can be to align the gNB measurements and UE reporting of ground truth during training.

UE sending ground truth UE location coordinate directly to gNB via RRC is an efficient way to support gNB-side data collection.

A window to report UE ground truth coinciding with SRS transmission periodicity is configured.

UE sends ground truth UE location coordinate to gNB via RRC for gNB-side data   collection.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	The benefit of event-based data collection is to allow the UE to start performing measurements on certain beams (e.g. narrow beams) only when the UE is in certain regions (e.g. under the coverage of one or more wide beams), thereby limiting the UE power consumption. Events based on cell level measurements are not useful to address this objective.

Observation 2	If we define events for the data logging exclusively based on cell level measurements, it means that during the inference at the gNB, the cell level measurement reports (RRM reports) received by the CU should be taken into account for the BM inference at the DU.

Observation 3	In legacy, the UEInformationRequest is mapped to SRB1, and the UEInformationResponse is mapped to SRB1 or SRB2 (when logged measurement information is included).

Observation 4	Case 3a/3b NW side data collection should be driven by RAN3 as it has primarily NRPPa impact. The main impact on RAN2 can be to align the gNB measurements and UE reporting of ground truth during training.

Observation 5	UE sending ground truth UE location coordinate directly to gNB via RRC is an efficient way to support gNB-side data collection.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 assumes that the radio resource configuration for the purpose of NW-side data collection consists of a list of logging configurations each associated to a CSI resource configuration (i.e. to a CSI-ResourceConfig IE) on which the UE shall perform radio measurements and the related logging.

Proposal 2	Along with the L1-RSRP and beams IDs, the UE includes the timestamp related to the point in time in which a reported measurement was taken.

Proposal 3	L1 procedures to perform radio measurements and logging for the NW-side data collection purpose are not captured in RAN2 specification, i.e. RAN1 to evaluate impact in TS 38.214. RAN2 will be only in charge of designing the configuration signalling and the procedures for transmitting via RRC the logged measurements.

Proposal 4	RAN2 to inform RAN1 about any agreements related to Proposal 1, Proposal 2, Proposal 3.

Proposal 5	The UE starts the data collection upon receiving a L3 logging configuration for NW-side data collection for periodic logging and when the corresponding periodic CSI resources are received for the first time.

Proposal 6	The logging periodicity/interval for both periodic and event-triggered data collection is configured in the L3 logging configuration.

Proposal 7	Besides the case of memory limitation issues (already agreed in RAN2#127), the UE may stop the event triggered NW-side data collection procedure when:

a.	data collection has been done for a certain configurable logging duration

b.	the event that triggered the data collection is no longer fulfilled.

Proposal 8	RAN2 to support the following events for the logging of NW-side data:

a.	L3 serving cell measurements becoming worse/better than a threshold

b.	Filtered L1 measurements associated to one or more CSI-RS/SSB resources of serving cell becomes worse/better than threshold.

Proposal 9	The UEInformationRequest requesting AIML data is transmitted via SRB1 (as any other data request contained in such message), and the UEInformationResponse is transmitted via SRB2, or via a new defined SRB (to be used for all AIML use cases).

Proposal 10	The availability indication transmitted by the UE should contain at least a reference to the type of data that are available for transmission (e.g. “AIML NW-side collected data”), and the UEInformationRequest should contain at least a reference to the type of data that are requested by the gNB (e.g. “AIML NW-side collected data”).

Proposal 11	As baseline, the data availability indication is transmitted at least in the UEAssistanceInformation message when there are data available for transmission. Whether it can be transmitted also in RRC complete messages (as in SON/MDT) depends on the handling of collected data during mobility.

Proposal 12	As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT.

Proposal 13	The UE memory for data collection is shared for NW- and UE-side data collection.

Proposal 14	Define in a UE capability some different minimum memory values that a UE can support for the logging of collected data. RAN2 to wait RAN1 inputs on the type of data to be logged by the UE, before discussing the right values.

Proposal 15	For Rel.19, as baseline, RAN2 assumes that the UE discards/clears the AIML collected data at mobility events (e.g. handover to a different gNB than the source gNB, RLF/HOF, RRC state transitions).

Proposal 16	A window to report UE ground truth coinciding with SRS transmission periodicity is configured.

Proposal 17	UE sends ground truth UE location coordinate to gNB via RRC for gNB-side data   collection.