3GPP TSG-RAN WG2 Meeting #129	R2-2501306

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	8.8.4

Source:	Ericsson

Title:	Support for broadcast services in NR NTN

Document for:	Discussion, Decision

# 1	Introduction

The third WID on NTN for NR was agreed in [1], which includes an objective on specifying signalling for broadcast services, as follows:

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify SIB signalling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

- Specify the necessary signalling between CN and NG-RAN. [RAN3]

So far, RAN2 has reached several agreements regarding the shape/format of the intended service area as well as some of the related UE behaviours when located inside and outside the intended service area (ISA). However, signalling means as well as service continuity are still under discussion. This contribution presents our views on the open issues and on supporting PWS in NTN cells.

# 2	Support for MBS broadcast

### 2.1	Intended Service Area (ISA) and Service Announcement

#### 2.1.1	Background

The Service Announcement provides essential information that enables users to discover and access available multicast and broadcast services. Among other parameters, it includes the MBS session identifier (TMGI), the type of content being offered, the starting and ending times of broadcast, the area of broadcast (MBS Service Area), the notification of changes, and service continuity information (MBS FSAI). In simple terms, it is much like how a TV guide helps viewers plan what they want to watch and when. The Service Announcement helps UEs navigate and make use of the services offered by the network efficiently.

- The Service Announcement provides essential information to discover and access available multicast and broadcast services.

- Much like a TV guide, the Service Announcement helps UEs navigate and make use of the services offered by the network efficiently.

The Service Announcement may be pre-configured at the UE, provided by the Application Function (AF) or MBS Service Function (MBSF) or retrieved by the UE from those entities. According to TS 26.502, the Service Announcement can be delivered via application specific means, an MBS Session, or a regular PDU Session. Figure 1 illustrates a detailed signalling flow of the phases for MBS broadcast session establishment. In our understanding of the whole process, the network starts a broadcast service by creating the MBS sessions, allocating TMGIs, and sending the Service Announcement. The first step for UE is to receive or acquire the Service Announcement to become aware of the available MBS services (e.g., content, identifiers, start and stop times). Once this information is obtained, depending on user’s choice, the UE may become interested in an MBS service and trigger RAN procedures such as MCCH acquisition or MRB establishment.

- It can be assumed that a UE interested in an MBS service has already acquired the Service Announcement to know the available services.

Figure 1: Broadcast session establishment flow (adapted from Figure 7.3.1-1 in TS 23.247 V18.6.0).

In conclusion, the Service Announcement plays a pivotal role in the MBS architecture. A UE acquires the Service Announcement to become aware and access the available MBS services in the cell. In RRC termsn, a UE needs to receive the Service Announcement to become “interested to receive MBS broadcast services”. In fact, this may happen even before attempting to decode any MBS related SIBs and obtain the related MBS RAN configuration via MCCH.

#### 2.1.2	MCCH acquisition relaxation

A UE interested in MBS acquires the Service Announcement to become aware of the existing MBS services and their respective characteristics (see Figure 1). This may happen even before attempting to decode the MBS configuration provided in MCCH. In fact, the current RRC specification allows the UE to skip MCCH monitoring based on USD/Service Announcement:

[NOTE in TS 38.331 V18.3.0 - Section 5.9.2.2]

“It is up to UE implementation to use the cell/tracking area list in the USD to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service.”

On a related note, Stage-2 specification allows the UE to decide when to start monitoring the MCCH based on the information signalled in the Service announcement:

[NOTE in TS 38.300 V18.2.0 - Section 16.10.6.2]

It is up to UE implementation to use the start and stop times in the USD to determine when to start monitoring the MCCH for the session the UE is interested in.

Finally, if the ISA ID is provided in the Service Announcement, UE behaviour would be similar to legacy. A UE interested in an MBS broadcast session may skip MCCH acquisition when it is located outside the intended service area where this service is provided. A similar note could be captured in Stage 2 so that the evaluation of the conditions (SIB acquisition, GNSS positioning, and the mapping between TGMI and ISA ID) is left to UE implementation.

A UE interested in an MBS broadcast service may skip MCCH acquisition when it is located outside the intended service area where this service is provided.

#### 2.1.3	ISA ID in the Service Announcement to relax MCCH acquisition

In previous RAN2 meetings, it was agreed that the mapping between an MBS service (represented with its TMGI) and its corresponding ISA is provided via MCCH (e.g., MBSBroadcastConfiguration IE). In addition, it was agreed that the geographical definition of the ISA(s) is provided in a list in a new SIB. This solution requires the UE to first acquire the new SIB and second decode MCCH (at least once) to be aware of which services are associated to each intended service area.

- RAN2 has agreed that the description of the ISA is provided via SIB, while the mapping between ISA and TGMI is provided in MCCH.

One open issue related to energy saving is to enable the UE to relax MCCH acquisition and monitoring when it is not located within an ISA associated with the service it is interested in obtaining. Indeed, the agreed signalling mechanism may potentially allow the UE to relax MCCH acquisition in such scenario. However, this would be done in a very coarse manner: if the UE is not located within any ISA, it may skip MCCH acquisition. One disadvantage is that the UE may acquire MCCH even if the MBS service which it is interested in (discovered through the Service Announcement) is not intended for the specific ISA where the UE is located. Moreover, it is uncertain how the UE can handle MBS updates, for example, a new broadcast session starts.

- If the mapping between ISA and TMGI is only present in MCCH, a UE needs to decode MCCH to understand which MBS services are available in each ISA.

We see the benefits of additionally including the mapping between the ISA ID and MBS broadcast session (TMGI) in the Service Announcement. Mainly, this method secures that the UE becomes aware of the ISA ID of the specific MBS broadcast session the user is interested in obtaining. In this way, a UE can relax MCCH acquisition based on the combination of the geographical area (ISA) definition in the new SIB and the mapping between TMGI and ISA ID in the Service announcement. Specifically, including ISA ID in service announcement allows a UE to know whether it is within the ISA(s) of MBS session(s) that it is interested in and only consider those area(s) in MCCH-relaxation. Besides, this mechanism can handle MBS updates without uncertainty since the UE already monitors (up to implementation) the Service Announcement.

- Including the ISA ID in the Service Announcement helps MBS interested UEs skip MCCH acquisition benefiting both energy saving and user experience.

Note that the current RRC specification already allows the UE to skip MCCH monitoring based on area information in the USD/Service Announcement. Thus, minimum specification changes are needed to adopt this legacy mechanism (MCCH acquisition relaxation) in the NTN case by including the intended service area (ISA) in the Service Announcement.

- A legacy UE may skip MCCH monitoring based on geographical information (tracking area/cell ID) provided in the Service Announcement.

Figure 2 represents the end-to-end process of MBS session creation from the application function (AF) to the UE with the intermediate steps and different functions involved. In case of NTN, it illustrates how the UE can obtain the geographical area definition from system information broadcast and the ISA ID from either MCCH or, optionally, the Service Announcement. As shown in section 3, the adoption of this enhancement has minimum specification impact in RAN2. Similarly, minor changes are expected for other working groups (SA2) and it would not constitute an increased workload since these groups are already discussing the topic of intended service area in MBS NTN.

Figure 2. End-to-end flow of MBS session creation associated with an intended service area in NTN.

In conclusion, including the intended service area (ISA) ID in the Service Announcement allows the UE to save energy by skipping MCCH acquisition based on the specific MBS session (TMGI) the UE is interested in receiving. Compared to the proposed alternatives, it does not suffer from uncertainty when receiving MBS updates. Provided that RAN2 decides to adopt this enhancement, a draft LS to SA2 is provided in Annex to inform them of this feature and can be considered as a baseline.

Provide ISA ID in the MBS Service Announcement.

RAN2 informs SA2. Consider the draft LS in the annex as a baseline.

### 2.2	MBS service continuity in NTN

#### 2.2.1	Background

MBS Service continuity for UEs in RRC_IDLE or RRC_INACTIVE was enhanced with the following mechanisms to reduce service interruption and user experience degradation (section 16.10.6.5 in TS 38.300):

- A list of neighbour cells providing the one or more MBS broadcast services as the serving cell. It is informed with a list provided in MCCH (MBSBroadcastConfiguration IE) through the parameter mbs-NeighbourCellList.

- Information about which frequency is providing which MBS broadcast services. It is informed through User Service Description (USD) and SIB21. Specifically, SIB21 contains MBS Frequency Selection Area ID (FSAI) which identifies a preconfigured area within, and in proximity to, which the cell(s) announces the FSAI and the associating frequency.

The first mechanism allows a UE to request unicast reception of the service before moving to a cell not providing the MBS broadcast service(s) using point-to-multipoint (PTM) transmission. The second mechanism helps the UE avoid the need to read MBS broadcast related system information and potentially MCCH on neighbour frequencies. The UE can figure out the TMGI-Frequency mapping from USD/Service Announcement or from the combination of USD/Service Announcement and SIB21. In addition, the UE is allowed to make the desired frequency highest priority in cell reselection.

#### 2.3.2	NTN specific changes

In previous RAN2 meetings, there has been an increased interest in enhancing service continuity considering the concept of Intended Service Area, which is specific to NTN. In our view, the introduction of ISA does not entail any necessary changes to legacy mechanisms for service continuity in 5G MBS broadcast, which can also be applied in NTN. That means, the list of neighbour cells providing the same MBS broadcast services (now with the associated ISA) in MCCH, the FSAI in SIB21, and the TMGI-frequency mapping or TMG-SAI mapping in the USD (which is delivered to the UE via Service announcement) can be reused.

- Legacy (TN) mechanisms for service continuity due to UE mobility in 5G MBS Broadcast can be reused for MBS broadcast in NTN.

RAN2 has agreed that the ISA(s) may expand beyond the coverage of a cell and overlap neighbour cells, i.e., they are not tied to cell boundaries but represent geographical locations. In addition, RAN2 concluded that there are no requirements for the UE to start or release MBS Radio Bearers (MRB) upon entering or leaving the ISA. Therefore, when the UE moves between cells providing an MBS service, it can perform frequency prioritization and cell reselection following legacy mechanisms. Once the UE is camping in the neighbour cell, it may acquire MCCH and the new SIB and evaluate whether it is located within the ISA(s) specific to its service. In summary, the UE can continue obtaining an MBS service upon cell change in RRC_IDLE/INACTIVE mode without considering the neighbour cell’s ISA(s), yielding no benefit in informing the UE of its neighbour cells ISA(s) nor their relation of ISA and FSAI, which would only increase complexity and signalling overhead.

- A UE can continue obtaining an MBS service upon cell change in RRC_IDLE/INACTIVE mode without considering the neighbour cell’s ISA(s).

- There is no benefit for the UE to know the ISA of neighbour cells.

Service continuity enhancements due to UE movement between NTN cells are down prioritized in Rel-19.

We see the need to address service continuity in NTN originated due to satellite movement. Specifically, one aspect where NTN differs from terrestrial networks (TN) is that the time duration that an NGSO satellite serves a quasi-earth-fixed cell is just a few minutes. Each new serving satellite typically implies a new cell, therefore, an MBS UE in RRC_IDLE or RRC_INACTIVE needs to perform cell reselection and potentially acquire MBS related configuration and the list of neighbour cells that providing the same MBS services (carried in MCCH) quite often. This can be a challenge in terms of signalling overhead and service interruption since it is difficult from network perspective to align satellite switches with the MCCH notification period, and there is a risk the UE does not obtain the MCCH update in time, leading to service interruption. Therefore, we suggest RAN2 to discuss this issue and find potential solutions.

- In NTN systems, the need to enhance service continuity mechanisms comes from the frequent satellite switches rather than UE movement.

- The continuous switch of satellites/cells in an NTN deployment may lead to both increased signalling overhead and service interruption in MBS.

RAN2 focuses on service continuity enhancements required due to satellite movement.

A potential solution to the foregoing problem is to allow a serving cell to signal a list of upcoming satellites/cells (e.g., indicated via satellite IDs) providing the same MBS services as the serving cell. This indication would be equivalent to the existing mbs-NeighbourCellList in terrestrial deployments. This information can be complemented with the satellite assistance information provided in other System Information blocks (e.g., SIB19). In combination with the legacy solution using SIB21 and FSAI, this small enhancement aims to enable UEs interested in MBS broadcast to know well in advance whether the same services are provided by upcoming satellites, thereby preparing the UEs to either continue using the same MBS configuration or request a unicast delivery of the same service in the new cells/satellites to ensure service continuity.

In a quasi-Earth fixed cell, the network provides information about the MBS broadcast configuration and MBS services provided by the upcoming satellite(s) that will serve the area.

### 2.4	Public warning systems in NTN

The scope of the work item to include a service area smaller than a cell in NTN is not limited to MBS but also encompasses other broadcast services such as Public Warning Systems (PWS). The two most prominent PWS are ETWS and CMAS. During RAN2#127, the feasibility of applying the geofencing solution used in CMAS to ETWS was discussed. RAN2 concluded to send an LS to SA1 to inquire about the requirements and clarify whether such capability can be extended to ETWS.

In this meeting, RAN2 has received the LS response from SA1 [3] indicating the creation of geofencing requirements for PWS in NTN

## 5.6	Additional PWS Requirements Specific to ETWS

Subject to local/regional regulatory requirements, in addition to ETWS Primary and Secondary Notifications, ETWS shall be able to support broadcasting Warning Notification Area(s). This requirement applies to Satellite E-UTRAN and Satellite NG-RAN.

Subject to local/regional regulatory requirements, an ETWS-capable UE shall present the received ETWS Notification(s) (Primary or Secondary) if there is a corresponding Warning Notification Area(s) and if the ETWS-capable UE is able to determine that its location is within the corresponding Warning Notification Area(s). Otherwise, if an ETWS-capable UE is unable to or fails to determine its location, the ETWS-capable UE shall present the ETWS Notification(s) (Primary or Secondary) to the user.

. The following changes have been introduced in SA1 specifications [4].

Given the positive response from SA1 and the inclusion of new requirements for both NR and IoT NTN, it becomes clear that RAN2 should introduce geofencing of ETWS notification for NTN deployments. In addition, a new work item revision [5] was approved in RAN#105 that includes the support of PWS for NB-IoT NTN and the potential extension of the intended service area concept. Therefore, we advocate that, unless technical impediments are found, RAN2 adopts a common solution for both RATs. A detailed account of a signalling proposal is provided in [6].

Introduce geographic information in ETWS notification for geo-fencing in NTN cells.

RAN2 to study a common solution to geofence ETWS notifications in both NR NTN and NB-IoT NTN RATs.

# 3	Text Proposals

## 3.1	TP to TS 38.300 v18.3.0

#### 16.10.6.2	Configuration

MBS broadcast can be received by UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state. A UE can receive the MBS configuration for broadcast session (e.g., parameters needed for MTCH reception) via MCCH in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state. The parameters needed for the reception of MCCH are provided via System Information.

The following principles govern the MCCH structure:

-	MCCH provides the list of all broadcast services with ongoing sessions transmitted on MTCH(s) and the associated information for broadcast session includes MBS session ID, associated G-RNTI scheduling information and information about neighbouring cells providing certain service on MTCH(s). MCCH content is transmitted within periodically occurring time domain windows, referred to as MCCH transmission window defined by MCCH repetition period, MCCH window duration and radio frame/slot offset;

-	MCCH uses a modification period and MCCH contents are only allowed to be modified at each modification period boundary; a notification mechanism is used to announce the change of MCCH contents due to broadcast session start, modification or stop and due to neighbouring cell information modification;

NOTE:	It is up to UE implementation to use the start and stop times and the ID of the intended service area in the USD to determine when to start monitoring the MCCH for the session the UE is interested in.

-	When the UE receives an MCCH change notification, it acquires the updated MCCH in the same MCCH modification period where the change notification is sent.

## 3.2	TP to TS 38.331 v18.3.0

#### 5.9.2.2	Initiation

A UE shall apply the MCCH information acquisition procedure upon becoming interested to receive MBS broadcast services. A UE interested to receive MBS broadcast services shall apply the MCCH information acquisition procedure upon entering the cell providing SIB20 (e.g. upon power on, following UE mobility), upon receiving SIB20 of an SCell via dedicated signalling and upon receiving a notification that the MCCH information has changed due to the start of new MBS service(s). A UE that is receiving data via broadcast MRB shall apply the MCCH information acquisition procedure upon receiving a notification that the MCCH information has changed due to MCCH information modification other than the change caused by the start of new MBS service(s).

NOTE 1:	It is up to UE implementation how to address a possibility of the UE missing an MCCH change notification.

NOTE 2:	It is up to UE implementation to use the cell/tracking area list or the intended service area ID in the USD along with the UE's GNSS position to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service.

Unless explicitly stated otherwise in the procedural specification, the MCCH information acquisition procedure overwrites any stored MCCH information, i.e. delta configuration is not applicable for MCCH information and the UE discontinues using a field if it is absent in MCCH information.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	The Service Announcement provides essential information to discover and access available multicast and broadcast services.

Observation 2	Much like a TV guide, the Service Announcement helps UEs navigate and make use of the services offered by the network efficiently.

Observation 3	It can be assumed that a UE interested in an MBS service has already acquired the Service Announcement to know the available services.

Observation 4	RAN2 has agreed that the description of the ISA is provided via SIB, while the mapping between ISA and TGMI is provided in MCCH.

Observation 5	If the mapping between ISA and TMGI is only present in MCCH, a UE needs to decode MCCH to understand which MBS services are available in each ISA.

Observation 6	Including the ISA ID in the Service Announcement helps MBS interested UEs skip MCCH acquisition benefiting both energy saving and user experience.

Observation 7	A legacy UE may skip MCCH monitoring based on geographical information (tracking area/cell ID) provided in the Service Announcement.

Observation 8	Legacy (TN) mechanisms for service continuity due to UE mobility in 5G MBS Broadcast can be reused for MBS broadcast in NTN.

Observation 9	A UE can continue obtaining an MBS service upon cell change in RRC_IDLE/INACTIVE mode without considering the neighbour cell’s ISA(s).

Observation 10	There is no benefit for the UE to know the ISA of neighbour cells.

Observation 11	In NTN systems, the need to enhance service continuity mechanisms comes from the frequent satellite switches rather than UE movement.

Observation 12	The continuous switch of satellites/cells in an NTN deployment may lead to both increased signalling overhead and service interruption in MBS.

Based on the discussion in the previous sections we propose the following:

Proposal 1	A UE interested in an MBS broadcast service may skip MCCH acquisition when it is located outside the intended service area where this service is provided.

Proposal 2	Provide ISA ID in the MBS Service Announcement.

Proposal 3	RAN2 informs SA2. Consider the draft LS in the annex as a baseline.

Proposal 4	Service continuity enhancements due to UE movement between NTN cells are down prioritized in Rel-19.

Proposal 5	RAN2 focuses on service continuity enhancements required due to satellite movement.

Proposal 6	In a quasi-Earth fixed cell, the network provides information about the MBS broadcast configuration and MBS services provided by the upcoming satellite(s) that will serve the area.

Proposal 7	Introduce geographic information in ETWS notification for geo-fencing in NTN cells.

Proposal 8	RAN2 to study a common solution to geofence ETWS notifications in both NR NTN and NB-IoT NTN RATs.

# 5	References

RP-240775, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3” 3GPP TSG RAN#103, Mar. 2024.

R2-2409184, Support for broadcast services in NR NTN, Ericsson, RAN2#127bis, Hefei, China, October 2024.

S1-244857, Reply LS on requirements for ETWS primary notification, S1#108, Novamint, Orlando, US, November 2024.

RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, RAN#105, Sep. 2024.

S1-244753, Warning notification with geofencing for ETWS over satellite access, S1#108, Qualcomm, Novamint, SyncTechno Inc, CATT, Orlando, US, November 2024.

R2-XXXXX, XYZ, Ericsson, RAN2#129, Athens, Greece, February 2025.

# 6	Annex

## 6.1	Draft LS to SA2

3GPP TSG-RAN WG2 Meeting #128	draft-R2-XXXXXX

Orlando, FL, USA, November 18th – 22nd, 2024

Title:	[draft LS] Intended Service area for MBS NTN

Response to:	-

Release:	Rel-19

Work Item:	NR_NTN_Ph3-Core

Source:	RAN2

To:	SA2

Cc: 	-

# Contact Person:

# Name:	Ignacio Pascual

E-mail Address:	 Ignacio.pascual.pelayo@ericsson.com

# Attachments:	None

# 1. Overall Description:

RAN2 is working on a solution to provide UEs with geographical information of where within an NTN cell an MBS broadcast service is applicable. This area is called the “intended service area” whose geographical definition is provided to the UE via system information broadcast and is associated with an identifier.

For the purpose of MCCH acquisition relaxation, RAN2 is considering the inclusion of the “intended service area” identifier in the Service Announcement. RAN2 understands that there may be an impact in SA2 specifications and would kindly request SA2 to consider the necessary updates.

# 2. Actions:

# To SA WG 1:

# RAN2 kindly request SA2 to take into consideration and update the corresponding specifications, if needed.

# 3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #129bis		7th April – 11th April. 2025				TBC, China

TSG RAN WG2 Meeting #130		19th May – 23rd May 2025			Malta