3GPP TSG-RAN WG2 Meeting #129								R2-2501337

Athens, Greece, Feb. 17th – 21st, 2025			(revision of R2-2501112)

Title: 	Discussion on logging suspension due to IDC

Source: 	Huawei, HiSilicon, CMCC, OPPO

Agenda item:	6.1.3.1

Document for:	Discussion/Decision

- Introduction

In the revision, we made some updates on proposals due to some offline discussions.

At RAN2#128 meeting, the following contributions were discussed [1]. There were diverse views regarding the issue, so this discussion was postponed. In this contribution, we provide our views regarding the issue and potential specification impacts.

6.1.3	Control Plane corrections

IDC

R2-2409664	Clarification for IDC solutions of logged MDT (Rel-17)	Xiaomi, CMCC	CR	Rel-17	38.331	17.10.0	5109	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2409665	Clarification for IDC solutions of logged MDT (Rel-18)	Xiaomi, CMCC	CR	Rel-18	38.331	18.3.0	5110	-	A	NR_ENDC_SON_MDT_enh-Core

-	LG agrees with the intention but want to change in procedural text rather than capturing this with a NOTE. Huawei think this CR is an optimization and no change is needed. Qualcomm agrees with Huawei and think this is not testable and do not see a need for this. Samsung think this can be left to UE implementation and there is no need to overspecify here

Postponed

- Discussion

Firstly, the relevant text is copied in section 5 Annex (see highlighted parts).

Secondly, in order to better discuss the issue as well as understandings, we provide some figures for analysis. In figure 1, the UE performs logging at some time intervals, and there is no IDC issue.

Figure 1

In the following sections, we are going to analyze the issue for two scenarios:

Scenario 1: IDC is detected and not resolved at the time interval X, and IDC is resolved at later time interval

Scenario 2: IDC is detected and resolved at the time interval X

IDC is detected at the time interval X, and resolved at later time interval

At the time interval X, if IDC is detected and not resolved, there are two cases:

(1) if measResultsServingCell in the VarLogMeasReport is not empty, the UE sets IDC flag for the logging at the time interval X, and suspends logging from X+1. See figure 2

(2) if measResultsServingCell in the VarLogMeasReport is empty, it means the UE logs nothing at the time interval X, and we think the UE should suspend logging from X, but it needs RAN2 confirmation and it may have some impacts to TS 38.331. See figure 3

Figure 2

Figure 3

In figure 3, the UE suspends logging from the time interval X. Further, if IDC is solved at the time interval X+2, we think the UE just suspends logging for X+2 (i.e. logs nothing, see Figure 4), and the UE starts logging from X+3.

Figure 4

During offline discussion, another understanding is that the UE should start logging just from X+2 (i.e. change the status from suspend to logging at the time interval X+2, see Figure 5), and the UE starts logging from X+3. For this understanding, the UE needs to check the measurement results, i.e.:

(1) if measResultsServingCell in the VarLogMeasReport is not empty, the UE sets IDC flag for the logging at the time interval X+2

(2) if measResultsServingCell in the VarLogMeasReport is empty, it means the UE logs nothing at the time interval X+2

Figure 5

Here is a summary for two understandings:

|  | UE behaviours at the time interval X+2 | UE behaivours at the time interval X+3 | Specification impacts |
| --- | --- | --- | --- |
| Understanding 1 (See Figure 4) | suspension, and log nothing | logging | Should be no impacts to TS 38.331 |
| Understanding 2 (See Figure 5) | check measResultsServingCell at X+2, and if it is not empty, the UE sets IDC flag for the logging | logging | There may be impacts to TS 38.331 due to UE behaivours on X+2 |

We slightly prefer understanding 1, as it is simple and we do not think the logged results at X+2 is helpful to NW side.

Proposal 1: If IDC is resolved at the time interval X (IDC is detected at the previous time interval), the UE suspends logging for the time interval X, and the UE performs logging from the time interval X+1 (if no IDC issues are detected).

IDC is detected and resolved at the time interval X

Figure 6 shows this scenario.

Figure 6

At the time interval X, if IDC is detected and resolved, there are two cases:

(1) if measResultsServingCell in the VarLogMeasReport is not empty

(2) if measResultsServingCell in the VarLogMeasReport is empty

For case (1), the UE should log the results and IDC flag for the time interval X. At the time interval X+1, understanding 1 is that the status is suspension so that the UE should not do logging for it (while the UE resumes logging from X+2), and understanding 2 is that the UE resumes logging from X+1.

Here is a summary for two understandings:

|  | UE behaviours at the time interval X | UE behaivours at the time interval X+1 | UE behaivours at the time interval X+2 | Specification impacts |
| --- | --- | --- | --- | --- |
| Understanding 1 | logging. The UE logs the results and IDC flag. | suspension | resume logging | There may be impacts to TS 38.331 |
| Understanding 2 | logging. The UE logs the results and IDC flag. | Suspend logging and then resume logging | logging | There may be impacts to TS 38.331 |

For case (2), understanding 1 is that the UE changes the status from logging to suspension for the time interval X and the UE logs nothing at the time interval X. The UE resumes logging from the time interval X+1.

Here is a summary for understanding 1:

|  | UE behaviours at the time interval X | UE behaivours at the time interval X+1 | Specification impacts |
| --- | --- | --- | --- |
| Understanding 1 | the status is changed from logging to suspension. The UE logs nothing | resume logging | There may be impacts to TS 38.331 |

Based on the above analysis, we think a unified solution can be considered, because it is simple for UE implementation and it may have minimal specification impacts. So it is proposed:

Proposal 2: If IDC is detected and resolved at the same time interval X:

- if measResultsServingCell in the VarLogMeasReport is not empty, the UE logs the result and IDC flag for the time interval X

- if measResultsServingCell in the VarLogMeasReport is empty, the UE suspends logging for the time interval X

The UE performs logging from the time interval X+1 (if no IDC issues are detected).

- Conclusion

In this contribution, we discuss logging suspension due to IDC, and we have the following proposals:

Proposal 1: If IDC is resolved at the time interval X (IDC is detected at the previous time interval), the UE suspends logging for the time interval X, and the UE performs logging from the time interval X+1 (if no IDC issues are detected).

Proposal 2: If IDC is detected and resolved at the same time interval X:

- if measResultsServingCell in the VarLogMeasReport is not empty, the UE logs the result and IDC flag for the time interval X

- if measResultsServingCell in the VarLogMeasReport is empty, the UE suspends logging for the time interval X

The UE performs logging from the time interval X+1 (if no IDC issues are detected).

- References

Draft_RAN2_128_Meeting_Report_v1

- Annex

### 5.5a.3	Measurements logging

#### 5.5a.3.1	General

This procedure specifies the logging of available measurements by a UE in RRC_IDLE and RRC_INACTIVE that has a logged measurement configuration. The actual process of logging within the UE, takes place in RRC_IDLE state could continue in RRC_INACTIVE state or vice versa.

#### 5.5a.3.2	Initiation

While T330 is running and SDT procedure is not ongoing, the UE shall:

1>	if measurement logging is suspended:

2>	if during the last logging interval the IDC problems detected by the UE is resolved, resume measurement logging;

1>	if not suspended, perform the logging in accordance with the following:

2>	if the reportType is set to periodical in the VarLogMeasConfig:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]):

4>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

3>	if the UE is in camped normally state on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport; or

3>	if the UE is in camped normally state on an NR cell and if the registered SNPN identity is included in snpn-ConfigID-List stored in VarLogMeasReport:

4>	if areaConfiguration is not included in VarLogMeasConfig; or

4>	if the serving cell is part of the area indicated by areaConfig in areaConfiguration in VarLogMeasConfig; or

4>	if the serving cell is part of the area indicated by cag-ConfigList in areaConfiguration in VarLogMeasConfig; or

4>	if the serving cell is part of the area indicated by snpn-ConfigList in areaConfiguration in VarLogMeasConfig:

5>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

2>	else if the reportType is set to eventTriggered, and eventType is set to outOfCoverage:

3>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the UE is in any cell selection state;

3>	upon transition from any cell selection state to camped normally state in NR:

4>	if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport, or if the registered SNPN identity is included in snpn-ConfigID-List stored in VarLogMeasReport; and

4>	if areaConfiguration is not included in VarLogMeasConfig or if the current camping cell is part of the area indicated by areaConfig of areaConfiguration in VarLogMeasConfig, or if the current camping cell is part of the area indicated by cag-ConfigList of areaConfiguration in VarLogMeasConfig, or if the current camping cell is part of the area indicated by snpn-ConfigList of areaConfiguration in VarLogMeasConfig:

5>	perform the logging;

2>	else if the reportType is set to eventTriggered and eventType is set to eventL1:

3>	if the UE is in camped normally state on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport; or

3>	if the UE is in camped normally state on an NR cell and if the registered SNPN identity is included in snpn-ConfigID-List stored in VarLogMeasReport:

4>	if areaConfiguration is not included in VarLogMeasConfig; or

4>	if the serving cell is part of the area indicated by areaConfig in areaConfiguration in VarLogMeasConfig; or

4>	if the current serving cell is part of the area indicated by cag-ConfigList of areaConfiguration in VarLogMeasConfig, or if the current camping cell is part of the area indicated by snpn-ConfigList of areaConfiguration in VarLogMeasConfig;

5>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the conditions indicated by the eventL1 are met;

2>	when performing the logging:

3> if InterFreqTargetInfo is configured and if the UE detected IDC problems on at least one of the frequencies included in InterFreqTargetInfo or any inter-RAT frequency during the last logging interval, or

3>	if InterFreqTargetInfo is not configured and if the UE detected IDC problems during the last logging interval:

4>	if measResultServingCell in the VarLogMeasReport is not empty:

5>	include inDeviceCoexDetected;

5>	suspend measurement logging from the next logging interval;

4>	else:

5>	suspend measurement logging;

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if location information became available during the last logging interval, set the content of the locationInfo as in 5.3.3.7:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]):

4>	set anyCellSelectionDetected to indicate the detection of no suitable or no acceptable cell found;

4>	if the reportType is set to eventTriggered in the VarLogMeasConfig; and

4>	if the RPLMN at the time of entering the any cell selection state is included in plmn-IdentityList stored in VarLogMeasReport or if the registered SNPN identity at the time of entering the any cell selection state is included in snpn-ConfigID-List stored in VarLogMeasReport; and

4>	if areaConfiguration is not included in VarLogMeasConfig or if the last suitable cell that the UE was camping on is part of the area indicated by areaConfig of areaConfiguration in VarLogMeasConfig, or if last suitable cell that the UE was camping on is part of the area indicated by cag-ConfigList of areaConfiguration in VarLogMeasConfig, or if last suitable cell that the UE was camping on is part of the area indicated by snpn-ConfigList of areaConfiguration in VarLogMeasConfig:

5>	set the servCellIdentity to indicate global cell identity of the last suitable cell that the UE was camping on;

5>	set the measResultServingCell to include the quantities of the last suitable cell the UE was camping on;

4>	else if the reportType is set to periodical in the VarLogMeasConfig:

5>	set the servCellIdentity to indicate global cell identity of the last logged cell that the UE was camping on;

5>	set the measResultServingCell to include the quantities of the last logged cell the UE was camping on;

3>	else:

4>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

4>	set the measResultServingCell to include the quantities of the cell the UE is camping on;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include measurements of neighbouring cell that became available during the last logging interval and according to the following:

4>	include measurement results for at most 6 neighbouring cells on the NR serving frequency and for at most 3 cells per NR neighbouring frequency and for the NR neighbouring frequencies in accordance with the following:

5>	if interFreqTargetInfo is included in VarLogMeasConfig:

6>	if earlyMeasIndication is included in VarLogMeasConfig;

7>	include measurement results for NR neighbouring frequencies that are included in both interFreqTargetInfo and either in measIdleCarrierListNR (within the VarMeasIdleConfig) or SIB4;

6>	else:

7>	include measurement results for NR neighbouring frequencies that are included in both interFreqTargetInfo and SIB4;

5>	else:

6>	if earlyMeasIndication is included in VarLogMeasConfig;

7>	include measurement results for NR neighbouring frequencies that are included in either measIdleCarrierListNR (within the VarMeasIdleConfig) or SIB4;

6>	else:

7>	include measurement results for NR neighbouring frequencies that are included in SIB4;

4>	include measurement results for at most 3 neighbours per inter-RAT frequency in accordance with the following:

5>	if earlyMeasIndication is included in VarLogMeasConfig:

6>	include measurement results for inter-RAT neighbouring frequencies that are included in either measIdleCarrierListEUTRA (within the VarMeasIdleConfig) or SIB5;

5>	else:

6>	include measurement results for inter-RAT frequencies that are included in SIB5;

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 1:	The UE includes the latest results of the available measurements as used for cell reselection evaluation in RRC_IDLE or RRC_INACTIVE, which are performed in accordance with the performance requirements as specified in TS 38.133 [14].

NOTE 2:	For logging the measurements on frequencies (indicated in measIdleCarrierListNR/ measIdleCarrierListEUTRA) in the logged measurement, the qualityThreshold in measIdleConfig should not be applied, and how the UE logs the measurements on the frequencies is left to the UE implementation.

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.5a.1.4.