3GPP TSG-RAN WG2 Meeting #129	R2- 2501338

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.1.1

Source:	Nokia

Title:	On the Running Stage 3 (38.331) CR

WID/SID:	NR_AIML_air-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

We would like to thank the RAN2 rapporteur for NR_AIM_air-Core, who provided a running CR for stage 3, TS38.331, in R2-2501263 [1], to concretize agreements made since the start of the WI phase. However, while many of the additions can be matched to agreements, there are several significant procedures and ASN.1 structures which have been defined without agreements, and not marked as FFS. In this document, we discuss three prominent examples of unagreed content, propose to endorse the CR only after a post meeting email discussion, and a way to ensure consistency with agreements moving forward such that it is easier to see that the running CR matches the agreements, and clearly marks items as FFS or rapporteur’s assumptions that have not yet been agreed.

# 2	Discussion

In this section, we discuss three examples showing significant inconsistency between the running CR and agreements.

## 2.1	Applicability Reporting

The following ASN.1 definition was provided in the section entitled “ApplicabilityReportList”.

ApplicabilityReport-r19 ::= CHOICE { predictionConfigApplicabilityReport-r19 SEQUENCE { applicabilityCellId-r19 ServCellIndex, applicabilityReportConfigIdList-r19 SEQUENCE (SIZE (1..maxNrofApplicabilityReports)) OF CSI-ReportConfigId, } assistanceApplicabilityReport-r19 SEQUENCE { applicabilityInfoId-r19 INTEGER (0..maxNrofApplicabilityReports-1-r19), applicabilityAssociatedIdList-r19 SEQUENCE (SIZE (1..maxNrofAssociatedIDs-r19)) OF AssociatedId-r19, } }

Figure 2.1-1: ApplicabilityReport-r19 ASN.1 Definition [1]

The ApplicabilityReport-r19 defined in Figure 2.1-1 contains a CHOICE between two types of applicability reports: one which reports CSI-ReportConfigIds and one which reports applicabilityInfoIds.

The first type of report contains a serving cell index and a list of CSI-ReportConfigIds and cell IDs. According to the LS from RAN1 [2], at least for reporting the applicability of a full inference configuration, the list of CSI-ReportConfigIds is in alignment.

However, there are two issues with this first type of report. We have not agreed to specify a serving cell index in the applicability report. This needs to be discussed as we neither have agreements on whether AI/ML-enabled configurations are restricted to the primary cell only, or if secondary cells are also eligible and then whether their applicability reports are sent in aggregate or in individual reports as in this CR.

Observation 1: RAN2 has not agreed to support AI/ML enabled configurations for secondary cells.

Observation 2: RAN2 has not agreed to include a serving cell index in the applicability report.

The second type of report, assistanceApplicabilityReport, contains an applicabilityInfoId-r19 and a list of AssociatedId-r19. The applicabilityInfoId points to ApplicabilityInfoConfig-r19, contained in ApplicabilityReportConfig-r19, which are captured in Figure 2.1-2 below. The field description or applicabilityReportConfig is provided in Figure 2.1-3.

ApplicabilityReportConfig-r19 ::= SEQUENCE { applicabilityInfoList-r19 SEQUENCE (SIZE (1..maxNrofApplicabilityReports-r19)) OF ApplicabilityInfoConfig-r19 OPTIONAL -- Need R } ApplicabilityInfoConfig-r19 ::= SEQUENCE { reportedCellIdentity-r19 FFS, predictionType-r19 ENUMERATED {FFS}, associatedIdList-r19 SEQUENCE (SIZE (1..maxNrofAssociatedIDs-r19)) OF AssociatedId-r19 OPTIONAL -- Need R }

Figure 2.1-2: ApplicabilityReportConfig-r19 [1]

applicabilityReportConfig Configuration for the UE to indicate in the ApplicabilityReportList the applicability of prediction configurations or sets of prediction related parameters.

Figure 2.1-3: ApplicabilityReportConfig Field Description [1]

First, we can address the non-FFS parts. It is implied that it has been agreed that the UE would be configured with a list of AssociatedId-r19 and a predictionType-r19, where the prediction types are FFS. We haven’t made any agreements yet in RAN2 about how inference-related parameters could be provided to the UE, nor what those parameters are. The field description for applicabilityReportConfig, shown in Figure 2.1-3, indicates that the configuration applies to “prediction configurations” or “sets of prediction related parameters”.

RAN1 indicated in an LS to RAN2 [2] that the NW can provide the UE with inference-related parameters, which at least include the following, shown in Figure 2.1-4.

- The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction

Figure 2.1-4: Definition of inference-related Parameters from RAN1 LS [2]

RAN1 indicated that inference related parameters, not “prediction related parameters”, can be signalled, and that they could be selected from IEs contained in CSI-ReportConfig or that they could be in CSI-ReportConfig. RAN2 has not discussed the container for inference-related parameters, which based on the RAN1 LS could be a new container or a CSI-ReportConfig.

Observation 3: RAN1 informed RAN2 that inference related parameters could be selected from IEs contained in CSI-ReportConfig or that the inference related parameters could be inside of CSI-ReportConfig. RAN2 has not made an agreement based on the two options.

Observation 4: The CR is using a different terminology from RAN1 for inference related parameters.

As mentioned in Observation 2, RAN2 has not agreed to include a cell ID in the applicability report. It isn’t clear from the CR whether the content of the reportedCellIdentity-r19 is FFS or if the field is FFS.

Additionally, instead of using the list provided by RAN1 as a starting point, the ApplicabilityReportConfig-r19 only includes prediction type, not mentioned or defined by RAN1 and a list of associated IDs. We have not agreed to report applicability based on associated IDs.

Observation 5: RAN2 has not agreed to report applicability based on associated IDs.

Therefore, the ASN.1 captured for configuring the applicability report, ApplicabilityReportConfig-r19, and the contents of the contents of the applicability report, ApplicabilityReport-r19, is not fully aligned with agreements made in RAN2 and RAN1 thus far.

Observation 6: The ASN.1 captured for configuring the applicability report, ApplicabilityReportConfig-r19, and the contents of the contents of the applicability report, ApplicabilityReport-r19, is not fully aligned with agreements made in RAN2 and RAN1 thus far

## 2.2	Measurement Logging Configuration

A configuration IE was provided for configuring the logging of measurements for beam prediction, CSI-LoggedMeasurementConfig-r19, and is shown in Figure 2.2-1 below. The only item marked as FFS is the contents of the EventTriggredConfig-r19 IE.

CSI-LoggedMeasurementConfig-r19 ::= SEQUENCE { csi-LoggedMeasurementConfigId-r19 CSI-LoggedMeasurementConfigId-r19, csi-LoggedResourceConfig-r19 CSI-ResourceConfigId, eventTriggeredConfig-r19 EventTriggeredConfig-r19 OPTIONAL –Need R … } EventTriggeredConfig-r19 ::= SEQUENCE { FFS }

Figure 2.2-1: CSI-LoggedMeasurementConfig-r19 [1]

An email discussion, the report for which was provided in R2-2500999 [3], was held between RAN2#128, and this present meeting to discuss event-triggered measurement logging, and many companies provided proposals with significantly different approaches to supporting event-triggered measurement logging. The solution provided in Figure 2.2-1 does not align with any discussed or proposed implementations, which could include configuring event-based reporting and logging using the RRC measurement reporting framework, i.e., ReportConfigNR, MeasId, and MeasObjectNR, configuring events inside of CSI-ReportConfig with a configuration for logging, or supporting event-based triggering by using MeasuremetnReports from the UE to help the NW decide when to activate logging.

Observation 7: The provided ASN.1 implementation for CSI-LoggedMeasurementConfig-r19 is not backed up by any agreements in RAN2 and is still being actively discussed, such as using ReportConfigNR, CSI-ReportConfig, or another mechanism to support event-based triggering of measurement logging.

## 2.3	Measurement Logging Reporting

The CR provides an ASN.1 definition for reporting logged measurements and is shown in Figure 2.3-1.

CSI-LogMeasReport-r19 ::= SEQUENCE { csi-LogMeasInfoList-r19 CSI-LogMeasInfoList-r19, csi-LogMeasAvailable-r19 ENUMERATED {true} OPTIONAL, FFS … } CSI-LogMeasInfoList-r19 ::= SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF CSI-LogMeasInfo-r19 CSI-LogMeasInfo-r19 ::= SEQUENCE { pci-arfcn-r19 PCI-ARFCN-NR-r16, refCSI-LoggedMeasurementConfigId-r19 CSI-LoggedMeasurementConfigId-r19, csi-RS-MeasResultList-r19 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF CSI-MeasResults-r19 OPTIONAL, csi-SSB-MeasResultList-r19 SEQUENCE (SIZE (1..maxNrofSSBs)) OF CSI-MeasResults-r19 OPTIONAL FFS … } CSI-MeasResults-r19 ::= SEQUENCE { rs-Index-r19 CHOICE { csi-RS-Index-r19 NZP-CSI-RS-ResourceId, ssb-Index-r19 SSB-Index } l1RSRP-r19 RSRP-Range OPTIONAL … }

Figure 2.3-1: CSI-LogMeasReport-r19 [1]

CSI-LogMeasReport-r19 basically captures some assumptions, which have not been formally agreed, despite being simple: to support the transmission of a list of measurements from the logged measurement buffer in the UE and to include an indication of additional measurement availability.

Observation 8: The provided ASN.1 implementation for CSI-LogMeasReport-r19 captures assumptions from discussion on the inclusion of a list of measurements and an indicator of further data availability, but it isn’t based on agreements.

Secondly, no agreement has been made as to the nature of the logged measurement configurations. Figure 2.3-1, refCSI-LoggedMeasurementConfigId-r19 implies the existence of multiple logging configurations, because if there were only one logging configuration or only one logging configuration per cell, then refCSI-LoggedMeasurementConfigId-r19 would not be needed. However, the support of multiple logging configurations has not been agreed, and papers were submitted to this meeting that discuss that issue.

Observation 9: The ASN.1 definition of CSI-LogMeasInfo-r19 implies that multiple logging configurations, either one per cell for multiple cells or more than one per cell is supported, despite not having made any agreement and that the topic is under discussion.

Lastly, RAN1 is still discussing the content of the report, and will discuss it further during the present meeting, RAN1#120. The report of RAN1#119 [4] discussed proposals, Proposal 6.2A and Proposal 6.2B, covering “content for data collection for training for NW-sided model via higher layer signaling”.

Proposal 6.2A provided options such as “L1-RSRPs measured based on one resource set (for Set A and Set B)”, “L1-RSRPs measured based on one resource set (for Set B) configured to UE, and beam information of Top K measured based on another resource set (for Set A)”, “differential L1-RSRP reporting”, and suggested for future study “whether/how to omit some of L1-RSRPs in the report”.

Proposal 6.2B provided further options: “Type1: All L1-RSRPs of measured based the resources configured to UE”; “Type 2: L1-RSRPs measured based on some resources configured to UE, and beam information of K1 beams based on some other resources configured to the UE”; and “Type 3: L1-RSRPS measured based on some resources configured to UE, and beam information and L1-RSRPs of K2 beams based on some other resources configured to UE”.

As these proposals were provided in the summary report without associated agreements, it is clear that RAN1 has not yet concluded on data content for data collection for NW-side beam prediction models.

Observation 10: RAN1 has not yet concluded on data content for data collection for NW-side beam prediction models.

Observation 11: RAN2 has no basis for defining the logged CSI measurement report contents, i.e., in CSI-MeasResults-r19, until RAN1 concludes.

Note as well that PCI-ARFCN-NR-r16 is signalled in CSI-LogMeasInfo-r19, while ServCellIndex and reportedCellIdentity are reported for ApplicabilityReport-r19 and ApplicabilityReportConfig-r19, respectively.

Observation 12: The unagreed inclusion of some kind of cell identifier in the applicability report, the configuration for applicability reporting, and the CSI measurement logging configuration is inconsistent, indicating ServCellIndex, reportedCellIdentity, and PCI-ARFCN-NR-r16, respectively.

# 3	Way Forward

In this document, we provided three example instances, which do not represent the full set of content in the running Stage 3 CR, which are not supported by agreements made in RAN1 or RAN2. It is understandable to make some small next step assumptions such as the inclusion of a data availability indicator in a logged measurement report. However, topics still well under discussion such as applicability reporting, logged measurement configuration, and the contents of the logged measurement reports should not have been defined so elaborately in the running CR. Each unagreed item included in the CR without at least an FFS indication is a cause for discussion, which could consume more time than if we were to build the procedures and ASN.1 definitions only based on agreements.

Therefore, we propose to postpone the endorsement of the running Stage 3 CR, TBD whether to discuss some sections in a post meeting email discussion.

Proposal 1: Postpone the endorsement of the running Stage 3 CR for this WI. TBD whether to discuss some sections in a post meeting email discussion.

In the spirit of transparency, we would also like to propose that for each change in the running CR for this WI, include the agreement which supports each change. For example, R2-2500918 [5] includes relevant agreements using Microsoft Word comment bubbles. An example is shown below.

Proposal 2: For each change in the running CR for this WI, include the agreement which supports the change.

# 4	Conclusion

This document has made the following observations:

Observation 1: RAN2 has not agreed to support AI/ML enabled configurations for secondary cells.

Observation 2: RAN2 has not agreed to include a serving cell index in the applicability report.

Observation 3: RAN1 informed RAN2 that inference related parameters could be selected from IEs contained in CSI-ReportConfig or that the inference related parameters could be inside of CSI-ReportConfig. RAN2 has not made an agreement based on the two options.

Observation 4: The CR is using a different terminology from RAN1 for inference related parameters.

Observation 5: RAN2 has not agreed to report applicability based on associated IDs.

Observation 6: The ASN.1 captured for configuring the applicability report, ApplicabilityReportConfig-r19, and the contents of the contents of the applicability report, ApplicabilityReport-r19, is not fully aligned with agreements made in RAN2 and RAN1 thus far

Observation 7: The provided ASN.1 implementation for CSI-LoggedMeasurementConfig-r19 is not backed up by any agreements in RAN2 and is still being actively discussed, such as using ReportConfigNR, CSI-ReportConfig, or another mechanism to support event-based triggering of measurement logging.

Observation 8: The provided ASN.1 implementation for CSI-LogMeasReport-r19 captures assumptions from discussion on the inclusion of a list of measurements and an indicator of further data availability, but it isn’t based on agreements.

Observation 9: The ASN.1 definition of CSI-LogMeasInfo-r19 implies that multiple logging configurations, either one per cell for multiple cells or more than one per cell is supported, despite not having made any agreement and that the topic is under discussion.

Observation 10: RAN1 has not yet concluded on data content for data collection for NW-side beam prediction models.

Observation 11: RAN2 has no basis for defining the logged CSI measurement report contents, i.e., in CSI-MeasResults-r19, until RAN1 concludes.

Observation 12: The unagreed inclusion of some kind of cell identifier in the applicability report, the configuration for applicability reporting, and the CSI measurement logging configuration is inconsistent, indicating ServCellIndex, reportedCellIdentity, and PCI-ARFCN-NR-r16, respectively.

And the following proposals:

Proposal 1: RAN2 to have a post email discussion on the running Stage 3 CR for this WI after this meeting before taking it as the baseline.

Proposal 2: For each change in the running CR for this WI, include the agreement which supports the change.

# 5	References

[1] R2-2501263, “Draft running RRC CR for AIML PHY”, Ericsson, Athens, Greece, February 2025

[2] R2-2500011, “Reply LS on applicable functionality reporting for beam management UE-sided model”, TSG WG1, Athens, Greece, February 2025

[3] R2-2500999, “Report on [POST128][019][AI PHY] NW side data collection (Nokia)”, Nokia, Athens, Greece, February 2025

[4] R1-2410737, “FL summary #4 for AI/ML in beam management”, Samsung (moderator), Orlando, US, November 2024

[5] R2-2500918 “Running CR for SONMDT features”, Ericsson, Athens, Greece, February 2025