3GPP TSG-RAN WG2 Meeting #129	R2-250xxxx

Athens, Greece, 17-21 February 2025

Agenda item:		8.4.4

Source:				CATT

Title:			[AT129][204][LPWUS] Proposals for reply to R1 LS R2-2500012/R1-2410909 (CATT)

Document for:	Discussion and Decision

1	Introduction

This is to discuss the questions in R1 LS R2-2500012/ R1-2410909 [1] and whether short DRX is supported together with LP-WUS according to the offline discussion [204] scope as following.

[AT129][204][LPWUS] Proposals for reply to R1 LS R2-2500012/ R1-2410909 (CATT)

Scope: Discuss R1 questions and try to form answers to the questions, can also discuss whether short DRX is supported together with LPWUS

Intended outcome: Summary/Proposals in R2-2501444.

Deadline: before Thursday CB

2	Participants

| Company Name | Participant name | Contact/email address |
| --- | --- | --- |
| Qualcomm | Jianhua Liu | jianhua@qti.qualcomm.com |
| Apple | Fangli XU | Fangli_xu@apple.com |
| LGE | Hanul Lee | hanul.lee@lge.com |
| NTT docomo | Kenichiro Aoyagi | Kenichirou.aoyagi.yv@nttdocomo.com |
| Huawei, HiSilicon | Yiru Kuang | kuangyiru@huawei.com |
| Sony | Anders Berggren | Anders.Berggren@sony.com |
| InterDigital | Samuli Turtinen | samuli.turtinen@interdigital.com |
| NEC | Rao | shi_rao@nec.cn |
| Lenovo | Shwetha Sreejith | ssreejith1@lenovo.com |
| Vivo | Chenli | Chenli5g@vivo.com |
| Nokia | Sunyoung Lee | sunyoung.lee@nokia.com |
| Ericsson | Martin van der Zee | martin.van.der.zee@ericsson.com |
| CATT | Da Wang | wangda@catt.cn |
| Sharp | LIU Lei | lei.liu@cn.sharp-world.com |
| CMCC | Fang Xie | xiefang@chinamobile.com |
| Xiaomi | Haitao Li | Lihaitao11@xiaomi.com |
| Samsung | Byounghoon Jun | bh14.jung@samsung.com |

3	Discussion

According the LS, RAN1 asked RAN2 about the following questions:

- Overall Description: In RAN#119, RAN1 made following conclusion and agreement to use existing L2 signalling to stop C-DRX related timers and resuming LP-WUS monitoring: Conclusion - For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask whether RAN2 has same understanding or not. LS in R1-2410909. Agreement - For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution - Use existing DRX Command MAC CE and Long DRX Command MAC CE - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask the feasibility of the above potential solution. LS in R1-2410909.. Regarding the above conclusion on Option 1-1, RAN1 would like to ask RAN2 whether RAN2 has same understanding or not. Regarding the above agreement on Option 1-2, RAN1 would like to ask RAN2 the feasibility of the above potential solution.

The questions in RAN1 LS focus on the following aspects:

- Whether short DRX is supported together with LP-WUS;

- Whether existing (long) DRX Command MAC CE can be used to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group in Option 1-1 and stop the new timer and drx-InactivityTimer for the DRX group in Option 1-2.

3.1	Short DRX cycle

According to the online discussion, RAN2 discussed whether short DRX cycle is supported together with LP-WUS.

Discussions Whether short DRX is supported with LPWUS - Ericsson suggest to discuss whether short DRX is support with LPWUS from R2 point of view. ZTE think it is support. - CATT think for O1-2 short DRX is not needed, while for O1-1 we need some further analysis because there is potential impact on PS gain and also complexity. OPPO, LG E agree. OPPO and LG E think DCP and short DRX are not configured simultaneously in legacy. Nokia agree on O1-1. - Xiaomi do not see a need to restrict the use of short DRX, and think it is R1 decision. - Apple do not see a benefit of support short DRX, there is no motivation. NEC agree. QC ok to not have short DRX at all. Lenovo. - Nokia think for O1-2 it can be supported. - Ericsson think we need further discussion. - DCM and Lenovo think this issue is not directly related to R1 questions.

In the following, we discuss whether short DRX cycle supported together with LP-WUS for Option 1-1 and Option 1-2 separately. This issue is related with RAN1 LS [1] for the conclusion/agreement that when UE is not in C-DRX active time, UE monitors LP-WUS.

3.1.1	Option 1-1

It has been agreed for Option 1-1 that:

For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline.

The behaviour of DCP and short DRX cycle in TS 38.321 [2]and TS 38.213[3] is shown as following:

- In TS 38.321, when short DRX cycle is used, the UE will start drx-onDurationTimer based on the short DRX cycle without checking DCP. When long DRX cycle is used and DCP is configured, the UE will check if DCP indication associated with the current DRX cycle received from lower layer indicated to start drx-onDurationTimer.

- In 38.213, the value “0” or “1” for the Wake-up indication bit in DCI format 2_6 (DCP in MAC) indicates not to start or start the drx-onDurationTimer for the next long DRX cycle.

Based on the spec, the rapporteur makes the following assumptions:

- DCP is only used for long DRX cycle indicating whether to start the drx-onDurationTimer for the next long DRX cycle. I.e., DCP is only applied when UE is using long DRX cycle.

Companies are invited to discuss the following question.

Q1: Do you agree that DCP is only applied when UE is using long DRX cycle?

The intention of Q1 is just align the common understanding of DCP behaviour among companies. If companies have the same understanding, maybe we don’t need agreement on this.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes |  |
| Apple | Yes |  |
| LGE | Yes |  |
| NTT docomo | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Sony | Yes |  |
| InterDigital | Yes |  |
| NEC | Yes |  |
| Lenovo | Yes |  |
| vivo | Yes | RAN1 has the corresponding agreement in Rel-16 that: The PDCCH-based power saving signal/channel is not applicable for the short DRX cycle If there is also short DRX cycle configured for the UE |
| Nokia | Yes |  |
| Ericsson | Yes | We also have the understanding that the NW would not configure dcp-Config-r16 when short DRX cycle is configured. During the DCP time frame RAN2 assumed that DCP is also supported with short DRX: RAN2#107bis meeting (Chongqing, 14th – 20th October 2019): Agreements => From RAN2 perspective, it is desirable to support WUS for both short and long DRX and it can be configurable by the network. However, RAN2 will follow the final RAN1 decision. => Ask RAN1 if there are technical feasibility concerns to support WUS for short DRX RAN1 replied in RAN2#109-e (R2-2000017): The following opinions were expressed (not necessary exhaustive) - 1st opinion: - No RAN1 spec impacts were identified - 2nd opinion: Potential RAN1 spec impacts - Whether SearchSpace could be used for short and long DRX - Whether one PS_offset could be configured for short and long DRX - Whether UE procedures with dynamic triggering of short DRX RAN1 can not reach consensus on whether or not there is benefit, technical feasibility concern, or RAN1 spec impact of introducing WUS for short DRX. RAN1’s working assumption as communicated in LS R1-1911475 that WUS is not applicable for the short DRX cycle If both long and short DRX cycles are configured for the UE still stands. |
| CATT | Yes |  |
| Sharp | Yes |  |
| CMCC | Yes |  |
| Xiaomi | Yes |  |
| Samsung | Yes |  |

Summary:

Yes: 17; No: 0.

Observation: Companies have the same understanding on the DCP behaviour, i.e., DCP is only applied when UE is using long DRX cycle.

RAN2 assumed the solutions/ operations introduced for DCP mechanism was taken as baseline for Option 1-1. Companies are invited to discuss the following questions.

Q2: Do you agree LP-WUS is only applied when UE is using long DRX cycle for Option 1-1?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | RAN2 agreed taking DCP-like mechanism for Option 1-1. |
| Apple | Yes |  |
| LGE | Yes |  |
| NTT docomo | Yes |  |
| Huawei, HiSilicon | Maybe No? | From RAN2 perspective we understand short DRX cycle can be supported with LP-WUS without additional spec impacts, so it is unnecessary to have such restriction. For option 1-1, some companies mentioned there is no motivation to support LP-WUS in short DRX cycle, but we understand short DRX cycle only means the short latency, but it dose not mean high data rate, when the data arrives at gNB it needs to transmit to the UE ASAP, but it does mean UE will have data for every cycle/onduration, thus, LP-WUS is useful to skip the frequent onduration when there is no real data. On another hand, if the NW really thinks the LP-WUS is useless for short DRX cycle, it can disable LP-WUS when configures short DRX cycle, we can leave it to NW implementation. But if the majority prefer to do the exactly like DCP, we can accept. |
| Sony | Yes |  |
| InterDigital |  | From RAN2 point of view, applying LP-WUS also with Short DRX cycle should allow improved power saving opportunity for the UE rather than always starting the drx-onDurationTimer with Short DRX cycle. |
| NEC | Yes | We also need another question that whether short DRX cycle can be configured with LP-WUS option 1-1. If short DRX cycle can be configured, LP-WUS is only applied for long DRX cycle (this is just to follow DCP mechanism). If short DRX cycle cannot be configured, then no doubt that LP-WUS is only applied for long DRX cycle. |
| Lenovo | Yes | Following legacy DCP principle is fine for Option 1-1. |
| vivo | Yes | DCP mechanism could be reused for Option 1-1 |
| Nokia | Yes |  |
| Ericsson | No | From RAN2 perspective there is no reason to disallow short DRX with option 1-1. |
| CATT | Yes | To answer Huawei and Ericsson, if LP-WUS can apply with short DRX, from spec impact, the key problem is how to design the offset between LP-WUS and short DRX cycle. Based on current RAN1 design, the offset between LP-WUS and short DRX cycle may be larger than the duration of short DRX cycle, which may impact RAN1 design. Thus, we prefer to follow DCP mechanism. To answer NEC, the DCP mechanism is on matter NW configure short DRX or not, LP-WUS is only applied for long DRX cycle. Whether configure short DRX is depended on NW implementation. Thus, in this question, we don’t touch the configuration issue. |
| Sharp | Yes |  |
| CMCC | Yes |  |
| Xiaomi | No | RAN2 has no reason not allowing short DRX with option 1-1. |
| Samsung | Yes |  |

Summary:

Yes: 14; No: 3.

Proposal 1: Propose to make a working assumption, i.e., LP-WUS is only applied when UE is using long DRX cycle for Option 1-1, which is following DCP behaviour. And send LS to RAN1 to further check.

If the answer to Q2 is no, i.e. LP-WUS can be applied to short DRX cycle, there are some concerns in RAN1 including:

- How to define location of LP-WUS, e.g., it may have relationship with short DRX cycle;

- It requires a huge overhead in the DL.

Companies are invited to discuss the following questions.

Q2a: If your answer to Q2 is no, do you agree that RAN2 ask RAN1 about whether LP-WUS can be applied to short DRX for Option 1-1?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | See comments | We don’t think there are such concerns, since option 1-1 is similar as DCP, the LP-WUS occasion(s) is derived based on the location of onduration, which is irrelevant to DRX cycle. For the signalling overhead, if the NW cannot accept it, it can disable LP-WUS when configures short DRX cycle. So we don’t think we need to raise these concerns in the LS. But we are ok to ask RAN1 to feedback if they have concern for supporting LP-WUS and short DRX cycle, just a simple question, RAN1 can further discuss if real issues are identified. |
| Ericsson | See comments | LP-WUS with short DRX and option 1-1 can provide further savings. The LP-WUS location with short DRX is defined in a similar way as it is defined with long DRX. We do not think there is any issue. The network resources are only used when NW sends PEI, i.e. does not necessarily increase the DL load. |
| Xiaomi | yes |  |

Summary:

See above.

In RAN1 LS, RAN1 made the following conclusion and ask RAN2 for further check:

Conclusion - For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask whether RAN2 has same understanding or not. LS in R1-2410909.

Based on the discussion in Q2, companies are invited to discuss the following question.

Q3: Do you agree UE monitors LP-WUS outside C-DRX active time when long DRX cycle is used for Option 1-1?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes |  |
| Apple | Yes | Based on the assumption that the simultaneous configuration of short DRX cycle and LP-WUS is not supported, we can simply confirm that UE monitors LP-WUS when UE is not in C-DRX active time. |
| LGE | Yes |  |
| NTT docomo | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Sony | Yes |  |
| InterDigital | Yes |  |
| NEC | Yes |  |
| Lenovo | Yes |  |
| vivo | with comments | Generally, we agree with it. However, for the case that PDCCH skipping is used together with LP-WUS, UE may not monitor PDCCH within the C-DRX actice time, it is possible that UE could also monitor the LP-WUS within C-DRX active time if UE doesn’t monitor PDCCH. |
| Nokia | Yes |  |
| Ericsson | Yes |  |
| CATT | Yes | To answer Apple, based on the discussion on Q1 and Q2. The currently assumption is short DRX cycle and LP-WUS can be simultaneous configured. But LP-WUS doesn’t apply for the short DRX cycle. Thus, we cannot simply confirm the RAN1 agreement. If Q2 is agree, we can say UE monitors LP-WUS outside C-DRX active time only when long DRX cycle is used for Option 1-1. But it depends on the discussion of Q2. |
| Sharp | Yes |  |
| Xiaomi | yes |  |
| Samsung | Yes |  |

Summary:

Yes: 17; No: 0.

Proposal 2: For Option 1-1, UE monitors LP-WUS outside C-DRX active time at least when long DRX cycle is used. FFS on PDCCH skipping or short DRX cycle is used.

3.1.2	Option 1-2

In RAN1 LS, RAN1 made the following agreements and ask RAN2 for further check:

Agreement - For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution - Use existing DRX Command MAC CE and Long DRX Command MAC CE - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask the feasibility of the above potential solution. LS in R1-2410909.

It was agreed in RAN2 that the drx-onDurationTimer is not started with Option 1-2 LP-WUS. Hence, the UE behaviour regarding PDCCH monitoring for the long DRX cycle and short DRX cycle is the same. And it has been agreed that LP-WUS monitoring is performed at least outside legacy C-DRX Active Time in RAN2 for Option 1-2. Thus, the UE will monitor LP-WUS at least outside legacy C-DRX Active Time no matter long DRX cycle or short DRX cycle is used for Option 1-2.

Companies are invited to discuss the following question.

Q4: Do you agree the UE will monitor LP-WUS outside C-DRX Active Time no matter long DRX cycle or short DRX cycle is used for Option 1-2?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | See comments | Since LP-WUS option 1-2 is introduced to reduce the latency due to the legacy long DRX cycle, and it is assumed option 1-2 can achieve the effect of short DRX cycle. So short DRX does not to be configured for LP-WUS option 1-2. That means, e.g, if LP-WUS is applied for short DRX cycle, the LP-WUS cycle should be shorter than short DRX cycle, then short DRX cycle has no effect. |
| Apple | Comments | In LP-WUS option 1-2, UE doesnot start onduration-timer for PDCCH monitoring, so that the configuration of short DRX cycle and long DRX cycle does not impact the CDRX active time. If we only consider the impact on CDRX active time, we donot need either long DRX cycle and short DRX cycle. |
| LGE | Comment | For Long DRX cycle, YES, but for Short DRX cycle, we don’t think that Short DRX cycle is not used for Option 1-2, with following reasons. - From PDCCH monitoring perspective, two DRX cycle is not needed for Option 1-2 because PDCCH monitoring is performed based on LP-WUS occasion. - From CSI reporting perspective, there is no strong motivation to use two DRX cycle. If RAN2 agreed not to use Short DRX cycle for Option 1-2, it seems reasonable not to use Short DRX cycle to simplify modelling. |
| Huawei, HiSilicon | Yes | From RAN2 perspective we understand short DRX cycle can be supported with LP-WUS without additional spec impacts, so it is unnecessary to have such restriction. For option 1-2, onduration timer won’t be started, the operation in long or short DRX cycle is the same. |
| Sony | Comment | Agree with Apple |
| InterDigital | Yes |  |
| NEC | Yes | It is common understanding that the UE will monitor LP-WUS outside of active time, and in option 1-2, no matter it is long DRX cycle or short DRX cycle, onD timer (which is not started) does not count as active time. Even if we don’t support short DRX cycle with LP-WUS for option 1-2… |
| Lenovo | Yes, with comments | Generally, we agree the principle that UE will monitor LP-WUS outside C-DRX Active Time no matter long DRX cycle or short DRX cycle is used for Option 1-2. But whether LP-WUS can be configured with short DRX cycle for option 1-2 is another issue that needs to be clarified. In our view, we do not see strong motivation to configure the short DRX cycle for LP-WUS in option1-2. Short DRX cycle is configured by network depending on the expected traffic in the UE (most likely when the data traffic pattern is bursty). Using LP-WUS with short cycle will degrade the power saving gain since the UE is frequently waking up. Additionally, if short DRX cycle is configured with LP-WUS for option1-2, it needs to be further designed how to apply the LP-WUS for the short DRX cycle, e.g., whether LP-WUS configuration based on short DRX cycle is needed or not in addition to the LP-WUS configuration for Long DRX Cycle, which may cause complex design. Some companies raised the point regarding CSI reporting with short cycle in Option 1-2, but we think that Option 1-2 is anyway intended for latency reduction and hence the CSI reporting will not be largely impacted even when CSI-mask is configured. Furthermore, the network can always probe the UE for aperiodic or semi-persistent CSI reports regardless of periodic CSI reporting configuration. A unified solution for opt1-1 and opt1-2 on short DRX cycle is preferred, i.e., LP-WUS is not configured with short DRX cycle for opt1-2. |
| vivo | Yes | Same as Q3. |
| Nokia | Yes |  |
| Ericsson | Yes | Short DRX and option 1-2 might make sense if we would introduce a “short” and “long” LP-WUS periodicity. The short LP-WUS periodicity is used when the UE is in short DRX, otherwise the long LP-WUS periodicity is used. Otherwise we are not sure what the benefit would be i.e. similar view as QC above. |
| CATT | Yes |  |
| Sharp | Yes |  |
| CMCC | Comments | It’s common understanding that UE will monitor LP-WUS outside C-DRX Active Time no matter long DRX cycle or short DRX cycle is used for Option 1-2, since drx onduration timer is not started in Option 1-2. But whether to support the simultaneous configuration of Option 1-2 and short DRX should be further discussed. Now that drx onduration timer is not started in Option 1-2, we think there’s no need to configure the two features simultaneously, considering the potential complexity like CSI/L1-RSRP reporting. |
| Xiaomi | yes |  |
| Samsung | Yes |  |

Summary:

Yes: 11; Comments: 5.

Regarding the companies view, most comments are for the configuration of Option 1-2 and short DRX cycle. Rapporteur thinks it’s depended on the below discussion on Q5, which is no consensus. Rapporteur suggests we can first agree the UE behaviour of Option 1-2 then discuss the configuration issues.

Proposal 3: For Option 1-2, UE monitors LP-WUS at least outside C-DRX Active Time. FFS on whether the short DRX cycle is used.

And it was agreed in RAN2#128 meeting that:

For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time.

The intention is to specify how the UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time.

Some companies think if short DRX cycle can work simultaneously with LP-WUS in Option 1-2, the short DRX cycle will be used without starting drx-onDurationTimer when DRX command MAC CE is received. This may bring impact on periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer. However, some companies think it is not necessary to report periodic CSI/L1-RSRP with short DRX cycle, which brings complexities to the network implementation and low power saving gain for Option 1-2. Even though the periodic CSI/L1-RSRP need shorter period to be reported, the network can configure shorter Long DRX cycle to report the periodic CSI/L1-RSRP.

Companies are invited to discuss the following questions.

Q5: Which option do you prefer for periodic CSI/L1-RSRP report during the time given by the configured drx-onDurationTimer of short DRX cycle for Option 1-2:

Option 1: The UE needs to report periodic CSI/L1-RSRP during time given by the configured drx-onDurationTimer of short DRX cycle;

Option 2: The UE does not need to report periodic CSI/L1-RSRP during time given by the configured drx-onDurationTimer of short DRX cycle;

| Company | Option 1 or Option 2 | Comments |
| --- | --- | --- |
| Qualcomm | See comment | As commented in Q4, we think short DRX does not need to be configured for LP-WUS Option 1-2 |
| Apple | See comment | From CSI reporting during onduration duration perspective, we think only one DRX cycle (i.e. long DRX cycle) is sufficient. Therefore, in LP-WUS Option 1-2, the short DRX cycle configuration is not needed from CSI reporting perspective. |
| LGE | Comment | As mentioned in Q4, we think that Short DRX cycle is not used with Option 1-2. |
| Huawei, HiSilicon | See comments | No special handling is needed for short DRX cycle, the operation of periodic CSI/L1-RSRP reporting should be same regardless of short or long DRX cycle. |
| InterDigital | Comment | The CSI/L1-RSRP reporting would be the only reason why to configure Short DRX cycle for Option 1-2. So it should be rather discussed whether there are benefits of UE transmitting more frequent CSI/L1-RSRP reports after data activity (ie., based on the Short DRX cycle) or not. |
| NEC | See comment | First we need to discuss whether short DRX cycle can be configured with LP-WUS option 1-2 simultaneously. Since we already agreed that legacy onD timer will not be started for option 1-2, it is useless to configure short DRX cycle. While some companies think it can be also used for CSI report. In our understanding, as long as the UE is in active timer (e.g., new active timer or other active timer, e.g., IAT timer), the UE can report CSI, also the UE can report CSI during legacy long DRX cycle onD timer by NW control. More report window triggered by short DRX cycle is not necessary. We don’t see the benefit of supporting short DRX cycle for option 1-2. But if short DRX is supported, we prefer option 1 by the agreed NW control (i.e., For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, including long DRX and short DRX cycle, for the case when UE is outside C-DRX active time. ) |
| Lenovo | Refer comments | We prefer not to configure LP-WUS and short DRX cycle simultaneously for option 1-2. |
| vivo | See comments | We prefer follow the same mechanism on CSI/L1-RSRP report for both short DRX cycle and long DRX cycle for simplicity. Anyway, the configuration is up to network implementation and we don’t see the complexities to the network implementation. As for the power saving gain, NW could disable the report in this case based on the following agreement: network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer. If NW configures short DRX cycle with the LP-WUS Option 1-2, and enable the periodic CSI/L1-RSRP report in this case, it means NW needs the report, there is no motivation for UE not to follow the NW configuration. |
| Nokia | Comments | First, DRX should be configured at least to determine the location of on duration since RAN2 agreed that CSI/L1-RSRP is reported based on the configured on duration (even without starting the timer itself). Second, the question should rather be what the harm is to report the CSI/L1-RSRP according to the Short DRX cycle while the UE is monitoring the PDCCH. In legacy, while the UE is in Active Time, with CSI mask, the network is able to distribute the CSI/L1-RSRP reporting from multiples UEs so that they are not overlapped each other. Even with LP-WUS configuration, if the UE receives a PDCCH while monitoring PDCCH during the new active time, the UE starts legacy active time. Unless any critical problem identified using short Cycle during legacy active time, it should be allowed for the network to configure short cycle to get frequent CSI/L1-RSRP reporting for better scheduling. |
| Ericsson | Option 1 | In case we agree to support short DRX with LP-WUS we prefer to keep the legacy behavior w.r.t. CSI/L1-RSRP reporting. |
| CATT | Comments | We also don’t support to configure LP-WUS and short DRX cycle simultaneously for option 1-2. And we don’t see the necessary to report periodic CSI/L1-RSRP during short DRX cycle. |
| Sharp | Comments | If there is no obvious benefits, it is better to keep the same mechanism as option 1-1, i.e. not be configured with short DRX cycle simultaneously. |
| CMCC | Comments | As answered in Q4, we prefer to not configure Option 1-2 and short DRX simultaneously. |
| Xiaomi | Option 1 |  |

Summary:

Regarding the different views from companies, Rapporteur proposes to discuss this issue in the next meeting.

Q6: If Option 2 in Q5 is agreed, do you agree that short DRX cycle and LP-WUS can’t be configured simultaneously in Option 1-2?

| Company | Yes/No | Comments |
| --- | --- | --- |
| NEC | Yes | Short DRX should not be configured with LP-WUS simultaneously for option 1-2. Since we already agreed that legacy onD timer will not be started for option 1-2, it is useless to configure short DRX cycle. While some companies think it can be also used for CSI report. In our understanding, as long as the UE is in active timer (e.g., new active timer or other active timer, e.g., IAT timer), the UE can report CSI, also the UE can report CSI during legacy long DRX cycle onD timer. More reporting window triggered by short DRX cycle is not necessary. We don’t see the benefit of supporting short DRX cycle. |
| vivo | No | See above. |
| CATT | Yes |  |
| Nokia | No | It is always up to NW to configure it or not. Why should we remove it at all? No justification is shown to remove the configurability. |

Summary:

See above.

3.2	DRX timers

In RAN1 LS [1], RAN1 made the following conclusion/agreement and ask RAN2 for further check:

Conclusion - For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask whether RAN2 has same understanding or not. LS in R1-2410909. Agreement - For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution - Use existing DRX Command MAC CE and Long DRX Command MAC CE - When UE is not in C-DRX active time, UE monitors LP-WUS - Send LS to RAN2 to ask the feasibility of the above potential solution. LS in R1-2410909.

Regarding (Long) DRX command MAC CE, they can be used to stop drx-onDurationTimer and drx-InactivityTimer for the DRX group in legacy DRX operation. For Option 1-2, it was agreed in RAN2#128 meeting as following:

For option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS.

Hence, it is straightforward that RAN2 can confirm the RAN1 conclusions/agreements and feedback to RAN1.

Companies are invited to discuss the following questions.

Q7: Do you agree that (Long) DRX command MAC CE can be used with option 1-1 to stop drx-onDurationTimer and drx-InactivityTimer?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes |  |
| Apple | Yes |  |
| LGE | Yes |  |
| NTT docomo | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Sony | Yes |  |
| InterDigital | Yes |  |
| NEC | Yes |  |
| Lenovo | Yes |  |
| vivo | Yes | It is aligned with the current MAC spec to stop drx-onDurationTimer and drx-InactivityTimer when (Long)DRX command MAC CE is received. |
| Nokia | Yes |  |
| Ericsson | Yes |  |
| CATT | Yes |  |
| Sharp | Yes |  |
| CMCC | Yes |  |
| Xiaomi | yes |  |
| Samsung | yes |  |

Summary:

Yes: 17; No: 0.

Proposal 4: RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-1 to stop drx-onDurationTimer and drx-InactivityTimer.

Q8: Do you agree that (Long) DRX command MAC CE can be used with option 1-2 to stop the new timer and drx-InactivityTimer?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes |  |
| LGE | Yes |  |
| NTT docomo | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Sony | Yes | But not sure about the legacy drx_InactivityTimer |
| InterDigital | Yes |  |
| NEC | Yes |  |
| Lenovo | Yes |  |
| vivo | Yes |  |
| Nokia | Yes to both timers |  |
| Ericsson | Yes |  |
| CATT | Yes |  |
| Sharp | Yes |  |
| CMCC | Yes |  |
| Xiaomi | yes |  |
| Samsung | yes |  |

Summary:

Yes: 17; No: 0.

Proposal 5: RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-2 to stop the new timer and drx-InactivityTimer.

4	Conclusion

Based on company feedback, the following is proposed:

Observation: Companies have the same understanding on the DCP behaviour, i.e., DCP is only applied when UE is using long DRX cycle.

Easily to be agreed:

Proposal 2: For Option 1-1, UE monitors LP-WUS outside C-DRX active time at least when long DRX cycle is used. FFS on PDCCH skipping or short DRX cycle is used.

Proposal 4: RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-1 to stop drx-onDurationTimer and drx-InactivityTimer.

Proposal 5: RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-2 to stop the new timer and drx-InactivityTimer.

Potential to be agreed

Proposal 1: Propose to make a working assumption, i.e., LP-WUS is only applied when UE is using long DRX cycle for Option 1-1, which is following DCP behaviour. And send LS to RAN1 to further check.

Proposal 3: For Option 1-2, UE monitors LP-WUS at least outside C-DRX Active Time. FFS on whether the short DRX cycle is used.

Proposal 6: Reply to RAN1 LS R2-2500012/R1-2410909, including corresponding RAN2 agreements, if any.

5	References

- R2-2500012	LS on LP-WUS operation in CONNECTED mode (R1-2410909; contact: NTT DOCOMO)	RAN1	LS in;

- 3GPP TS 38.321 Medium Access Control (MAC) protocol specification;

- 3GPP TS 38.213 Physical layer procedures for control.