3GPP TSG-RAN WG2 Meeting #128	draft R2-2501332

Athens, Greece, Feb. 17th – 21st, 2025

Source: 	RAN2 Vice Chairman (CATT)

Title:	Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others

## Organizational email discussion

[AT129][200] Organizational – Rel-18 MIMO, Rel-19 MIMO, LPWUS, and SBFD (RAN2 VC)

Scope:

a) Share plans for online/offline discussions during the meeting, and

b) Share draft session notes and agreements for review

#### 7.0.2.13 NR MIMO evolution

(NR_MIMO_evo_DL_UL-Core; leading WG: RAN1; REL-18; WID: RP-233028)

On precoder indication for 8-Port CG-PUSCH

R2-2500009	LS on Precoder Indication for 8-Port CG-PUSCH (R1-2410836; contact: Google)	RAN1	LS in	Rel-18	NR_MIMO_evo_DL_UL-Core	To:RAN2

Noted

R2-2500107	Introduction of RRC parameters for 8-port CG-PUSCH	Google	CR	Rel-18	38.331	18.4.0	5199	-	F	NR_MIMO_evo_DL_UL-Core

Revised in R2-2501441

R2-2500726	Correction on Precoder Indication for 8-Port CG-PUSCH	Ericsson	CR	Rel-18	38.331	18.4.0	5221	-	F	NR_MIMO_evo_DL_UL-Core

Merged in R2-2501441

Discussion

-	OPPO think Google CR is good baseline but point out some minor issue in CR coversheet. ZTE likes Ericsson CR better since it is simple.

[AT129][201][MIMOevo] Updated CR on RRC parameters for 8-port CG-PUSCH  (Google)

Scope: Updated the CR, taking into account R2-2500726 and comments from companies

Intended outcome: Updated CR in R2-2501441.

Deadline: before CB

R2-2501441	Introduction of RRC parameters for 8-port CG-PUSCH	Google, Ericsson	CR	Rel-18	38.331	18.4.0	5199	1	F	NR_MIMO_evo_DL_UL-Core

The CR is agreed.

On applied TCI states for PDCCH reception

R2-2500013	LS on condition of applying both indicated TCI states for PDCCH reception (R1-2410916; contact: Samsung)	RAN1	LS in	Rel-18	NR_MIMO_evo_DL_UL-Core	To:RAN2

Noted

R2-2500157	Correction to applied TCI state for mTRP PDCCH reception	Samsung	CR	Rel-18	38.331	18.4.0	5200	-	F	NR_MIMO_evo_DL_UL-Core

Revised in R2-2501447

Discussion

-	For change #1, HW suggest to use the usual wording for configuration restriction. Samsung OK to change but think the current wording is from R1 and similar wording is used today. Nokia think we can go with R1 wording.

-	For change #2, HW and Nokia think it is fine, while CATT think this is already clear in R1 spec, so no need to change.

R2-2501447	Correction to applied TCI state for mTRP PDCCH reception	Samsung	CR	Rel-18	38.331	18.4.0	5200	1	F	NR_MIMO_evo_DL_UL-Core

The CR is agreed.

On TDD UL/DL configuration for two TA

R2-2500709	Remaining details on TDD UL/DL Configuration for Two TA	Ericsson	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Noted

Proposal 1	Agree to add configuration for TDD-UL-DL-ConfigCommon for each additionalPCI.

Proposal 2	Agree to add the configuration for TDD-UL-DL-ConfigCommon in IE ServingCellConfig by extending the IE MIMOParam.

Proposal 3	Agree to use a new AddModList additionalTDDConfig-perPCI-ToAddModList-r18 for the configuration for TDD-UL-DL-ConfigCommon.

Proposal 4	Agree CR in R2-2500710 as baseline for the discussion.

R2-2500710	Correction on TDD UL/DL Configuration for Two TA	Ericsson	CR	Rel-18	38.331	18.4.0	5219	-	F	NR_MIMO_evo_DL_UL-Core

Revised in R2-2501442

[AT129][202][MIMOevo] Proposal on TDD UL/DL Configuration for Two TA (Ericsson)

Scope: Updated CR or set of questions to ask R1, if needed

Intended outcome: Updated CR (if agreeable) or summary with proposals in R2-2501442

Deadline: before CB

R2-2501442	Correction on TDD UL/DL Configuration for Two TA	Ericsson	CR	Rel-18	38.331	18.4.0	5219	1	F	NR_MIMO_evo_DL_UL-Core

Revised the CR in R2-2501448, adding “,” btw ‘Need N’ and ‘for a TDD cell’.

R2-2501448	Correction on TDD UL/DL Configuration for Two TA	Ericsson	CR	Rel-18	38.331	18.4.0	5219	2	F	NR_MIMO_evo_DL_UL-Core

The CR is agreed.

On UE capability pusch-DMRS8Tx-r18

R2-2501117	Correction on pusch-DMRS8Tx-r18	Huawei, HiSilicon	CR	Rel-18	38.331	18.4.0	5260	-	F	NR_MIMO_evo_DL_UL-Core

The CR is agreed

On supporting 8Tx in MAC

R2-2500410	Correction on supporting 8Tx in MAC specification - method 2	ASUSTeK	CR	Rel-18	38.321	18.4.0	1990	1	F	NR_MIMO_evo_DL_UL-Core	R2-2410174

-	Assustek explains that we do not have new agreement from R1.

The CR is postponed.

R2-2501238	Clarification On SP CSI Reporting Activation/Deactivation For 8Tx	R2	CR	Rel-18	38.321	18.4.0	2040	-	F	NR_MIMO_evo_DL_UL-Core	Withdrawn

R2-2501297	Clarification On SP CSI Reporting Activation/Deactivation For 8Tx	R2	CR	Rel-18	38.321	18.4.0	2043	-	F	NR_MIMO_evo_DL_UL-Core	Withdrawn

R2-2501301	Clarification On SP CSI Reporting Activation/Deactivation For 8Tx	R2	CR	Rel-18	38.321	18.4.0	2046	-	F	NR_MIMO_evo_DL_UL-Core

=> Withdrawn

## 8.4	Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

(NR_LPWUS-Core; leading WG: RAN1; REL-19; WID RP-241824)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.4.1	Organizational

LS, Rapporteur input, including workplan, etc.

LSin

R2-2500012	LS on LP-WUS operation in CONNECTED mode (R1-2410909; contact: NTT DOCOMO)	RAN1	LS in	Rel-19	NR_LPWUS	To:RAN2

=> The document is moved to 8.4.4

R2-2500050	LS Reply on LP-WUS subgrouping (S2-2412876; contact: Huawei)	SA2	LS in	Rel-19	NR_LPWUS-Core	To:RAN2, RAN3, CT1

R2-2500150	LR and MR operating frequencies	Vodafone, Huawei, HiSilicon,Vivo	discussion	Rel-19

R2-2500302	Discussion on SA2 LS on LP-WUS subgrouping	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

=> The above 3 documents are moved to 8.4.2

Running CR

Chair: No detailed discussions expected in this meeting. All running CRs for this WID should be created/updated after this meeting, and submitted to the next meeting for discussions/endorsement.

R2-2501092	Introduction of LP-WUS/LP-WUR	Ericsson	draftCR	Rel-19	38.331	18.4.0	F	NR_LPWUS-Core

Noted

Post meeting email discussions

[Post129][209][LPWUS] Running CR for TS 38.300 (Ericsson)

Intended outcome: Running CR for submission to the next meeting

Deadline:  Long

[Post129][210][LPWUS] Running CR for TS 38.321 (Apple)

Intended outcome: Running CR for submission to the next meeting

Deadline:  Long

[Post129][211][LPWUS] Running CR for TS 38.331 (vivo)

Intended outcome: Running CR for submission to the next meeting

Deadline:  Long

[Post129][212][LPWUS] Running CR for TS 38.304 (CATT)

Intended outcome: Running CR for submission to the next meeting

Deadline:  Long

[Post129][213][LPWUS] Running CR for TS 37.340 (ZTE)

Intended outcome: Running CR for submission to the next meeting

Deadline:  Long

### 8.4.2	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

Procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping, entry/exit condition for LP-WUS monitoring, and separate band issues following WF in RP‑243266

Separate band issues

R2-2500150	LR and MR operating frequencies	Vodafone, Huawei, HiSilicon,Vivo	discussion	Rel-19

Noted

Proposal 1: Standardize solution 2 from RAN Plenary WF in release 19.

R2-2501006	Discussion on RRC CONNECTION load balancing for LP-WUS capable UEs	NTT DOCOMO INC..	discussion	Rel-19

Noted

Proposal 1: Indicate some information on SIB(SIB1) for the LP-WUS capable UEs to prioritize the current LP-WUS band is the highest priority in which the UE monitors LP-WUS.

Proposal 2: Dedicated signal can be applied to support LP-WUS capable UE to camp on LP-WUS Band

R2-2501017	Further considerations on LP-WUS operation in IDLE INACTIVE mode	CMCC	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 4: Solution 3 is used for LR and MR operating on different bands case in Rel-19.

R2-2501089	Procedure and configuration of LP-WUS for IDLE and INACTIVE mode	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 9: For the case that UE support LP-WUS reception in a subset of bands among all the bands supported by UE, both solution 1 and solution 3 are supported. It is up to network to prioritize frequencies supporting both MR and LR, i.e. solution 1, and/or redirect UE to a frequency supporting MR only if frequencies supporting both MR and LR are overloaded, i.e. solution 3.

R2-2501252	LP-WUS operation in IDLE/Inactive state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

Noted

Proposal 1	Reuse existing dedicated frequency priority to distribute UEs to other frequency bands to reducing LP-WUS cell overload.

Discussion

- VDF think the problem is the load in low band and think only solution 2 can solve it. VDF think it is critical to solve this issue otherwise it is useless. HW agree. HW think if we only allows some of the UEs to enjoy the feature then the benefit is less. Apple see some benefit of O2.

- IDT thinks solution 2 will impact paging procedure and think it is not within scope of the work.

- LG E think the number of UEs implementing LPWUS may not be huge, so the load issue is not so serious. QC share this view, and think UE has different capabilities and this may be issue for some UEs. ZTE, CATT agree.

- OPPO think it is beneficial for UEs to camp on LR bands, but on the other hand the load issue may not be so critical. So OPPO prefer solution 1.

- Samsung have sympathy to VDF proposal but would like to understand why the issue is so critical, and think solution 1 is better option.

- CATT think solution 2 may change RACH procedure and may involve R4 work, not sure if we can finish this in R19. MTK, Apple agree.

- MTK not sure about motivation of solution 2.

- Xiaomi do not support solution 2 and think it is new feature with big impact.

- Xiaomi think solution 1 cannot work alone, so we also need solution 3.

- Lenovo think the solution should be future proof, even if we will not have large UE number in the near future but it can grow. Lenovo prefer long term solution.

- Ericsson think we should confirm from UE vendor first whether there is real issue that UE only support LPWUS in low band. Ericsson do not think we need to discuss solution 2 before the issue itself is confirmed.

- NEC support solution 2, but ok with solution 1 + 3. Vivo has similar view.

- Vivo think we should respect operator concern and think different solutions are for different scenarios.

- Nokia ask whether solution 1 is needed for solution 2.

[AT129][203][LPWUS] Proposals for LPWUS separate band issue (vivo)

Scope: Discuss the issue, pros and cons of existing solutions, and try to form a WF

Intended outcome: Summary/Proposals in R2-2501443.

Deadline: before Thursday CB

R2-2501443	Discussion report on [AT129][203][LPWUS]	vivo	NR_LPWUS-Core

Noted

Proposal 1: RAN2 understands that UE can report which band(s) supported by LR to NW.

Proposal 2: RAN2 understands that any potential overload issues could be addressed by current mechanism in spec.

Proposal 3: Discuss whether we need to inform RAN1/RAN4 in CB session.

Discussion

-	HW think solution 3 has some drawback of limited useage of LPWUS and want to also show that in the conclusion.

RAN2 understands that UE can report which band(s) is supported by LR to NW.

RAN2 understands that any potential overload issues could be addressed by current mechanism in spec.

Send LS to RAN1 and RAN4 to inform the agreements.

[Post129][214][LPWUS] LS to RAN1/RAN4 on agreements of separate band issue (vivo)

Intended outcome: Approve the LS.

Deadline: short

Sub-grouping

R2-2500456	Discussion on LP-WUS procedure and configuration	OPPO	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 3	The formula for UE_ID based PEI subgrouping is reused for UE_ID based subgrouping for LP-WUS.

R2-2501252	LP-WUS operation in IDLE/Inactive state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

Noted

Proposal 9	The formula for UE_ID based PEI subgrouping is taken as baseline for LP-WUS UE_ID based PEI subgrouping.

Proposal 10	If enhancement is needed, then option 1 can be acceptable.

R2-2500143	General considerations on the procedure for RRC_IDLE_INACTIVE	Xiaomi Communications	discussion

Noted

Proposal 3	For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e.,

SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where

-	UE_ID is related to 5G-S-TMSI,

-	N is the number of total paging frames in one DRX cycle,

-	Ns is the number of the PO for a PF,

-	Np is the number of subgroupNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1,

-	subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

R2-2500589	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 5: Agree the UE_ID based LP-WUS subgrouping formula as below:

SubgroupID = (floor (UE_ID/(N*Ns*K)) mod subgroupsNumForUEID_LP-WUS) + (subgroupsNumPerPO_LP-WUS – subgroupsNumForUEID_LP-WUS)

•	UE_ID is related to 5G-S-TMSI.

•	N is the number of total paging frames in one DRX cycle.

•	Ns is the number of the PO for a PF.

•	K is the configured value in SIB1; if not configured, K is 1.

•	subgroupsNumForUEID_LP-WUS is the subgroup number for UE_ID based LP-WUS subgrouping.

•	subgroupsNumPerPO_LP-WUS is the total number of LP-WUS subgroups in a PO.

Discussions on the options

- Samsung, LG E support O3. LG E think by Option 2 the number of subgroup is different for PEI and LPWUS. IDT agree.

- ZTE support Xiaomi proposal.

- CATT think Apple proposal allows full configuration flexibility. HW support Apple proposal.

- Lenovo agree with QC proposal. Lenovo think NW implementation can choose proper number of subgroups and it is easy to achieve.

- Ericsson do not agree with O3.

- NEC slightly prefer O1.

Discussion on whether we send LS

- CATT suggest to send this agreement to R3. Xiaomi, NEC agree that we can send LS. Ericsson think we can discuss option to avoid potential impact to other WG.

- CATT reports that in offline discussion one company not OK to send LS to R1. CATT explains RAN3 is now waiting for our progress.

For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e.,

SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where

-	UE_ID is related to 5G-S-TMSI,

-	N is the number of total paging frames in one DRX cycle,

-	Ns is the number of the PO for a PF,

-	Np is the number of subgroupNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1,

-	subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

Issues related to SA2 LS (R2-2500050/S2-2412876)

R2-2500050	LS Reply on LP-WUS subgrouping (S2-2412876; contact: Huawei)	SA2	LS in	Rel-19	NR_LPWUS-Core	To:RAN2, RAN3, CT1

Noted

On LP-WUS related terminology

R2-2500302	Discussion on SA2 LS on LP-WUS subgrouping	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

“LP-WUS” terminology

Proposal 1: RAN2 to confirm that the “LP-WUS” terminology can be used, since LP-WUS is received with low power consumption by LP-WUR.

Proposal 2: RAN2 sends reply LS to SA2/RAN3/CT1, and CC RAN1 and RAN4 on “LP-WUS” terminology.

Noted

RAN2 sends reply LS to SA2/RAN3/CT1, and CC RAN1 and RAN4 to confirm the “LP-WUS” terminology.

On UE Radio Capability for Paging Information

R2-2500343	Discussion on LP-WUS WUR in RRC_IDLE INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 21: Reply LS to SA2 that it is up to network implementation to address the misalignment issue between the gNB and the UE on the UE capability of supporting LP-WUS in case the Initial Registration is performed on a pre-Rel-19 gNB, e.g., all legacy gNB is upgraded to support the latest ASN.1 for the UE capability on LP-WUS. No specification impact is foreseen.

R2-2500302	Discussion on SA2 LS on LP-WUS subgrouping	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 3: RAN2 to confirm that the UE will miss the paging message due to the mismatch of using LP-WUS between UE and gNB, if UE does not know the gNB is not able to forward the LP-WUS capability to CN.

Proposal 4: It is suggested to consider to specify a solution to solve the mismatch of LP-WUS capability between the UE and the network.

Proposal 5: It is suggested to focus on LP-WUS aspect first and a consistent solution for all identified features with the same issue can be considered in general agenda item.

R2-2501093	LP-WUS in Idle and Inactive	Ericsson	discussion	Rel-19	NR_LPWUS-Core	R2-2410085

Noted

Proposal 5	The LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container.

R2-2501089	Procedure and configuration of LP-WUS for IDLE and INACTIVE mode	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 8:RAN2 reply the SA2 LS to indicate that:

…

RAN2 will include the LP-WUS capability(e.g. AS capability) in ue-RadioPagingInfo-r17 IE of UECapabilityInformation message, the ue-RadioPagingInfo-r17 IE is already included in UERadioPagingInformation container(which is included in RAN3 IE of UE Radio Capability for Paging IE) and is transparently passed from gNB to AMF, from AMF to gNB and from one gNB to another gNB. Thus, the LP-WUS capability will not be lost if the gNB is rel-17 or later.

RAN2 confirms that the LP-WUS capability is independent from the other features (e.g. can work with or without other features that are signalled within the UE Radio Capability for Paging Information).

Discussion

- QC think this can be solved by NW implementation, and think it is mainly based on supporting the IE structure in ASN. Samsung, NEC agree.

- OPPO share HW’s view that we need to solve the paging missing issue and we need a specified solution.

- Xiaomi think we had similar discussion in R17 PEI and think similar here in R2 we do not need to do anything.

- CATT think we first discuss whether there is an issue, and think for pre R17 gNB there may be issue. HW agree.

- Lenovo not sure what is the problem and think the capability is anyway stored by NW.

- HW agree with Ericsson proposal.

- HW think we need to also consider x-vendor cases so it is not always ok to rely on implementation.

- Ericsson think NW should handle this case when new feature is introduced, and think if we rely on standardized solution there is impact for both UE and NW.

- VDF pointed out that they are initiating this discussion in SA2, and think if we do not solve it there is paging failure.

- Ericsson think the issue also has R3 impact.

- WI Rapp suggest we can conclude first that there is no issue for after R17 NW, and for pre R17 it is better to check in main session.

Regarding the SA2 raised issue on UE Radio Capability for Paging Information, R2 understand that there is no issue for NW after Release 17 (in which case the LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container). Whether there is issue for the other cases (for the features mentioned by SA2 LS R2-2500050) can be further discussed in the main session.

[Post129][215][LPWUS] Reply LS to SA2 (Huawei)

Scope: Discuss LS to reply to SA2 LS R2-250005/S2-2412876 based on the agreements

Intended outcome: Approved LS

Deadline: Short

R2-2500135	Discussion of LR and MR operating in same/different frequency band	MediaTek Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2500143	General considerations on the procedure for RRC_IDLE_INACTIVE	Xiaomi Communications	discussion

R2-2500158	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

R2-2500246	LP-WUS in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

R2-2500282	Discussion on LP-WUS in RRC_IDLE/INACTIVE 	NEC	discussion	NR_LPWUS-Core

R2-2500343	Discussion on LP-WUS WUR in RRC_IDLE INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

R2-2500456	Discussion on LP-WUS procedure and configuration	OPPO	discussion	Rel-19	NR_LPWUS-Core

R2-2500589	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

R2-2500645	Discussion on LP-WUS in RRC_IDLE/INACTIVE	HONOR	discussion	Rel-19	NR_LPWUS-Core

R2-2500663	On Idle/Inactive mode procedures for LP-WUS	Tejas Network Limited	discussion	Rel-19

R2-2500740	RAN2 aspects on LP-WUS/WUR in RRC Idle/Inactive mode	Sony	discussion	Rel-19	NR_LPWUS-Core

R2-2500857	Procedure and Configuration of LP-WUS in RRC IDLE/INACTIVE	Lenovo	discussion	NR_LPWUS-Core

R2-2500868	LP-WUS operation in RRC_IDLE and RRC_INACTIVE	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2500943	Discussion on LP-WUS operation in RRC_IDLE/INACTIVE modes	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2500993	LP-WUS in IDLE and INACTIVE	Nokia	discussion	Rel-19	NR_LPWUS-Core

R2-2501002	Discussion on the LP-WUS handling for Emergency call back 	NTT DOCOMO INC..	discussion	Rel-19

R2-2501006	Discussion on RRC CONNECTION load balancing for LP-WUS capable UEs	NTT DOCOMO INC..	discussion	Rel-19

R2-2501017	Further considerations on LP-WUS operation in IDLE INACTIVE mode	CMCC	discussion	Rel-19	NR_LPWUS-Core

R2-2501075	Discussion on LP-WUS in RRC_IDLE and RRC_INACTIVE	Sharp	discussion	Rel-19

R2-2501089	Procedure and configuration of LP-WUS for IDLE and INACTIVE mode	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

R2-2501093	LP-WUS in Idle and Inactive	Ericsson	discussion	Rel-19	NR_LPWUS-Core	R2-2410085

R2-2501132	Procedure and Configuration of LP-WUS in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

R2-2501252	LP-WUS operation in IDLE/Inactive state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

Agreements on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

On separate band issue RAN2 understands that UE can report which band(s) is supported by LR to NW. RAN2 understands that any potential overload issues could be addressed by current mechanism in spec. Send LS to RAN1 and RAN4 to inform the agreements. On sub-grouping For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e., SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where - UE_ID is related to 5G-S-TMSI, - N is the number of total paging frames in one DRX cycle, - Ns is the number of the PO for a PF, - Np is the number of subgroupNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1, - subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively. On SA2 LS (R2-2500050/S2-2412876) RAN2 sends reply LS to SA2/RAN3/CT1, and CC RAN1 and RAN4 to confirm the “LP-WUS” terminology. Regarding the SA2 raised issue on UE Radio Capability for Paging Information, R2 understand that there is no issue for NW after Release 17 (in which case the LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container). Whether there is issue for the other cases (for the features mentioned by SA2 LS R2-2500050) can be further discussed in the main session.

### 8.4.3	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions

Serving cell RRM relaxation criteria, to include LR measurement or not

R2-2500869	RRM relaxation and RRM offloading	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 6	[For case #3] If the condition for measurement relaxation is met, UE performs serving cell measurements using only MR with relaxed requirement. No LR measurements.

R2-2500944	Discussion on RRM measurement relaxation and offloading	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 4: For partially offloading case, support the exit condition based on LR measurement.

R2-2501254	LP-WUS RRM measurement relaxation and offloading	Qualcomm Incorporated	discussion	NR_LPWUS-Core

Noted

Proposal 4	The entry condition for partially offloading:

-	When both MR and LR measurement are above the thresholds defined for partially offloading, and,

-	When any of LR and MR measurement is below the threshold which is defined for totally offloading

Discussion

-	CMCC think LG E P6 is about UE behaviour. LG E think it is correct but no LR is needed. NEC, Lenovo and HW think as per R4, LR is needed, and think we should follow R4 decision.

-	Nokia, OPPO, ZTE support QC proposal.

-	Lenovo prefer LR measurement result is included.

-	OPPO think MR results may not be timely. HW, CATT share this view, given the R4 agreement on larger scaling factor.

-	HW think IDT P4 is more reliable. Samsung agree IDT P4 and want to have simple design.

-	Ericsson do not understand the case that some companies mentioned, where UE is first in partial offloading and then move on to fully offloading. Samsung share this question.

-	CATT think for new we first discuss the relaxation case, and we can check full offloading later.

-	Apple agree with 1st bullet of QC proposal, but 2nd bullet not needed.

-	HW and Apple wonders whether LR criteria is always configured. QC explain both are configured, and think if LR based condition is not configured then different UEs may have different behaviour.

-	QC wonders what is the UE behaviour if LR measurement criteria is not configured, and think it may cause ping-pong switching btw different cases. Xiaomi share this question.

-	Ericsson want to confirm again that serving cell RRM relx is only allowed for UEs that support LP-WUS.

The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements.

If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

Serving cell RRM relaxation criteria, whether separate exit condition is needed

R2-2501043	Discussion on RRM measurement relaxation and offloading in RRC_IDLE INACTIVE	CMCC	discussion	Rel-19	NR_LPWUS-Core

Not treated

Proposal 6: No separate exit condition is needed, and the exit condition can be defined as failing to meet the entry condition.

R2-2501133	RRM measurement relaxation and offloading in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

Not treated

Proposal 3: Similar to serving cell offloading, the exit conditions of MR serving cell RRM measurement relaxation are:

	If the serving cell quality from LR is below a threshold

	FFS low mobility criterion

R2-2500247	RRM Relaxation and Offloading in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

Not treated

Proposal 4: RAN2 discuss which option is adopted for MR RRM relaxation

-	Option 1: Serving cell quality measured by MR is higher than relaxation threshold, while serving cell quality measured by LR is higher than relaxation threshold. No separate exit condition.

-	Option 2: Entry condition: based on MR serving cell quality and optional LR serving cell quality.

Exit condition: based on LR serving cell quality.

RRM relaxation criteria, whether it is the same for serving cell and neighbour cell

R2-2500247	RRM Relaxation and Offloading in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

Not treated

Proposal 2: Same relaxation criteria are applied to both MR serving cell and neighbor cell measurement for UEs capable of LP-WUS.

R2-2501076	Discussion on RRM measurement relaxation and offloading	Sharp	discussion	Rel-19

Not treated

Proposal 4: The entry conditions for serving cell RRM measurement relaxation and for neighbour cell RRM measurement relaxation can be different.

R2-2500608	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Lenovo	discussion	Rel-19

Not treated

Proposal 5: The entry condition for serving cell RRM measurement relaxation is configured independently with the neighboring cell RRM measurement relaxation.

R2-2500144	Discussion on RRM measurement relaxation for RRC_IDLE_INACTIVE	Xiaomi Communications	discussion

R2-2500201	Further discussion on the criteria for RRM measurement relaxation and offloading	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

R2-2500247	RRM Relaxation and Offloading in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

R2-2500283	Discussion on LP-WUS RRM relaxation and offloading 	NEC	discussion	NR_LPWUS-Core

R2-2500344	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

R2-2500457	Discussion on RRM measurement in RRC IDLE and INACTIVE	OPPO	discussion	Rel-19	NR_LPWUS-Core

R2-2500590	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

R2-2500608	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Lenovo	discussion	Rel-19

R2-2500869	RRM relaxation and RRM offloading	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2500944	Discussion on RRM measurement relaxation and offloading	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2500994	RRM measurement relaxation in RRC_IDLE/INACTIVE	Nokia	discussion	Rel-19	NR_LPWUS-Core

R2-2501043	Discussion on RRM measurement relaxation and offloading in RRC_IDLE INACTIVE	CMCC	discussion	Rel-19	NR_LPWUS-Core

R2-2501068	Discussion on RRM measurement relaxation and offloading for RRC_IDLE/INACTIVE	China Telecom	discussion

R2-2501076	Discussion on RRM measurement relaxation and offloading	Sharp	discussion	Rel-19

R2-2501090	RRM measurement relaxation and offloading in RRC_IDLE and RRC_INACTIVE mode	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

R2-2501094	LP-WUS and RRM measurements	Ericsson	discussion	Rel-19	NR_LPWUS-Core	R2-2410086

R2-2501131	Discussion on neighboring cell measurement with LR	InterDigital, Ericsson, Nokia, Sony, Vodafone	discussion	Rel-19	NR_LPWUS-Core

R2-2501133	RRM measurement relaxation and offloading in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

R2-2501254	LP-WUS RRM measurement relaxation and offloading	Qualcomm Incorporated	discussion	NR_LPWUS-Core

Agreement on serving cell RRM relaxation criteria

The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements. If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

### 8.4.4	Procedures for LP-WUS in RRC_CONNECTED

Procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring.

R2-2500012	LS on LP-WUS operation in CONNECTED mode (R1-2410909; contact: NTT DOCOMO)	RAN1	LS in	Rel-19	NR_LPWUS	To:RAN2

Noted

R2-2501003	Discussion on the LS from RAN1 on LP-WUS CONNECTED	NTT DOCOMO INC..	discussion	Rel-19

Noted

Proposal1: RAN2 confirms that existing DRX Command MAC CE and Long DRX Command MAC CE can stop drx-onDurationTimer and drx-InactivityTimer for the DRX group in Option 1-1.

Proposal2: RAN2 confirms that it is feasible for existing DRX Command MAC CE and Long DRX Command MAC CE to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer

R2-2501095	LP-WUS in Connected	Ericsson	discussion	Rel-19	NR_LPWUS-Core	R2-2410087

Noted

Proposal 1	(Long) DRX command MAC CE can be used with option 1-1 and 1-2 to stop drx-onDurationTimer (option 1-1), lpwus-OnDurationTimer (option 1-2) and drx-InactivityTimer. When outside Active Time the UE starts monitoring LP-WUS.

R2-2501201	LP-WUS in RRC_CONNECTED	Nokia, Nokia Shanghai Bell	discussion	NR_LPWUS-Core

Noted

Proposal 1: RAN2 sends a reply LS to RAN1, indicating that the (Long) DRX Command MAC CE stops drx-onDurationTimer and drx-InactivityTimer for option 1-1, as well as the new timer for option 1-2.

R2-2500345	Discussion on LP-WUS WUR in RRC_Connected	vivo	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1: RAN2 confirms in Option 1-1, LP-WUS is only applied when UE using Long DRX cycle.

Proposal 2: If P1 is agreed, reply RAN1 that for Option 1-1:

−	If UE receives DRX Command MAC CE, UE will use Short DRX cycle (if configured) according to current specification, since Option 1-1 is only applied to Long DRX cycle, Option 1-1 won’t be used in this case.

−	If UE receives Long DRX Command MAC CE or DRX Command MAC CE in case Short DRX cycle is not configured, UE will use Long DRX cycle according to current specification. When Long DRX Command MAC CE is received, the UE will leave the C-DRX Active Time, and will monitor LP-WUS.

Proposal 3: Reply RAN1 that for Option 1-2, if DRX command MAC CE or Long DRX command MAC CE is received, UE will stop the new timer and drx-InactivityTimer. In this case, the UE will leave C-DRX Active Time and go back to LP-WUS monitoring.

Discussions

Whether short DRX is supported with LPWUS

- Ericsson suggest to discuss whether short DRX is support with LPWUS from R2 point of view. ZTE think it is support.

- CATT think for O1-2 short DRX is not needed, while for O1-1 we need some further analysis because there is potential impact on PS gain and also complexity. OPPO, LG E agree. OPPO and LG E think DCP and short DRX are not configured simultaneously in legacy. Nokia agree on O1-1.

- Xiaomi do not see a need to restrict the use of short DRX, and think it is R1 decision.

- Apple do not see a benefit of support short DRX, there is no motivation. NEC agree. QC ok to not have short DRX at all. Lenovo.

- Nokia think for O1-2 it can be supported.

- Ericsson think we need further discussion.

- DCM and Lenovo think this issue is not directly related to R1 questions.

[AT129][204][LPWUS] Proposals for reply to R1 LS R2-2500012/ R1-2410909 (CATT)

Scope: Discuss R1 questions and try to form answers to the questions, can also discuss whether short DRX is supported together with LPWUS

Intended outcome: Summary/Proposals in R2-2501444.

Deadline: before Thursday CB

R2-2501444	[AT129][204][LPWUS] Proposals for reply to R1 LS R2-2500012/R1-2410909 CATT	discussion	Rel-19	NR_LPWUS-Core

Noted

For Option 1-1, UE monitors LP-WUS outside C-DRX active time at least when long DRX cycle is used. FFS on short DRX cycle is used.

RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-1 to stop drx-onDurationTimer and drx-InactivityTimer.

RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-2 to stop the new timer and drx-InactivityTimer.

[Post129][216][LPWUS] Reply on LP-WUS operation in CONNECTED mode (NTT DOCOMO)

Scope: Discuss LS to reply to RAN1 R2-2500012/R1-2410909

Intended outcome: Approved LS

Deadline: Short

R2-2500074	Discussing on LP-WUS monitoring in Connected mode	Xiaomi	discussion	Rel-19	NR_LPWUS-Core

R2-2500248	Analysis on LP-WUS for RRC_CONNECTED	CATT	discussion	Rel-19	NR_LPWUS-Core

=> Revised in R2-2501326

R2-2501326	Analysis on LP-WUS for RRC_CONNECTED	CATT	discussion	Rel-19	NR_LPWUS-Core

R2-2500284	Discussion on LP-WUS in RRC_CONNECTED 	NEC	discussion	NR_LPWUS-Core

R2-2500303	Further discussion on LP-WUS for RRC_CONNECTED mode	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

R2-2500345	Discussion on LP-WUS WUR in RRC_Connected	vivo	discussion	Rel-19	NR_LPWUS-Core

R2-2500458	Discussion on LP-WUS in RRC_CONNECTED	OPPO	discussion	Rel-19	NR_LPWUS-Core

R2-2500591	Procedures for LP-WUS in RRC_CONNECTED	Apple	discussion	Rel-19	NR_LPWUS-Core

R2-2500666	LP-WUS operation in Connected mode	Tejas Network Limited	discussion	Rel-19

R2-2500717	LP-WUS in CONNECTED mode	InterDigital	discussion	Rel-19	NR_LPWUS-Core

R2-2500741	Considerations on LP-WUS/WUR in RRC Connected mode	Sony	discussion	Rel-19	NR_LPWUS-Core

R2-2500827	Discussion on LP-WUS in RRC_CONNECTED	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2500860	LP-WUS in RRC Connected Mode	Lenovo	discussion	NR_LPWUS-Core

R2-2501003	Discussion on the LS from RAN1 on LP-WUS CONNECTED	NTT DOCOMO INC..	discussion	Rel-19

R2-2501018	Discussion on LP-WUS operation in CONNECTED mode	CMCC	discussion	Rel-19	NR_LPWUS-Core

R2-2501077	Discussion on LP-WUS in RRC_CONNECTED	Sharp	discussion	Rel-19

R2-2501091	Procedures for LP-WUS in RRC_CONNECTED	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

R2-2501095	LP-WUS in Connected	Ericsson	discussion	Rel-19	NR_LPWUS-Core	R2-2410087

R2-2501134	Procedures for LP-WUS in RRC Connected Mode	Samsung	discussion	Rel-19

R2-2501201	LP-WUS in RRC_CONNECTED	Nokia, Nokia Shanghai Bell	discussion	NR_LPWUS-Core

R2-2501253	LP-WUS operation in CONNECTED state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

Agreements on LP-WUS in RRC_CONNECTED

For Option 1-1, UE monitors LP-WUS outside C-DRX active time at least when long DRX cycle is used. FFS on short DRX cycle is used. RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-1 to stop drx-onDurationTimer and drx-InactivityTimer. RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-2 to stop the new timer and drx-InactivityTimer.

## 8.11	Evolution of NR duplex operation: Sub-band full duplex (SBFD)

(NR_duplex_evo-Core; leading WG: RAN1; REL-19; WID: RP‑241614)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdocs

### 8.11.1	Organizational

Incoming LS, Rapporteur input, including workplan, etc..

LSin

R2-2500036	LS on CSI-RS measurement with SBFD operation (R4-2420165; contact: MediaTek)	RAN4	LS in	Rel-19	NR_duplex_evo-Core	To:RAN1	Cc:RAN2

Noted

Running CR

Chair: No detailed discussions expected in this meeting. All running CRs for this WID should be created/updated after this meeting, and submitted to the next meeting for discussions/endorsement.

R2-2500273	38300 Running CR for SBFD	CATT	draftCR	Rel-19	38.300	18.4.0	NR_duplex_evo-Core

Noted

Discussion

-	Nokia think we didn’t agree on type 2 rach, but it is in the draft. CATT explains that it is not the intention.

R2-2500886	SBFD UE capabilities running CR	Ericsson	discussion	Rel-19	NR_duplex_evo-Core	Late

=> withdrawn

Post meeting email discussions

[Post129][217][SBFD] List of open issues of RRC impact (Huawei)

Intended outcome: List of open issues of RRC impact to submit to the next meeting, taking into account related proposals in this meeting

Deadline:  Long

[Post129][218][SBFD] Running CR for TS 38.321 (Samsung)

Intended outcome: Running CR for submission to the next meeting

Deadline:  Long

[Post129][219][SBFD] Running CR for TS 38.300 (CATT)

Intended outcome: Running CR for submission to the next meeting

Deadline:  Long

### 8.11.2	Random access in SBFD

RAN2 impacts to support SBFD operation to support random access in SBFD symbols by UEs in RRC _CONNECTED mode and RRC_IDLE/INACTIVE mode.

RACH configuration

R2-2500884	SBFD RA aspects 	Ericsson	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 2	When a SBFD aware UE supporting both SBFD RACH configuration options accesses a cell, the UE applies the matching SBFD RACH configuration option configured in the cell.

Proposal 3	When a SBFD aware UE supporting a SBFD RACH configuration option accesses a cell configured with a different SBFD RACH configuration option, the UE applies the legacy RA operation instead of SBFD RA operation.

Discussion

-	HW support P2 and P3, think it is correct behaviour.

-	OPPO wonders whether legacy RA configuration is always present.

-	ZTE has concern on P1, and think it means in some cases UE has to apply the SBFD configuration. ZTE think it should be possible for UE to not use SBFD configuration based on some selection procedure.

When a SBFD aware UE supporting one or both SBFD RACH configuration options accesses a cell, the UE can apply the supported SBFD RACH configuration option in the cell.

When a SBFD aware UE supporting a SBFD RACH configuration option accesses a cell configured with a different SBFD RACH configuration option, the UE applies the legacy RA operation, and does not apply the SBFD RACH configuration.

RACH procedure, RO selection criteria

On Network indication of RO selection priority

R2-2500298	Random Access for SBFD Operation	NEC	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal-3: The network can indicate the RO types (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA.

Proposal-4: The network indication of RO type for the case of CBRA is optional present.

R2-2500339	Discussion on Random Access operation in SBFD	InterDigital, Inc.	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1. Support indicating the additional-ROs with higher priority over legacy-ROs to the SBFD-aware UEs.

R2-2500588	Remaining issues for RACH in SBFD	Apple	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 5: RAN2 to select Option 2 (no indication on prioritization of SBFD RO over legacy RO) as baseline.

Discussion

-	Apple explains their P5 assume it is cell specific priority indication. And Apple do not think there is a need to introduce indication per UE either.

-	CATT, Ericsson, QC, CMCC support NEC proposals.

-	CATT think this indication if configured, is cell specific.

-	Ericsson support think network can configure either legacy RO or additional RO as priority, and it should be up to network choice in different case. QC, CMCC, IDT agree.

-	Nokia think the indication is for SBFD UEs whether additional RO is used or not.

-	Sony agree with NEC P1, think the configuration is useful depending on number of SBFD UEs, think it can be multiple level indication, and want to put it FFS.

-	CMCC think network has the choice taking into account factors like coverage.

-	OPPO think we need RSRP criteria.

-	LG E ok to have this indication, and want this indication to be more dynamic than SIB1. Apple share this view.

For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS.

Detailed RO selection criteria (without RO selection priority, e.g., if such priority is not supported or not configured)

R2-2500298	Random Access for SBFD Operation	NEC	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal-5: UE selects the first available RO type(legacy RO or additional RO) among all of the RACH resources valid to the UE for its CBRA based access, when the indication of RO type from the network is absent.

R2-2500606	Discussion on random access procedure in SBFD	ZTE Corporation	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 3: RAN2 to support option 1 for RO type selection, and the SSB RSRP should not be a criteria for selecting RO type.

Proposal 4: Introduce a time window to let UE determine whether to select SBFD RO or legacy RO, in order to balance the RA latency and RA opportunity:

	If UE finds the SBFD RO exists in the time window, no matter whether legacy RO is the nearest or not, UE should choose the nearest SBFD RO in the time window;

	If UE finds there is no SBFD RO in the time window, UE should select the nearest RO based on legacy rule.

R2-2501244	Discussion on Random Access in SBFD	LG Electronics Inc.	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 4. For the initial attempt of the RA procedure, if the network indication is not configured/agreed, SBFD-aware UE prioritizes SBFD RACH occasion if the channel condition is better than the RSRP threshold as a baseline. Otherwise, SBFD-aware UE performs RA procedure using legacy RACH occasion.

R2-2500884	SBFD RA aspect		Ericsson	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 10	Adopt the below option for RO prioritization in case of CBRA:

a.	The network provides a cell specific SIB1 indication whether SBFD-aware UEs shall use SBFD ROs, legacy ROs or first available RO (of any type).

b.	If there is no such cell specific indication on the RO type, a UE selects RO based on a SSB RSRP threshold and a SIB1-provided indicator whether to use additional ROs below or above this SIB1-provided SSB RSRP threshold.

c.	If RO prioritization information is not provided in SIB1, RO prioritization is up to UE implementation.

Discussion

-	HW indicate they have similar proposal as Ericsson, but think given that we already agreed the priority indication then perhaps there is no need to indicate whether ‘above’ or ‘below’ RSRP threshold. HW think if we combine the agreed priority indication and LG E proposal based on RSRP then it is complete solution.

-	Nokia agree with Ericsson proposal, since it support different scenarios.

-	Charter not sure if it can be leave to UE implementation, think RSRP based criteria is good.

-	Qualcomm do not prefer to have RSRP based threshold, since cell edge UE may also use SBFD resources for RA. Sony share this view.

-	vivo think RSRP threshold is needed for SSB selection so it is important. Vivo think if there is no RO prioritization UE should follow consider legacy RO and SBFD RO equally.

-	OPPO support LG E proposal.

-	CATT think a possible compromise is to first agree we will use RSRP, how to use can FFS. IDT agree.

-	Apple wonders if we can consider couple RSRP threshold configuration with RACH configuration option.

If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold.

RACH procedure, RO selection before or after selecting a set of RA resources

R2-2500298	Random Access for SBFD Operation	NEC	discussion	Rel-19	NR_duplex_evo-Core

Proposal-2: The RO type selection is performed after the RA type selection.

Noted

R2-2501244	Discussion on Random Access in SBFD	LG Electronics Inc.	discussion	Rel-19	NR_duplex_evo-Core

Proposal 6. The SBFD-aware UE selects between SBFD RO and legacy RO before selecting a set of Random Access resources.

Noted

Discussion

-	ZTE support NEC proposal, since if we go with LG E proposal the fallback mechanism will cross the set of resources, which is complicated. ZTE also do not want to put RO selection priority higher than resource set selection.

-	Ericsson think the existing feature combination based resource partitioning, and think it is not reasonable to switch the RO type before selecting the partitioning.

-	Nokia support LG E proposal.

-	Samsung wonders if there is NW indicated RO type priority then there is no need to select RO.

FFS whether RO type selection is performed before or after the RA type selection.

RACH procedure, fallback behaviour

R2-2500098	Impacts on the random access by the evolution of duplex operation		Huawei, HiSilicon	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 6: Support that the UE is allowed to switch from the PRACH resources in SBFD symbols to the PRACH resources in non-SBFD symbol. Support to switch from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols.

R2-2500274	Discussion on Random Access in SBFD symbols	CATT	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 14: For CBRA, if legacy RO is selected for initial RACH transmission, it cannot fallback to SBFD RO after a number of times of RACH attempts.

Discussion

-	ZTE do not see clear benefit of P6 from HW paper, e.g., ping-pong type of fallback.

-	Ericsson, Nokia, Charter, Sharp support HW proposal, and think we can have limitation on the maximum number of fallback times mentioned by ZTE.

-	Nokia think there is no ping-pong problem if we only allow one time fallback. Charter share this view.

-	OPPO think given what we already agreed it is better to avoid complicated fallback mechanism.

-	Sharp think either option has similar MAC spec impact.

-	Samsung think if we allow only fallback once then the specification complexity is similar for the two options. CMCC share this view.

-	Sony think we should limit the case for cell edge, e.g., only cell edge UEs can select RO and then do fallback.

-	Lenovo prefer CATT proposal, and is not sure what is the benefit to allow fallback from legacy RO to SBFD RO. Lenovo think already today we have a lot of tools to handle RACH performance, e.g., power control. HW think there is benefit in coverage limited case, since SBFD RO may give UE more UL resources. Lenovo still not sure about benefit in cell edge.

-	Nokia also see benefit in avoiding RA preamble collision.

-	Samsung and LG E not ready to making HW proposal as working assumption. LG E think the issue also depends on FFS on whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold.

FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

R2-2500098	Impacts on the random access by the evolution of duplex operation	Huawei, HiSilicon	discussion	Rel-19	NR_duplex_evo-Core

R2-2500105	Random Access Operation of SBFD	Nokia Corporation	discussion	Rel-19	NR_duplex_evo-Core

R2-2500109	SBFD Configuration for initial random access and operations	Charter Communications, Inc	discussion

R2-2500191	Discussion on RACH in SBFD	Xiaomi	discussion	Rel-19

R2-2500274	Discussion on Random Access in SBFD symbols	CATT	discussion	Rel-19	NR_duplex_evo-Core

R2-2500298	Random Access for SBFD Operation	NEC	discussion	Rel-19	NR_duplex_evo-Core

R2-2500337	Discussion on random access procedure in SBFD	vivo	discussion	Rel-19	NR_duplex_evo-Core

R2-2500339	Discussion on Random Access operation in SBFD	InterDigital, Inc.	discussion	Rel-19	NR_duplex_evo-Core

R2-2500376	Random Access Issues for SBFD	Sharp	discussion	Rel-19	NR_duplex_evo-Core

R2-2500588	Remaining issues for RACH in SBFD	Apple	discussion	Rel-19	NR_duplex_evo-Core

R2-2500606	Discussion on random access procedure in SBFD	ZTE Corporation	discussion	Rel-19	NR_duplex_evo-Core

R2-2500751	Random access for SBFD Operation	Sony	discussion	Rel-19	NR_duplex_evo-Core

R2-2500884	SBFD RA aspects	Ericsson	discussion	Rel-19	NR_duplex_evo-Core

R2-2501029	Discussion on random access in SBFD	CMCC	discussion	Rel-19	NR_duplex_evo-Core

R2-2501129	Views on random access for SBFD	Qualcomm Incorporated	discussion	Rel-19	NR_duplex_evo-Core

R2-2501135	Random access in SBFD	Samsung	discussion	Rel-19

R2-2501244	Discussion on Random Access in SBFD	LG Electronics Inc.	discussion	Rel-19	NR_duplex_evo-Core

Agreements on random access in SBFD

When a SBFD aware UE supporting one or both SBFD RACH configuration options accesses a cell, the UE can apply the supported SBFD RACH configuration option in the cell. When a SBFD aware UE supporting a SBFD RACH configuration option accesses a cell configured with a different SBFD RACH configuration option, the UE applies the legacy RA operation, and does not apply the SBFD RACH configuration. For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS. If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold. FFS whether RO type selection is performed before or after the RA type selection. FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

### 8.11.3	Other aspects

Other RAN2 impacts with SBFD if not covered by the previous agenda items.

R2-2501130	Other aspects of SBFD	Qualcomm Incorporated	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1: For UE-specific dedicated RRC configuration on SBFD time/frequency configuration, RAN2 wait RAN1 further decision on whether support.

Proposal 3: A new SP CLI measurement resource set activation/deactivation MAC CE is introduced to activate/deactivate the SP CLI measurement resource.

Proposal 6:  For L1 based UE-to-UE CLI reporting configuration, at least AP trigger based CLI reporting is supported.

Proposal 7: For L1 based UE-to-UE CLI reporting configuration, RAN2 waits for RAN1 further conclusion on whether supports periodic based CLI reporting.

Proposal 10: RAN2 to study the MAC impacts on the corresponding MAC features like SR and CG with enabling the SBFD operation.

A new SP CLI measurement resource set activation/deactivation MAC CE is introduced to activate/deactivate the SP CLI measurement resource.

R2-2501170	Other Aspects of SBFD	Samsung	discussion	Rel-19	NR_duplex_evo-Core

Not treated

Proposal 6: RAN2 confirm that exchange of the information for the activation/deactivation of SRS resources between gNBs is not supported in Rel-19.

Proposal 7: RAN2 does not support the separate BFD/BFR procedure for SBFD symbols.

Proposal 8: RAN2 consider to support the separate Layer3 measurement report for CSI-RS resources in SBFD symbol.

R2-2500637	Other impacts by the evolution of duplex operation	Huawei, HiSilicon	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1: Introduce an optional capability with signaling to indicate whether UE supports UL transmission and DL reception across SBFD symbols and non-SBFD symbols in different slots.

Proposal 2: Send an LS to RAN1 on whether SBFD can work with DC.

Discussion on P1

-	ZTE, OPPO  think if it is part of R1 feature list than we do not need to discuss. HW think we can discuss whether is for common or separate capability for DL and UL.

Chair: Other detailed proposals in the following two contributions can be discussed if time allows.

R2-2501130	Other aspects of SBFD	Qualcomm Incorporated	discussion	Rel-19	NR_duplex_evo-Core

R2-2501170	Other Aspects of SBFD	Samsung	discussion	Rel-19	NR_duplex_evo-Core

R2-2500110	Cross link interference handling in SBFD networks	Charter Communications, Inc	discussion

R2-2500275	Discussion on other aspects of SBFD	CATT	discussion	Rel-19	NR_duplex_evo-Core

R2-2500338	SBFD other aspects	vivo	discussion	Rel-19	NR_duplex_evo-Core

R2-2500340	Discussion on resource configuration in SBFD	InterDigital, Inc.	discussion	Rel-19	NR_duplex_evo-Core

R2-2500403	Other aspects of SBFD	Xiaomi	discussion	Rel-19	NR_duplex_evo-Core

R2-2500554	Other aspects of SBFD	Nokia	discussion	Rel-19	NR_duplex_evo-Core

R2-2500607	Discussion on L3 and L1 measurement in SBFD	ZTE Corporation	discussion	Rel-19	NR_duplex_evo-Core

R2-2500637	Other impacts by the evolution of duplex operation	Huawei, HiSilicon	discussion	Rel-19	NR_duplex_evo-Core

R2-2500885	CSI-RS measurements for RLM/BFD/CBD in SBFD	Ericsson	discussion	Rel-19	NR_duplex_evo-Core

R2-2501130	Other aspects of SBFD	Qualcomm Incorporated	discussion	Rel-19	NR_duplex_evo-Core

R2-2501170	Other Aspects of SBFD	Samsung	discussion	Rel-19	NR_duplex_evo-Core

Agreement on other aspects for SBFD

A new SP CLI measurement resource set activation/deactivation MAC CE is introduced to activate/deactivate the SP CLI measurement resource.

## 8.12	NR MIMO Phase 5

(NR_MIMO_Ph5-Core; leading WG: RAN1; REL-19; WID: RP-242394)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdocs

### 8.12.1	Organizational

LSs and rapporteur input, including workplan, etc.

LSin

R2-2500008	LS to RAN2 on RRC and MAC impacts for Rel-19 NR MIMO Ph5 (R1-2410758; contact: Samsung)	RAN1	LS in	Rel-19	NR_MIMO_Ph5	To:RAN2

Noted

Discussion

-	Samsung explains that we can note this and discuss based on contribution

Work plan

R2-2501026	Work Plan for Rel-19 on NR MIMO Phase 5	CMCC,Samsung,MediaTek	Work Plan	Rel-19	NR_MIMO_Ph5-Core

Noted

Running CR

Chair: All running CRs for this WID should be created/updated after this meeting, and submitted to the next meeting for discussions/endorsement.

Post meeting email discussions

[Post129][206][MIMO_Ph5] Running CR for TS 38.321 (Samsung)

Intended outcome: Running CR for submission to the next meeting

Deadline:  Long

[Post129][207][MIMO_Ph5] Running CR for TS 38.331 (Ericsson)

Intended outcome: Running CR for submission to the next meeting

Deadline:  Long

### 8.12.2	Asymmetric DL sTRP/UL mTRP

To identify RRC/MAC aspects that need to be discussed for asymmetric DL sTRP/UL mTRP

MAC CE design

R2-2500179	Discussion on asymmetric DL sTRP/UL mTRP	China Telecom	discussion

Noted

Proposal 2: The new MAC CE can have a variable size consisting of following fields:

-	Serving cell ID

-	BWP ID

-	TCI state ID

-	Absolute PL offset value

R2-2500825	Consideration on Asymmetric DL sTRP/UL mTRP	LG Electronics Inc.	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1. RAN2 discusses how to indicate TCI state ID considering following options

-	Option 1. Bitmap mapping for TCI state ID + PL offset value

-	Option 2. Configured TCI state ID + PL offset value

Proposal 2. RAN2 design the new MAC CE considering follows.

-	One Serving Cell field and one BWP field are included

-	CORESET Pool ID field and D/U field are not included

R2-2501222	Consideration on the Asymmetric DL sTRP/UL mTRP	ZTE Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 2: The mechanism “multiple TCI states (either joint or UL) are indicated per PL offset” is not supported.

Proposal 5: Up to 16 joint/UL TCI states can be indicated in the PL offset MAC CE.

R2-2501177	Discussion on Asymmetric DL sTRP/UL mTRP	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1: The new MAC CE for PL offset update includes flexible number of PL offsets.

One PL offset value is indicated for each TCI state included in the new MAC CE.

Discussion on P2 from R2-2500179

-	CMCC think TCI state ID can be indicated by a bitmap. LG E agree and think bitmap can save some overhead when the number of indicated TCI states is large.

-	Sony wonders why we need BWP ID. Samsung think we need this since TCI states are configured per BWP.

The new MAC CE contains one serving cell ID and one BWP ID

Discussion on P1 from R2-2500825

-	Ericsson think it is meaningful to reduce the overhead, suggest to first confirm we need a TCI state to PL value association and leaves exact way FFS.

-	LG E think it depends on how many TCI states can be indicated by this MAC CE.

-	OPPO think we should consider real deployment, so do not think the # is very large.

-	Nokia slightly prefer O2, and not sure why bitmap way saves overhead. ZTE also do not see benefit of bitmap.

-	HW think it is very important to limit the size of this MAC CE. HW prefer to put 8 as the maximum number of TCI states in one MAC CE. Vivo think we already agree that any configured TCI state can be indicated, so do not see why we can limit to 8.

-	Samsung think if NW want to indicate more TCI states, it is possible to send multiple MAC CEs. Samsung prefer Option 2.

TCI state ID is used to indicate a TCI state in the new MAC CE (i.e., no bitmap for TCI states is needed)

R2-2501222	Consideration on the Asymmetric DL sTRP/UL mTRP	ZTE Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 2: The mechanism “multiple TCI states (either joint or UL) are indicated per PL offset” is not supported.

Proposal 5: Up to 16 joint/UL TCI states can be indicated in the PL offset MAC CE.

R2-2501177	Discussion on Asymmetric DL sTRP/UL mTRP	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1: The new MAC CE for PL offset update includes flexible number of PL offsets.

Discussion on P5 in R2-2501222 and P1 in R2-2501177

-	HW suggest to first agree that this MAC CE can contain flexible number of PL offset values and decide maximum number later.

-	Samsung suggest to discuss now.

-	Apple agree with HW on the flexible number part, but not sure if we need to conclude on maximum number.

-	IDT think we do not need FFS on the maximum number of PL offset values.

The new MAC CE can include flexible number of PL offset values.

On PL offset update

R2-2501024	Discussion on Asymmetric DL sTRP/UL mTRP	CMCC	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1:  We propose three options to address the conflicts between MAC CE and RRC when updating PL offset:

-	Option1: For the same joint/UL TCI state, the UE should apply the latest PL offset value, whether it is represented by RRC configuration or MAC CE update.

-	Option2: For the same joint/UL TCI state, UE needs to determine which signaling update is the latest between RRC and MAC CE, and the criteria for this determination are to be discussed (taking latency as an example)

-	Option3: UE updates the PL offset based only on the newly received MAC CE after the initial PL offset is configured by RRC, regardless of the TCI state.

R2-2500217	Discussion on Asymmetric DL sTRP/UL mTRP	Samsung	discussion	Rel-19	NR_MIMO_Ph5

Noted

Proposal 7: UE updates the RRC configured PL offset with MAC CE indicated value. UE applies the latest PL offset value (e.g., upon BWP switching) received in RRC or MAC CE.

Proposal 8: Upon receiving a PL offset update MAC CE, the MAC entity indicates to lower layer the information in the MAC CE.

R2-2501177	Discussion on Asymmetric DL sTRP/UL mTRP	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 7: Ask RAN1 whether, after an RRC reconfiguration in which the path loss offset values are neither released nor modified by RRC, the UE continues using the values modified by MAC CEs, or reverts to the values configured by RRC.

Proposal 8: Ask RAN1 whether, after BWP switching, the UE reverts to the RRC-configured path loss offset values for the new BWP or uses the last value that were used on this BWP i.e., including modifications by MAC CEs.

Discussions

-	OPPO agree with Samsung understanding and think in the case of BWP switching UE also use the latest value as indicated by the new MAC CE. Xiaomi agree.

-	Ericsson think UE follow the latest value no matter it is RRC configured or MAC CE indicated value. Nokia, LG E, ZTE, Apple, vivo agree. QC also agree and think it is up to UE capability to process the indicated value.

-	HW think the ‘latest indicated value’ may have ambiguity period issue.

- Working assumption

UE applies the latest PL offset value received in RRC or MAC CE. Can revisit if new issue is found.

On PHR trigger

R2-2500217	Discussion on Asymmetric DL sTRP/UL mTRP	Samsung	discussion	Rel-19	NR_MIMO_Ph5

Noted

Proposal 9: For PHR trigger, add a note in MAC specification: If a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset.

R2-2500104	RAN2 Aspects of Asymmetric DL sTRP/UL mTRP	Nokia Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1: RAN2 to confirm that PL offset has no impact on PHR triggering condition as the PL offset is applied in the PHY during the calculation of PL value.

Discussions

-	Ericsson agree the intention of both paper, and think it is nice to use a note in MAC to clarify how it works. Ericsson think there is already a note in MAC mentioning this.

-	Qualcomm think the PL calculation in 38.213 which is R1 spec, so do not think we need to capture in MAC. LG E agree and think MAC just refer to PL value from PHY when needed. Xiaomi agree. Xiaomi think having a note is clear.

-	OPPO open to confirming this understanding and think we can do either way.

-	Qualcomm suggest to send our understanding to R1.

RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset. FFS whether/how to capture this.

R2-2500104	RAN2 Aspects of Asymmetric DL sTRP/UL mTRP	Nokia Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2500179	Discussion on asymmetric DL sTRP/UL mTRP	China Telecom	discussion

R2-2500217	Discussion on Asymmetric DL sTRP/UL mTRP	Samsung	discussion	Rel-19	NR_MIMO_Ph5

R2-2500249	Discussion on Asymmetric DL sTRP UL mTRP	CATT	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2500268	Discussion on asymmetric DL sTRP and UL mTRP	Xiaomi	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2500354	Discussion on MAC CE impact for asymmetric DL sTRP/UL mTRP scenarios	vivo	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2500450	Discussion on asymmetric DL sTRP and UL mTRP	SHARP Corporation	discussion	NR_MIMO_Ph5-Core

R2-2500635	Discussion on PL offset	Lenovo	discussion	Rel-19

R2-2500752	Enhancement for Asymmetric DL sTRP/UL mTRP	Sony	discussion	Rel-19	NR_MIMO_Ph5

R2-2500825	Consideration on Asymmetric DL sTRP/UL mTRP	LG Electronics Inc.	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2500975	Asymmetric DL/UL mTRP user plane impact from MIMO ph. 5	Ericsson	discussion	Rel-19	38.321	NR_MIMO_Ph5-Core

R2-2501024	Discussion on Asymmetric DL sTRP/UL mTRP	CMCC	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2501166	Discussion on UL only mTRP	Qualcomm Incorporated	discussion

R2-2501177	Discussion on Asymmetric DL sTRP/UL mTRP	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2501222	Consideration on the Asymmetric DL sTRP/UL mTRP	ZTE Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

Agreements on Asymmetric DL sTRP/UL mTRP

One PL offset value is indicated for each TCI state included in the new MAC CE. The new MAC CE contains one serving cell ID and one BWP ID TCI state ID is used to indicate a TCI state in the new MAC CE (i.e., no bitmap for TCI states is needed) The new MAC CE can include flexible number of PL offset values. - Working assumption: UE applies the latest PL offset value received in RRC or MAC CE. Can revisit if new issue is found. - Agreement RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset. FFS whether/how to capture this.

### 8.12.3	Others

To identify R2 impact on other objectives

UE initiated beam reporting, modelling

R2-2500930	Impacts from other NR MIMO Phase 5 objectives	Ericsson	discussion

Noted

Proposal 1	The event evaluation for UE-initiated beam reporting is captured in 38.321, where the evaluation is based on indications from measurements described in a RAN1 specification.

Proposal 2	RAN2 to inform RAN1 on its preference of specifying the triggering event determination for Event 2.

R2-2500210	Discussion on UEI beam reporting	OPPO	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1	The event evaluation for UE initiated beam reporting is specified in PHY.

R2-2500218	Discussion on UE-initiated Beam Reporting and CSI enhancement	Samsung	discussion	Rel-19	NR_MIMO_Ph5

Noted

Proposal 1: RAN2 confirms that UEI beam reporting does not share common framework of LTM event-triggered L1 measurement.

Proposal 2: Send a LS to RAN1 to ask whether the procedure of handling time window and counter for event evaluation/triggering is to be captured in RAN1 or RAN2 specification. Adopt the draft LS in the appendix as baseline.

Discussions

-	CMCC echo Samsung proposals. CMCC think anyway R2 can start discussion in the next meeting, and we can revisit if needed.

-	ZTE think R1 will not make any conclusion on this issue. ZTE think we discuss for two meetings and agree with CMCC that we start in the next meeting.

-	Nokia prefer to wait for R1, and think we cannot make assumption in R2. CATT, LG E agree.

-	CATT, LG E share the same understanding as P1 from Samsung.

-	QC agree with Ericsson proposal to capture the procedure in MAC.

-	LG E has concern that if we already start to write MAC then perhaps R1 decide otherwise.

-	vivo, MTK suggest that we decide now instead of waiting longer, and think we can ask R1 if they have concern on our decision.

-	Vivo and QC think ‘framework’ is not clear in Samsung P1.

-	Samsung, LG E, and Nokia do not think we need a working assumption, and think we can discuss in the next meeting based on existing R1 agreement anyway. Apple think we just wait.

-	Huawei do not see a need to ask the Q1 as Samsung suggested in their contribution.

-	Nokia has concern to make a working assumption.

-	CMCC, QC think working assumption is a good compromise.

-	LG E ok to have this working assumption but do not want to

Working assumption: RAN2 assume the event evaluation for UE-initiated beam reporting is captured in 38.321, where the evaluation is based on indications from measurements described in a RAN1 specification. The final decision is up to RAN1, we can revisit this if R1 has a different decision.

No need to send a LS to R1 on this.

UE initiated beam reporting, other potential issues

R2-2501176	Enhancements for UE-initiated/event-driven beam management	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 6: Send a LS to RAN1 to ask whether the UEI beam reporting for mode-B is visible to the MAC layer.

Proposal 7: In mode-A of UEIBM, the UE should remain in DRX active time to receive DCI for the second UL transmission.

Proposal 8: In mode-B of UEIBM, the second UL transmission is not subject to DRX active time.

RRC impact

Chair: the following RRC related proposals can be discussed if time allows. Or, post meeting email discussions can be used to prepare for further discussions in the next meeting(s).

R2-2500930	Impacts from other NR MIMO Phase 5 objectives	Ericsson	discussion

R2-2500218	Discussion on UE-initiated Beam Reporting and CSI enhancement	Samsung	discussion	Rel-19	NR_MIMO_Ph5

R2-2501223	Consideration on the UEIBM	ZTE Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2500103	RAN2 Aspects of the NR MIMO	Nokia Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2500250	Discussion on UE-initiated Beam Reporting and CSI Enhancement	CATT	discussion	Rel-19	NR_MIMO_Ph5-Core

Not treated

[Post129][208][ MIMO_Ph5] Issues and proposals on RRC impact (Ericsson)

Scope: Discuss RRC impact taking into account proposals in R2-2500930, R2-2500218, R2-2501223, R2-2500103, and R2-2500250, and also include new RRC parameters if R1 agreed further, identify main issues and try to form proposals for next meeting’s discussions

Intended outcome: Summary document with proposals

Deadline: Long

R2-2500103	RAN2 Aspects of the NR MIMO	Nokia Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2500180	Discussion on UE initiated beam reporting	China Telecom	discussion

R2-2500210	Discussion on UEI beam reporting	OPPO	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2500218	Discussion on UE-initiated Beam Reporting and CSI enhancement	Samsung	discussion	Rel-19	NR_MIMO_Ph5

R2-2500250	Discussion on UE-initiated Beam Reporting and CSI Enhancement	CATT	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2500269	Discussion on modelling of UE-initiated beam report	Xiaomi	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2500355	Discussion on UE-initiated/event-driven beam management	vivo	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2500451	Discussion on UE-initiated/event-driven beam management	SHARP Corporation	discussion	NR_MIMO_Ph5-Core

R2-2500636	Discussion on UEIBR	Lenovo	discussion	Rel-19

R2-2500826	Discussion on UEI beam reporting impact	LG Electronics Inc.	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2500930	Impacts from other NR MIMO Phase 5 objectives	Ericsson	discussion

R2-2501025	Discussion on UE-initiated/event-driven beam management	CMCC	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2501167	Discussion on UE Initiated Beam Report	Qualcomm Incorporated	discussion

R2-2501176	Enhancements for UE-initiated/event-driven beam management	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2501223	Consideration on the UEIBM	ZTE Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

Agreement on UE initiated beam reporting modelling

Working assumption: RAN2 assume the event evaluation for UE-initiated beam reporting is captured in 38.321, where the evaluation is based on indications from measurements described in a RAN1 specification. The final decision is up to RAN1, we can revisit this if R1 has a different decision. No need to send a LS to R1 on this.

## 8.19	NR Others

Tdoc limit: 1

Contributions addressing LS from RAN4 R4-2420410 and any RAN4 LSs not related to any of the AIs above.

On UE capability for FDD-FDD inter-band CA

R2-2500041	LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx (R4-2420410; contact: CATT)	RAN4	LS in	Rel-19	NR_CADC_SUL_R19	To:RAN2	Cc:RAN1

Noted

R2-2500229	Discussion on FDD-FDD inter-band CA simultaneous RxTx (LS R4-2420410)	CATT, Huawei, HiSilicon, China Telecom, China Unicom	discussion	Rel-19	NR_CADC_SUL_R19

Noted

Proposal 1: RAN2 to confirm that the 2UL/2DL inter-band FDD-FDD CA_n5-n8 with the scheduling restriction of non-concurrent n5 DL and n8 UL is feasible, and there is no RAN2 spec impact (i.e., the necessary scheduling restriction can be all up to network implementation).

Proposal 2: For a FDD-FDD inter-band NR CA band combination, define per BC UE capabilities to indicate:

1)	For each band pair (X, Y) in this band combination, whether simultaneous transmission of band X and reception of band Y is supported;

2)	For each band pair (X, Y) in this band combination, whether simultaneous transmission of band Y and reception of band X is supported.

Proposal 3: RAN2 to send out a reply LS with RAN2 agreements.

Discussion on P1

-	Nokia think the proposal only focus on a specific example and it may not be general enough.

-	CATT think it is possible to add something to make it more general, e.g., ‘also apply to other FDD-FDD BCs’

-	QC not sure if we should generalize that way.

-	Ericsson think the understanding is ok to capture.

RAN2 under that for the example band the 2UL/2DL inter-band FDD-FDD CA_n5-n8 with the scheduling restriction of non-concurrent n5 DL and n8 UL is feasible, and there is no RAN2 spec impact (i.e., the necessary scheduling restriction is up to network implementation)

Discuss on need of UE capability

-	Xiaomi do not support having the ue cap, since no UE will support this combination anyway.

-	HW think the intention of the R4 LS to let us introduce cap signalling, and even if no UE support this today we should be future proof. HW prefer to introduce signalling. CATT has same view.

-	QC not ready to introduce capability. QC not sure what the capability means, e.g., it could be full duplex in uplink and downlink. ZTE, vivo share this view.

-	Nokia think there could be the case some UE support simultaneous tx/rx in some part of those band. Nokia also ok for us to check further before doing the singaling.

-	vivo think even if we introduce the UE cap for now, network may still need to update to support scheduling of simultaneous tx/rx in those BCs.

-	China Telecom ok to introduce the UE cap, since it helps network to know and make the scheduling decision.

-	CATT think having UE cap help us to support more cases, e.g., more BCs, higher order of CA. CATT think detailed signalling design can FFS and if needed we can further ask R4. Ericsson, HW agree.

-	QC OK to check with R4.

-	OPPO want to check with R4 before agree anyway.

R2-2500230	DRAFT Reply LS on UE capability for FDD-FDD inter-band CA simultaneous RxTx	CATT	LS out	Rel-19	NR_CADC_SUL_R19	To:RAN4	Cc:RAN1

Noted

R2-2500113	Discussion on n5-n8 simultaneous operation (R4-2420410)	OPPO	discussion	Rel-19	NR_CADC_SUL_R19

Noted

Proposal 1	R2 discuss whether there is a need to define capability bits for 2UL/2DL CA_n5-n8 case, considering so far simultaneous (n8UL, n5DL) is anyway not supported.

Proposal 2	If R2 decides to define capability bit for 2UL/2DL CA_n5-n8 case, and if R2 decides to rely on existing simultaneousRxTxInterBandCA to cover FDD-FDD case, extend it in a way that it is present only if the concerned FDD-FDD BC includes (n5, n8) band pair, and when simultaneous (n8UL, n5DL) is supported.

Proposal 3	If R2 decides to define capability bit for 2UL/2DL CA_n5-n8 case, and if R2 decides to rely on existing - simultaneousRxTxInterBandCAPerBandPair to cover FDD-FDD case, extend it in a way that the bit for the concerned band pair (n5, n8) indicates 0 if the simultaneous (n8UL, n5DL) is not supported, or 1 if it is supported (in both case, assuming simultaneous (n8DL, n5UL) is supported).

R2-2500932	On UE capability for FDD-FDD inter-band CA simultaneous RxTx	Ericsson	discussion

Noted

Proposal 1	Inform RAN4 that enforcing non-simultaneous UL and DL to existing FDD-FDD band combinations would cause backwards compatibility issues, and thus it is expected that such restriction is limited to band combinations defined onwards and not applicable to already existing band combinations.

Proposal 2	FDD-FDD band combinations where simultaneous UL and DL is not possible due to overlapping bands can be handled by NW implementation, as long as this restriction is limited to band combinations defined onwards and not applicable to already existing band combinations.

Proposal 3	RAN2 assumes that, if there are other cases defined in the future, they would also be a single FDD-FDD band pair. If there is an intention of any different combination, then RAN2 may need to discuss it more on the signaling solutions for this non-simultaneous transmission (including the case of a single FDD-FDD band pair).

Proposal 4	The restriction on non-simultaneous UL and DL for CA_n5-n8 is expected to be captured in RAN4 specifications.

Discussion on P1

-	CATT think P1 is already clear based on R4 LS. Nokia agree.

R2-2501191	Capability signalling for overlapping FDD-FDD inter-band CA	Nokia	discussion	Rel-19	NR_CADC_SUL_R19

Noted

R2-2501225	Consideration on UE capability for FDD-FDD inter-band CA Simultaneous Rx/Tx	ZTE Corporation	discussion	Rel-19	NR_CADC_SUL_R19

Noted

[AT129][205][NR_Others] Questions to R4 on potential UE capability for FDD-FDD inter-band CA (CATT)

Scope: Discuss and form questions to R4 on UE capability for FDD-FDD inter-band CA

Intended outcome: Draft LS in R2-2501445 for CB, including our agreements and questions. Discussion summary in R2-2501446.

Deadline: Friday CB

On SSB position restrictions for less-than-5MHz Scells

R2-2500040	LS on SSB position restrictions for less-than-5MHz Scells (R4-2420383; contact: Qualcomm)	RAN4	LS in	Rel-19	NR_FR1_lessthan_5MHz_BW_Ph2-Core	To:RAN2	Cc:RAN1

Noted

R2-2500950	SSB position restrictions for less-than-5MHz SCells	Qualcomm Incorporated	CR	Rel-18	38.331	18.4.0	5249	-	F	NR_FR1_lessthan_5MHz_BW-Core

Discussion

-	QC explain why this is Rel-18 CR even though the LS is Rel-19. ZTE, Nokia has not strong view whether this is from R18 or it is in R19.

-	vivo not sure if ‘transmission bandwidth configuration’ is best wording in a R2 spec.

-	Ericsson ok to have it from Rel-18.

[CB]

- Changes in R2-2500950 is technically correct and needed. The CR is postponed, can further discuss if it is for Rel-19 or Rel-18.

On Ku band numerology

To be treated in NTN breakout session

R2-2500034	LS on Ku band numerology (R4-2419902; contact: Rumney Telecom)	RAN4	LS in	Rel-19	NR_NTN_Ku_bands-Core	To:RAN1, RAN2	Cc:RAN

R2-2500087	Discussion on RAN4 LS on Ku Band Numerology	vivo 	discussion	Rel-19	NR_NTN_Ku_bands-Core

R2-2500694	Discussion on Ku band numerology	Huawei, HiSilicon	discussion	Rel-19	NR_NTN_Ku_bands-Core

R2-2500937	Draft Reply LS on Ku band numerology	THALES	LS out	Rel-19	NR_NTN_Ku_bands-Core	To:RAN4	Cc:RAN1, RAN

R2-2500979	Reply LS on Ku band numerology	Eutelsat Group	LS out	Rel-19	NR_NTN_Ku_bands-Core	To:RAN4	Cc:RAN, RAN1

Location services

To be treated in positioning breakout session

R2-2500047	LS on Location service of UEs served by MWAB (S2-2412625; contact: Huawei)              SA2       LS in     Rel-19    VMR_Ph2           To:RAN3            Cc:RAN2

Moved to AI 8.0 General

R2-2500051	LS on energy saving indication from CN to RAN (S2-2413034; contact: Ericsson)            SA2       LS in     Rel-19    EnergySys          To:RAN2, RAN3

R2-2500055	LS on Time Synchronization for MBS (S4-242169; contact: Qualcomm)            SA4       LS in     Rel-19   FS_AMD    To:SA2, RAN2

R2-2500066	Reply LS on Time Synchronization for MBS (S2-2501327; contact: Ericsson) SA2       LS in     Rel-19   FS_AMD    To:SA4, RAN2

R2-2501096	Time Synchronization for MBS	Ericsson	discussion	Rel-19	FS_AMD

## List of post meeting email discussions

Short

[Post129][214][LPWUS] LS to RAN1/RAN4 on agreements of separate band issue (vivo)

Intended outcome: Approve the LS.

Deadline: short

[Post129][215][LPWUS] Reply LS to SA2 (Huawei)

Scope: Discuss LS to reply to SA2 LS R2-250005/S2-2412876 based on the agreements

Intended outcome: Approved LS

Deadline: Short

[Post129][216][LPWUS] Reply on LP-WUS operation in CONNECTED mode (NTT DOCOMO)

Scope: Discuss LS to reply to RAN1 R2-2500012/R1-2410909

Intended outcome: Approved LS

Deadline: Short

Long

[Post129][206][MIMO_Ph5] Running CR for TS 38.321 (Samsung)

Intended outcome: Running CR for submission to the next meeting

Deadline:  Long

[Post129][207][MIMO_Ph5] Running CR for TS 38.331 (Ericsson)

Intended outcome: Running CR for submission to the next meeting

Deadline:  Long

[Post129][208][ MIMO_Ph5] Issues and proposals on RRC impact (Ericsson)

Scope: Discuss RRC impact taking into account proposals in R2-2500930, R2-2500218, R2-2501223, R2-2500103, and R2-2500250, and also include new RRC parameters if R1 agreed further, identify main issues and try to form proposals for next meeting’s discussions

Intended outcome: Summary document with proposals

Deadline: Long

[Post129][209][LPWUS] Running CR for TS 38.300 (Ericsson)

Intended outcome: Running CR for submission to the next meeting

Deadline:  Long

[Post129][210][LPWUS] Running CR for TS 38.321 (Apple)

Intended outcome: Running CR for submission to the next meeting

Deadline:  Long

[Post129][211][LPWUS] Running CR for TS 38.331 (vivo)

Intended outcome: Running CR for submission to the next meeting

Deadline:  Long

[Post129][212][LPWUS] Running CR for TS 38.304 (CATT)

Intended outcome: Running CR for submission to the next meeting

Deadline:  Long

[Post129][213][LPWUS] Running CR for TS 37.340 (ZTE)

Intended outcome: Running CR for submission to the next meeting

Deadline:  Long

[Post129][217][SBFD] List of open issues of RRC impact (Huawei)

Intended outcome: List of open issues of RRC impact to submit to the next meeting, taking into account related proposals in this meeting

Deadline:  Long

[Post129][218][SBFD] Running CR for TS 38.321 (Samsung)

Intended outcome: Running CR for submission to the next meeting

Deadline:  Long

[Post129][219][SBFD] Running CR for TS 38.300 (CATT)

Intended outcome: Running CR for submission to the next meeting

Deadline:  Long