3GPP TSG-RAN WG2 Meeting #129bis	R2-2501762

Wuhan, China, April 7-11, 2025

Agenda item:	8.7.4.1

Source:	Qualcomm Incorporated

Title:	Discussion on LCP enhancements

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In this paper, we discuss open issues related to conditions for priority adjustment and priority during intra-UE prioritization.

# Discussion

Priority in intra-UE prioritization

BSR is triggered by new data arriving in an empty LCH. BSR then triggers SR. During SR re-/transmission, if the data associated with the pending SR becomes priority adjustable, i.e. it has become time critical, then it is important for UE to get a UL grant as soon as possible, so that it can send the data before its delay budget runs out. What this entails is that

- The LCH which triggered the SR should be allowed to use a SR configuration associated with a higher priority;

- During intra-UE prioritization, the SR should be given higher priority than its default one so that its transmission is not delayed.

A natural choice for the higher priority in the above two scenarios is the additional priority configured for the LCH which triggered the SR.

Observation 1.	If data which triggered an SR has become priority adjustable, giving the SR a higher priority can help the UE get a UL grant sooner.

Proposal 1.	If data which triggered an SR has become priority adjustable, the UE can select the SR configuration that matches the additional priority of the corresponding LCH, for the PUCCH transmission and intra-UE prioritization of the SR.

In legacy, for the purpose of intra-UE prioritization, MAC layer indicates the priority of a MAC PDU when delivering it to PHY layer. Then the same priority is used throughout the HARQ process for the MAC PDU. In Rel-19, it is up to discussion whether the priority of a MAC PDU may change over time, if it contains data from an LCH which is configured with an additional priority.

For UE implementation, once a MAC PDU is submitted to the lower layer, it is very challenging for either MAC layer or PHY layer to track remaining time of data in that MAC PDU. That is because MAC layer itself does not have any buffer and thus does not keep any state information about a MAC PDU once it submits a MAC PDU to the lower layer. What is inside a MAC PDU is transparent to PHY layer. Therefore, we do not think it is practical for UE to upgrade the priority of a MAC PDU after it is submitted to PHY.

Proposal 2.	After the LCP procedure, the priority of a MAC PDU does not change over time.

Conditions for priority adjustment

At the RAN2#129 meeting, it was agreed that PBR is not adjusted when there is priority adjustable data in a logical channel. However, it was left FFS whether the priority-adjustable data should be subject to the restriction Bj <0.

- No additional PBR is needed for priority adjusted data - FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold

From scheduling perspective, lifting the restriction Bj>0 is equivalent to changing PBR, because they have the same effect on LCP, i.e. give an LCH more tokens/credits so that its data can be scheduled sooner. Since there is already an agreement that no additional PBR is needed for priority adjusted data, then the restriction Bj>0 should not be removed.

Observation 2. Lifting the restriction Bj>0 for an LCH is equivalent to increasing its PBR. But it is already agreed that no additional PBR is needed for priority adjusted data.

Proposal 3.	LCHs with priority-adjustable data are still subject to the restriction Bj>0.

If an LCH is allowed to upgrade its priority from p1 to p2, it will compete UL resources with the LCH(s) which has the same priority p2. If this LCH is already in congestion (i.e. PSI based discard has been activated), then the priority upgrade can worsen the congestion, as less UL resources are available to every LCH with priority p2. Therefore, it makes sense to add to the priority adjustment criteria that the LCHs with the same priority as the additional priority should not be in congestion.

Proposal 4.	If an LCH with priority p is in congestion (i.e. PSI based discard has been activated), then no other LCHs are allowed to upgrade their priorities to priority p, even if they meet the priority adjustment criteria.

At RAN2#128, the following agreements on LCH prioritization were made:

- As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. - As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP

We think the above agreements may have negative impacts on resource allocation.

First, these agreements imply that there is no limit on how much data in an LCH may be given a higher priority. This lack of restriction can lead to reduced fairness for other LCHs.

For example, consider a configuration in which low bit rate flows (e.g. haptic and control) are mapped to LCH #1 and other high bit rate flows (e.g. video) are mapped to LCH #2. LCH #2 can adjust its priority to the same level as LCH #1 when it has priority-adjustable data.

Priority adjustment is most likely to occur when there is congestion, because data in low priority LCH can’t get enough UL grants to be scheduled in time. In addition, most UL grants during congestion may not be large enough to accommodate all data. In such cases, LCH #1 is penalized after LCH #2 adjusts its priority to the same level even in round 1. This happens because LCH #1 no longer has priority over LCH #2, even when its Bj is still positive (Otherwise, there would be no benefit in adjusting the priority of LCH #2).

Even when there is no congestion and sufficient UL resources are available, a high-priority LCH can still be affected, if a UE implementation does not support fallback to default priority during the second round. This happens because if a low-priority LCH continue to use the adjusted (higher) priority in the second round, the high-priority LCH has to share resources that it originally has strict priority over.

Observation 3.	Priority adjustment can penalize high-priority logical channels if applied without restriction.

The above analysis clearly indicates the need for restrictions on how often or how much priority adjustments an LCH may have. We think a simple yet effective approach is to limit the amount of data that can be adjusted to higher priority within a time window. For example, an LCH is allowed to adjust its priority at start of the LCP procedure only if the total amount of data sent with higher priority in the last T msec is less than a configured threshold. Of course this is only one of the possible options. Companies can discuss other possible approaches.

Proposal 5. 	At the start of LCP, an LCH with priority-adjustable data is allowed to apply its additional priority only if the total amount of data transmitted with the additional priority within the last T msec is less than a configured threshold.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Observation 1.	If data which triggered an SR has become priority adjustable, giving the SR a higher priority can help the UE get a UL grant sooner.

Proposal 1.	If data which triggered an SR has become priority adjustable, the UE can select the SR configuration that matches the additional priority of the corresponding LCH, for the PUCCH transmission and intra-UE prioritization of the SR.

Proposal 2.	After the LCP procedure, the priority of a MAC PDU does not change over time.

Observation 2. 	Lifting the restriction Bj>0 for an LCH is equivalent to increasing its PBR. But it is already agreed that no additional PBR is needed for priority adjusted data.

Proposal 3.	LCHs with priority-adjustable data are still subject to the restriction Bj>0.

Proposal 4.	If an LCH with priority p is in congestion (i.e. PSI based discard has been activated), then no other LCHs are allowed to upgrade their priorities to priority p, even if they meet the priority adjustment criteria.

Observation 3.	Priority adjustment can penalize high-priority logical channels if applied without restriction.

Proposal 5. 	At the start of LCP, an LCH with priority-adjustable data is allowed to apply its additional priority only if the total amount of data transmitted with the additional priority within the last T msec is less than a configured threshold.