3GPP TSG-RAN WG2 Meeting #129bis	R2-2501764

Wuhan, China, April 7-11, 2025

Agenda item:	8.7.5

Source:	Qualcomm Incorporated

Title:	Discussion on RLC enhancements

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In this paper, we discuss issues related to timely retransmission and avoidance of unnecessary retransmissions.

# Discussion

## Timely retransmission

Status report

To enable the full benefits of polling enhancements in reducing RLC round-trip time, it is important to expedite the corresponding status report too. Otherwise, polls triggered based on remaining time would still be subject to the regulation of legacy mechanisms such as the prohibit timer t-StatusProhibit. As a result, status reports are not returned as soon as they should be.

It is not a good idea to configure a very small t-StatusProhibit for all status reports. For example, when no RLC PDUs have low remaining time, status reports do not need to be sent frequently. They need to be expediated or sent more frequently only when there are time-critical PDUs (those with low remaining times).

Observation 1.	To take full advantage of the polling enhancement in reducing RLC round-trip time, it is important to enhance the corresponding status reports too.

With the enhancements studied so far, there can be a few options for a RLC receiver to recognize there are time-critical PDUs.

For example, a poll is triggered when remaining time of an SDU dropping below a threshold. In this case, if the transmitter can inform the receiver that it is a time-critical poll, the receiver should send a status report immediately without the restriction by t-StatusProhibit or apply a shorter prohibit timer.

For the subsequent discussion, we denote such a status report a time-critical status report.

Proposal 1.	When sending a poll, the transmitter includes an indication whether it is a time-critical poll (i.e. triggered due to remaining time below a threshold).

Proposal 2.	Upon receiving a time-critical poll, the receiver triggers a time-critical status report without the restriction by t-StatusProhibit (or apply a shorter t-StatusProhibit).

Another possible option is to have receiver trigger a time-critical status report itself. This is possible because of the enhancement for avoiding unnecessary retransmission. If this enhancement is enabled, the receiver starts t-RxDiscard whenever it detects a gap in the RLC sequence numbers. This timer can approximate the remaining time of a missing RLC PDU. Therefore, when the remaining value of this timer drops below a threshold, it implies that the missing PDU has become time critical, and a time-critical status report should be sent back asap.

Proposal 3.	When the remaining value of t-RxDiscard is below a configured threshold, the receiver sends a time-critical status report without the restriction by t-StatusProhibit (or apply a shorter t-StatusProhibit).

In addition to timely triggering, transmission of time-critical status reports should be performed with extra priority to achieve further reduction in latency. Possible enhancements to consider can include buffering them at the head of the RLC transmission queue before any other types of RLC PDUs, special handling in the LCP procedure, etc.

Proposal 4.	Discuss RLC and MAC enhancements that can expediate the transmission of time-critical status reports.

Autonomous retransmission

At the RAN2#129 meeting, it was left FFS whether the remaining time used in the autonomous retransmission should be based on the PDCP discard timer or a new timer managed by RLC should be introduced.

- Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC

What we observe is that it is not easy for UE to use PDCP discard timer for the purpose. That is because there is no simple mapping between the sequence number of PDCP PDUs and RLC PDUs (for example, in DC configuration). If the remaining time was based on PDCP discard timer, the RLC needs to keep track of the association between sequence numbers of every PDCP PDU and its corresponding RLC SDU. This would require significant amount of memory and frequent lookups in this association.

On the other hand, from purely technical perspective, it also makes more sense to use a timer managed by RLC. That is because when to trigger autonomous retransmissions should be more related to RLC-layer transmission time (i.e. traversal time from a RLC transmitter to the RLC receiver) than end-to-end delay budget.

Therefore, we think the remaining time should be based on a new timer managed by RLC. This timer should be per RLC SDU and should be started when its associated RLC SDU is received from the lower layers. It should be canceled when the RLC SDU is positively acknowledged.

Proposal 5.	The remaining time for autonomous retransmission and/or polling should be based on a new timer managed by RLC (Timer A).

Proposal 6.	Timer A, which is per RLC SDU, is started when a new RLC SDU is received from PDCP. It is canceled when its associated RLC SDU is positively acknowledged.

At the RAN2#129 meeting, RAN2 have agreed to support autonomous retransmission based on remaining time. One shortcoming with this approach is that a transmitter blindly initiates retransmission without considering transmission status at the lower layers. That sometimes may result in unnecessary retransmissions. For example, when an SDU is being transmitted over the air in an active HARQ process, it may not be necessary to immediately triggers a new RLC retransmission when remaining time threshold is reached.

A simple fix to this shortcoming can be based on a new timer. The purpose of this new timer, which works like a prohibit timer, is to give the lower layer some time to go through the usual HARQ process. A retransmission is held back if this timer is running, even if the remaining time has already dropped below the threshold (autonomousReTxThreshold).

It works as follows:

- This timer is started when a RLC PDU is submitted to the lower layers;

- When remaining time of this RLC PDU drops below autonomousReTxThreshold (i.e. the threshold for autonomous retransmission), UE triggers a retransmission only if this timer is not running. Otherwise, the retransmission is initiated until after this timer expires or is cancelled.

Proposal 7. 	Define a new timer (Timer B) for a RLC PDU, which is started when a RLC PDU is submitted to the lower layers.

Proposal 8. 	When remaining time of a RLC PDU drops below autonomousReTxThreshold, UE triggers a retransmission only if Timer B is not running. Otherwise, the retransmission is initiated after timer B expires or is cancelled.

This timer can be cancelled based on the outcome of underlying HARQ process. For example, after N times of HARQ retransmissions, UE can determine with high confidence that this HARQ process is not likely to succeed (more details of this argument can be found in R2-2409558 [1]). Once upon that determination, UE should cancel the timer to allow UE to initiate retransmission as soon as possible.

Proposal 9. 	Timer B is cancelled after the TB which contains the associated RLC PDU has failed N HARQ re-/transmissions.

## Avoid unnecessary retransmission

UE capability

For avoiding unnecessary retransmissions, RAN2 have adopted a combined approach, i.e. a transmitter stops retransmissions of an outdated SDU based on upper-layer indication of PDU discard, and the receiver abandons obsolete SDUs based on a local timer. However, it is worth noting that the transmitter side enhancement is independent of the receiver side enhancement. Even if there is no upper-layer indication, retransmissions of an outdated SDU can still be avoided, because the transmitter can do so based on the “fake ACKs” from the receiver. The transmitter side enhancement is useful only in that it helps the transmitter stop retransmission of outdated SDUs sooner.

Observation 3. 	Retransmission of outdated SDUs can still be avoided even if only the receiver side enhancements are applied.

Besides the technical feasibility discussed above, it is also beneficial from a UE implementation perspective to define a separate UE capability for the transmitter side enhancement. For example, in UE implementations that prebuild RLC PDUs, PDU discard may require significant changes to the data path and hence are not desired. Therefore, it would greatly facilitate development and testing if the transmitter and receiver enhancements are supported by two separate UE capabilities.

Proposal 10.	Define separate UE capabilities for transmitter and receiver enhancements for avoiding unnecessary retransmissions.

Impact on PDCP SN gap report

At the RAN2#129 meeting, companies discussed an issue with PDCP SN gap report during mobility events, triggered by an LS from RAN3 (R3-247891). Lost of a gap report may be caused by a RLC reset when a mobility event is initiated. One example scenario that illustrates the issue can be found in R2-2500491 [2], which for the convenience of discussion, is copied below:

0. All PDCP SDUs before SDU #X have been successfully delivered from UE to the network; 1. SDU #X is discarded, and PDCP SN Gap report is triggered and transmitted by RLC, together with some SDUs from #(X+1) onwards; 2. UE receives and executes an RRC command for handover, including RLC re-establishment for the DRB; - The RLC re-establishment takes place before successful delivery of any PDCP PDU transmitted in step 1; 3. In line with the above, XnAP SN Status transfer from source RAN node to target RAN node indicates #X as the First missing UL SDU (and no bitmap indicating higher-numbered received SDUs); 4. After handover completion by the UE (and the related delay), as part of PDCP Re-establishment, UE PDCP retransmits the SDUs from #(X+1) onwards. At this point, reception of these SDUs while SDU #X remains missing starts t-Reordering at the target RAN node, which will run until expiry before the received SDUs are delivered to upper layers.

In our view, the issue described above is valid. Since a PDCP SN gap report is lost, there is no way for source/target gNB to learn or detect that a missing SDU is due to discard, not even with any enhancements for inter-gNB coordination. In addition, no legacy procedure can be used to help the target gNB recover it and restart the reordering timer, because the transmitter does not have anything to retransmit.

Observation 4. 	If a PDCP discard occurs before a mobility event and the corresponding PDCP SN gap report is lost, it is not possible for source/target gNB to learn or detect that a missing SDU is due to PDCP discard.

In our view, while it works to require UE always resend a PDCP SN gap report after completion of a mobility event, it is a very inefficient approach. First, PDCP discard due to expiry of the discard timer is already a small probability event. The probability is even smaller that it happens before a mobility event and its corresponding PDCP SN gap report is lost. Hence the occurrence of these two events together should be quite rare and, by any measure, a corner case. Obviously, it is very inefficient to have UE send a SN gap report after every mobility event just to handle a corner case.

Instead, a good solution should rely on UE to detect a lost SN gap report. Such a detection is not hard to implement. For example, if UE find an SDU indicated in a previous SN gap report is NACKed in a status report, it can infer with high confidence that the previous SN gap report must have lost, and the report should be resent.

An advantage of this scheme is that it not only works in scenarios associated with mobility events, but also works in any scenarios where a SN gap report may be lost, e.g. due to errors on DU-CU interface. It is a solution with general applicability. Therefore, it can be defined as a new trigger for PDCP SN report.

Proposal 11. 	UE triggers a PDCP SN gap report when it receives a status report in which an SDU indicated in a previous SN gap report is negatively acknowledged.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Status report

Observation 1.	To take full advantage of the polling enhancement in reducing RLC round-trip time, it is important to enhance the corresponding status reports too.

Proposal 1.	When sending a poll, the transmitter includes an indication whether it is a time-critical poll (i.e. triggered due to remaining time below a threshold).

Proposal 2.	Upon receiving a time-critical poll, the receiver triggers a time-critical status report without the restriction by t-StatusProhibit (or apply a shorter t-StatusProhibit).

Proposal 3.	When the remaining value of t-RxDiscard is below a configured threshold, the receiver sends a time-critical status report without the restriction by t-StatusProhibit (or apply a shorter t-StatusProhibit).

Proposal 4.	Discuss RLC and MAC enhancements that can expediate the transmission of time-critical status reports.

Autonomous retransmission

Proposal 5.	The remaining time for autonomous retransmission and/or polling should be based on a new timer (Timer A) managed by RLC.

Proposal 6.	Timer A, which is per RLC SDU, is started when a new RLC SDU is received from PDCP. It is canceled when its associated RLC SDU is positively acknowledged.

Proposal 7. 	Define a new timer (Timer B) for RLC PDU, which is started when a RLC PDU is submitted to the lower layers.

Proposal 8. 	When remaining time of a RLC PDU drops below autonomousReTxThreshold, UE triggers a retransmission only if Timer B is not running. Otherwise, the retransmission is initiated only after this timer expires or is cancelled.

Proposal 9. 	Timer B is cancelled after the TB that contains the associated RLC PDU has failed N HARQ re-/transmissions.

Avoiding unnecessary retransmissions

Observation 2. 	Retransmission of outdated SDUs can still be avoided if only the receiver side enhancements are applied.

Proposal 10.	Define separate UE capabilities for transmitter and receiver enhancements for avoiding unnecessary retransmissions.

Impact on PDCP SN gap reporting

Observation 3. 	If a PDCP discard occurs before a mobility event and the corresponding PDCP SN gap report is lost, it is not possible for source/target gNBs to learn or detect that a missing SDU is due to PDCP discard.

Proposal 11. 	UE triggers a PDCP SN gap report when it receives a status report in which an SDU indicated in a previous gap report is negatively acknowledged.

# References

- R2-2409558, Discussion on RLC enhancements, Qualcomm, November 2024.

- R2-2500491, Discussion of RAN3 LS on PDCP SN Gap report handling, Nokia, Nokia Shanghai Bell, February 2025.