3GPP TSG-RAN WG2 #129bis	R2-2501803

Wuhan, China, April 2025

Agenda Item:	8.7.3

Source:	OPPO

Title:	Discussion on Measurement Gap Enhancements

Document for:	Discussion, Agreement

Introduction

In previous meeting, RAN2 discussed potential L2 impacts due to skipping of measurement and the need for semi-static/UE-initiated solution besides the DCI-based solution to enable measurement gap skipping. In this paper, the RAN2 impacts will be further discussed.

Discussion

Issue-1: UAI

Firstly, on how to enable the transmission and reception in MG/scheduling restriction occasions, RAN1 has agreed on the following solution based on dynamic indication from the network.

For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it.

As shown in the above agreements, it is the network that determines whether to enable Tx/Rx during a specific gap or restriction caused by RRM measurements, and there was a discussion in RAN1 on whether the assistance information from the UE is needed for network’s decision. There was no consensus in RAN1, one LS[1] was sent to RAN4 and CC RAN2. In the LS, the candidate assistance information was listed.

In the last RAN4 meeting, they agreed to define UAI as follows but the content of UAI will be further discussed

Agreement: Agree to define UAI based on the following assumptions, and further discuss the content of UAI: - RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective) - Not define UE behavior and the corresponding requirements based on UAI - Supporting UAI is optional for UE with optional UE capability

RAN4 agreed to define UAI with optional UE capability, and the content of UAI will be discussed further.

For the detailed content, the following candidates are discussed in RAN1

- FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not

Among the above information, the first FFS on assistance information related to measurement occasions is more of RAN4 scope, and the last three FFS related to channel quality and traffic are more of RAN2 scope which can be discussed in RAN2.

Among the UAI candidates identified by R1, the first is more related to measurement occasions and falls within the RAN4 scope, while the others pertain more to the RAN2 scope.

Regarding the UAI associated with UL traffic, specifically PSI, it is mainly utilized for PDU set-based discard, which does not relate to RRM and scheduling. Furthermore, we already have DSR to indicate the remaining time and data volume to the gNB, assisting the network for good scheduling.

For UAI related to traffic, PSI is not related to scheduling or RRM. We already have DSR to report the remaining delay budget and data volume to the network.

Then, for the channel conditions related information, i.e., RSRP is below/above search threshold (s-MeasureConfig), it seems easy and beneficial to support as S-MeasureConfig is a legacy mechanism. The only enhancement is to report the measurement result to the network indicating that the measurement of non-serving cells is not required and the scheduling during those measurement gaps can be performed.

For UAI related to channel condition, S-MeasureConfig is a legacy mechanism, the only enhancement is to report the measurement result to the network indicating that the measurement of non-serving cells is not required and the scheduling during those measurement gaps can be performed.

Lastly for the UE mobility related information, it aligns with the RRM relaxation mechanism defined for redcap, i.e., the UE monitors and reports fulfillment status of RRM relaxation criteria to the network. The RRM relaxation criterion is determined by tracking serving cell RSRP variations over an observation period.

#### 5.7.4.4 Relaxed measurement criterion for a stationary (e)RedCap UE The relaxed measurement criterion for a stationary UE is met when: - (SS-RSRPRefStationaryConnected – SS-RSRP) < SSearchDeltaP-StationaryConnected, Where: - SS-RSRP = current L3 RSRP measurement of the PCell based on SSB (dB). - SS-RSRPRefStationaryConnected = reference SS-RSRP value of the PCell (dB), set as follows: - at the end of RRC reconfiguration procedure as specified in 5.3.5.3, when rrm-MeasRelaxationReportingConfig is included in the RRCReconfiguration message; or - after MAC successfully completes a Random Access procedure after applying a reconfigurationWithSync in spCellConfig while stationary criterion is configured; or - upon an indication from lower layers that a RACH-less handover or an LTM cell switch is successfully completed for the CG while stationary criterion is configured, or - if (SS-RSRP – SS-RSRPRefStationaryConnected) > 0; or - if the relaxed measurement criterion has not been met for TSearchDeltaP-StationaryConnected: - UE shall set the value of SS-RSRPRefStationaryConnected to the current SS-RSRP value of the serving cell.

The UE mobility related information is also beneficial for the network to know the RRM can be saved/skipped and scheduling can be performed when the UE is at stationary state. But the new criterion needs to be defined.

For UAI related to UE mobility, RRM relaxation mechanism defined for redcap can be used as reference. The information is beneficial for the network to know the RRM can be saved/skipped and scheduling can be performed when the UE is at stationary state.

In summary, for the contents of UAI, the analysis of each information is as follows:

| UAI information | Pros | Cons | Analysis |
| --- | --- | --- | --- |
| UAI related to measurement occasions | - | - | Up to RAN4 to evaluate the Pros and Cons |
| UAI related to traffic | No | No additional information to assist the network for RRM configuration and scheduling | Can be excluded |
| UAI related to channel condition | - S-MeasureConfig can be reused, spec impact is small - No impact to RRM performance, since the related measurement is not performed anyway | No | RAN2 to discuss whether to support |
| UAI related to UE mobility | The unnecessary measurement can be saved and scheduling can be performed | New criterion for the stationary state needs to be defined | RAN2 to discuss whether to support |

For the UAI content related to measurement gap skipping, RAN2 to

- Rely on RAN4 on the UAI related to measurement occasions.

- Exclude the UAI related to traffic.

- Discuss the UAI related to channel condition (i.e., S-MeasureConfig) and UE mobility.

Issue-3: Capability

As agreed by RAN1 and RAN4, for DCI-based measurement gap skipping, there is a minimum time offset between indication to skip and the skipped measurement occasion.

RAN1 Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s). RAN4 conclusion: RAN4 discussed the above RAN1 working assumption and confirms that Alt 1-1 is feasible from RAN4 perspective with X=5ms as baseline, and X=3ms with optional UE capability.

As shown in the above RAN1/4 agreements, the RRC configuration for the minimum time offset is needed. Further details on can wait for RAN1/4 progress.

As concluded by RAN1 and RAN4, the RRC configuration for minimum time offset is needed.

In addition, as indicated in RAN4 LS, at least for X=3ms, the optional UE capability is needed.

For measurement gap skipping, new UE capability will be introduced for DCI-based approach, details pending RAN1/RAN4 further conclusion.

- Conclusion

We have the following observations:

Observation 1	RAN4 agreed to define UAI with optional UE capability, and the content of UAI will be discussed further.

Observation 2	Among the UAI candidates identified by R1, the first is more related to measurement occasions and falls within the RAN4 scope, while the others pertain more to the RAN2 scope.

Observation 3	For UAI related to traffic, PSI is not related to scheduling or RRM. We already have DSR to report the remaining delay budget and data volume to the network.

Observation 4	For UAI related to channel condition, S-MeasureConfig is a legacy mechanism, the only enhancement is to report the measurement result to the network indicating that the measurement of non-serving cells is not required and the scheduling during those measurement gaps can be performed.

Observation 5	For UAI related to UE mobility, RRM relaxation mechanism defined for redcap can be used as reference. The information is beneficial for the network to know the RRM can be saved/skipped and scheduling can be performed when the UE is at stationary state.

Observation 6	As concluded by RAN1 and RAN4, the RRC configuration for minimum time offset is needed.

We have the following proposals:

Proposal 1	For the UAI content related to measurement gap skipping, RAN2 to

-	Rely on RAN4 on the UAI related to measurement occasions.

-	Exclude the UAI related to traffic.

-	Discuss the UAI related to channel condition (i.e., S-MeasureConfig) and UE mobility.

Proposal 2	For measurement gap skipping, new UE capability will be introduced for DCI-based approach, details pending RAN1/RAN4 further conclusion.

Reference

- R1-2405736, LS on UE assistance information

- R4-2420198, Response LS on gaps/restrictions that are caused by RRM measurements