3GPP TSG-RAN WG2 Meeting #129bis	R2-2501815

Wuhan, China, 7th – 11st Apr, 2025

Agenda Item:	8.7.4.1

Source: 	OPPO

Title:  	Discussion on LCH priority adjustment for XR

Document for:	Discussion and Decision

# Introduction

In Rel-19 XR WID [1] revised in RANP#105 meeting, the objectives of scheduling enhancements are updated as follows.

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

This contribution would like to provide our considerations on LCP enhancements for scheduling.

# Discussion on LCH priority adjustment

Impact on SRB

In RAN2#125bis meeting, the following agreement was made on SRB impact due to LCP enhancement, but no further conclusion on this.

RAN2 should consider potential impact on traffic from SRBs.

In our understanding, a proper network implementation would solve this issue easily. Networks can ensure the LCH of SRB always has higher LCH priority than the additional LCH priority used for the LCH with LCH priority-adjusted data.

- Up to network implementation to avoid SRB impact due to LCH prioritization-based LCP enhancement, i.e., configure higher priority for LCH of SRBs than LCH with LCH priority-adjusted data.

Impact on intra-UE prioritization

For LCH with LCH priority-adjusted data, one issue is whether/how additional LCH priority is applied for intra-UE prioritization procedure or not.

In legacy intra-UE prioritization, for the MAC entity configured with lch-basedPrioritization, the priority of an uplink grant is determined based on the priorities of the LCHs that are multiplexed or can be multiplexed. In detail, it is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions. Similarly, for the MAC entity configured with lch-basedPrioritization, the priority of an SR is also determined based on the priority of the LCH triggering the SR.

In RAN2#129 meeting, the following agreements were made on impact on intra-UE prioritization.

There is no impact on BSR/SR/DSR triggering and reporting due to adjusted priority. Intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. FFS whether this has specifications impact FFS Intra-UE prioritization shall also use the additional LCP priority for SR priority determination

At least the additional LCH priority applies for UL grant priority determination for both new transmission and retransmission. It is still FFS whether the additional LCH priority applies for SR priority determination or not.

What we understand for this issue includes

1/ In the current running CR, UE only determines to use the additional LCP priority during LCP procedure when UL grant received. In case of SR triggered by BSR/DSR, UE may not have any UL grant, so there is no chance for LCHs to determine to use the additional LCP priority. And we are negative to allow additional priority adjustment beside LCP procedure, which adds much UE complexity and specification effort.

2/ Although there is motivation to use the additional LCP priority for SR priority determination, considering the same remaining time threshold used for priority adjustment in LCP procedure and SR priority determination in intra-UE prioritization, the benefit is doubted since the priority-adjusted data is likely to expire before UE receives UL grant requested by SR.

3/ Besides, we have agreed that no impact on BSR/SR/DSR triggering and reporting due to additional priority, therefore it is reasonable to not use additional priority also for SR priority determination.

Therefore, we propose not use additional LCP priority for SR priority determination.

- The additional LCP priority is NOT used for SR priority determination in intra-UE prioritization.

Bj/PBR impact

Another issue discussed before is the enhancement of PBR/Bj in case of an LCH with LCH priority-adjusted data. The main direction is to allow LCH with LCH priority-adjusted data to not follow PBR-based token bucket algorithm to some extent during 1st-round of LCP procedure.

In RAN2#129 meeting, the following agreements were made.

No additional PBR is needed for priority adjusted data FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold.

Additional PBR configuration is excluded. But it is FFS on allowing an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network).

In legacy, during 1st-round of LCP procedure, MAC entity only allocates resource for LCHj with positive Bj, and update Bj according to Bj = Bj - total size of MAC SDUs served to LCHj. For MAC entity, it is fully up to UE implementation to determine how many MAC SDUs to be served in LCHj. A typical UE implementation is to serve MAC SDU of LCHj one by one until Bj < 0 or its PBR requirement is satisfied.

- In legacy 1st-round of LCP procedure, it is up to UE implementation to determine how many MAC SDUs to be served in an LCH with Bj > 0. A typical UE implementation is to serve MAC SDU of this LCH one by one until Bj < 0 or its PBR requirement is satisfied.

If it is allowed to allocate resource for LCHj with an upgraded priority in case of Bj < 0, one issue has to be considered is how for MAC entity to determine how many MAC SDUs to be served for LCHj. If we still leave it to UE implementation, since Bj < 0 or PBR requirement has already been satisfied, no guidance for MAC entity to determine when to stop serving MAC SDU of LCHj. We understand it would break the logic of token bucket algorithm in the LCP procedure, which leads to significant negative impacts on fairness for other LCHs.

- If allowing to allocate resource for LCH with an upgraded priority in case of Bj < 0 in 1st-round of LCP procedure, one issue is how for MAC entity to determine how many MAC SDUs to be served for LCHj, which cannot leave to UE implementation.

Therefore, if allocating resource for LCHj with an upgraded priority in case of Bj < 0 is allowed, we propose to provide a guidance for UE’s MAC entity to determine how many MAC SDUs to be served for this LCHj, i.e., limit total size of MAC SDUs served to LCHj with an upgraded priority. It cannot be fully up to UE implementation. For example, the limitation includes

Alt-1) a data volume configured by network, or a data volume defined based on PBR (e.g., PBR * T)

Alt-2) the size of priority-adjusted data available for transmission buffered for this LCH,

Alt-3) the minimum value between Alt-1) and Alt-2).

- Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network) and RAN2 to decide how for UE’s MAC entity to determine how many MAC SDUs to be served for this LCH.

Issue in running CR

We notice that an EN is captured in clause 5.4.3.1.3 Allocation of resources in the current MAC running CR, as shown below.

Editor’s Note: The above conditions on priority adjustment may need further clarifications, due to a corner case which has not been discussed yet. In this case, the data with the smallest remaining discard timer is queued behind non-delay-critical data and the UL grant is not large enough to include it in the transmission. It is FFS whether this logical channel should apply its additionalPriority or its Priority in this case.

As mentioned in this EN, it is indeed a corner case. Besides, the existing BSR/DSR as well as Rel-19 DSR can already help NW to have the knowledge to schedule the sufficient UL resources. Therefore, we understand that the enhancement on this corner case is not necessary.

- The existing BSR/DSR as well as Rel-19 DSR can already help NW to have the knowledge to schedule the sufficient UL resources.

- In case that the UL grant is not large enough to include the delay-adjusted data (queued behind non-delay-adjusted data) in the transmission, use additionalPriority without additional enhancement.

UE capability

Another issue needed to discuss is the capability for LCH priority adjustment. The LCH priority adjustment introduced in Rel-19 XR should be an optional feature for a Rel-19 UE to support, thus needing an optional capability reported to network to keep a common understanding between UE and network. R2 can confirm the introduction of the optional capability with signalling.

- Introduce a new capability on whether the UE supports LCH priority adjustment for Rel-19 XR, which is an optional capability with signalling.

# 3. Conclusion

Based on the discussion above, we made the following observations and proposals:

- Up to network implementation to avoid SRB impact due to LCH prioritization-based LCP enhancement, i.e., configure higher priority for LCH of SRBs than LCH with LCH priority-adjusted data.

- The additional LCP priority is NOT used for SR priority determination in intra-UE prioritization.

- In legacy 1st-round of LCP procedure, it is up to UE implementation to determine how many MAC SDUs to be served in an LCH with Bj > 0. A typical UE implementation is to serve MAC SDU of this LCH one by one until Bj < 0 or its PBR requirement is satisfied.

- If allowing to allocate resource for LCH with an upgraded priority in case of Bj < 0 in 1st-round of LCP procedure, one issue is how for MAC entity to determine how many MAC SDUs to be served for LCHj, which cannot leave to UE implementation.

- Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network) and RAN2 to decide how for UE’s MAC entity to determine how many MAC SDUs to be served for this LCH.

- The existing BSR/DSR as well as Rel-19 DSR can already help NW to have the knowledge to schedule the sufficient UL resources.

- In case that the UL grant is not large enough to include the delay-adjusted data (queued behind non-delay-adjusted data) in the transmission, use additionalPriority without additional enhancement.

- Introduce a new capability on whether the UE supports LCH priority adjustment for Rel-19 XR, which is an optional capability with signalling.

# 4. Reference

[1] RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia