3GPP TSG-RAN WG2 Meeting #129bis	R2-2501820

Wuhan, China, 7th – 11th April, 2025

Source:	ZTE Corporation, Sanechips

Title:	Remaining issues on PWS support for NB-IoT over NTN

Agenda Item:	8.9.4

Document for:	Discussion and Decision

Introduction

In RAN#102, a new WID for further enhancements on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) has been agreed. In RAN#105, the WID is further refined in [1] and a new objective for PWS support has been added as below:

The aim of this WI is enhancement of IoT-NTN with the following objectives: <<skip the unrelated parts>> - Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: <skip the unrelated part> - Support of Capacity enhancements for uplink <skip the unrelated part> - Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In RAN2#127bis meeting, RAN2 focus the discussion about new SIB for PWS and new PWS indication in direct indication information, and achieved the following agreements:

Agreements: - RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS - The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes. - For NB-IoT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/ skip some unnecessary fields): - SystemInformationBlockType10-NB for primary ETWS notification; - SystemInformationBlockType11-NB for secondary ETWS notification; - SystemInformationBlockType12-NB for CMAS notification. - Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB): - etws-Indication; - cmas-Indication. - Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (can come back to clarify further what immediately means) - Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases)

In RAN2#128 meeting, RAN2 focus the discussion about the remaining issues, and achieved the following agreements:

Agreements: - We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements. - We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed - Add the following PWS indication in Paging-NB: - etws-Indication; - cmas-Indication. - We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed - We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements. - Send a LS to RAN1 asking whether it’s possible to specify support for PWS reception in RRC_CONNECTED for NB-IoT in Rel-19.

In RAN2#129 meeting, RAN2 focus the discussion about the remaining issues, and achieved the following agreements:

Agreements: - Introduce geographic information in ETWS messages for geo-fencing in IoT NTN cells - Introduce the Warning Area Coordinates IE for ETWS to describe an emergency area in system information. - Introduce the Warning Area Coordinates IE to existing ETWS SIBs, i.e., SIB10-NB/11-NB, without introducing the need for segmentation of SIB10-NB - We don’t introduce a new SIB for carrying geographical information for emergency areas

In this document, we will give analysis on the remaining issues for broadcasting PWS message for NB-IoT by re-using the LTE mechanisms. Based on the analysis, we also give our proposals.

Discussion

  - PWS acquisition for UE in idle mode

As reference, the IE SystemInformationBlockType10 contains an ETWS primary notification.

SystemInformationBlockType10 information element

-- ASN1START

SystemInformationBlockType10 ::=	SEQUENCE {

messageIdentifier					BIT STRING (SIZE (16)),

serialNumber						BIT STRING (SIZE (16)),

warningType							OCTET STRING (SIZE (2)),

dummy								OCTET STRING (SIZE (50))	OPTIONAL,		-- Need OP

...,

lateNonCriticalExtension			OCTET STRING				OPTIONAL

}

-- ASN1STOP

SystemInformationBlockType10 field descriptions

messageIdentifier Identifies the source and type of ETWS notification. The leading bit (which is equivalent to the leading bit of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.44) contains bit 7 of the first octet of the equivalent IE, defined in and encoded according to TS 23.041 [37], clause 9.4.3.2.1, while the trailing bit contains bit 0 of the second octet of the same equivalent IE.

serialNumber Identifies variations of an ETWS notification. The leading bit (which is equivalent to the leading bit of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.45), contains bit 7 of the first octet of the equivalent IE, defined in and encoded according to TS 23.041 [37], clause 9.4.3.2.2, while the trailing bit contains bit 0 of the second octet of the same equivalent IE.

dummy This field is not used in the specification. If received it shall be ignored by the UE.

warningType Identifies the warning type of the ETWS primary notification and provides information on emergency user alert and UE popup. The first octet (which is equivalent to the first octet of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.50) contains the first octet of the equivalent IE defined in and encoded according to TS 23.041 [37], clause 9.3.24, and so on.

It can be seen that there is a dummy IE in legacy SIB10 for eMTC. In NR, the dummy IE has been deleted from SIB6 for an ETWS primary notification. There is no use case for keeping the dummy IE. Hence, we suggest to also delete the dummy IE for SystemInformationBlockType10-NB.

Proposal 1: It’s suggested to delete the dummy IE for SystemInformationBlockType10-NB.

In general, when the network changes (some of the) system information, it first notifies the UEs about this change, e.g., via paging message. And then in the next modification period, the network transmits the updated system information. But for ETWS notification and/or CMAS notifications, the network can transmit the SIB10 for ETWS primary notification and/or SIB11 for ETWS secondary notification, and/or SIB12 for one or more CMAS notifications in the nearest scheduled subframes, without waiting for the next modification period.

The corresponding UE behavior in the specification is as below:

TS 36.331 5.3.2.3 Reception of the Paging message by the UE //skip the unrelated part 1> if the etws-Indication is included and the UE is ETWS capable: 2> re-acquire SystemInformationBlockType1 immediately, i.e., without waiting until the next system information modification period boundary; 2> if the schedulingInfoList indicates that SystemInformationBlockType10 is present: 3> acquire SystemInformationBlockType10; NOTE 2: If the UE is in CE, it is up to UE implementation when to start acquiring SystemInformationBlockType10. 2> if the schedulingInfoList indicates that SystemInformationBlockType11 is present: 3> acquire SystemInformationBlockType11; 1> if the cmas-Indication is included and the UE is CMAS capable: 2> re-acquire SystemInformationBlockType1 immediately, i.e., without waiting until the next system information modification period boundary as specified in 5.2.1.5; 2> if the schedulingInfoList indicates that SystemInformationBlockType12 is present: 3> acquire SystemInformationBlockType12;

As above, the scheduling information for SIB10, SIB11 and SIB12 is contained in SIB1 (e.g., schedulingInfoList) . In order to simplify the receiving process and quickly acquire PWS, some companies suggested to skip the reception of SIB1. Instead, UE could store the scheduling information and utilize it to require SIB10~12 upon receiving of ETWS notification and/or CMAS notifications. However, when a UE that has just entered into a new cell receives the scheduling information in SIB1 and cannot distinguish whether it originates from the ETWS notification and/or CMAS notifications. In this case, the UE will attempt to acquire SIB10~12, resulting in the unnecessary UE actions. Besides, considering the flexible implementation of eNB, it can adjust the scheduling for SIB based on current load. Thus, the scheduling information may change. In final, it’s suggested that, upon receiving of ETWS notification and/or CMAS notifications, ETWS or CMAS capable UE firstly needs to re-acquire SIB1 immediately without waiting until the next system information modification period boundary, and then acquire SIB10, SIB11 and SIB12 according to updated scheduling information in SIB1.

Proposal 2: Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE firstly needs to re-acquire schedulingInfoList contained in SIB1 immediately without waiting until the next system information modification period boundary, and then acquire the corresponding PWS messages.

According to the description in section “10.1.4 Paging and C-plane establishment” in TS 36.300, when extended DRX (eDRX) is used in idle mode, the following exceptional case is mentioned for LTE/eMTC UE regarding PWS function:

- ETWS, CMAS, PWS requirement may not be met when a UE is in eDRX. For EAB, if the UE supports SIB14, when in extended DRX, it acquires SIB14 before establishing the RRC connection;

Also according to TS 22.268, the requirement for ETWS is as below

5.2 Duration of delivery time Duration of the delivery time for PLMN operators is the time from the receipt of the Warning Notification by the PLMN operator, i.e. the edge of the 3GPP network, to the time that the Warning Notification is successfully delivered to the UEs. Provisioning of delivery of Primary and Secondary Notification may be required: - Primary Notification shall be delivered within 4 seconds to the UE in the Notification Area even under congestion situation. - Secondary Notification is delivered to the users in the Notification Area even under congestion situation.

For NB-IoT, the defaultPagingCycle value is {rf128, rf256, rf512, rf1024}. If the defaultPagingCycle is set as rf512, or rf1024, it is longer than 4s. In this case, the requirement of EMTS primary notification can’t be satisfied. Furthermore, if PSM is enabled and UE enters into sleep mode, any notification will be missed. Hence, we suggest to add two above specific cases where ETWS, CMAS, PWS requirement may not be met.

Proposal 3: For NB-IoT UE, the ETWS, CMAS, PWS requirement may not be met in some cases. Besides the case when the UE is in eDRX, there are also other cases, e.g., when the UE is configured with a large DRX cycle (e.g., rf512, rf1024), or when PSM is enabled.

  - PWS acquisition for UE in RRC_CONNECTED

In RAN#107, it was agreed that ‘PWS notifications in paging, direct indication, and system information in RRC connected mode is not supported’. As a result, in R19, according to the views from various companies, the available solutions are list as below for further discuss:

- NB-IoT UE is provided ETWS/CMAS notification via dedicated signaling: upon reception of ETWS/CMAS notification, eNB could send the dedicated message, e.g., RRC reconfiguration message, carrying SIB10 and SIB11 or SIB12 to each capable NB-IoT UE in RRC_CONNECTED. The biggest advantage of this solution is that the UE remains in RRC_CONNECTED and can quickly resume scheduling after reception of ETWS/CMAS notification. However, from network resource standpoint, using dedicated signaling is highly inefficient and consumes excessive radio interface resources. Even such scheduling is possible, the delay may be large which would finally result in that ETWS reception cannot meet the requirement, e.g., within 4 secs.

- NB-IoT UE is released to RRC_IDLE to acquire ETWS/CMAS notification: upon reception of ETWS/CMAS notification, eNB could release all capable NB-IoT UEs in RRC_CONNECTED to RRC_IDLE to trigger them to acquire SIB messages in RRC_IDLE. This solution is simplest. However, the potential issue that could arise subsequently is that, those UEs may trigger access to the network later almost at the same time, causing serious signalling storm problem.

- In order to ensure the PWS acquisition timely, it is recommended to adopt the second solution for R19.

Proposal 4: In R19, NB-IoT UE in RRC_CONNECTED is released to RRC_IDLE to acquire ETWS/CMAS notification.

In the current TS 36.306, the related capabilities for support of the ETWS and CMAS are defined as follows:

6.2.1 ETWS It is optional for UE to support ETWS reception as specified in TS 36.331 [5]. 6.2.2 CMAS It is optional for UE to support CMAS reception as specified in TS 36.331 [5]. It is optional for a CMAS-capable UE to support Geofencing information (warningAreaCoordinates-r15) as specified in TS 36.331 [5].

In R19, those two capabilities can be introduced for NB-IoT UE.

- Proposal 5: Two capabilities to support ETWS or CMS reception can be introduced for NB-IoT UE.

Conclusion

Based on the above analysis, the following proposals are given:

Proposal 1: It’s suggested to delete the dummy IE for SystemInformationBlockType10-NB.

Proposal 2: Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE firstly needs to re-acquire schedulingInfoList contained in SIB1 immediately without waiting until the next system information modification period boundary, and then acquire the corresponding PWS messages.

Proposal 3: For NB-IoT UE, the ETWS, CMAS, PWS requirement may not be met in some cases. Besides the case when the UE is in eDRX, there are also other cases, e.g., when the UE is configured with a large DRX cycle (e.g., rf512, rf1024), or when PSM is enabled.

Proposal 4: In R19, NB-IoT UE in RRC_CONNECTED is released to RRC_IDLE to acquire ETWS/CMAS notification.

- Proposal 5: Two capabilities to support ETWS or CMS reception can be introduced for NB-IoT UE.

Reference

[1] RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc., RAN#105, September, 2024