3GPP TSG-RAN WG2 Meeting #129bis	R2-2501829

Wuhan, China, April 7th - 11th, 2025

Agenda Item:	8.2.3

Source:	HONOR

Title: 	Discussion on A-IoT random access

Document for:	Discussion and decision

1	Introduction

During RAN2#129 meeting, RAN2 discussed the A-IoT random access and made several agreements as:

Agreements

- For Rel-19, only 3-step CBRA is supported for A-IoT

- We will specify both CBRA and CFRA.

- Re-use the subsequent paging message to trigger re-access.  There is no need to differentiate msg1 resource for initial access vs re-access.

- NACK based mechanism is supported for D2R messages to determine re-access for at least msg3.  FFS details including whether we need a timer or explicit message and when reader sends feedback.

- RAN2 assumes that device randomly selects among FDMA occasions as the baseline.

In this contribution, we analyze the remaining issues of the A-IoT random access and provide proposals related to A-IoT random access procedure and the failure handling.

- Discussion

Ambient IoT, which is introduced as a new 3GPP IoT technology, aims to rely on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications and address some representative use cases. In Release 19 A-IoT WI [1], the contention-based random access is limited to the 3-step only procedure.

As shown in the TR [2], the A-IoT random access procedure consists of Random access type and access occasion/resource determination, (optional) Contention resolution of contention-based random access, and the upper layer data transmission with the reader. Furthermore, the failure handling for the random access is also important. Below, we analyse some issues found in the random access and provide the WF accordingly.

2.1 Failure detection

Timer based failure detection

For 3-step CBRA, the device considers the contention resolution to be a success only when the reader acks the same ID received in Msg1. In another word, the device considers the contention resolution as failed, if the Msg2 including the same ID in Msg1 is not received before the next occasion/slot. This device would join the subsequent paging procedure since it has not completed the RA for the previous paging with the same paging ID.

Observation 1: The device would regard the D2R transmission as failure if it has not received the corresponding mandatory R2D transmission, for example, the Msg2 is not received after sending the Msg1.

For 2 step random access, it is up to the reader implementation whether A-IoT Msg2 is sent in random access as shown in the TR. The device waits for the optional R2D transmission until the reception the Msg2 with its own RN or the next slot. What’s more, the device would not perform the re-access even if it has not received the Msg2 with its RN in Msg1. This issue is more severe for the 2-step CBRA resulted from the contention than CFRA. It is the same situation for the Msg3 in 3-step CBRA since the R2D failure/success feedback indication is optional. The device will not perform re-access in the following rounds if it is not aware of whether the Msg1/Msg3 is not well-received by the reader. That is, it is hard for the device to detect the failure without an implicit indication for Msg3 in 3-step CBRA and 2-step RA.

Observation 2: It is hard for the device to detect D2R data transmission failure if it has not received explicit R2D failure/success feedback indication since the subsequent R2D transmission is optional based on the reader implementation.

However, since RAN1 has studied A-IoT processing time aspects in terms of the following timing relationships:

TR2D_min: Minimum time between a R2D transmission and the corresponding D2R transmission following it. TD2R_min: Minimum time between a D2R transmission and the corresponding R2D transmission following it. TD2R_max: Maximum time between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max], when a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device. See clause 6.3 for message descriptions. TR2D_R2D_min: Minimum time between two different consecutive R2D transmissions to the same A-IoT device. TD2R_D2R_min: Minimum time between two different consecutive D2R transmissions from the same A-IoT device. For the time interval between a R2D transmission and the corresponding D2R transmission following it, there are two options studied: Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D ≥ TR2D_min.

As shown in the chart, the device expects the R2D transmission corresponding to the D2R transmission ahead would be sent before the TD2R_max.

Observation 3: Failure could be detected from perspective of device based on the TD2R_max (maximum time between the D2R transmission and the corresponding R2D transmission following it) defined by RAN1.

For example, Msg2 following the Msg1 is mandatory in the 3-step CBRA. Considering the failure occurs shortly before, it is possible that the explicit indication may be missed by the device. Furthermore, the reader could detection the contention without identifying the specific device(s) in this case. Based on the observation before, from the perspective of the device, it could regard the corresponding D2R transmission as failure if the R2D transmission is not received within the certain time from the corresponding D2R transmission. Thus, even though the explicit indication is missed by the device, it could determine the failure based on the gap between the D2R transmission and its following R2D transmission. For example, the TD2R_max could be used to determine the failure when there supposes to be a corresponding R2D transmission after the D2R transmission. The A-IoT processing time TD2R_max seems to be the pre-defined timer based on the UE/NW capability. In addition, the maximum time between the D2R transmission and the corresponding R2D transmission following it could be pre-configured by the reader based on its scheduling strategy or load condition. Thus, based on pre-configured/pre-defined timer, the failure detection from the device side could be the supplementary solution for the agreed explicit indication under the control of the network.

Proposal 1: Support the failure detection based on a timer which could be pre-configured time in the R2D trigger or the pre-defined time in RAN1.

Signalling based failure detection

In addition to the data transmission without contention, there may be contention resolution failure from perspectives of both reader and device for the following two cases: 1) more than one device selects the same access occasion; 2) the decoding failure of the received D2R message for one device. Unlike the D2R transmission failure, it is hard to locate only one device for the contention resolution failure for the first case. The TR only defines the default success operation that the device only determines the success of the contention resolution failure if it receives one Msg2 contains the same random number sent in its Msg1.

Proposal 2: Support NACK message for contention resolution.

The NACK message after Msg3 could be used to indicate the Msg3 transmission failure and provide some performance improvement. That is, the device would perform the re-access or re-transmission based on the indication from the reader. Furthermore, there are some circumstances that the device could determine the success/failure based on the absent/present of the subsequent R2D transmission. On one hand, the absent of the mandatory subsequent R2D message indicates the failure of the last D2R message. For example, during 3-step CBRA, the Msg2 is mandatory to ack the RN16 in the Msg1. The absent of the Msg2 would indicate the failure of the Msg1. On the other hand, the present of the subsequent R2D message indicates the success of the last D2R message. For example, in the inventory+command use case, the command would be conveyed to the device after the reader receives the device ID in the last D2R message (i.e., Msg3 in CBRA or Msg1 in CFRA). From the perspective of the device, the Msg3 is well received if it receives the command for it and it could skip the failure handling caused by Msg3 failure (e.g., the re-access in the subsequent paging procedure). Thus, the failure detection could be performed based on the signalling in the current procedure as following.

Proposal 3a: The R2D transmission indicates that there is no failure handling for the last D2R message.

Proposal 3b: The absent of subsequent R2D transmission (e.g., Msg2 after Msg1 in CBRA) indicates the device should perform failure handling caused by D2R failure.

In addition, RAN2 already agreed that the NACK message is introduced for the device to determine re-access for at least msg3. If the NACK based mechanism as in the proposal 2 could work, the NACK message could be used to confirm the failure of Msg1 and Msg3. Thus, the present of the NACK message indicates the device to perform the failure handling. For Msg3 failure, the reader could identify the specific device and send the NACK message to the specific device. However, for Msg1 failure, the reader could not identify the specific when the failure is caused by contention and the NACK message may not be target to the specific device.

Proposal 3c: The present of the NACK indicates the device(s) to perform the failure handling.

2.2 Failure handling

To handling the detected/indicated failure, one solution for the device is to perform re-access during the subsequent paging procedure with the same transaction ID. Another solution is to re-transmit the failed message. Especially for the timer-based failure detection on the device side, the device could trigger the re-transmission. For example, when the device has not received the Msg2 after a certain time from the Msg1, it could trigger the retransmission of Msg1. However, since the retransmission requires some resources, the device could only trigger the retransmission before the next slot (e.g., R2D trigger). Whether to apply the offset to the retransmission for the unsegmented packet is left as FFS when discussing the segment retransmission. We think it is natural and beneficial to reuse the offset to schedule the failed D2R again by setting the offset as zero.

Proposal 3a: Device could perform re-access upon failure detection by the device or when it is indicated by the reader (e.g., NACK for re-access).

Proposal 3b: Device could perform D2R re-transmission upon failure detection by the device if allowed or when it is indicated by the reader (e.g., NACK for re-transmission).

In case of the D2R transmission after contention resolution or the failure reception of a specific Msg1, the reader could identify one specific device in this access occasion. It is possible for the reader to provide access occasion to one device and this device could re-send the failed D2R transmission (e.g., Msg1/Msg3). Otherwise, the reader needs to trigger another paging procedure to find the failed device(s) which results in more signalling overhead and longer latency to complete the service request and may cause possible contention in the subsequent paging. In addition, if the failure is informed timely, the failure handling could be performed with less latency. When the reader detects the D2R transmission failure, it is beneficial to perform the re-transmission in the same paging procedure.

Proposal 4: NACK messages could be used by the reader to provide occasion(s) for re-transmission between A-IoT paging messages.

For both D2R transmission failure and contention resolution failure, the failure may be caused by the unavailability due to energy harvesting, coverage weakness and so on. The potential way to handle this kind of failure is to send the paging whenever the device stores enough energy and starts monitoring. Based on our understanding, higher layer repetition or multiple/subsequent paging could be used to collect all the targeted devices during one round or multiple rounds.

Proposal 5: Higher layer repetition of paging and subsequent paging could be used to handle the failure caused by un-availability.

2.3 Msg content/format

Before discussing the content/format of a specific msg in the random-access procedure. Multi-reader scenario is one of the controversial points when designing the mechanism for the AS ID, some companies argues that some ID needs to be included to identify the D2R response is for the specific reader/procedure. For the Msg(s) in the random access procedure, similar issue about the whether and how to solve the collision when the device receives R2D messages from different reader and even R2D messages from parallel procedures of one reader. Since the format of the transaction ID is still under discussion, we could delay the discussion about whether the transaction ID is enough to solve the multi-reader scenario and .

Observation 4: Unless multi-reader scenario could be solved by implementation, more information (e.g., transaction ID) is needed to resolve the collision issue.

CBRA

For CBRA, Rel-19 only focus on the 3-step procedure which means Msg1 would only carry the RN16 without higher layer data. Considering the multi-reader scenario, some ID needs to be included to identify the response is for the specific reader.

Proposal 6: For CBRA, Msg1 would carry at least the RN16.

For Msg2, RAN1 already come to the agreement about the resource allocation for the Msg3 as following:

Agreement Support the following option(s) for frequency domain resource for Msg3 transmission: · Option 2: The frequency domain resource for Msg3 can be determined based on explicit indication in the PRDCH for Msg2 transmission for one or multiples devices. · FFS: support option 1 as a default if PRDCH does not provide the indication or based on a rule o Option 1: the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device

In addition to the RN16 as the one received by Msg1, Msg2 could carry the frequency domain resource for Msg3. Furthermore, to support AS ID mechanism, Msg2 could be used to confirm/assign the AS ID to enable the scheduling of the D2R transmission as soon as possible. Thus, for the content of Msg2, we propose that:

Proposal 7: For CBRA, Msg2 would carry the RN16 as in Msg1, (optional) frequency resource information, (optional) confirm/assignment of the AS ID.

CFRA

For CFRA, the device ID would be included in the Msg1 and to algin the AS ID allocation mechanism, the RN16 is also needed in the Msg1 for the AS ID allocation/confirmation and the following scheduling.

Proposal 8: For CFRA, Msg1 would carry the device ID and RN16.

3	Conclusions

In this contribution, we discussed the issues related to the random access procedure and failure handling aspects on Ambient IoT. Based on the  discussion, the following observation and proposals are concluded:

Observation 1: The device would regard the D2R transmission as failure if it has not received the corresponding mandatory R2D transmission, for example, the Msg2 is not received after sending the Msg1.

Observation 2: It is hard for the device to detect D2R data transmission failure if it has not received explicit R2D failure/success feedback indication since the subsequent R2D transmission is optional based on the reader implementation.

Observation 3: Failure could be detected from perspective of device based on the TD2R_max (maximum time between the D2R transmission and the corresponding R2D transmission following it) defined by RAN1.

Proposal 1: Support the failure detection based on a timer which could be pre-configured time in the R2D trigger or the pre-defined time in RAN1.

Proposal 2: Support NACK message for contention resolution.

Proposal 3a: The R2D transmission indicates that there is no failure handling for the last D2R message.

Proposal 3b: The absent of subsequent R2D transmission (e.g., Msg2 after Msg1 in CBRA) indicates the device should perform failure handling caused by D2R failure.

Proposal 4: NACK messages could be used by the reader to provide occasion(s) for re-transmission between A-IoT paging messages.

Proposal 5: Higher layer repetition of paging and subsequent paging could be used to handle the failure caused by un-availability.

Observation 4: Unless multi-reader scenario could be solved by implementation, more information is needed to resolve the collision issue.

Proposal 6: For CBRA, Msg1 would carry at least the RN16.

Proposal 7: For CBRA, Msg2 would carry the RN16 as in Msg1, (optional) frequency resource information, (optional) confirm/assignment of the AS ID.

Proposal 8: For CFRA, Msg1 would carry the device ID and RN16.

4	References

- RP-250796 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

- TR 38.876 Study on solutions for ambient IoT (Internet of Things) (Release 19).