3GPP RAN WG2 Meeting #129bis	R2-2501832

# Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.4.4

Source:	HONOR

Title:	Procedures for LP-WUS in RRC_CONNECTED

Document for:	Discussion and Decision

1. Introduction

In RAN2#129 meeting, the following agreements on procedures for LP-WUS in RRC_CONNECTED were made [1]:

- For Option 1-1, UE monitors LP-WUS outside C-DRX active time at least when long DRX cycle is used. FFS whether short DRX cycle is used. - RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-1 to stop drx-onDurationTimer and drx-InactivityTimer. - RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-2 to stop the new timer and drx-InactivityTimer.

In this contribution, we will discuss activation/deactivation procedure of LP-WUS monitoring and fallback mechanism of LP-WUS monitoring.

2. Discussion

2.1. Activation/deactivation of LP-WUS monitoring

RAN1 has the following options about activation/deactivation of LP-WUS monitoring [2].

- In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

Since gNB may activate/deactivate LP-WUS monitoring frequently based on measurement report throughout the data transmission, the semi-static reconfiguration of L3 seems not flexible.

From our perspective, the procedure of the activation/deactivation of LP-WUS monitoring is as follows.

- gNB reconfigures LP-WUS configuration to UE.

- gNB reconfigures A1 event (Serving becomes better than absolute threshold) or A2 event (Serving becomes worse than absolute threshold) to UE.

- When UE reports A1 event, gNB may activate the monitoring of LP-WUS by L1/L2 signalling.

- When UE reports A2 event, gNB may deactivate the monitoring of LP-WUS by L1/L2 signalling.

Proposal 1: gNB activate/deactivate the monitoring of LP-WUS by L1/L2 signalling.

Another left issue is whether assistance information from UE is needed when gNB activate/deactivate the monitoring of LP-WUS by L1/L2 signalling. In our view, LP-RSRP/LP-RSRQ should be used as the assistance information. For example, for activation of LP-WUS monitoring, even if serving cell becomes better than threshold, it does not mean that LP-WUS channel quality meets monitoring conditions. So, it is better to take LP-RSRP/LP-RSRQ to gNB as the assistance information. The same applies to the deactivation of LP-WUS monitoring.

Proposal 2: LP-RSRP/LP-RSRQ should be used as assistance information for gNB activate/deactivate the monitoring of LP-WUS.

2.2. Fallback of LP-WUS monitoring

Since LP-WUR detection performance is not as good as MR, it may miss to detect or falsely detect LP-WUS In some scenarios. In this case, gNB cannot wake up UE to monitor PDCCH. Although gNB can determine whether UE correctly receives LP-WUS by HARQ feedback, UE is not aware that whether it is missed detection or just because NW does not send anything. So, fallback mechanism may be considered to avoid misalignment between UE and gNB. For example, for periodic data transmission, gNB may configure a parameter N. UE should fall back to the legacy CDRX monitoring mechanism if it does not receive any LP-WUS targeted to itself within N LP-WUS cycles.

Proposal 3: RAN2 should discuss the fallback mechanism of LP-WUS monitoring.

3. Conclusions

In this contribution, we discussed activation/deactivation procedure of LP-WUS monitoring and fallback mechanism of LP-WUS monitoring. These proposals are summarised as below.

Proposal 1: gNB activate/deactivate the monitoring of LP-WUS by L1/L2 signalling.

Proposal 2: LP-RSRP/LP-RSRQ should be used as assistance information for gNB activate/deactivate the monitoring of LP-WUS.

Proposal 3: RAN2 should discuss the fallback mechanism of LP-WUS monitoring.

4. References

[1] R2-2501332

[2] Final_Minutes_report_RAN1#116_v100