3GPP TSG-RAN WG2 Meeting #129bis	R2-2501836

Wuhan, China, April 7th - 11th, 2025

Agenda Item:	8.6.3

Source:	HONOR

Title: 	Discussion on measurement event evaluation and report

Document for:	Discussion and decision

1	Introduction

The following agreements related to measurement event evaluation and measurement report for purpose of supporting LTM were made during RAN2#128 meeting.

Agreements on L1 event-driven MR:

- Confirm on the previous agreement that "For measurement resource configuration, R18 LTM CSI resource configuration is reused".

- Continue to carry the bits for SSB resource set when CSI-RS resource set is configured. FFS whether CSI-RS can be configured without SSB (up to R1 to decide).

- Do not create a new resource configuration just for the event triggered measurements.

- Confirm on the previous agreement "For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused".

- Ask RAN1 what should be the maximum number of beam measurement results that can be reported in event-triggered measurement report.

- Reuse the existing RSRP values and RSRP offset values and their interpretations in TS 38.133 for event-triggered measurement report. Can revisit if RAN4 identifies any issue.

- TimeToTrigger and reportInterval of L3 measurement report can be reused. FFS on additional value(s) of reportInterval.

- Cell specific offset is not supported for LTM L1 event evaluation.

- The trigger quantity for LTM L1 event triggered measurement is the same as the report quantity.

- Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling.

- Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam.

- R18 differential RSRP encoding is used as baseline.

- LTM MR MAC CE only include one triggered measurement event and the LTM MR MAC CE can be multiplexed in any available UL grant.

- Select the beam not satisfying the event condition, based on the measured quality (i.e. according to the best beam(s))

- Truncated MR MAC CE includes the triggered beam information:

One report config ID (event config ID)

- At least one triggered beam id (SSBRI or CRI) + L1 RSRP

- Common format for normal MR MAC CE and truncated MR MAC CE is used. Details are handled by MAC running CR rapporteur.

- For truncated LTM MR MAC CE, an LCID separate from that for non-truncated LTM MR MAC CE is used.

- S-measureConfig is applicable to in the L1 event-triggered MR. No additional change is expected on RRC and MAC.

In this contribution, we further discuss the L1 measurement event evaluation and report and provide proposals accordingly.

- Discussion

Signalling burden

For the report of the LTM MR MAC CE, the signalling burden is more severe since the number of the measured SSB/CSI-RS is much bigger than the legacy. Several companies mentioned the signalling burden brought by the SSB/CSI-RS based LTM MR for candidate cells. It is straight forward to introduce a prohibit timer for the report. However, the improper timer configuration may reduce effectiveness by delaying the LTM execution. Considering the motivation of the LTM MR is to switch the cell with early sync, more factors could be considered when design the mechanism to reduce the signalling burden of LTM MR. For example, similar to the L3 measurement, the number of the good beams could be considered.  To improve the possibility of RACH-less and reduce the signalling burden of LTM MR, the early-sync status could also be one of the events to be evaluated.

Proposal 1: RAN2 to discuss how to reduce the signalling burden caused by L1 MR:

- prohibit timer,

- event LTM2~5 combined with other conditions (early-sync status),

- the number of satisfying beams,

- Others.

Additional information of the candidate cell

For UE-triggered L1 measurement report for the LTM, RAN2 needs to further discuss which beam is included in the report. Beam(s) of neighbour satisfying the event is the basic option since it is useful to help the NW determine the target beam. During the RAN2#129 meeting, it is already agreed that beam(s) not satisfying the condition could be reported within the number limitation configured by the NW. The beam(s) not satisfying the condition are ranked based on the measured quality and the best ones are selected within the limitation. That is, the UE would report some measured beams within the number limitation N. However, if some of the reported beams are with bad quality, the information is useless to the NW and may cause signalling overhead by the useless information. For the L1 measurement report of the serving cell carried by UCI, the NW needs to know the full picture of the serving cell. Since the MAC CE MR is designed for LTM, only beams good enough are useful for the NW to perform the LTM beam selection. Thus, we propose one more selection criterion for the beam not satisfying the condition.

Proposal 2: RAN2 to discuss the additional criterion for the reported beam not satisfying the condition. FFS the criterion is a pre-configured RSRP/RSRQ threshold, not satisfying the leaving condition, satisfying the entering condition, and so on.

During RAN2#129 meeting, RAN2 come to an initial consensus that the explicit indication in MR MAC CE to inform the leaving event condition is met with FFS whether it is a per beam indication. Based on the previous definition of the report on leave (e.g., leaving event), the leaving condition of the beam is also evaluated and always satisfied during the TTT. Similar to the event condition with TTT, the evaluation for the leaving event is based on a timer running in the UE side. Although the NW may distinguish the beam met the leaving event, the UE could provide a more accuracy information and reduce the NW implement complexity. Thus, we prefer the indication could distinguish which beam(s) satisfies the leaving event.

Proposal 3: For the explicit indication in MR MAC CE to inform the leaving event is met, RAN2 to further discuss the following options:

a) the MR MAC CE only includes the beam satisfying the leaving event.

b) the indication is per beam if there is no separate MR for beams satisfying entering and leaving event.

In addition, UE may perform DL/UL sync phase before the LTM execution. Especially, the UE may obtain the TA based on its own measurement or receive the TA from the previous serving cell. It is beneficial to report additional information like sync status (e.g., UL sync of the candidate cell). This assists the NW by providing sync status information, for example, to indicate whether the CG could be used in the following execution.

Proposal 4: RAN2 to discuss what additional information (e.g., UL sync status) of the candidate cell needs to be reported by the UE. FFS in the MR or other message.

Additional information of the serving cell

Additionally, during the RAN2#127-bis meeting, it was agreed that the current beam could be reported based on the NW configuration. This provides up-to-date information of the current beam to assist the NW for target beam/cell of the LTM cell switch. In addition, on one hand, beam management within the serving cell causes less interruption than the cell switch to another candidate cell; on the other hand, if there is a good enough beam with the serving cell upon the LTM cell switch, the ping pong between the previous serving cell and target candidate cell may occurs. Thus, it is beneficial to provide more latest information of the current cell to assist the .

Proposal 5: RAN2 to discuss what additional information of the serving cell needs to be reported in the MR MAC CE.

Measurement result in the MR

During RAN1#118-bis meeting, they further clarified the agreement made in RAN1#118 meeting and made the following agreement.

Agreement The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows: - UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI. - Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18) - The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used - No L1 specified filtering for time and spatial domain is introduced - No enhancement on how to report L cells x M beams - Periodic reporting on PUCCH is supported - FFS: semi-persistent reporting on PUCCH/PUSCH, and aperiodic reporting on PUSCH

Based on RAN1 agreement, it is observed that L1 filter may not be introduced. To take the robust into account, RAN2 assumed filtering was needed during RAN2#126 meeting.

Agreement

RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

There may be rapid variation of L1 measurement. It may lead to misunderstanding of the beam quality if the latest measurement result of the beam is reported when the event is met. Thus, if L1 specified filter is not introduced, RAN2 needs to further discuss what measurement results to report to further improve the robust without L1 filtering.

Proposal 6: RAN2 to discuss whether the reported L1 result needs to be filtered/averaged in MAC.

3	Conclusions

In this contribution, we discussed the L1 measurement enhancement for Rel-19 mobility WI. Based on the discussion, the following observation and proposals are concluded:

Proposal 1: RAN2 to discuss how to reduce the signalling burden caused by L1 MR:

- prohibit timer,

- event LTM2~5 combined with other conditions (early-sync status),

- the number of satisfying beams,

- Others.

Proposal 2: RAN2 to discuss the additional criterion for the reported beam not satisfying the condition. FFS the criterion is a pre-configured RSRP/RSRQ threshold, not satisfying the leaving condition, satisfying the entering condition, and so on.

Proposal 3: For the explicit indication in MR MAC CE to inform the leaving event is met, RAN2 to further discuss the following options:

a) the MR MAC CE only includes the beam satisfying the leaving event.

b) the indication is per beam if there is no separate MR for beams satisfying entering and leaving event.

Proposal 4: RAN2 to discuss what additional information (e.g., UL sync status) of the candidate cell needs to be reported by the UE. FFS in the MR or other message.

Proposal 5: RAN2 to discuss what additional information of the serving cell needs to be reported in the MR MAC CE.

Proposal 6: RAN2 to discuss whether the reported L1 result needs to be filtered/averaged in MAC.

4	References