3GPP RAN WG2 Meeting #129bis	R2-2501838

# Wuhan, China, Apr. 7th – 11th , 2025

Agenda Item:	8.7.4.1

Source:	HONOR

Title:	Discussion on LCP enhancements

Document for:	Discussion and Decision

1. Introduction

In the last meeting, the following agreements were agreed [1]:

Agreements on LCP enhancements - Only one additional priority is configured to an LCH for LCP enhancement. - We keep an existing agreement (remaining time threshold is configured per LCH) - There is no impact on BSR/SR/DSR triggering and reporting due to adjusted priority. - Intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. FFS whether this has specifications impact - FFS Intra-UE prioritization shall also use the additional LCP priority for SR priority determination - No additional PBR is needed for priority adjusted data - FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold.

As shown in above, we discuss the left issues in this document.

2. Discussion

In the last meeting, it was raised whether to allow an LCH with an upgraded priority to be transmitted if Bj is negative (if configured by the network), while the remaining time is less the configured threshold.

For the resource allocation, the standard allows the UE to allocate additional resources to logical channels during the first-round resource allocation, resulting in exceeding their Bj. A negative Bj of a logical channel indicates that it has been over-allocated resources in the short term. In order to ensure fairness, a limitation of Bj less than 0 is set in the first round of resources allocation. If this restriction is removed, the Bj value of the logical channel may be negative for a period of time. This is not benefit for the overall process of logical channel resource allocation. Therefore, it is suggested that not allowing an LCH with an upgraded priority to be transmitted if Bj is negative, while the remaining time is less the configured threshold.

Proposal 1: Do not support an LCH with an upgraded priority to be transmitted even if Bj is negative, while the remaining time is less the configured threshold.

In the running CR discussion, it is FFS whether the additional priority is explicitly specified in the standard. Since the LCH could applies two priorities in the LCP process, it is needed to clarify that the highest priority should be used in the intra-UE prioritization.

Proposal 2: The priority used in intra-UE prioritization is explicitly specified when the additional priority is applied for the LCH in the LCP procedure.

And there was an issue about the retransmission case for the intra-UE prioritization, whether priority of a MAC PDU may change over time if it includes data from a logical channel configured with additionalPriority. In genernal, the priority determination of LCH for resource allocation is applied for the new transmission. It is complex for the UE to determine the priority of LCH with priority-adjusted data every times when the data of that LCH is transmitted. For simplicity, the highest priority applied in the LCP procedure can be used in the retransmission case for intra-UE prioritization.

Proposal 3: The highest priority applied in the LCP procedure is used in intra-UE  prioritization for new transmission and retransmission.

3. Summary

Proposal 1: Do not support an LCH with an upgraded priority to be transmitted even if Bj is negative, while the remaining time is less the configured threshold.

Proposal 2: The priority used in intra-UE prioritization is explicitly specified when the additional priority is applied for the LCH in the LCP procedure.

Proposal 3: The highest priority applied in the LCP procedure is used in intra-UE  prioritization for new transmission and retransmission.

4. References

[1]	RAN2 #129 Chairman notes