3GPP TSG-RAN WG2 Meeting #129bis                                                   R2-2501849

Wuhan, China, 7th – 11th April, 2025

Source:              CATT

Title:	             Discussion on Random Access in SBFD Symbols

Agenda Item:	8.11.2

Document for:	Discussion and Decision

Introduction

In the last RAN2 meeting, random access in SBFD was further discussed. But there are still some leftover issues, listed below:

- When to perform RO type selection during the RA procedure?

- For CBRA, how to perform the RO type fallback?

- For CBRA initial transmission, how to design the signaling for indicating the RO type?

- For CBRA initial transmission, if no RO type is provided by NW,  how to select the RO type based on RSRP？

- For CFRA, which signaling is used to indicate the RO type？

- For CFRA, how to determine the RO type in case of CFRA fallbacks to CBRA?

- Whether the working assumption on supported RACH triggers can be confirmed?

- Whether early identification in Msg3 needs to be supported when legacy RO is selected?

- Whether RA-RNTI collision needs to be solved?

Discussion

Order of RO type selection

In legacy release, the RA resource selection procedure is as below:

Figure-1 Legacy RA procedure for initial RACH transmission

With the introduction of SBFD, RO type selection should be introduced in the above RA procedure. The key issue needs to be discussed is the order between the selection of RO type and the selection of the set of Random Access resources. In the following, the detailed analysis on this issue will be given.

According to the description in subcaluse5.1.1b of TS38.321, there are three main steps during the selection of the set of Random Access resources:

- Step 1:  determining whether msg3 repetition should be applied to the current RA procedure;

- Step 2:  determining whether msg1 repetition should be applied to the current RA procedure and the applicable msg1 repetition number(s);

- Step 3:  determining which set of Random Access resources should be applied for the current RA procedure considering RACH partition for feature/featurecombination.

Take CBRA as an example, the corresponding details of Step 1, Step 2 and Step 3 are shown in the following Figure-2/3/4.

Figure-2 Determining whether msg3 repetition is applied (Step 1)

Figure-3 Determining whether msg1 repetition is applied/applicable msg1 repetition number(s) (Step 2)

Figure-4 Set of Random Access resources selection for feature/featurecombination (Step 3)

Step 1

- For RACH configuration Option 1, it is unclear whether msg3-Repetitions configuration is same for legacy RO and additional RO in RAN1.

- For RACH configuration Option 2, since the additional RO is a separate set of Random Access resources, hence msg3-Repetitions configuration can be same or different from other set of Random Access resources.

Observation 1:

- For RACH configuration Option 1, it is unclear whether msg3-Repetitions configuration is same or different for legacy RO and additional RO. If it can be different, RO type selection may impact the determination of whether msg3 repetition is applied or not.

- For RACH configuration Option 2, RO type selection will impact the determination whether msg3 repetition is applied or not.

Step 2

In RAN1#119 meeting, the following agreements were reached regarding to msg1 repetition:

Agreement For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. - msg1-RepetitionNum is configured in RACH-ConfigCommon in current specification, i.e., for RACH configuration Option 2, msg1-RepetitionNum can already be separately configured for legacy-ROs and additional-ROs. Agreement For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. - Same msg1-RepetitionNum is used for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.

Based on RAN1 agreements, it is obvious that:

- For RACH configuration Option 2, both rsrp-ThresholdMsg1-RepetitionNumX and msg1-RepetitionNum can be configured differently for additional RO and legacy RO.

- For RACH configuration Option 1, only rsrp-ThresholdMsg1-RepetitionNumX can be configured differently for additional RO and legacy RO.

Hence, for either RACH configuration Option 1 or RACH configuration option 2, RO type selection may impact the determination of msg1 repetition number determination. Furthermore, for RACH configuration Option 2, RO type selection also impact whether msg1 repetition is used or not.

Observation 2:

- For RACH configuration Option 1, RO type selection will impact the determination of the applicable msg1 repetition number(s).

- For RACH configuration Option 2, RO type selection will not only impact whether msg1 repetition is applied, but also impact the determination of applicable msg1 repetition number(s).

Step 3

Based on Observation 1/2, since the RO type selection impacts the determining of whether msg1/msg3 repetition is applied and the applicable msg1 repetition number(s). Hence, it will impact the final set of Random Access resources selection because msg1/msg3 repetition are both feature.

Observation 3: RO type selection will impact the selection of the set of Random Access resources for feature/featurecombination.

Based on the above observation 1/2/3, it had better perform RO type selection before the selection of set of Random Access resources.

Proposal 1: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources.

Proposal 2: For SBFD-aware UE, whether msg1/3 repetition is applied is determined based on the selected RO type.

RO type fallback for CBRA

Based on the Figure-1 in section 2.1, it is observed that  during the RA re-attempts, the set of Random Access resources keeps unchanged even if 2-step RA fallback to 4-step RA.

Observation 4: In legacy RA procedure, only one set of Random Access resources is selected and the selected set of Random Access resources keeps unchanged during RA re-attempts even if 2-step/4-step fallback happens.

Hence, RAN2 should first discuss whether RA re-attempts should be within the same set of Random Access resources as the initial RA attempt with the introduction of SBFD.

Proposal 3: RAN2 confirms that the RA re-attempts should be within the same set of Random Access resources as the initial RA attempt with the introduction of SBFD.

With the introduction of SBFD, there are two kinds of RACH configuration:

- For RACH configuration Option 1, there is both legacy RO and additional RO within one set of Random Access resources, RO type fallback is possible without change the set of Random Access resources.

- For RACH configuration Option 2, additional RACH configuration is used. The potential additional RACH configuration can be configured as below. Similar as 2-step/4-step RA fallback, if both legacy RO and additional RO are configured within the selected set of Random Access resources, fallback is feasible; otherwise, fallback is not feasible.

Figure-5 ASN.1 structure of additional RACH configuration in RACH configuration option 2

Proposal 4: With the introduction of SBFD, for RACH configuration Option 1, RO type fallback is feasible; and for RACH configuration Option 2, RO type fallback is feasible only when there are both legacy RO and additional RO within the selected set of Random Access Resources.

Legacy RO to additional RO fallback

In RAN2#129 meeting, this issue was discussed but no conclusion was reached:

FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

In our understanding, this fallback is not preferred due to the following reasons:

- The benefit of fallback needs justification. Considering additional RO may experience larger interference due to eNB-eNB CLI, hence, the benefit of fallback from legacy RO to additional RO is doubtable.

- It may introduce more specification efforts.

  - Whether ping-pong fallback is supported, that is additional RO->legacy RO->additional RO? If this ping-pong fallback is not allowed, fallback number restriction should be introduced.

  - RAN1 already agreed 2-step RA on SBFD symbol is not supported. If 2-step RA fallback to 4-step RA, it should further discuss whether RO type fallback is supported.

Considering, the benefit of fallback from legacy RO to additional RO is unclear and it will introduce additional specification efforts, it had better not to support the fallback from legacy RO to additional RO.

Proposal 5: For CBRA, if legacy RO is selected for initial RACH transmission, fallback to additional RO is not allowed.

Signaling of the RO type indication for CBRA initial transmission

In RAN2#129 meeting, it was agreed that:

For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS.

Based on the above agreement, for CBRA initial transmission, it is obvious that the network can indicate the RO type to SBFD-aware UE. But according to the discussion in last RAN2 meeting, there is divergence on whether the indication is cell-specific or more dynamic. Hence, the detailed signaling is FFS. In the following, we will analyze whether dynamic indication is beneficial.

- For initial access, there is no RRC connection between UE and NW, hence only cell-specific indication is feasible.

- For RRC release or RRC resume, it is possible for the NW to provide UE-specific indication considering the UE position. But considering the UE may move during IDLE/INACTIVE state, the UE-specific indication may be not suitable, e.g., UE moves from the cell center to cell edge.

Hence, the benefit of UE-specific RO type indication needs justification and it is slightly prefers that the RO type indication is cell-specific.

Proposal 6: For CBRA initial RA transmission, the RO type as indicated by NW should be cell-specific.

RO type selection when no NW indication for CBRA initial transmission

In RAN2#129 meeting, it was agreed that:

If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold.

Based on the above agreement, it is only agreed RSRP can be used for RO type selection. But the details are still FFS.

Considering there are two different motivations to initiate RA on SBFD symbol, one is to reduce latency and another is to improve coverage. If the motivation is to reduce latency, UE in the center can perform RA on SBFD symbols; while if the motivation is to improve the coverage, UE in the cell edge can perform RA on SBFD symbols. Hence, it is reasonable for NW to configure how to use the RSRP threshold.

Proposal 7: In the absence of RO type indication from the NW, it can configure how to utilize the RSRP threshold for RO type selection, such as determining the RO type when RSRP is above the threshold.

Signaling of the RO type indication for CFRA

In RAN2#128 meeting, regarding to initial RO type selection of CFRA, the following agreement was reached:

The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed).

Based on the above agreements, it is obvious that the RO type is indicated by NW for CFRA. But the detailed signaling is FFS.

In NR, CFRA can be triggered by the following cases:

- Case 1: PDCCH order;

- Case 2: Msg1-based SI request;

- Case 3: BFR;

- Case 4: RRCReconfigurationWithSync;

- Case 5: Early UL synchronization with an LTM candidate cell;

- Case 6: RACH-based LTM cell switch.

For Case 1, the dedicated CFRA resources are indicated in PDCCH order. In RAN1#119 meeting, RAN1 already agreed that:

Agreement For CFRA triggered by PDCCH order for SBFD aware-UEs, SBFD-aware UE is explicitly indicated to use additional-RO or legacy-RO via a 1-bit field in the DCI of PDCCH order

Based on the above RAN1 agreement, it is obvious that RAN1 already agreed that the RO type can be indicated in the DCI of PDCCH order.

Proposal 8: For CFRA triggered by PDCCH order, follow RAN1 agreement, the RO type is explicitly indicated in the DCI of PDCCH order.

For Case 2, the CFRA resources will be indicated in SIB1. Hence, it is obvious that the RO type should also be indicated in SIB1, e.g., indicated in SI-RequestConfig and SI-RequestConfigRepetition of SIB1.

Proposal 9: For CFRA triggered by Msg1-based SI request, the RO type is indicated in both SI-RequestConfig and SI-RequestConfigRepetition.

For Case 3, the beam failure recovery configuration is delivered by dedicated RRC signalling, e.g., BeamFailureRecoveryConfig in RRCReconfiguration. Hence, the RO type should also be indicated in the BeamFailureRecoveryConfig.

Proposal 10: For CFRA triggered by BFR, the RO type is indicated in BeamFailureRecoveryConfig.

For Case 4, the dedicated RACH resource is configured via the RACH-ConfigDedicated in ReconfigurationWithSync. Hence, it is obvious that the RO type should be indicated in RACH-ConfigDedicated.

Proposal 11: For CFRA triggered by ReconfigurationwithSync, the RO type is indicatedin RACH-ConfigDedicated.

For Case 5, for early UL sync with an LTM candidate cell, the dedicated RACH resource is configured via EarlyUL-SyncConfig. Hence, the RO type should be indicated in EarlyUL-SyncConfig.

Proposal 12: For early UL sync with an LTM candidate cell, the RO type is indicated in EarlyUL-SyncConfig.

For Case 6, for RACH-based LTM cell switch, the dedicated RACH resource is configured via LTM Cell Switch Command MAC CE as below:

Figure-6 LTM Cell Switch Command MAC CE

Considering there is still R bit in the LTE Cell Switch Command MAC CE, one R bit can be used to indicate RO type.

Proposal 13: For CFRA triggered RACH-based LTM cell switch, a reserved bit in the LTM Cell Switch Command MAC CE can be used to indicate the RO type.

RO type selection in case of CFRA/CBRA fallback

According to [2], the following two kinds of RA type fallback are supported for CFRA:

- Case 1: CFRA fallbacks to CBRA, and only CBRA is performed afterwards.

  - Case1.1: CFRA is triggered, but there is always no suitable SSB/CSI-RS (i.e. no SSB/CSI-RS satisfy the RSRP requirement) during each RACH attempts or the preamble index indicated in PDCCH order is 0b000000;

  - Case 1.2: CFRA is performed for N(N≥ 1) times and when performs afterwards RA re-attempts, there is always no SSB/CSI-RS satisfy the RSRP requirement.

- Case 2: CFRA fallbacks to CBRA, and CBRA fallbacks to CFRA again…….

For Case 2, after CFRA fallback to CBRA, for RACH re-attempt, once there is suitable SSB/CSI-RS, it can fallback to CFRA again. And during CFRA, if no SSB/CSI-RS satisfy the RSRP threshold, CFRA can fallback to CBRA again…….

According to RAN2 agreement, for CFRA, the RO type can be indicated by NW. In case of CFRA fallback to CBRA, there are two kinds of RO type selection methods:

- Alternative 1:  Always use the RO type indicated by NW for CFRA;

- Alternative 2:  When performing CFRA, uses the RO type indicated by NW; when performs CBRA, it follows the RO type selection rule used for CBRA.

If Alternative 2 is adopted, RO type “ping-pong” switch is unavoidable at least for Case 2. In case of “ping-pong” switch happens, it should further discuss how to handle the power ramping. As discussed in the last RAN2 meeting, RO type “ping-pong” switch is unexpected. Hence, Alternative 1 is slightly preferred.

Furthermore, there is another issue needs to be addressed. If CFRA fallback to CBRA, and during the consecutive CBRA re-attempts, whether RO type fallback discussed in section 2.5 needs to be supported or not. If this needs to be supported, some new issues need to be discussed:

- How to define “a certain number”? Whether it includes the CFRA (re-)attempts before CBRA or not? If not considering the CFRA (re-)attempts before CBRA, a new PREAMBLE_TRANSMISSION_COUNTER need to be introduced.

- Whether there is really benefit of RO type fallback considering the next RACH re-attempt may fallback to CFRA again?

From the perspective of simplification, Alternative 1 is slightly preferred.

Proposal 14: If CFRA is triggered, UE always uses the RO type indicated for CFRA even if CFRA/CBRA fallback is performed.

Whether the working assumption on supported RACH triggers can be confirmed?

In RAN2#127 meeting, regarding to the supported RACH triggers, the following working assumption was reached:

- Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

In[1], all the RACH triggers are listed as below:

### 9.2.6 Random Access Procedure The random access procedure is triggered by a number of events: - Initial access from RRC_IDLE; - RRC Connection Re-establishment procedure; - DL or UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure (see clause 18.0) is ongoing, when UL synchronisation status is "non-synchronised"; - UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when there are no PUCCH resources for SR available; - Handover, except for when RACH-less HO is configured; - SR failure; - Explicit request by RRC upon synchronous reconfiguration; - RRC Connection Resume procedure from RRC_INACTIVE; - To establish time alignment for a primary or a secondary TAG; - Request for Other SI (see clause 7.3); - Beam failure recovery; - Consistent UL LBT failure on SpCell; - SDT in RRC_INACTIVE (see clause 18); - Positioning purpose during RRC_CONNECTED requiring random access procedure, e.g., when timing advance is needed for UE positioning; - Early UL synchronization with an LTM candidate cell; - RACH-based LTM cell switch.

From RAN2 perspective, allowing SBFD-aware to use RACH resources in SBFD symbol can reduce the RACH latency and improve RACH capacity. There is no motivation to prohibit SBFD-aware UE to use RACH resource on SBFD symbol for any of the above RACH trigger events. In last RAN2 meeting, two concerns are mentioned regarding to the working assumption in [1]:

- RACH resource for Position SI request and SI request are not needed to be extended to SBFD symbols since Position SI request and SI request are not delay-sensitive procedure.

- RACH resource configurations for Position SI request and SI request are separately configured from RACH-ConfigCommon. Supporting Position SI request and SI request on SBFD symbols will increase the on-demand SI RACH resource and specification effort.

But in our understanding, RACH on SBFD symbol is not only used to reduce latency but also to improve coverage. At least for the cell edge UE, performing Position SI request and SI request on SBFD symbol can increase the SI request successful probability. Hence, it is unnecessary to prohibit using RACH resource on SBFD symbols for Position SI request and SI request.

Proposal 15: Confirm the working assumption: random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Whether early identification is needed when legacy RO is selected?

In RAN2#127bis meeting, the following agreements were reached:

RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

Based on the above agreements, it is obvious that if additional RO is selected, early identification via Msg1 is possible. If legacy RO is selected, since SBFD is not introduced as a feature combination, early identification via Msg1 is not supported.

Based on the above background, what needs to be discussed next is whether early identification via Msg3 needs to be supported in case of legacy RO is selected. According to [1], the RRC connection setup procedure is as below:

Figure-7 UE triggered transition from RRC_IDLE to RRC_CONNECTED

If early identification via Msg3 (step 1 in the Figure-1) is supported, the NW can allocate UL resource on SBFD symbols for RRCSetupComplete(step2a), ULInformationTransfer(step5) and securityModeComplete(step7a). It can reduce the UE access latency. On the contrary, if early identification via Msg3 is not supported, the NW can only aware whether the UE supports SBFD via UE capability enquiry procedure which is performed after AS security activation (e.g., after step 7a in Figure-1). It means step 2a/step 5/step 7a can only on transmitted on UL subframes. In order to reduce the access latency, it is slightly preferred to support early identification via Msg3 in case of legacy RO is selected.

Proposal 16: If legacy RO is selected, early identification via Msg3 can be introduced.

Furthermore, early identification via Msg3 has already been supported for other features, such as (e)RedCap UE and PUCCH repetition of Msg4 HARQ-ACK. In order to support early identification via Msg3 for these features, new LCIDs for CCCH were introduced. For SBFD, if early identification via Msg3 needs to be supported, the same early identification method can be reused, that is to introduce new LCIDs for CCCH of SBFD-aware UE.

Proposal 17: If legacy RO is selected and early identification via Msg3 is supported for SBFD-aware UE, new LCIDs for CCCH SDU specific for SBFD-aware UE can be introduced.

RA-RANTI collision

In the last RAN2 meeting, some companies mentioned the RA-RNTI collision issue in case of the additional RO and legacy RO overlapped. In RAN1#120 meeting, the following conclusion was reached in RAN1:

Conclusion For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, support Alt-1: - Alt-1: do not optimize for this case

Based on the above conclusion, RA-RNTI collision can be solved by gNB implementation, no additional specification effort is needed.

Proposal 18: The RA-RNTI collision issue can be left to gNB implementation, no spec impact.

Conclusion

According to the analysis in section 2, it is proposed:

Order of RO type selection/set of Random Access resources/RA type

Proposal 1: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources.

Proposal 2: For SBFD-aware UE, whether msg1/3 repetition is applied is determined based on the selected RO type.

RO type fallback principle

Proposal 3: RAN2 confirms that the RA re-attempts should be within the same set of Random Access resources as the initial RA attempt with the introduction of SBFD.

RO type fallback (CBRA)

Proposal 4: With the introduction of SBFD, for RACH configuration Option 1, RO type fallback is feasible; and for RACH configuration Option 2, RO type fallback is feasible only when there are both legacy RO and additional RO within the selected set of Random Access Resources.

Proposal 5: For CBRA, if legacy RO is selected for initial RACH transmission, fallback to additional RO is not allowed.

Signaling for indicating the RO type (CBRA)

Proposal 6: For CBRA initial RA transmission, the RO type as indicated by NW should be cell-specific.

RO type selection without NW indication (CBRA)

Proposal 7: In the absence of RO type indication from the NW, it can configure how to utilize the RSRP threshold for RO type selection, such as determining the RO type when RSRP is above the threshold

Signaling for the RO type indication for CFRA

Proposal 8: For CFRA triggered by PDCCH order, follow RAN1 agreement, the RO type is explicitly indicated in the DCI of PDCCH order.

Proposal 9: For CFRA triggered by Msg1-based SI request, the RO type is indicated in both SI-RequestConfig and SI-RequestConfigRepetition.

Proposal 10: For CFRA triggered by BFR, the RO type is indicated in BeamFailureRecoveryConfig.

Proposal 11: For CFRA triggered by ReconfigurationwithSync, the RO type is indicatedin RACH-ConfigDedicated.

Proposal 12: For early UL sync with an LTM candidate cell, the RO type is indicated in EarlyUL-SyncConfig.

Proposal 13: For CFRA triggered RACH-based LTM cell switch, a reserved bit in the LTM Cell Switch Command MAC CE can be used to indicate the RO type.

RO type selection for CFRA/CBRA fallback

Proposal 14: If CFRA is triggered, UE always uses the RO type indicated for CFRA even if CFRA/CBRA fallback is performed.

RACH triggers

Proposal 15: Confirm the working assumption: random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Early identification in Msg3

Proposal 16: If legacy RO is selected, early identification via Msg3 can be introduced.

Proposal 17: If legacy RO is selected and early identification via Msg3 is supported for SBFD-aware UE, new LCIDs for CCCH SDU specific for SBFD-aware UE can be introduced.

RA-RNTI collision

Proposal 18: The RA-RNTI collision issue can be left to gNB implementation, no spec impact.

Reference

- R2-2500588  Remaining issues for RACH in SBFD   Apple

- TS38.300  NR; NR and NG-RAN Overall Description; Stage 2; (Release-18)

- TS38.321  NR; Medium Access Control (MAC) protocol specification (Release-18)