3GPP TSG-RAN WG2 Meeting #129bis	R2-2501859

Wuhan, China, 7-11 April 2025

Agenda Item:	8.2.3

Source:	NEC

Title:	Random Access for Ambient IoT device

Document for:	Discussion, Decision

# Introduction

In RAN2#129 meeting, the following agreements were reached for the random access procedure for AIoT device:

Agreements on AIoT RACH

- For Rel-19, only 3-step CBRA is supported for A-IoT - We will specify both CBRA and CFRA. - Re-use the subsequent paging message to trigger re-access. There is no need to differentiate msg1 resource for initial access vs re-access. - NACK based mechanism is supported for D2R messages to determine re-access for at least msg3. FFS details including whether we need a timer or explicit message and when reader sends feedback. - RAN2 assumes that device randomly selects among FDMA occasions as the baseline.

This contribution further discusses A-IoT random access.

# Discussion

## Msg.1 resource selection for Ambient IoT device

RAN1#120 meeting achieved the following agreement for FDMA of Msg.1 in CBRA:

- Support FDMA with Y≥1 D2R transmissions for Msg.1 from multiple devices in response to a R2D transmission triggering random access.

For the TDMA of Msg.1, TR38.769 summarized that,

- A R2D transmission triggering random access determines X time domain resource(s) for D2R transmission(s) for Msg.1, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s).

- Observation-1: Both TDMA and FDMA are supported for Msg.1 for A-IoT CBRA.

The slotted-ALOHA based access in which TDMA (X time domain resources) and FDMA (Y freq. domain resources) are assumed as shown in Fig.1.

Fig.1 Slotted-ALOHA based access: TDMA (X time domain resources), FDMA (Y freq. domain resources) for Msg.1

An opportunity of time-frequency resource for A-IoT device(s) to perform access (i.e., transmitting the A-IoT Msg.1 by the device) is defined as “Access Occasion” in TR38.769. A set of access occasion(s) (referred to as an Access Occasion Set in Fig.1) contains X time domain resources and Y frequency domain resources with the assumption that time synchronization of X time domain resources is maintained between the reader and the devices. It is proposed to define access occasion set on top of access occasions.

- Proposal-1: Access Occasion Set is defined as a set of access occasions. Access occasion set hosts a number of access occasions with time synchronization maintained.

RAN2#129 meeting agreed that “the A-IoT paging message can include a number of Msg.1 resources.” We assume there are following factors for the calculation of Msg.1 resources for AIoT devices:

- the number of Access Occasion Sets (in time domain)

- the number of time domain resources within an access occasion set (X)

- the number of frequency domain resources within an access occasion set (Y)

In order to simplify the indication of Access Occasions for AIoT device, it is assumed that the same number of time domain resources (X) and frequency domain resources (Y) applies to all Access Occasion Sets.

Based on the above factors, in our understanding, the number of Msg.1 resources included in the A-IoT paging message can be represented by both X and Y for each Access Occasion Set. We suggest that the A-IoT paging message include these factors to notify the number of Msg.1 resources for the devices.

- Proposal-2: The number of Msg.1 resources included in the A-IoT paging message is represented by the following parameters:
- the number of Access Occasion Sets in time domain (M)
- the number of time domain resources within an access occasion set (X)
- the number of frequency domain resources within an access occasion set (Y)

Upon receiving the above three parameters, there needs to be a discussion on how AIoT devices matching filter criteria should select their Msg.1 resources accordingly.

There may be three options for the AIoT device to randomly choose a Msg.1 resource.

Option-1: the AIoT device randomly picks three random values within the ranges (0, M-1), (0, X-1), and (0, Y-1) for the selection of the access occasion set, and the time and frequency resources within the selected access occasion set.

Option-2: the AIoT device firstly randomly choose a Access Occasion Set among all of the available Access Occasion Sets (i.e., M sets), and then randomly choose an Access Occasion among the available access occasions within that Access Occasion Set (i.e., X*Y resources ).

Option-3: the devices can randomly choose a value among (0, (M*X*Y-1)) and map that random value to an access occasion among all of the available access occasions.

- Proposal-3: RAN2 to discuss the following options for the AIoT device matching filter criteria upon receiving A-IoT paging message for CBRA on selection of Msg.1 resources

- Option1: randomly picks three random values within the ranges (0, M-1), (0, X-1), and (0, Y-1) for the selection of the access occasion set, and the time and frequency resources within the selected access occasion set.

- Option 2: firstly randomly choose a Access Occasion Set among all of the available Access Occasion Sets, and then randomly choose an Access Occasion among the available access occasions within that Access Occasion Set.

- Option-3:  the devices can randomly choose an access occasion among all of the available access occasions.

As we assume that time synchronization is maintained within an access occasion set, in our understanding, only one counter is needed for determination of the access occasion set (similar to the slot counter in RFID).

- Proposal-4: One counter is needed for determination of the Access Occasion set (similar to the slot counter in RFID).

If a device, in response to the A-IoT paging message, loads its Access Occasion Set counter with zero, it replies to the A-IoT paging message immediately (i.e., using the randomly picked time/frequency domain resources within the first Access Occasion Set). Otherwise, the device remains silent.

- Proposal-5: If a device, in response to the A-IoT paging message, loads its Access Occasion Set counter with zero, it replies to the A-IoT paging message immediately (i.e., using the randomly picked time/frequency domain resources within the first Access Occasion Set).

An R2D message is assumed to indicate the start of an access occasion set (similar to QueryRep in RFID), except for the set that immediately follows an A-IoT paging message.

The expected device behavior upon receiving such an R2D message is:

- decrement its Access Occasion Set counter by 1, and

- if the Access Occasion Set counter=0 after the decrement, reply to the A-IoT paging message immediately (i.e., using the randomly picked time/frequency domain resources within the current Access Occasion Set).

Proposal-6: The expected device behaviour upon receiving the R2D message which determines the start of one access occasion set is:
- decrement its Access Occasion Set counter by 1, and
- if the Access Occasion Set counter=0 after the decrement, reply to the A-IoT paging message immediately (i.e., using the randomly picked time/frequency domain resources within the current Access Occasion Set).

On the R2D message in Proposal 6, RAN2#129 meeting reached the following agreements:

From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later:

Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or

Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field.

Considering the expected behaviour upon receiving the R2D message which indicates the start of one access occasion set is quite different from that upon receiving the paging message, we prefer to introduce a new R2D message, i.e., Way-1.

Proposal-7: After initial paging message, introduce a new R2D message for determining the Msg1 resource(s), i.e., the start of one access occasion set.

## Msg.2 transmission in response to multiple Msg.1 transmission

RAN1#120 meeting achieved following agreement on Msg.2 transmission in response to multiple Msg.1 transmission:

Support following two options for Msg2 transmission in response to multiple Msg1 transmissions, which are initiated by a R2D transmission triggering random access.

- Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device.

- Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices.

FFS the maximum number of Msg1 responses that can be carried within a PRDCH for Msg2.

When Option 1 is applied, the reader sends separate Msg.2 to echo different RN16 which received in the Msg.1 time/frequency resources, as shown in Fig.2.

Fig.2 Msg.2 Option 1

Therefore, devices that have sent an RN16 to the reader within the current access occasion set need to continuously monitor Msg.2 until they detect their own RN16 or the next access occasion set begins. If a device receives an A-IoT Msg.2 that includes its own RN16, it considers the contention resolution successful and stops monitoring Msg.2 for the current access occasion set.

- Proposal-8: In case of Msg2 transmission corresponding to one device, devices that have sent an RN16 to the reader within the current access occasion set need to continuously monitor Msg.2 until they detect their own RN16 or the next access occasion set begins.

- Proposal-9: In case of Msg2 transmission corresponding to one device, if a device receives an A-IoT Msg.2 that includes its own RN16, it considers the contention resolution successful and stops monitoring Msg.2 for the current access occasion set.

For Option 2, a PRDCH for Msg.2 transmission corresponds to multiple A-IoT Msg.1 messages received from different devices. It accommodates the maximum number of Msg.1 responses that can be carried within a PRDCH for Msg.2. If the maximum number of Msg.1 responses is greater than or equal to the total time/frequency resources of Msg.1 within the access occasion set, a single R2D message can carry all received RN16s. In this case, devices that sent RN16 to the reader within the current access occasion set do not need to continuously monitor Msg.2.

- Proposal-10: In case of Msg2 transmission corresponding to multiple device, if a single R2D message can carry all received RN16, devices that sent RN16 to the reader within the current access occasion set do not need to continuously monitor Msg.2.

When multiple RN16s received via Msg.1 from different devices are included in a single Msg.2, it is preferable to arrange the RN16 sequence according to the time/frequency resources of Msg.1, as illustrated in Fig. 3, to simplify RN16 detection by the devices.

Fig.3 RN16 arrangement in a single Msg.2 according to the time/frequency resources of Msg.1

Proposal-11: To simplify RN16 detection by the devices, the reader arranges the received RN16 sequence in a single Msg.2 according to the time/frequency resources of Msg.1.

## D2R transmission of Msg.3

RAN1#120 meeting achieved following agreement on D2R transmissions of Msg.3:

Support FDMA of D2R transmissions for Msg3 from multiple devices in response to a PRDCH for Msg2 transmission corresponding to multiple Msg1 during access procedure.

Support the following option(s) for frequency domain resource for Msg3 transmission:

•	Option 2: The frequency domain resource for Msg3 can be determined based on explicit indication in the PRDCH for Msg2 transmission for one or multiples devices.

•	FFS: support option 1 as a default if PRDCH does not provide the indication or based on a rule

Based on above RAN1 agreements, our proposal is

Proposal-12: After the A-IoT device considers the contention resolution as successful for CBRA, it performs D2R transmissions of Msg.3 towards the reader (i.e., the upper layer data transmission) using the frequency domain resources explicitly indicated in the Msg.2.

## Msg.3 transmission failure

RAN2#129 meeting agreements related to re-access are:

Re-use the subsequent paging message to trigger re-access.

NACK based mechanism is supported for D2R messages to determine re-access for at least msg3.  FFS details including whether we need a timer or explicit message and when reader sends feedback.

For the CBRA case, after the transmission of Msg.2, there may be two cases where the reader fails to receive Msg.3 from a device.

- Case 1: The device has not received Msg.2 from the reader.

- Case 2: The device received Msg.2 and initiated Msg.3 transmission, but the reader fails to receive it successfully, possibly due to poor channel quality.

Considering the potential causes of failure, the reader has two alternatives: it can either resend Msg.2 to prompt the device to retransmit Msg.3, or it can allow the device to respond (by sending Msg.1) to a re-access procedure initiated by the reader's subsequent paging message. The choice of action is entirely up to the reader.

- Observation-2: Upon Msg.3 reception failure, the reader can either resend Msg.2 to prompt the device to retransmit Msg.3 or allow the device to respond (by sending Msg.1) to a re-access procedure initiated by the reader's subsequent paging message. The choice of action is entirely up to the reader.

In case of resending Msg.2 to trigger the device to transmit Msg.3 again, as stated in Cause 2.3, FDMA of D2R transmissions for Msg.3 is supported. From the device's perspective, the frequency domain resource for Msg.3 can be determined based on explicit indication in the PRDCH for Msg2 transmission.

- Proposal-13: In case of reader re-sending Msg.2 to trigger the device to transmit Msg.3 again, Msg.2 explicitly indicates the frequency domain resource for Msg.3.

For the cases where the reader intends to have the device respond to a re-access paging, we understand that an indication from the reader is needed to ensure the device to confirm the failure status of the current random access (i.e., the need for re-access).

Our understanding of the "NACK-based mechanism for D2R messages to determine re-access for Msg.3" is as follows:

- After sending Msg.3, devices typically assume the transmission is successful, as the system is designed to achieve a high success ratio. In other words, devices assume that re-access is not needed.

- When the device receives a "NACK" indication, it can no longer assume a successful Msg.3 transmission and must change the re-access status to "needed," indicating that re-access is required.

- Upon receiving subsequent paging with resources for re-access, a device whose re-access status is "needed" can respond to the re-access procedure by sending Msg.1. Devices with a re-access status of "not needed" (i.e., devices that do not receive a "NACK" indication) will continue to assume a successful Msg.3 transmission and will not need to respond to re-access triggered by a subsequent paging message.

Based on above understanding, we propose followings:

- Proposal-14: Support a new R2D message to convey NACK feedback for Msg.3 reception.

- Proposal-15: Reader sends the NACK feedback for Msg.3 reception upon detecting Msg.3 reception failure.

- Proposal-16: Upon receiving a NACK feedback, the device considers Msg.3 transmission failure.

- Proposal-17: Devices that consider Msg.3 transmission failure can respond to a re-access (starting from Msg.1) initiated by reader’s subsequent paging message.

Regarding subsequent paging, from the device's perspective, we think there are two possible cases:

- After a device has responded to a paging, it then receives a subsequent paging associated with the same service that contains no specific device identifier. The message targets for all devices and  CBRA based re-access is assumed in this case.

- After a device has responded to a paging, it then receives a subsequent paging associated with the same service that contains the device's specific identifier. As this message targets only for one device, CFRA based re-access is assumed in this case.

For case a), the subsequent paging for all devices may intend to provide resources for multiple devices that previously failed to access the reader. Therefore, if the device considers it has successfully responded to the same service before, it should skip sending a response to the A-IoT paging message.

Proposal-18: When the subsequent paging is for all devices (i.e., CBRA re-access assumed), a device should skip responding to this subsequent paging if it considers it has successfully responded to the same service before.

For case b), it is assumed that the reader is aware of the identities of devices that have successfully responded and wishes to allow them to respond again for certain reasons. Therefore, when the subsequent paging message contains a specific device identifier, that device should respond to the subsequent paging message as requested by the reader.

- Proposal-19: When the subsequent paging message contains specific a device identifier (i.e., CFRA re-access assumed), the device that matches the identifier should respond to the subsequent paging message as requested by the reader.

# Conclusion and Proposal

We have the following proposals:

- Observation-1: Both TDMA and FDMA are supported for Msg.1 for A-IoT CBRA.

- Observation-2: Upon Msg.3 reception failure, the reader can either resend Msg.2 to prompt the device to retransmit Msg.3 or allow the device to respond (by sending Msg.1) to a re-access procedure initiated by the reader's subsequent paging message. The choice of action is entirely up to the reader.

- Proposal-1: Access Occasion Set is defined as a set of access occasions. Access occasion set hosts a number of access occasions with time synchronization maintained.

- Proposal-2: The number of Msg.1 resources included in the A-IoT paging message is represented by the following parameters:
- the number of Access Occasion Sets in time domain (M)
- the number of time domain resources within an access occasion set (X)
- the number of frequency domain resources within an access occasion set (Y)

- Proposal-3: RAN2 to discuss the following options for the AIoT device matching filter criteria upon receiving A-IoT paging message for CBRA on selection of Msg.1 resources

- Option1: randomly picks three random values within the ranges (0, M-1), (0, X-1), and (0, Y-1) for the selection of the access occasion set, and the time and frequency resources within the selected access occasion set.

- Option 2: firstly randomly choose a Access Occasion Set among all of the available Access Occasion Sets, and then randomly choose an Access Occasion among the available access occasions within that Access Occasion Set.

- Option-3:  the devices can randomly choose an access occasion among all of the available access occasions.

- Proposal-4: One counter is needed for determination of the Access Occasion set (similar to the slot counter in RFID).

- Proposal-5: If a device, in response to the A-IoT paging message, loads its Access Occasion Set counter with zero, it replies to the A-IoT paging message immediately (i.e., using the randomly picked time/frequency domain resources within the first Access Occasion Set).

Proposal-6: The expected device behaviour upon receiving the R2D message which determines the start of one access occasion set is:
- decrement its Access Occasion Set counter by 1, and
- if the Access Occasion Set counter=0 after the decrement, reply to the A-IoT paging message immediately (i.e., using the randomly picked time/frequency domain resources within the current Access Occasion Set).

Proposal-7: After initial paging message, introduce a new R2D message for determining the Msg1 resource(s), i.e., the start of one access occasion set.

- Proposal-8: In case of Msg2 transmission corresponding to one device, devices that have sent an RN16 to the reader within the current access occasion set need to continuously monitor Msg.2 until they detect their own RN16 or the next access occasion set begins.

- Proposal-9: In case of Msg2 transmission corresponding to one device, if a device receives an A-IoT Msg.2 that includes its own RN16, it considers the contention resolution successful and stops monitoring Msg.2 for the current access occasion set.

- Proposal-10: In case of Msg2 transmission corresponding to multiple device, if a single R2D message can carry all received RN16, devices that sent RN16 to the reader within the current access occasion set do not need to continuously monitor Msg.2.

Proposal-11: To simplify RN16 detection by the devices, the reader arranges the received RN16 sequence in a single Msg.2 according to the time/frequency resources of Msg.1.

Proposal-12: After the A-IoT device considers the contention resolution as successful for CBRA, it performs D2R transmissions of Msg.3 towards the reader (i.e., the upper layer data transmission) using the frequency domain resources explicitly indicated in the Msg.2.

- Proposal-13: In case of reader re-sending Msg.2 to trigger the device to transmit Msg.3 again, Msg.2 explicitly indicates the frequency domain resource for Msg.3.

- Proposal-14: Support a new R2D message to convey NACK feedback for Msg.3 reception.

- Proposal-15: Reader sends the NACK feedback for Msg.3 reception upon detecting Msg.3 reception failure.

- Proposal-16: Upon receiving a NACK feedback, the device considers Msg.3 transmission failure.

- Proposal-17: Devices that consider Msg.3 transmission failure can respond to a re-access (starting from Msg.1) initiated by reader’s subsequent paging message.

Proposal-18: When the subsequent paging is for all devices (i.e., CBRA re-access assumed), a device should skip responding to this subsequent paging if it considers it has successfully responded to the same service before.

Proposal-19: When the subsequent paging message contains a specific device identifier (i.e., CFRA re-access assumed), the device that matches the identifier should respond to the subsequent paging message as requested by the reader.

# Reference

- Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece

- TR38.769