3GPP TSG-RAN WG2 Meeting #129	bis					R2-2501869

Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.18

Source:	Samsung

Title:	Handling downlink BWP restriction for MT-SDT

Document for:	Discussion and Decision

# Introduction

RAN2 discussed the downlink BWP restriction for MT-SDT in RAN2#129. Two solutions were discussed: a) early CQI reporting in Msg3 and b) BWP restriction removal. Due to lack of consensus on a solution, discussion was postponed to next meeting.

The intention of early CQI reporting is to facilitate efficient resource utilization e.g. by adapting MCS based on reported CQI. In our view, early CQI reporting in Msg3 affects the UL coverage of Msg3 due to increased size of Msg3. One time sending CQI reporting is not sufficient due to subsequent transmissions/receptions during the SDT procedure. Additional mechanism such as periodic/event based CQI reporting are needed. Beam management may also be needed in addition to CQI reporting for efficient communication during the SDT procedure. Considering these issues, in this paper we look at the details of solution based on BWP restriction removal.

# Discussion

Current Procedure

- UE initiate random access procedure and uses the regular (i.e. not defined for MO-SDT) RA resources. A resume cause indicating MT-SDT is included in the RRCResumeRequest/RRCResumeRequest1.

- UE transmit RA preamble on Initial Uplink BWP

- UE receives RAR on Initial Downlink BWP. UE transmits Msg3 on Initial Uplink BWP based on received UL grant in RAR.

- UE monitors PDCCH addressed to TC-RNTI on Initial Downlink BWP. UE receives Msg4 on Initial Downlink BWP. Contention resolution is successful. Random access procedure is successfully completed. TC-RNTI is promoted to C-RNTI.

- UE monitors PDCCH addressed to C-RNTI on Initial Downlink BWP.

- UL and DL data transmission/reception on Initial Uplink BWP/Initial Downlink BWP

To address the issue of BWP restriction for MT-SDT, a downlink BWP for MT-SDT can be introduced. It can be signalled in SIB1. Modified procedure for RA-based MT-SDT can be as follows:

- Network configures a downlink BWP for MT-SDT

- UE initiate random access procedure. A resume cause indicating MT-SDT is included in the RRCResumeRequest/RRCResumeRequest1. UE may also need to indicate the it supports new DL BWP for MT-SDT. New resume cause indicating MT-SDT using DL BWP for MT-SDT can be defined for this purpose. Other option could be separate RA resources/partition for MT-SDT using DL BWP for MT-SDT. Resume cause approach is simpler than separate RA resources/partition for MT-SDT as it works for both CG based and RA based MT-SDT.

- UE transmit RA preamble on Initial Uplink BWP

- UE receives RAR on Initial Downlink BWP. UE transmits Msg3 on Initial Uplink BWP based on received UL grant in RAR.

- UE monitors PDCCH addressed to TC-RNTI on Initial Downlink BWP. UE receives Msg4 on Initial Downlink BWP. Contention resolution is successful. Random access procedure is successfully completed. TC-RNTI is promoted to C-RNTI.

- UE switches to DL BWP configured for MT-SDT after the completion of RA procedure. UE monitor PDCCH addressed to C-RNTI on DL BWP configured for MT-SDT. UL and DL data transmission/reception on Initial Uplink BWP/dedicated DL BWP for MT-SDT

CG-based MT-SDT can follow the similar approach. UE switches to DL BWP configured for MT-SDT after receiving the acknowledgment (i.e. new DL assignment or UL grant for new transmission) for the initial uplink transmission.

An alternate approach would be to also configure an uplink BWP dedicated for SDT in addition to dedicated downlink BWP. UE uses them for UL/DL transmission/reception from the start of SDT procedure. No new resume cause is needed in this case. Network can know that UE will receive DL in dedicated downlink BWP as it receives the UL from UE in uplink BWP dedicated for SDT.

Proposal 1: Introduce a dedicated downlink BWP for MT-SDT. This DL BWP is configured in SIB1.

Proposal 2: Introduce a new resume cause indicating MT-SDT using downlink BWP configured for MT-SDT.

Proposal 3: For RA based MT-SDT, UE switches to DL BWP configured for MT-SDT after the completion of RA procedure.

Proposal 4: For CG based MT-SDT, UE switches to DL BWP configured for MT-SDT after receiving the acknowledgment (i.e. new DL assignment or UL grant for new transmission) for the initial uplink transmission.

In last meeting, some companies mentioned that DCI format 1_0 with CRC scrambled by C-RNTI can schedule DL resource only within the coreset 0.

Based on RAN1 spec (TS 38.212), DCI format 1_0 with CRC scrambled by C-RNTI can schedule DL resource over a bandwidth larger than coreset 0 i.e. over the full bandwidth of DL BWP as shown in highlighted green part below. The restriction to schedule DL resource only within the coreset 0 is for the case, search space for monitoring is DCI format 1_0 with CRC scrambled by C-RNTI is CSS. If the search space is USS there is no issue.

#### ----------------------------------------------------Text from TS 38.212 Start--------------------------------------------------------

#### 7.3.1.2	DCI formats for scheduling of PDSCH

##### 7.3.1.2.1	Format 1_0

DCI format 1_0 is used for the scheduling of PDSCH in one DL cell.

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats - 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format

-	Frequency domain resource assignment -  bits where  is given by Clause 7.3.1.0

#### 7.3.1.0	DCI size alignment

If necessary, padding or truncation shall be applied to the DCI formats according to the following steps executed in the order below:

Step 0:

-	Determine DCI format 0_0 monitored in a common search space according to clause 7.3.1.1.1 where  is the size of the initial UL bandwidth part.

-	Determine DCI format 1_0 monitored in a common search space according to clause 7.3.1.2.1 where  is given by

-	the size of CORESET 0 if CORESET 0 is configured for the cell; and

-	the size of initial DL bandwidth part if CORESET 0 is not configured for the cell.

-	If DCI format 0_0 is monitored in common search space and if the number of information bits in the DCI format 0_0 prior to padding is less than the payload size of the DCI format 1_0 monitored in common search space for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 0_0 until the payload size equals that of the DCI format 1_0.

-	If DCI format 0_0 is monitored in common search space and if the number of information bits in the DCI format 0_0 prior to truncation is larger than the payload size of the DCI format 1_0 monitored in common search space for scheduling the same serving cell, the bitwidth of the frequency domain resource assignment field in the DCI format 0_0 is reduced by truncating the first few most significant bits such that the size of DCI format 0_0 equals the size of the DCI format 1_0.

Step 1:

-	Determine DCI format 0_0 monitored in a UE-specific search space according to clause 7.3.1.1.1 where  is the size of the active UL bandwidth part.

-	Determine DCI format 1_0 monitored in a UE-specific search space according to clause 7.3.1.2.1 where  is the size of the active DL bandwidth part.

#### ----------------------------------------------------Text from TS 38.212 end--------------------------------------------------------

Observation 1: For USS, DCI format 1_0 with CRC scrambled by C-RNTI can schedule DL resource over a bandwidth larger than coreset 0 i.e. over the full bandwidth of DL BWP.

Regarding search space which are monitored by UE for SDT are specified as follows in TS 38.213. Currently, USS is not configured for RA based SDT as shown below.

#### ----------------------------------------------------Text from TS 38.213 Start--------------------------------------------------------

## 19.1	Configured-grant based PUSCH transmission

:

A UE can be provided a USS set by SearchSpace, or a CSS set by sdt-SearchSpace, to monitor PDCCH for detection of DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI for scheduling PUSCH transmission or of DCI format 1_0 with CRC scrambled by C-RNTI for scheduling PDSCH receptions [12, TS 38.331].

## 19.2	Random-access based PUSCH transmission

:

A UE can be provided by sdt-SearchSpace a CSS set to monitor, after contention resolution as described in clause 8.4, PDCCH for detection of a DCI format 0_0 or DCI format 1_0 with CRC scrambled by C-RNTI for scheduling respective PUSCH transmissions or PDSCH receptions; otherwise, if the UE is not provided sdt-SearchSpace, the UE monitors PDCCH according to a Type1-PDCCH CSS set as described in clause 10.1.

#### ----------------------------------------------------Text from TS 38.213 end --------------------------------------------------------

Observation 2: In the current specification, only CSS is configured for RA based SDT. Both USS/CSS can be configured for CG-SDT.

No new signalling is needed to configure USS for SDT. We need to only remove restriction that sdt-SearchSpace can configure a search space of type USS, which is possible from signalling point of view as shown below.

#### ----------------------------------------------------Text from TS 38.331 Start--------------------------------------------------------

sdt-SearchSpace-r17                 CHOICE {

newSearchSpace                      SearchSpace,

existingSearchSpace                 SearchSpaceId

}

SearchSpace information element

-- ASN1START

-- TAG-SEARCHSPACE-START

SearchSpace ::=                         SEQUENCE {

searchSpaceId                           SearchSpaceId,

controlResourceSetId                    ControlResourceSetId                                        OPTIONAL,   -- Cond SetupOnly

monitoringSlotPeriodicityAndOffset      CHOICE {

sl1                                     NULL,

sl2                                     INTEGER (0..1),

sl4                                     INTEGER (0..3),

sl5                                     INTEGER (0..4),

sl8                                     INTEGER (0..7),

sl10                                    INTEGER (0..9),

sl16                                    INTEGER (0..15),

sl20                                    INTEGER (0..19),

sl40                                    INTEGER (0..39),

sl80                                    INTEGER (0..79),

sl160                                   INTEGER (0..159),

sl320                                   INTEGER (0..319),

sl640                                   INTEGER (0..639),

sl1280                                  INTEGER (0..1279),

sl2560                                  INTEGER (0..2559)

}                                                                                                   OPTIONAL,   -- Cond Setup4

duration                                INTEGER (2..2559)                                           OPTIONAL,   -- Need S

monitoringSymbolsWithinSlot             BIT STRING (SIZE (14))                                      OPTIONAL,   -- Cond Setup

nrofCandidates                          SEQUENCE {

aggregationLevel1                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel2                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel4                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel8                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel16                      ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}

}                                                                                                   OPTIONAL,   -- Cond Setup

searchSpaceType                         CHOICE {

common                                  SEQUENCE {

dci-Format0-0-AndFormat1-0              SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-0                           SEQUENCE {

nrofCandidates-SFI                      SEQUENCE {

aggregationLevel1                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel2                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel4                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel8                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel16                      ENUMERATED {n1, n2}                         OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-1                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-2                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-3                           SEQUENCE {

dummy1                                  ENUMERATED {sl1, sl2, sl4, sl5, sl8, sl10, sl16, sl20}  OPTIONAL,   -- Cond Setup

dummy2                                  ENUMERATED {n1, n2},

...

}                                                                                           OPTIONAL    -- Need R

},

ue-Specific                                 SEQUENCE {

dci-Formats                                 ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1},

...,

[[

dci-Formats-MT-r16                   ENUMERATED {formats2-5}                                OPTIONAL,    -- Need R

dci-FormatsSL-r16                    ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1, formats3-0, formats3-1,

formats3-0-And-3-1}                        OPTIONAL,    -- Need R

dci-FormatsExt-r16                   ENUMERATED {formats0-2-And-1-2, formats0-1-And-1-1And-0-2-And-1-2}

OPTIONAL     -- Need R

]],

[[

dci-FormatsMC                        ENUMERATED {formats0-3, formats1-3, formats0-3-And-1-3} OPTIONAL,   -- Need R

dci-FormatsNCR-r18                   ENUMERATED {formats2-8}                                OPTIONAL     -- Need R

]]

}

}                                                                                                   OPTIONAL    -- Cond Setup2

}

#### ----------------------------------------------------Text from TS 38.331 end--------------------------------------------------------

Observation 3: No new signalling is needed to configure USS for SDT.

# Conclusion

Proposal 1: Introduce a dedicated downlink BWP for MT-SDT. This DL BWP is configured in SIB1.

Proposal 2: Introduce a new resume cause indicating MT-SDT using DL BWP for MT-SDT.

Proposal 3: For RA based MT-SDT, UE switches to DL BWP configured for MT-SDT after the completion of RA procedure.

Proposal 4: For CG based MT-SDT, UE switches to DL BWP configured for MT-SDT after receiving the acknowledgment (i.e. new DL assignment or UL grant for new transmission) for the initial uplink transmission.

Observation 1: For USS, DCI format 1_0 with CRC scrambled by C-RNTI can schedule DL resource over a bandwidth larger than coreset 0 i.e. over the full bandwidth of DL BWP

Observation 2: In the current specification, only CSS is configured for RA based SDT. Both USS/CSS can be configured for CG-SDT.

Observation 3: No new signalling is needed to configure USS for SDT.

# References