3GPP TSG-RAN WG2 Meeting #129bis	R2-2501879

# Wuhan, China, 7 – 11 April 2025

Agenda item:		8.6.3

Source:	Xiaomi

Title:	Remaining issues of L1 event triggered measurement reporting

Document for:		Discussion and Decision

Introduction

In RAN2#129 meeting, following was agreed regarding L1 event triggered measurement reporting [1]:

Agreements on L1 event-triggered MR:

- Confirm on the previous agreement that "For measurement resource configuration, R18 LTM CSI resource configuration is reused".

- Continue to carry the bits for SSB resource set when CSI-RS resource set is configured. FFS whether CSI-RS can be configured without SSB (up to R1 to decide).

- Do not create a new resource configuration just for the event triggered measurements.

- Confirm on the previous agreement "For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused".

- Ask RAN1 what should be the maximum number of beam measurement results that can be reported in event-triggered measurement report.

- Reuse the existing RSRP values and RSRP offset values and their interpretations in TS 38.133 for event-triggered measurement report. Can revisit if RAN4 identifies any issue.

- TimeToTrigger and reportInterval of L3 measurement report can be reused. FFS on additional value(s) of reportInterval.

- Cell specific offset is not supported for LTM L1 event evaluation.

- The trigger quantity for LTM L1 event triggered measurement is the same as the report quantity.

- Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling.

- Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam.

- R18 differential RSRP encoding is used as baseline.

- LTM MR MAC CE only include one triggered measurement event and the LTM MR MAC CE can be multiplexed in any available UL grant.

- Select the beam not satisfying the event condition, based on the measured quality (i.e. according to the best beam(s))

- Truncated MR MAC CE includes the triggered beam information:

- One report config ID (event config ID)

- At least one triggered beam id (SSBRI or CRI) + L1 RSRP

- Common format for normal MR MAC CE and truncated MR MAC CE is used. Details are handled by MAC running CR rapporteur.

- For truncated LTM MR MAC CE, an LCID separate from that for non-truncated LTM MR MAC CE is used.

- S-measureConfig is applicable to in the L1 event-triggered MR. No additional change is expected on RRC and MAC.

- Re-use existing L1 events for CLTM.

- For field ltm3-Offset-r19, the granularity is 0.5 dB.

- The Measurement Report MAC CE is transmitted with a higher LCP than 'data from any Logical Channel, except data from UL-CCCH'.

- The Measurement Report MAC CE is transmitted with the same LCH priority as MAC CE for (Enhanced) BFR.

- The max number of N (included in the MR MAC CE) is the total number of all candidate beams without counting current serving beam.

- There is no restriction on which serving cell(s) L1 Measurement Report MAC CE can be transmitted on, i.e. not limited to SpCell.

- Truncated event-triggered L1 measurement report MAC CE can be used when the available grant is not sufficient to accommodate the normal measurement report MAC CE plus subheader.

In this contribution, we discuss the remaining issues of the L1 event triggered measurement reporting.

Discussion

MAC CE format

In RAN2#129 meeting, following was agreed:

- Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling.

- Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam.

- LTM MR MAC CE only include one triggered measurement event

From above agreements, one straightforward conclusion is that a single (Truncated) Event Triggered L1 MR MAC CE can include report of both beams fulfilling the event entry condition and beams fulfilling the event leaving condition, for the same measurement event.

Proposal 1: RAN2 to confirm that a single (Truncated) Event Triggered L1 MR MAC CE can include report of beams fulfilling the event entry condition and beams fulfilling the event leaving condition, for the same measurement event.

In RAN2#129 meeting, following was agreed: “R18 differential RSRP encoding is used as baseline.” With the agreement to support differential RSRP encoding, there is a need to indicate the information of the beam with the highest L1-RSRP value at the beginning of the MAC CE, i.e. immediately after Report Config ID. There is also a need to indicate whether the beam fulfills the event entry condition. However, there is no need to indicate whether the beam fulfills the event leaving condition given that the beam has the highest L1-RSRP value and the leaving condition has additional hysteresis.

Proposal 2: For the beam with highest L1-RSRP, following is indicated in Event Triggered L1 MR MAC CE: beam ID (SSBRI or CRI), absolute value of L1-RSRP, a flag whether the corresponding beam fulfills the event entry condition. The beam information is indicated immediately after Report Config ID.

Event Triggered L1 MR MAC CE includes the following, in the order of MAC CE format. The order considers the importance of the signalling to the network. Beams not fulfilling the event entry condition and serving beam L1-RSRP are put at the end since they are optionally configured. Note that if a beam belongs to the two beam lists below, it is only reported once (e.g. it is possible that a beam fulfills the event leaving condition does not fulfill the event entry condition).

- Report config ID

- Information of beam with highest L1-RSRP (absolute encoding for L1-RSRP)

- Information of beams fulfilling the event entry condition (differential encoding for L1-RSRP)

- Information of beams fulfilling the event leaving condition (differential encoding for L1-RSRP)

- Information of beams not fulfilling the event entry condition (differential encoding for L1-RSRP), if configured

- Serving beam L1-RSRP (absolute encoding), if configured

Proposal 3: Event Triggered L1 MR MAC CE includes the following fields, in the order of MAC CE format.

- Report config ID

- Information of beam with highest L1-RSRP (absolute encoding for L1-RSRP)

- Information of beams fulfilling the event entry condition (differential encoding for L1-RSRP)

- Information of beams fulfilling the event leaving condition (differential encoding for L1-RSRP)

- Information of beams not fulfilling the event entry condition (differential encoding for L1-RSRP), if configured

- Serving beam L1-RSRP (absolute encoding), if configured

One FFS from RAN2#129 meeting is on the detailed signaling for the explicit indication of which beams fulfills the event entry condition. There are mainly two options: 1) Indicating number of beams fulfilling the event entry condition; 2) Per beam signalling on whether the beam fulfills the event entry condition. Both options work, while the first option might require more signalling overhead. The reason is as follows. For each beam using differential RSRP encoding, the beam index (SSBRI or CRI, maximum 9 bits) and differential L1-RSRP (4 bits) needs to be indicated. Two octets are needed, and 3 bits are available for other indication. With per beam flag, there is no additional overhead. However with indication of number of beams, there can be additional overhead. The reason is that the signalling for highest L1-RSRP requires two octets and one bit to indicate whether the beam fulfills the event entry condition. Report config ID needs 6 bits. Therefore only 1 bit is left, which is not sufficient to indicate the number of beams.

Proposal 4: Per beam flag is used to indicate whether the corresponding beam fulfills the event entry condition.

Another open issue is whether to have per beam indication for beams fulfilling the event leaving condition. The main alternatives are per beam indication, or per MAC CE indication of whether there exist beams fulfilling the event leaving condition. With the order of fields and per beam indication of whether beams fulfill the event leaving condition, if network does not configure UE to report beams not fulfilling the event entry condition, then the beams after the last beam fulfilling the event entry condition are beams fulfilling the event leaving condition. If network configures UE to report beams not fulfilling the event entry condition, then with per MAC CE indication of whether there exist beams fulfilling the event leaving condition, the network can only be sure that the 1st beam after the last beam fulfilling the event entry condition is the beam fulfilling the event leaving condition, but do not know for sure whether other beams are beams fulfilling the event leaving condition or not. Per beam indication is helpful to provide full information without additional overhead, as analyzed before.

Proposal 5: Per beam flag is used to indicate whether the corresponding beam fulfills the event leaving condition.

With above analysis, we can design the Event Triggered L1 MR MAC CE as in Figure 1 below. Field BI indicates the beam index (SSBRI or CRI), Ei and Li are the per beam flag indicating whether the corresponding beam fulfills the event entry condition and leaving condition, respectively.

Figure 1: Example Event Triggered L1 MR MAC CE format

In email discussion “[POST129][109][MOB] (Vivo)”, there is an editor’s note in the running MAC CR: “Whether/How to introduce padding L1 measurement report will be updated based on the progress in RAN2, if any.” L1 measurement report is essential for mobility and already has very high priority as from RAN2#129 meeting agreement “The Measurement Report MAC CE is transmitted with the same LCH priority as MAC CE for (Enhanced) BFR.” Event Triggered L1 MR MAC CE can be also sent periodically after the event is triggered. There is no motivation to introduce padding Event Triggered L1 MR MAC CE.

Proposal 6: Padding Event Triggered L1 MR MAC CE is not needed.

Truncated Event Triggered L1 MR MAC CE and Multiple Event Triggered L1 MR MAC CEs

Following was agreed on Truncated Event Triggered L1 MR MAC CE in RAN2#128 meeting:

- Truncated MR MAC CE includes the triggered beam information:

- One report config ID (event config ID)

- At least one triggered beam id (SSBRI or CRI) + L1 RSRP

- Common format for normal MR MAC CE and truncated MR MAC CE is used. Details are handled by MAC running CR rapporteur.

- Truncated event-triggered L1 measurement report MAC CE can be used when the available grant is not sufficient to accommodate the normal measurement report MAC CE plus subheader.

After UE sends the Truncated Event Triggered L1 MR MAC CE, one open question is how UE handles the remaining part of the Event Triggered L1 MR MAC CE. Given that the information is helpful for the network, it is beneficial for the UE to send remaining part of the Event Triggered L1 MR MAC CE if the UE later on receives another UL grant. Given that Event Triggered L1 MR MAC CE has high logical channel priority and Truncated Event Triggered L1 MR MAC CE has a different eLCID from Event Triggered L1 MR MAC CE, it is expected that gNB can provide another UL grant after receiving Truncated Event Triggered L1 MR MAC CE.

Proposal 7: After sending Truncated Event Triggered L1 MR MAC CE, the UE can send the remaining part of the original Event Triggered L1 MR MAC CE to the network upon receiving another UL grant.

In RAN2#128 meeting, it was agreed that “LTM MR MAC CE only include one triggered measurement event and the LTM MR MAC CE can be multiplexed in any available UL grant.” In case that UL grant cannot accommodate all the LTM MR MAC CEs (similar to the case of truncated LTM MR MAC CE), discussion is needed on how to handle LTM MR MAC CE for different measurement events, e.g. whether to prioritize certain LTM MR MAC CEs, or the handling is left to UE implementation.

Proposal 8: RAN2 to discuss how to handle the case that UL grant cannot accommodate all the triggered Event Triggered L1 MR MAC CEs, e.g. whether to prioritize certain Event Triggered L1 MR MAC CEs, or the handling is left to UE implementation.

Event configuration

In email discussion “[POST129][108][MOB] (Huawei)”, one open issue is whether ltm-CSI-RS-ReportConfigToAddModList with ltm-ReportConfigType configured as eventTriggered should only be configured on SpCell.

Currently supported L1 measurement events are as follows:

- Event LTM2: Beam of serving cell becomes worse than absolute threshold;

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

- Event LTM4: Beam of candidate cell becomes better than absolute threshold;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

Whether the restriction is needed is related to above definition of the events. Event LTM4 is only related to candidate cell, Event LTM2 is only related to serving cell, while Event LTM 3/5 are related to both serving cell and candidate cell. Given that LTM-CSI-ReportConfig-r18 is configured within ServingCellConfig (as in the following chain: ServingCellConfig >> CSI-MeasConfig >> ltm-CSI-ReportConfigToAddModList-r18 >> LTM-CSI-ReportConfig-r18), it is expected that the serving cell is implicitly provided as ServingCellConfig which includes LTM-CSI-ReportConfig-r18.

In existing definition of Event LTM 2/3/5, “serving cell” is used instead of SpCell. In comparison, in Layer 3 event A3 and A5 are both defined for SpCell. Although layer 3 event A2 is defined for serving cell, the main purpose of event A2 is for SCell management, which is not the case for Event LTM 2. Therefore the question now is whether Event LTM 2/3/5 should be defined for SpCell only or not.

As Event LTM4 is only about candidate cell, it doesn’t matter whether the related configuration is in SpCell or not. If there is a restriction of only having configuration of Event LTM 2/3/5 in SpCell, then the same restriction can be applicable to Event LTM 4.

Proposal 9: RAN2 to discuss whether serving cell in Event LTM 2/3/5 refers to SpCell only. If yes, then ltm-CSI-RS-ReportConfigToAddModList with ltm-ReportConfigType configured as eventTriggered should only be configured on SpCell.

SR priority of Event Triggered L1 MR MAC CE

In the RAN2#127bis meeting, RAN2 made the following agreements for the SR used for the Event Triggered L1 MR MAC CE:

11. The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE. 12. NW can configure a dedicated SR configuration for MR MAC CE transmission.

In RAN#129 meeting, RAN2 agreed that “The Measurement Report MAC CE is transmitted with the same LCH priority as MAC CE for (Enhanced) BFR.”

In general, we think that the triggering/cancellation/priority of the dedicated SR for the Event Triggered L1 MR MAC CE can reuse the procedure defined for the dedicated SR for BFR MAC CE. This means that the dedicated SR is triggered when a L1 measurement report is triggered. When the L1 measurement report is cancelled (e.g. due to the report/cancellation of the Event Triggered L1 MR MAC CE), the dedicated SR is also cancelled. The related note in TS 38.321 is copied below:

NOTE 3:	When the MAC entity has pending SR for SCell beam failure recovery and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery of a BFD-RS set) overlapping with PUCCH resource for SCell beam failure recovery for the SR transmission occasion, the MAC entity considers only the PUCCH resource for SCell beam failure recovery as valid. When the MAC entity has pending SR for beam failure recovery of a BFD-RS set of Serving Cell and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery) overlapping with PUCCH resource for beam failure recovery of that BFD-RS set for the SR transmission occasion, the MAC entity considers only the PUCCH resource for beam failure recovery of that BFD-RS set as valid.

According to TS 38.321 as quoted above, the BFR SR has higher priority than the BSR SR, because the beam failure recovery is more urgent than the data transmission. From our understanding, the transmission of Event Triggered L1 MR MAC CE should also have higher priority, so that the LTM cell switch can be triggered in time. This means that the priority of the SR for Event Triggered MR MAC CE should be higher than that of the SR for BSR.

Proposal 10: The dedicated SR for Event Triggered L1 Measurement Report MAC CE has higher priority than the SR for BSR.

MAC CE to activate / deactivate semi-persistent CSI-RS resource for candidate cell measurement

In RAN1 LS R1-2501500, RAN1 made the agreement that semi-persistent CSI-RS resource is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting, and asks RAN2 to design related MAC CE to activate / deactivate the related CSI-RS resource “MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2”.

For serving cell, the related MAC CE is SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE, which is defined in TS 38.321 clause 6.1.3.12, as below:

Figure 6.1.3.12-1: SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE

The description for some fields in SP CSI-RS resource set ID is as below:

-	A/D: This field indicates whether to activate or deactivate indicated SP CSI-RS and CSI-IM resource set(s). The field is set to 1 to indicate activation, otherwise it indicates deactivation;

-	SP CSI-RS resource set ID: This field contains an index of NZP-CSI-RS-ResourceSet containing Semi Persistent NZP CSI-RS resources, as specified in TS 38.331 [5], indicating the Semi Persistent NZP CSI-RS resource set, which shall be activated or deactivated. The length of the field is 6 bits;

For Rel-19 LTM, RRC signalling for semi-persistent CSI-RS resource for candidate cell measurement is discussed in email discussion “[POST129][108][MOB] (Huawei)”, and rapporteur proposal on related ASN.1 signalling is as follows:

LTM-NZP-CSI-RS-ResourceConfig-r19 ::= SEQUENCE {

ltm-NZP-CSI-RS-ResourceConfigId-r19                  LTM-NZP-CSI-RS-ResourceConfigId-r19,

ltm-NZP-CSI-RS-ResourceType-r19                      ENUMERATED { periodic, semi-persistent},

ltm-NZP-CSI-RS-ResourceSetList-r19                   SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId    OPTIONAL,    -- Need R

...

}

LTM-NZP-CSI-RS-ResourceConfigId-r19 ::= INTEGER (0..maxNrofCSI-ResourceConfigurations-1)

maxNrofCSI-ResourceConfigurations       INTEGER ::= 112     -- Maximum number of resource configurations

maxNrofCSI-ResourceConfigurations-1     INTEGER ::= 111     -- Maximum number of resource configurations minus 1

Although detailed RRC signalling for CSI-RS resource configuration might be related to RAN1 RRC parameter list, RAN2 can agree on high level design principles for MAC CE to activate / deactivate semi-persistent CSI-RS resource for candidate cell measurement.

Since the activation / deactivation of SP CSI-RS resource is for candidate cell only, it is straightforward to only signal two fields: SP CSI-RS Resource Conifg ID and corresponding A/D field. There seems no need to indicate serving cell ID, BWP ID, SP CSI-IM resource set ID, TCI state ID, which are included in SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE.

Proposal 11: MAC CE to activate / deactivate semi-persistent CSI-RS resource for candidate cell measurement includes the following fields: SP CSI-RS Resource Config ID and corresponding A/D field.

Conclusion

In this contribution, we discuss the remaining issues of the L1 event triggered measurement reporting, and propose the following:

MAC CE format

Proposal 1: RAN2 to confirm that a single (Truncated) Event Triggered L1 MR MAC CE can include report of beams fulfilling the event entry condition and beams fulfilling the event leaving condition, for the same measurement event.

Proposal 2: For the beam with highest L1-RSRP, following is indicated in Event Triggered L1 MR MAC CE: beam ID (SSBRI or CRI), absolute value of L1-RSRP, a flag whether the corresponding beam fulfills the event entry condition. The beam information is indicated immediately after Report Config ID.

Proposal 3: Event Triggered L1 MR MAC CE includes the following fields, in the order of MAC CE format.

- Report config ID

- Information of beam with highest L1-RSRP (absolute encoding for L1-RSRP)

- Information of beams fulfilling the event entry condition (differential encoding for L1-RSRP)

- Information of beams fulfilling the event leaving condition (differential encoding for L1-RSRP)

- Information of beams not fulfilling the event entry condition (differential encoding for L1-RSRP), if configured

- Serving beam L1-RSRP (absolute encoding), if configured

Proposal 4: Per beam flag is used to indicate whether the corresponding beam fulfills the event entry condition.

Proposal 5: Per beam flag is used to indicate whether the corresponding beam fulfills the event leaving condition.

Proposal 6: Padding Event Triggered L1 MR MAC CE is not needed.

Truncated Event Triggered L1 MR MAC CE and Multiple Event Triggered L1 MR MAC CEs

Proposal 7: After sending Truncated Event Triggered L1 MR MAC CE, the UE can send the remaining part of the original Event Triggered L1 MR MAC CE to the network upon receiving another UL grant.

Proposal 8: RAN2 to discuss how to handle the case that UL grant cannot accommodate all the triggered Event Triggered L1 MR MAC CEs, e.g. whether to prioritize certain Event Triggered L1 MR MAC CEs, or the handling is left to UE implementation.

Event configuration

Proposal 9: RAN2 to discuss whether serving cell in Event LTM 2/3/5 refers to SpCell only. If yes, then ltm-CSI-RS-ReportConfigToAddModList with ltm-ReportConfigType configured as eventTriggered should only be configured on SpCell.

SR priority of Event Triggered L1 MR MAC CE

Proposal 10: The dedicated SR for Event Triggered L1 Measurement Report MAC CE has higher priority than the SR for BSR.

MAC CE to activate / deactivate semi-persistent CSI-RS resource for candidate cell measurement

Proposal 11: MAC CE to activate / deactivate semi-persistent CSI-RS resource for candidate cell measurement includes the following fields: SP CSI-RS Resource Config ID and corresponding A/D field.

- References

[1] R2-2501331, Vice Chairman (Samsung), "Report from session on R17/18 V2X/SL, R18/19 MOB and R19 NES"