3GPP TSG-RAN WG2 Meeting #129bis	 R2-2501927

Wuhan, China, Apr. 7th – 11th, 2025

Source:	CATT, Turkcell

Title:	Simulation results of measurement event prediction and handover performance

Agenda Item:	8.3.3

Document for:	Discussion and Decision

Introduction

In the previous meeting, conclusions were reached regarding measurement event prediction for temporal domain case B. It is confirmed that for temporal domain Case B, the AI-based HO has a minor/no system-level performance (i.e., HOF rate and HO number) decrease compared with the legacy HO mechanism. For temporal domain case A, it was concluded that the HOF rate of options 2 and 3 are better than the baseline and F1 score is very good. But there are some other aspects including different UE speed, TTT length and etc. need to be studied. In this document, we will focus on indirect measurement event prediction for temporal domain case A under these aspects.

Discussion

2.1 Simulation parameters

Regarding AI prediction, the AI model and the parameters used in our simulations for temporal domain case A are provided in Table 1.

Table 1 Simulation parameters for indirect measurement event prediction for temporal domain case A

| Report parameters |  | CATT |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |  |  |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |  |  |  |
|  | UE speed (30,60,90,120 Km/h) | 60, 90, 120 |  |  |  |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | N/A |  |  |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | N/A |  |  |  |
|  | Whether LOSsoft is modeled or not | Yes |  |  |  |
|  | spatial consistency option (A or B) | Procedure A |  |  |  |
|  | Number of TX beams | 32 |  |  |  |
|  | Number of RX beams | 4 |  |  |  |
|  | Measurement reduction rate(50%,…Note2) | N/A |  |  |  |
|  | Observation window(Note3) | 320ms | 400ms |  | 800ms |
|  | Prediction window (Note3) | 320ms | 400ms |  | 400ms |
|  | Any other parameters (Note 4) | Max ETD (ms, note1) |  | 80ms |  |
|  |  | A3 event offset (db) |  | 2 |  |
|  |  | TTT (ms) |  | 160, 320 |  |
| Data Size (Number of Samples) | Training/validity | ≈194K/48K |  |  |  |
|  | Testing | ≈48K |  |  |  |
| AI/ML model input/output | Model input (Note 5) | Cell level RSRP of all cells |  |  |  |
|  | Model output(Note 6) | Cell level RSRP of all cells |  |  |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |  |  |  |
|  | Model complexity in a number of parameters(M) | 0.97M | 5.601M |  | 5.601M |
|  | Model complexity in model size (e.g. Mbyte) | 11.7MB | 64.138MB |  | 64.138MB |
|  | Computational complexity [FLOPs] | 3.79M | 23.44M |  | 55.09M |

2.2	F1 score

The F1 score is the harmonic mean of precision and recall, and its value range is from 0 to 1. The higher the F1 score, the better the performance of the AI model. According to the formula, Precision refers to the proportion of true positive samples among all samples predicted as positive; Recall refers to the proportion of samples correctly predicted as positive among all samples that are actually positive. In AI prediction for A3 event, Precision means when AI predicts an A3 event, real A3 events may be triggered with Precision percentage. Recall means when a real A3 event is triggered, the AI model will predict an A3 event with Recall percentage.

During RAN2#129 meeting, the offline discussion reached the agreement to set the length of PW and TTT to 320ms. In our simulations, the OW length is set to 320ms based on the agreement, corresponding to the 4N:4N configuration (N represents a measurement period), as shown in Figure 1 and sliding L1/L3 filtering is used.

Figure 1: RSRP predication for measurement event prediction for temporal domain case A

During the previous meeting, some companies proposed using latest real RSRP measurement results for target cell selection. However, we need to emphasize that within the length of the PW, there are multiple L3 measurement period and RSRP is different at different samples within PW, particularly in high-speed scenarios. Assuming the latest real RSRP measurement result is used to estimate whether A3 enter condition is satisfied, and only one cell meets the A3 entering condition, the prediction result from AI or the real measurement result can both be used to select the target cell, since the same cell is estimated. However,  when multiple cells simultaneously satisfy the A3 entering condition based on the real measurement results, the optimal target cell may vary dynamically within PW based on AI prediction. Hence, the target cell selected based on the latest real RSRP measurement may not align with the one which is selected based on the AI prediction. Hence, using latest real RSRP measurement results for target cell selection is not suitable in this case. Taking these into consideration,  we suggest using the prediction results of AI to select the target cell. This approach better accommodates rapid channel variations and ensures more stable handover decisions in mobile environments.

Observation 0: The target cell selected based on the latest real RSRP measurement may not align the cell selected based AI prediction when multiple target cells meets the entry condition of A3 event. Hence, using the prediction results of AI to select the target cell is preferred.

For AI prediction, the RSRP can be predicted at multiple points in the future. And in RAN2#127bis meeting, it was agreed that:

- Companies can provide multiple real time RSRP value(s) and/or average RSRP value over the entire window and should indicate in their simulation results what they have used.   The companies should at least provide the results of only one value it should be the last value at the end of the PW.   We will add two columns in the spreadsheet to capture the last  value and the average value.

Correspondingly, the following 2 options are provided on how to select the target cell:

- Option1: the average RSRP within the TTT;

- Option2: the last RSRP value at the end of the TTT.

Normally, upon A3 event triggering, UE reports the latest RSRP of the corresponding measurement object to the network in the measurement report, and the network can select the target cell based on this real-time data. Option 2 aligns with this standard procedure. Therefore, we choose Option 2 in our simulation.

According to the analysis above, it is proposed that:

Proposal 1: For indirect measurement event prediction, when there are multiple predicted RSRP values within TTT, the following 2 options to select target cell can be considered:

- Option1: the average RSRP within the TTT;

- Option2: the last RSRP value at the end of the TTT.

- 2.2.1 Speed

To investigate the impact of different speeds on F1 scores, we conducted simulations under constant conditions where both TTT and PW were maintained at 320ms, and the performance metrics were evaluated at three different speeds: 60km/h, 90km/h, and 120km/h. The simulation results are shown in Table 2.

Table 2: F1 score/Precision/Recall at different speeds

| Speed | Precision | Recall | F1 score |
| --- | --- | --- | --- |
| 60 km/h | 94.28% | 95.21% | 94.74% |
| 90 km/h | 94.55% | 94.42% | 94.48% |
| 120 km/h | 94.44% | 94.34% | 94.39% |

Based on the simulation result above, we find that for temporal domain case A at different speeds:

- Precision of different speeds is greater than 94% when OW, PW and TTT are all equal to 320ms. It means when AI predicts an A3 event, real A3 event trigger rate is greater than 94%. I.e. when AI predicts 100 A3 events, 94 real A3 events will be triggered and 6 predicted A3 event is falsely predicted;

- Recall of different speeds is also greater than 94% when OW, PW and TTT are all equal to 320ms. It means when a real A3 event is triggered, the AI model has a prediction rate of greater than 94%. I.e. when 100 real A3 events are triggered, the AI model will predict 94 A3 events and 6 real A3 event is missed;

- Based on the Precision and Recall, F1 score is greater than 94%;

- F1 score decrease slowly as speed increases.

Observation 1: For indirect measurement event prediction for temporal domain case A with different speeds, it was observed that:

- Precision, Recall and F1 score are greater than 94%;

- F1 score decrease slowly as speed increases.

- 2.2.2 TTT

We also evaluate the effects of varying TTT values on F1 scores when PW equals to 320ms and UE speed is 90km/h. TTT equal to 160ms and 320ms are simulated. In order to ensure the same condition as TTT equal to 320ms, inference is initiated after entering condition of measurement event is met based on actual measurement result.

Table 3: F1 score/Precision/Recall at different TTT

| TTT | Precision | Recall | F1 score |
| --- | --- | --- | --- |
| 160ms | 96.79% | 95.84% | 96.31% |
| 320ms | 94.55% | 94.42% | 94.48% |

It can be seen that when TTT is reduced from 320ms to 160ms, all performance metrics showed significant improvement:

- Precision increased by 2.24% (94.55% -> 96.79%);

- Recall rose by 1.42 % (94.42% -> 95.84%);

- F1 score improved by 1.83% (94.48% -> 96.31%).

Observation 2: Precision, Recall and F1 score increase with the decreases of the TTT from 320 to 160ms, Precision increased by 2.24%, Recall increased by 1.42 %, and F1 score increased by 1.83%.

This is related to the AI prediction accuracy, the accuracy of RSRP difference increases as the prediction point index increases within the prediction window. Therefore, when both OW and PW are set to 320ms, the shorter TTT durations result in a smaller RSRP difference predicted by AI within the TTT time and a higher F1 score.

- 2.2.3 OW

Table 4 shows F1 score for different OW when UE speed is 90 km/h and PW is 400ms.

Table 4 F1 score/Precision/Recall of different OW

| [OW, PW](ms) | Precision | Recall | F1 score |
| --- | --- | --- | --- |
| [400, 400] | 89.46% | 89.05% | 89.25% |
| [800, 400] | 94.38% | 94.14% | 94.62% |

Results demonstrate progressive improvements in Precision, Recall, and F1 scores as the length of OW from 400ms to 800ms, with the following improvement rates. This is due to the enhanced accuracy of AI prediction by extending the observation window.

- Precision increases by 5.5% (89.46% -> 94.38%);

- Recall improves by 5.7% (89.05% -> 94.14%);

- F1 score rises by 5.9% (89.25% -> 94.62%).

Based on the above analysis, we have the following observation:

Observation 3: When PW is constant, Precision, Recall and F1 score increases with the increases of the OW length from 400 to 800ms, Precision increased by 5.5%, Recall increased by 5.7%, and F1 score increased by 5.9%.

2.3	Handover performance

It was agreed in RAN2#127bis meeting that “The HO model in 36.839 is taken as baseline”. And based on 36.839, the HoF is defined as below:

Definition 3: A handover failure is counted if a RLF occurs in state 2, or a PDCCH failure is detected in state 2 or state 3. For calculating the handover failures for the two states: - In state 2: when the UE is attached to the source cell, a handover failure is counted if one of the following criteria is met: 1) Timer T310 has been triggered or is running when the HO_CMD is received by the UE (indicating PDCCH failure) or 2) RLF is declared in the state 2 - In state 3: after the UE is attached to the target cell a handover failure is counted if the following criterion is met: - target cell downlink filtered average (the filtering/averaging here is same as that used for starting T310) wideband CQI is less than the threshold Qout (-8 dB) at the end of the handover execution time (Table 5.1.4.1) in state 3.

In our simulation, the HoF in each state is simulated, and whether/how AI improves the aspects of HO performance for HO number, HOF number and HoF rate are also shown. In order to facilitate the discussion in the document, we define the following terminologies:

- HoF 2_1: Timer T310 has been triggered or is running when the HO_CMD is received by the UE (indicating HO_CMD failure);

- HoF 2_2: RLF is declared in the state 2 (late HO).

- 2.3.1 Different options of HO procedure

For system level simulation, the following procedures were agreed upon at RAN2#128 meeting.

For System level simulation 2 For measurement event prediction for temporal domain case A, company focus on option 2 or option 3. Option 2: network transmit handover command purely based on actual measurement event regardless whether an actual measurement result(@t2) is earlier or later than predicted measurement event((@t1)) Option 3: For AI mobility, HO preparation starts when an event is predicted to happen (i.e., t0), and HO command is sent when A3 entering conditions are met based on actual/real measurement and an event is predicted to be met for the duration of TTT.

With assumption agreed in RAN2#128 and 129 meeting and the observation window length is 320ms in AI model, the HO number, HOF number and HoF failure rate are shown in Table 5.

Table 5: HO performance for FR2 to FR2 intra-frequency temporal domain case A

| Case | Handover number | Handover failure number | Handover failure rate | Handover failure number 2_1 | Handover failure rate 2_1 |
| --- | --- | --- | --- | --- | --- |
| Real HO procedure | 0.2356 per UE per second | 0.0406 per UE per second | 17.25% | 0.0297 per UE per second | 12.60% |
| HO procedure with AI prediction (Option2) | 0.2362 per UE per second | 0.0403 per UE per second | 17.07% | 0.0292 per UE per second | 12.37% |
| HO procedure with AI prediction (Option3) | 0. 2469per UE per second | 0.0316 per UE per second | 12.79% | 0.0200 per UE per second | 8.10% |

Based on the simulation result above, it can be seen that:

- The handover number of Option2 and Option3 are almost the same as the real handover number, ;

- The handover failure number of Option2 and Option3 are lower than the real handover failure number, i.e., AI prediction reduces the handover failure number;

- The handover failure rate of Option2 and Option3 are lower than the real handover failure rate, that is, AI prediction reduces the handover failure number;

- Option3 brings higher gain on handover failure number/rate reduction than that of option 2;

- For Option2 and Option3, the HoF reduction of HoF 2_1 contributes most of the gain for HoF reduction.

Observation 4: For indirect measurement event prediction for temporal domain case A, compared with Real handover procedure:

- The handover number of Option2 and Option3 is comparable to the Real handover procedure, which around 0.2 per UE per second with UE Speed = 90Km/h;

- The handover failure number are reduced by 0.73% for Option 2 and 22.1% for option 3 respectively;

- The handover failure rate is reduced by 1.46% for Option 2 and 25.8% for Option 3 respectively.

For Option2, handover command is sent with handover preparation duration in advance, which avoids the handover failure that the channel quality deteriorates during HO preparation process and the UE cannot receive the HO CMD.

For Option3, handover command is sent with TTT and handover preparation duration in advance. Normally, TTT is used to avoid measurement jilter so that a cell which shows stable signal performance can be selected as a target cell. In Option 3, the target cell is predicated to trigger A3 event, which means the target cell is stable to fulfill A3 event based on AI prediction. Considering the A3 event offset is set as 2dB in the simulation, the RSRP of the target cell is higher than that of the serving cell when A3 event is triggered. In this case, the channel condition of the target cell is better than that of the source cell. The UE should access to the target cell as early as possible to obtain the benefits of better channel condition of the target cell. It can be seen that the HoF number and HOF rate decreases if the UE accesses to the target cell with handover preparation and TTT duration in advance, and it is better than that if UE accesses to the target cell with handover preparation duration in advance, which proves the analysis above.

Observation 5: HoF rate and HOF number reduction for the HO_CMD failure case contributes most of the gain for HoF rate and HOF number reduction with HO model option 3 and option2 for FR2 to FR2 intra-frequency temporal domain case A.

Observation 6: Option3 is superior to option2 both in terms of handover failure rate and handover failure number.

Based on the above analysis, the handover performance of Option3 is better than that of option2. In the following sections, we simulate the HO performance for FR2 temporal domain case A with Option 3 under different speeds, TTT and OW.

- 2.3.2 Speed

When both the PW, OW and TTT are set to 320ms, the handover number, handover failure number, handover failure rates, and corresponding AI gain compared to real handover are illustrated in Figure 2.

Figure 2: HO performance at different speed

Based on the simulation result above, we find that the handover performance for temporal domain case A at different speeds:

- At the same speed, the handover number of option3 is slightly higher than that of real handover procedure which is around 0.3%;

- When the speed is 30, 60, and 90 km/h, the number of handover failures number per UE per second of option3 is decreased by 0.0038, 0.009, and 0.0154 respectively compared to real at the same speed;

- When the speed is 30, 60, and 90 km/h, the handover failure rate of option3 is decreased by 2.72, 4.46, 5.88 respectively compared to real at the same speed;

- The handover failure rate ((handover failure rate of real - handover failure rate of option3)/ handover failure rate of real) and handover failure number gain of option3 increases as the speed increases.

Observation 7: For indirect measurement event prediction for temporal domain case A with different speeds, the AI gain of handover failure rate and handover failure number increase with the increase of UE speed, e.g., from 18.29% to 25.86% and then to 31.13%, and from 14.45% to 22.17% and then to 27.8% when UE speed increases from 60 to 90 and to 120 km/h.

Our observations reveal an intriguing contradiction: both the HOF rate and HOF number demonstrate accelerated reduction as speed increases, while AI prediction accuracy conversely degrades with higher speeds. Through analysis, this is because the RSRP difference between the two consecutive moment increases with the increase of speed, and the RSRP difference between consecutive moments within TTT is shown in Figure 3. In actual handover process, the distance between the UE and the source cell increases with the increase of UE speed, which causes a more severe degradation in the signal quality of source cell, ultimately leading to a higher handover failure rate. For Option3, handover in advance of the HO preparation and TTT duration will reduce most of the handover failures caused by the deterioration of signal quality in the TTT time, so with the increase of the speed, the gain of AI is also increasing.

Figure 3: RSRP difference between consecutive moments within TTT

- 2.3.3 TTT

When PW is equal to 320ms, the handover performance of TTT equal to 160ms and 320ms is simulated. The simulation results are shown in Figure 4.

Figure 4: HO performance at different TTT

Based on the simulation result above, we find that the handover performance for temporal domain case A at different TTT:

- At the same TTT, the handover number of option3 is 0.009-0.011 per UE per second higher than that of real handover procedure;

- When the TTT is 160, 320 ms, the number of handover failures number per UE per second of option3 is decreased by 0.0041, 0.0096, respectively compared to real at the same TTT;

- When the TTT is 160, 320 ms, the handover failure rate of option3 is decreased by 1.95, 4.46 respectively compared to real at the same TTT;

- The handover failure rate ((handover failure rate of real - handover failure rate of option3)/ handover failure rate of real) and handover failure number gain of option3 increases as the TTT increases.

Observation 8: When TTT increases from 160 to 320 ms, the AI gain for handover failure rate and handover failure number increases, e.g., from 14.17% to 25.86%, and from 11.08% to 22.17%.

When PW is equal to 320ms and TTT changes from 160ms to 320ms, the AI prediction accuracy decreases, but the AI gain for handover failure rate and handover failure number are increases. This is because when the TTT increases, the channel quality of the serving cell will become worse, and the probability of handover failure will be increase. Therefore, as TTT becomes longer, the more handover failures number that option3 can reduce, the greater the gain of AI, because option3 can reduce the real handover failure in the TTT time, even if the RSRP predicted by AI becomes more accurate as the AI prediction window becomes shorter.

- 2.3.4 OW

The handover performance of different observation windows is simulated, and the simulation results  are shown in Table 6, with a fixed PW length  of 400ms and OW length of 400ms and 800ms.

Table 6: HO performance of Option 3 at different OW

| Case | Handover number | Handover failure number | HoF rate(%) |
| --- | --- | --- | --- |
| HO procedure with AI prediction (OW=400ms) | 0.2504 per UE per second | 0.0329 per UE per second | 12.74% |
| HO procedure with AI prediction (OW=800ms) | 0.2389 per UE per second | 0.0269per UE per second | 11.29% |

As shown in the Table 6, increasing the OW length leads to concurrent reductions in both HOF rate and HOF number. This is related to the relationship between the length of OW and the prediction accuracy. The prediction accuracy is improved with the increase of OW length, so that more timely and accurate handover can be achieved.

Observation 9: The HoF rate and HOF number decreases with the increase of the OW length from 400 to 800ms, e.g., from 12.74% to 11.29%, and from 0.2504 to 0.2389 per UE per second.

Conclusion

In this document, we shows the simulation results of F1 score for indirect measurement event prediction for temporal domain case A and case B, and it can be seen that:

Target cell selection:

Observation 0: The target cell selected based on the latest real RSRP measurement may not align the cell selected based AI prediction when multiple target cells meets the entry condition of A3 event. Hence, using the prediction results of AI to select the target cell is preferred.

Proposal 1: For indirect measurement event prediction, when there are multiple predicted RSRP values within TTT, the following 2 options to select target cell can be considered:

- Option1: the average RSRP within the TTT;

- Option2: the last RSRP value at the end of the TTT.

F1 score:

Observation 1: For indirect measurement event prediction for temporal domain case A with different speeds, it was observed that:

- Precision, Recall and F1 score are greater than 94%;

- F1 score decrease slowly as speed increases.

Observation 2: Precision, Recall and F1 score increase with the decreases of the TTT from 320 to 160ms, Precision increased by 2.24%, Recall increased by 1.42 %, and F1 score increased by 1.83%.

Observation 3: When PW is constant, Precision, Recall and F1 score increases with the increases of the OW length from 400 to 800ms, Precision increased by 5.5%, Recall increased by 5.7%, and F1 score increased by 5.9%.

Handover performance:

Different options of HO procedure

Observation 4: For indirect measurement event prediction for temporal domain case A, compared with Real handover procedure:

- The handover number of Option2 and Option3 is comparable to the Real handover procedure, which around 0.2 per UE per second with UE Speed = 90Km/h;

- The handover failure number are reduced by 0.73% for Option 2 and 22.1% for option 3 respectively;

- The handover failure rate is reduced by 1.46% for Option 2 and 25.8% for Option 3 respectively.

Observation 5: HoF rate and HOF number reduction for the HO_CMD failure case contributes most of the gain for HoF rate and HOF number reduction with HO model option 3 and option2 for FR2 to FR2 intra-frequency temporal domain case A.

Observation 6: Option3 is superior to Option2 both in terms of handover failure rate and handover failure number.

Speed

Observation 7: For indirect measurement event prediction for temporal domain case A with different speeds, the AI gain of handover failure rate and handover failure number increase with the increase of UE speed, e.g., from 18.29% to 25.86% and then to 31.13%, and from 14.45% to 22.17% and then to 27.8% when UE speed increases from 60 to 90 and to 120 km/h.

TTT

Observation 8: When TTT increases from 160 to 320 ms, the AI gain for handover failure rate and handover failure number increases, e.g., from 14.17% to 25.86%, and from 11.08% to 22.17%.

OW

Observation 9: The HoF rate and HOF number decreases with the increase of the OW length from 400 to 800ms, e.g., from 12.74% to 11.29%, and from 0.2504 to 0.2389 per UE per second.

Reference

- R2-2409651, Simulation results of RRM Measurement Prediction, CATT, Turkcell;

- R2-2500245, Simulation results of AI based Handover performance;