3GPP TSG RAN WG2 Meeting #129bis                                          R2-2501943

Wuhan, China, Apr 7th – 11th, 2025

Agenda Item:	8.12.2

Source:	Xiaomi

Title:	Discussion on asymmetric DL sTRP and UL mTRP

Document for:	Discussion and Decision

# Introduction

During RAN2 #128 meeting, following agreements were made for PL offset update:

Working assumption: - UE applies the latest PL offset value received in RRC or MAC CE. Can revisit if new issue is found. Agreement - RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset. FFS whether/how to capture this.

In this contribution, we further discuss the RAN2 impact of how to incorporate PL offset in PHR and RACH procedure.

# Discussion

## PHR trigger

During RAN2 #129 meeting, RAN2 agreed with following pathloss offset impact to PHR trigger:

RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset. FFS whether/how to capture this.

It is observed that, during RAN1 #120 meeting discussion, PL offset is not agreed to be considered during Type 3 PHR calculation, considering Type 3 PHR calculation is not a UL-only TRP scenario:

Conclusion There is no consensus to include PL offset in Type 3 PHR calculation.

In our understanding, if PL offset is not considered during Type 3 PHR calculation, there’s no change of Type 3 power headroom value after receiving the configured PL offset. In this scenario, UE does not need to send the unchanged PHR value to network again. Hence, PHR trigger condition for Type 3 PHR should not take PL offset into account.

On the other hand, RAN1 also agreed the PL offset is used in UL power control in a way that UE reuses the legacy uplink power control formulation by replacing legacy PL with a PL which is derived from the DL PL RS and the PL offset.

Agreement For the asymmetric DL sTRP/UL mTRP scenarios, support to include PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH

Therefore, RAN2 should further clarify that PHR trigger condition considering PL offset is only applicable to Type 1 PHR trigger.

Proposal 1: RAN2 add a NOTE in TS 38.321: ‘If a joint/UL TCI state is configured with a PL offset, PHR trigger for Type 1 PHR is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset’.

## RACH

PDCCH-order RACH procedure

According to the RAN1 agreements, the pathloss offset is indicated via the PDCCH-order RACH procedure.

Agreement Support applying PL offset on PDCCH-order PRACH towards a UL TRP in FR1. - Note: The DL reference timing determination for PDCCH-order PRACH transmission to an UL TRP is still based on the DL RS defined in current RAN4 specification - Above is subject to a separate UE capability signaling Agreement For indicating PL offset for PDCCH-order PRACH, introduce a new 1-bit DCI field in DCI format 1_0: - This DCI field exists when the corresponding RRC parameter (which is a new RRC used to configure the presence of this 1-bit DCI field) is enabled and at least one TCI state is configured with PL offset. - When one joint/UL TCI state is indicated in Rel-17 unified TCI, - the bit field index 0 of this field indicates that PL offset is not included in the PRACH transmission power calculation - the bit field index 1 of this field indicates that the PL offset associated with the indicated TCI state is included in the PRACH transmission power. - FFS: Whether the bit field can be used to indicate other information - FFS: When two joint/UL TCI states are indicated in Rel-18 unified TCI Agreement For indicating PL offset for PDCCH-order PRACH, when two joint/UL TCI states are indicated in Rel-18 unified TCI, down-select one from the following Alts for the 1-bit DCI field in DCI 1_0 which indicates the application of PL offset on PRACH transmission: - Alt2: - the bit field index 0 of this field indicates that the PL offset associated in the first indicated joint/UL TCI state is included in the PRACH transmission power calculation - the bit field index 1 of this field indicates that the PL offset associated in the second indicated joint/UL TCI state is included in the PRACH transmission power calculation. - FFS: Whether a restriction that only one of the indicated joint/UL TCI states can be configured with PL offset is needed. FFS: If other information can be indicated by this same 1-bit DCI field for the PDCCH-order PRACH transmission

According to the RACH procedure as quoted below, RSRP of downlink pathloss reference is used for the selection of different RACH procedures, NUL and SUL, and different PRACH resources.

TS 38.321 - rsrp-ThresholdSSB: an RSRP threshold for the selection of the SSB for 4-step RA type. If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdSSB used for the selection of the SSB within candidateBeamRSList refers to rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE; - rsrp-ThresholdCSI-RS: an RSRP threshold for the selection of CSI-RS for 4-step RA type. If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdCSI-RS is equal to rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE; - msgA-RSRP-ThresholdSSB: an RSRP threshold for the selection of the SSB for 2-step RA type; - rsrp-ThresholdSSB-SUL: an RSRP threshold for the selection between the NUL carrier and the SUL carrier; - msgA-RSRP-Threshold: an RSRP threshold for selection between 2-step RA type and 4-step RA type when both 2-step and 4-step RA type Random Access Resources are configured in the UL BWP; - rsrp-ThresholdMsg1-RepetitionNum2: an RSRP threshold for Msg1 repetition with repetition number 2 (see clause 5.1.1b); - rsrp-ThresholdMsg1-RepetitionNum4: an RSRP threshold for Msg1 repetition with repetition number 4 (see clause 5.1.1b); - rsrp-ThresholdMsg1-RepetitionNum8: an RSRP threshold for Msg1 repetition with repetition number 8 (see clause 5.1.1b); - rsrp-ThresholdMsg3: an RSRP threshold for Msg3 repetition (see clause 5.1.1b);

Calculation of pathloss for two TRPs of asymmetric DL sTRP/UL mTRP considering pathloss offset can be updated as below:

- PathlossUL_DL_TRP = PowerTx_UL_DL_TRP - RSRPPathloss_Reference

- PathlossUL _TRP

  - = PathlossUL_DL_TRP + Pathloss_Offset

  - = PowerTx_UL_DL_TRP – (RSRPPathloss_Reference - Pathloss_Offset)

  - = PowerTx_UL_DL_TRP – RSRPUL_TRP

Therefore, assuming that the UL_TRP is changed to UL_DL_TRP with the TRP transmission power of PowerTx_UL_DL_TRP, the virtual RSRP (i.e. RSRPUL_TRP reflecting the uplink coverage of the UL-only TRP) of the UL_TRP would be as follows:

- RSRPUL_TRP = RSRPPathloss_Reference - Pathloss_Offset

As PDCCH-order indicating pathloss offset is for the PRACH transmission to the UL-only TRP, in order to ensure the uplink coverage for the PRACH transmission to the UL-only TRP, the RSRP for the selection of the different RACH procedures, NUL and SUL, and different PRACH resources towards the UL-only TRP should combine the RSRP measured for the pathloss reference and the pathloss offset.

Proposal 2: When PDCCH-order indicates the pathloss offset, RSRP used for the selection of RACH procedure combines both RSRP measured for pathloss reference and pathloss offset.

# Conclusion

According to the above analysis, we have the following proposals:

Proposal 1: RAN2 add a NOTE in TS 38.321: ‘If a joint/UL TCI state is configured with a PL offset, PHR trigger for Type 1 PHR is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset’.

Proposal 2: When PDCCH-order indicates the pathloss offset, RSRP used for the selection of RACH procedure combines both RSRP measured for pathloss reference and pathloss offset.