3GPP TSG-RAN WG2 Meeting #129bis									R2-2501960

Wuhan, China, April. 7th – 11th, 2025                                 		Revision of R2-2501089

Agenda item:		8.4.2

Title: 	Procedure and configuration of LP-WUS for IDLE and INACTIVE mode

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In this contribution, we will further discuss the RAN2 related issues on procedure and configuration of LP-WUS for IDLE/INACTIVE modes and give our proposals.

- Discussion

2.1 Provision of LP-WUS related configuration

Based on the RAN1 agreement, the LP-SS design and LP-WUS design have been discussed, but has not concluded yet. Considering that LP-SS configuration and LP-WUS configuration highly depend on RAN1 agreement for LP-SS and LP-WUS design, RAN2 can postpone the discussion for LP-WUS and LP-SS related configuration until there is agreement for the LP-SS and LP-WUS design in RAN1.

Proposal 1: RAN2 can postpone the discussion on LP-WUS and LP-SS related configuration until there is agreement for the LP-SS and LP-WUS design in RAN1.

2.2 Activation/deactivation of LP-WUS

Since it has been agreed in RAN2#125bis that LP-WUS related configuration for IDLE/INACTIVE state is provided via system information, the provision of LP-WUS configuration via SIB can be seen as an implicit activation to apply LP-WUS configuration for all the UEs supporting this function. Meanwhile, if the LP-WUS configuration is no longer broadcast via SIB, it can be assumed that application of LP-WUS configuration (and also the LP-WUS monitoring) is deactivated.

However, it’s also possible that the UE would not apply LP-WUS configuration immediately on reception of LP-WUS configuration and the application of LP-WUS configuration will be activated later, e.g., per-UE via a dedicated signaling, e.g., RRC connection release message. During SI stage, such process has been discussed but no decision has been made.

The main motivation to activate/deactivate LP-WUS for a specific UE or part of UEs via dedicated signaling is that it may be not suitable to always activate the LP-WUS monitoring for all the UEs as soon as the LP-WUS configuration is broadcast. For example, LP-WUS can be applied to diverse UE types, e.g., IIoT UE, (e) Redcap UE, NR UE with XR traffic and so on, the device type or traffic type of UE may be another consideration for activating or deactivating LP-WUS for a UE. For example, the UEs with delay-sensitive services may not be suitable to monitor LP-WUS considering the possible large wake up latency caused by LP-WUS, which may have unexpected impacts on the delay-sensitive service. The exact latency that may be increased by LP-WUS is still under evaluation in RAN1, e.g., to take into account the LP-WUS processing time, MR transition time for ramp up, Time/frequency synchronization of MR etc. Per our understanding, if the additional latency caused by LP-WUS is far larger than the duration that random access procedure may use, e.g. if the increased latency is several seconds or more, the LP-WUS in IDLE/INACTIVE modes should be activated/deactivated by UE dedicated signaling, e.g., to activate LP-WUS only for the UE without delay-sensitive services or to deactivate LP-WUS when the UE starts to perform delay-sensitive services.

In a summary, from RAN2 perspective, we can see the need to activate/deactivate the application of LP-WUS configuration via dedicated signaling. But this can be later decided after the RAN1 has clear conclusion on the potential latency caused by LP-WUS monitoring.

Observation 1: LP-WUS monitoring may result in additional latency compared to legacy paging procedure and hence, it is useful to support per UE activation of LP-WUS monitoring in addition to the per cell configuration.

Proposal 2: In addition to the SIB signaling for LP-WUS configuration, dedicated activation/deactivation of LP-WUS per UE is supported.

If activation/deactivation of LP-WUS in IDLE/INACTIVE modes by dedicated signaling is supported, the gNB firstly needs to be aware of the UE capability of supporting LP-WUS in IDLE/INACTIVE modes and then can be able to activate/deactivate the LP-WUS for the UE by dedicated signaling.

Meanwhile, for the paging gNB, to only know the UE capability would be not sufficient as UE may not have been activated the use of LP-WUS, it also needs to know whether the LP-WUS is activated for this UE. Therefore, whether the LP-WUS monitoring has been activated for this UE (which can only be performed on UE supporting LP-WUS) needs to be transmitted to the paging gNB by Paging or RAN Paging.

Proposal 2a: If activation/deactivation of LP-WUS in IDLE/INACTIVE modes by UE dedicated signaling is supported, send LS to RAN3 to provide LP-WUS activation/deactivation indication to the paging gNB, e.g., via NGAP Signaling, NGAP PAGING and RAN PAGING message.

2.3 Time offset between LP-WUS and PO

As agreed in RAN1, UE reports one wake-up delay (e.g. one of candidate values) via UE capability reporting, and wake-up delay is minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. If UE reports, it can implicitly indicate that UE supports LP-WUS in RRC_IDLE/RRC_INACTIVE state. From the network perspective, as agree in RAN1, time offset value(s) between a LO and a reference PO/PF is/are configured. In order to support the different values for wake up delay, network can configure different time offset values, and a time offset value is larger than or equal to a reported wake-up delay + PEI offset if PEI is configured. However, the different time offset values may cause the multiple LP-WUS occasions for same PO scattered over the time domain, too many LP-WUS occasions per PO is not convenient for LP-WUS resource scheduling, network can configure some time offsets that can be used. In which case, UE should determine the time offset based on its reported wake-up delay from the time offsets configured by network, e.g. the UE selects the minimum time offset configured by network that is larger than or equal to its reported wake-up delay. If there is no time offset configured by network that is larger than or equal to its reported wake-up delay, the UE does not monitor LP-WUS.

Proposal 3a: If UE report its wake-up delay, it can implicitly indicate that UE supports LP-WUS in RRC_IDLE/RRC_INACTIVE state.

Proposal 3b: Network configures time offset(s) that can be used between LO and MR PDCCH monitoring.

Proposal 3c: UE determine the time offset to be used based on its reported wake-up delay and the time offset(s) configured by network, e.g. select the minimal time offset configured by network that is larger than or equal to its reported wake-up delay. If there is no time offset configured by network that is larger than or equal to its reported wake-up delay, the UE does not monitor LP-WUS.

Considering that the LP-WUS time offset capability will be used for paging gNB to determine the LP-WUS occasion, the LP-WUS wake-up delay capability should be included in ue-RadioPagingInfo-r17 IE of UECapabilityInformation message, since the ue-RadioPagingInfo-r17 IE is already included in UERadioPagingInformation container(e.g. which is included in RAN3 IE of UE Radio Capability for Paging IE) and is transparently passed from gNB to AMF, from AMF to gNB and from one gNB to another gNB, the LP-WUS capability will not be lost if the gNB is rel-17 or later.

Proposal 4: The UE reported wake-up delay is included in ue-RadioPagingInfo-r17 IE of UECapabilityInformation message so that it can be delivered to paging gNB transparently.

2.4 About whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session

Whether LP_WUS can be used when the UE has an emergency PDU session depends on the increased delay caused by LP-WUS, which should be evaluated in RAN1 first.

Observation 2: Whether LP_WUS can be used when the UE has an emergency PDU session depends on the increased delay caused by LP-WUS.

Proposal 5a: Send LS to RAN1 to evaluate the increased delay caused by LP-WUS.

Proposal 5b: RAN2 waits for RAN1’s conclusion on the increased delay caused by LP-WUS to decide whether RAN2 need to specify that UE having an emergency PDU session will not monitor LP-WUS.

2.5 About UE-ID in UE-ID based LP-WUS subgrouping determination

In the <38.304 Running CR for LP-WUS-draft>, there is an FFS for UE-ID in UE-ID based LP-WUS subgrouping as follows:

UE_ID: 5G-S-TMSI mod X, X is FFS

The UE-ID value range for UE-ID based LP-WUS subgrouping depends on the maximal number of POs, the maximal number of UE-ID based PEI sub-groups and the maximal number of UE-ID based LP-WUS subgroups.

In the current specification, the UE-ID for POs is defined as follows:

UE_ID: If the UE operates in eDRX as specified in clause 7.4: - 5G-S-TMSI mod 4096 else: - 5G-S-TMSI mod 1024

In the current specification, the UE-ID for UE-ID based PEI subgrouping is defined as follows:

UE_ID: 5G-S-TMSI mod X, where X is 32768, if eDRX is applied; otherwise, X is 8192

Which means that at the top of UE-ID for PO, the maximal number of POs in one DRX cycle * maximal number of PEI sub groups(e.g. 8) per PO are used for UE-ID to support UE-ID based PEI subgrouping.

In RAN1#120 meeting, the following agreement has been made [13]:

Agreement Confirm the following working assumption with the modification: Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 31

Which means that at the top of UE-ID for PO and UE-ID for PEI, the maximal number of LP_WUS sub groups(e.g. 31) per PO should be considered for UE-ID to support UE-ID based LP_WUS subgrouping.

Considering that 5G-S-TMSI is used for UE ID in NR, there is no security issue with more bits for UE-ID.

Observation 3: There is no security issue with more bits for UE-ID.

Since UE may monitor PO directly when detecting LP-WUS, or monitor PEI when detecting LP-WUS, but gNB cannot know whether UE will monitor UE-ID based PEI subgrouping, gNB can only know whether the monitoring condition of UE_ID based PEI subgrouping is met. Thus, the minimal UE-ID range in UE-ID based LP-WUS subgrouping is as follows:

UE_ID:

If the UE operates in eDRX as specified in clause 7.4, and the monitoring condition of UE_ID based PEI subgrouping is met as specified in clause 7.3.2:

-	5G-S-TMSI mod 1015808;  //at most 4096 POs per DRX cycle, 8 PEI subgroups per PO and 31 LP-WUS subgroups per PO

else if the UE operates in eDRX as specified in clause 7.4, and the monitoring condition of UE_ID based PEI subgrouping is not met as specified in clause 7.3.2:

-	5G-S-TMSI mod 126976; //at most 4096 POs per DRX cycle, and 31 LP-WUS subgroups per PO

else if the UE does not operate in eDRX as specified in clause 7.4, and the monitoring condition of UE_ID based PEI subgrouping is met as specified in clause 7.3.2:

-	5G-S-TMSI mod 253952;  //at most 1024 POs per DRX cycle, 8 PEI subgroups per PO and 31 LP-WUS subgroups per PO

else

-	5G-S-TMSI mod 32744;   //at most 1024 POs per DRX cycle, and 31 LP-WUS subgroups per PO

Considering that there is no security issue with more bits for UE-ID, for simplicity, the other option is to define a unified UE-ID for LP-WUS, e.g. the largest UE ID range (5G-S-TMSI mod 1015808) in all LP_WUS cases can be used for all LP-WUS monitoring cases.

Proposal 6: The following options can be considered for UE-ID derivation in UE-ID based LP-WUS subgrouping:

--Option 1:

UE_ID:

If the UE operates in eDRX as specified in clause 7.4, and the monitoring condition of UE_ID based PEI subgrouping is met as specified in clause 7.3.2:

-	5G-S-TMSI mod 1015808;

else if the UE operates in eDRX as specified in clause 7.4, and the monitoring condition of UE_ID based PEI subgrouping is not met as specified in clause 7.3.2:

-	5G-S-TMSI mod 126976;

else if the UE does not operate in eDRX as specified in clause 7.4, and the monitoring condition of UE_ID based PEI subgrouping is met as specified in clause 7.3.2:

-	5G-S-TMSI mod 253952;

else

-	5G-S-TMSI mod 32744;

--Option 2: UE_ID: 5G-S-TMSI mod 1015808.

- Conclusion

The following proposals are given based on the above analysis:

Provision of LP-WUS related configuration

Proposal 1: RAN2 can postpone the discussion on LP-WUS and LP-SS related configuration until there is agreement for the LP-SS and LP-WUS design in RAN1.

Activation/deactivation of LP-WUS

Observation 1: LP-WUS monitoring may result in additional latency compared to legacy paging procedure and hence, it is useful to support per UE activation of LP-WUS monitoring in addition to the per cell configuration.

Proposal 2: In addition to the SIB signaling for LP-WUS configuration, dedicated activation/deactivation of LP-WUS per UE is supported.

Proposal 2a: If activation/deactivation of LP-WUS in IDLE/INACTIVE modes by UE dedicated signaling is supported, send LS to RAN3 to provide LP-WUS activation/deactivation indication to the paging gNB, e.g., via NGAP Signaling, NGAP PAGING and RAN PAGING message.

Time offset between LP-WUS and PO

Proposal 3a: If UE report its wake-up delay, it can implicitly indicate that UE supports LP-WUS in RRC_IDLE/RRC_INACTIVE state.

Proposal 3b: Network configures time offset(s) that can be used between LO and MR PDCCH monitoring.

Proposal 3c: UE determine the time offset to be used based on its reported wake-up delay and the time offset(s) configured by network, e.g. select the minimal time offset configured by network that is larger than or equal to its reported wake-up delay. If there is no time offset configured by network that is larger than or equal to its reported wake-up delay, the UE does not monitor LP-WUS.

Proposal 4: The UE reported wake-up delay is included in ue-RadioPagingInfo-r17 IE of UECapabilityInformation message， so that it can be delivered to paging gNB transparently.

UE-ID based subgrouping handling when the UE has an emergency PDU session

Observation 2: Whether LP_WUS can be used when the UE has an emergency PDU session depends on the increased delay caused by LP-WUS.

Proposal 5a: Send LS to RAN1 to evaluate the increased delay caused by LP-WUS.

Proposal 5b: RAN2 waits for RAN1’s conclusion on the increased delay caused by LP-WUS to decide whether LP_WUS can be used when the UE has an emergency PDU session.

UE-ID derivation in UE-ID based LP-WUS subgrouping determination

Observation 3: There is no security issue with more bits for UE-ID.

Proposal 6: The following options can be considered for UE-ID derivation in UE-ID based LP-WUS subgrouping:

--Option 1:

UE_ID:

If the UE operates in eDRX as specified in clause 7.4, and the monitoring condition of UE_ID based PEI subgrouping is met as specified in clause 7.3.2:

-	5G-S-TMSI mod 1015808;

else if the UE operates in eDRX as specified in clause 7.4, and the monitoring condition of UE_ID based PEI subgrouping is not met as specified in clause 7.3.2:

-	5G-S-TMSI mod 126976;

else if the UE does not operate in eDRX as specified in clause 7.4, and the monitoring condition of UE_ID based PEI subgrouping is met as specified in clause 7.3.2:

-	5G-S-TMSI mod 253952;

else

-	5G-S-TMSI mod 32744;

--Option 2: UE_ID: 5G-S-TMSI mod 1015808.

- References

- RP-241824 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO, RAN#105, March, 2024

- Draft_R2_125bis_Meeting_Report_v1, RAN2#125bis, April, 2024

- Draft_R2_126_Meeting_Report_v1, RAN2#126, May, 2024

- Draft_Minutes_report_RAN1#116_v020-202402, RAN1#116, February, 2024

- Draft_Minutes_report_RAN1#116b_v030, RAN1#116bis, April, 2024

- Draft_Minutes_report_RAN1#117_v010, RAN1#117, February, 2024

- Draft_RAN2_127_Meeting_Report_v1

- Draft_Minutes_report_RAN1#118_v010, RAN1#118, August, 2024

- Draft_RAN2_127b_Meeting_Report_v1

- Draft_RAN2_128_Meeting_Report_v1

- R4-2416861 WF on RRM requirements for NR_LPWUS, RAN4 112bis, vivo

- Draft_RAN2_129_Meeting_Report_v2

- Draft_Minutes_report_RAN1#120_v030