3GPP TSG-RAN WG2 Meeting #129-bis					  R2-2501975

Wuhan, China, April 7 – April 11, 2025

Agenda item:	6.1.3.3

Source:			MediaTek Inc.

Title:	On emergency services and eDRX

Document for:	Discussion and Decision

- Background

As per the Reply LS on emergency call back and paging [1], SA2 analyzed and provided the feedbacks:

SA2 analysed the identified issue above and would like to provide the following feedback: - Existing specifications have specified that each PLMN/operator reserves a specific ARP value for emergency PDU session and provide to RAN node. See related text in TS 23.501 clause 5.16.4.3, 5.16.4.7, 5.16.4.9 and TS 23.502 clause 6.1.3.10. Therefore the RAN node can be aware of an emergency PDU session based on the reserved ARP value when the PDU session context exist in the RAN node. - In addition there is inactivity monitoring for the presence of user plane resources for the emergency PDU session in SMF as defined in TS 23.502 clause 4.4.2.2 and TS 23.503 clause 6.1.3.10. Based on the above, some companies in SA2 believe that the RAN node is capable through configuration to not release the emergency PDU session user plane resources based on existing implementation, and to not release the UE to RRC_INACTIVE state/RRC_IDLE state when the UE has an emergency PDU session. Therefore the issue highlighted in RAN2 LS may not exist. However there is no consensus in SA2 whether the UP of emergency PDU session can be deactivated, or whether to introduce new signalling to NG-RAN. SA2 would like to ask RAN2 to provide feedback.

In this contribution, we discuss whether RAN2 specs change is needed, and what feedback we can possibly provide.

- Discussion

  - What SA2 said

They indicated two things:

- There was already a standard interface along with the required information for the RAN node to be aware of an emergency PDU session exists. No consensus to introduce a new signaling to serve the same purpose.

- The issue highlighted in the RAN2 LS [2] may not exist. No consensus on whether the UP of emergency PDU session can be deactivated.

- An ongoing emergency service can be made known to the RAN node by network implementation.

- RAN2 to agree the RAN node can be aware of an ongoing emergency PDU session by network implementation.

For the above bullet 2, we understand that it might depend on the different network implementations, the UP of emergency PDU session could be deactivated so that there might be a chance to see the issue highlighted in the RAN2 LS [2] happen. However, there is no consensus whether the aforesaid scenario is valid so that no standard solution is considered in SA2.

- There was no SA2 consensus if anything to be done by CN and by RAN at all.

  - Recheck the RAN2 issue

To progress, we recheck the RAN2 issues and the questions outstanding as follows:

- Would the issue highlighted in the RAN2 LS [2] happen (i.e., caused by RAN) even if it is invalid in terms of SA2 viewpoint?

- Whether the current specification is sufficient to meet the service performance of an emergency call in the issue scenario under consideration. In other words, would eDRX be undesirably used during an ongoing emergency service?

- As mentioned above, is the RAN-configured eDRX case (in RRC_INACTIVE) not covered?

What SA2 didn’t explicitly state but it is worthwhile to know that, as per current CT (NAS) specifications, the UE can enter CM-IDLE mode and keep the emergency PDU session. From NAS (UE) viewpoint the reason can be e.g., Radio Link Failure, or RRC connection release.

For RLF, there are the RRC reestablishment procedures defined in RAN, and the NAS signalling connection recovery procedures defined in CN, we expect that the UE will go back to CM-CONNECTED state soon in most cases. If unfortunately, the abnormal condition persists, we assume the emergency PDU session will be released eventually (due to the guard timers in NAS).

For the case where RRC connection is released, we think it could be a valid case because SA2 cannot control if any data will flow through that emergency PDU session, and there was DataInactivityTimer guarding the inactivity situation to save radio resources in RAN. In addition to the aforesaid UE actions, it is also possible to see RAN releases RRC connection for the same purpose.

- There is no exception for the emergency service in the inactivity handling of RAN so that the issue highlighted is not precluded.

We still hear some interests to make the UE with emergency PDU session to be kept in RRC_CONNECTED (i.e., the issue highlighted in the RAN2 LS [2] will not happen). If this kind of high-level principle is agreed, without any top-down manner to control the UE/NW behavior in service or PDU session level, we think it will further introduce more RAN requirements. Take the inactivity handling as an example, a network-based solution is needed.

- To support keeping the UE with emergency PDU session to be in RRC_CONNECTED state, RAN2 to agree the network-based solution will be considered (No specs change is needed).

As RAN2 companies had discussed in the past meetings, we think the UE behavior on when not to use eDRX is clear according to the Stage 2 and Stage 3 specifications:

TS 23.501, the Stage 2 spec:

5.31.7.2 Extended Discontinuous Reception (DRX) for CM-IDLE and CM-CONNECTED with RRC-INACTIVE …… When the UE has PDU Session associated with emergency services, the UE and AMF follow regular discontinuous reception as defined in clause 5.4.5 and shall not use the extended idle mode DRX. Extended idle mode DRX parameters may be negotiated while the UE has PDU Session associated with emergency services. When the PDU Session associated with emergency services is released, the UE and AMF shall reuse the negotiated extended idle mode DRX parameters in the last Registration Update procedure. ……

TS 24.501, the Stage 3 spec:

5.3.16 Extended DRX cycle for UEs in 5GMM-IDLE and 5GMM-CONNECTED mode with RRC inactive indication …… The network accepts the request to use the eDRX by providing the Negotiated extended DRX parameters IE when accepting the registration procedure. The UE shall use eDRX only if it received the Negotiated extended DRX parameters IE during the last registration procedure and the UE does not have an emergency PDU session. NOTE 1: If the UE wants to keep using eDRX, the UE includes the Extended DRX parameters IE in each registration procedure. If the UE received the Negotiated extended DRX parameters IE during the last registration procedure, upon successful completion of the PDU session release procedure of the emergency PDU session, the UE shall resume eDRX. If the network has provided the Negotiated extended DRX parameters IE during the last registration procedure, upon successful completion of the PDU session release procedure of the emergency PDU session, the network shall resume eDRX. If the UE or the network locally releases an emergency PDU session, the UE or the network shall not use eDRX until the UE receives eDRX parameters during a registration procedure with PDU session context synchronization or upon successful completion of a service request procedure with PDU session context synchronization. ……

The above umbrella specifications basically covered the principles of using eDRX for the UE and the network in RRC_IDLE and RRC_INACTIVE of the possible scenarios including the involved CN, RAN nodes.

- The SA (stage 2) and CT (stage 3) specifications had covered the principles of not using eDRX either in RRC_IDLE or in RRC_INACTIVE if there is an emergency PDU session.

RAN2 also indicated that a RAN eDRX cycle can be configured by a RAN node when releasing the UE to RRC_INACTIVE state ONLY IF the UE is configured with an Idle eDRX cycle by CN, based on current specification [2]. Given that there’re also clear texts (NOTE) to indicate how the RRC and MAC layer to be aware of an ongoing emergency service is up to UE implementation in other Stage 3 specs i.e., TS 38.331 and TS 38.321. In light of the Proposal 1, if agreed by RAN2, we are ready to capture the corresponding requirement for the UE and the network.

- It’s been clear that, by implementation, the RAN node and the UE will know if there is an emergency PDU session and if it is the case, they both will not use the configured RAN eDRX cycle.

- RAN2 to agree on capturing that the UE shall not use eDRX if the UE has an emergency PDU session in RRC_INACTIVE state in TS 38.304.

- Conclusion

In previous sections we made the following observations and proposals:

Observation 1	An ongoing emergency service can be made known to the RAN node by network implementation.

Observation 2	There was no SA2 consensus if anything to be done by CN and by RAN at all.

Observation 3	There is no exception for the emergency service in the inactivity handling of RAN so that the issue highlighted is not precluded.

Observation 4	The SA (stage 2) and CT (stage 3) specifications had covered the principles of not using eDRX either in RRC_IDLE or in RRC_INACTIVE if there is an emergency PDU session.

Observation 5	It’s been clear that, by implementation, the RAN node and the UE will know if there is an emergency PDU session and if it is the case, they both will not use the configured RAN eDRX cycle.

Proposal 1	RAN2 to agree the RAN node can be aware of an ongoing emergency PDU session by network implementation.

Proposal 2	To support keeping the UE with emergency PDU session to be in RRC_CONNECTED state, RAN2 to agree the network-based solution will be considered (No specs change is needed).

Proposal 3	RAN2 to agree on capturing that the UE shall not use eDRX if the UE has an emergency PDU session in RRC_INACTIVE state in TS 38.304.

- References

- R2-2501752	Reply LS on emergency call back and paging

- R2-2411041	LS on emergency services and eDRX

- Appendix

  - Landscape

Figure 1 is shown for illustration purposes only and may not be an exact representation of the network nodes and CP/UP separation. It shows the differences between the UE and the network when they manage to align on whether eDRX should be used during an ongoing emergency service.

Table 1 summarizes the current landscape, but it does not contain exhaustive features and all combined scenarios. The key observation is that RAN doesn’t really have an unified principle for the possible clash between the power-saving and life-saving in RAN. So, each combined scenario is discussed on a case-by-case basis.

Yellow highlighted parts are the key discussion points in this paper and they’re RAN-centric.

|  | UE |  | RAN |  | CN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | NAS | emergency PDU session |  |  | NAS |  |
|  | ↑ (up to UE implementation) ↓ |  |  | (up to NW implementation) |  |  |
|  | RRC |  | RRC |  |  |  |

Figure 1: The difference on how the ongoing emergency service is made known to RAN protocol.

|  |  | NR eDRX |  | NR ePowSav |  |
| --- | --- | --- | --- | --- | --- |
|  |  | eDRX value configured by CN (for CN paging) | eDRX value configured by RAN (Up to the UE’s accepted idle mode value; for RAN paging) | PEI with CN controlled subgrouping | PEI with UE ID based subgrouping |
| Emergency PDU session exists | RRC_CONNECTED | N/A |  | N/A |  |
|  | RRC_IDLE | Not allowed | N/A | Not allowed | Allowed with latency concern |
|  | RRC_INACTIVE | Not allowed | Not allowed (by UE and network implementation) | Not allowed | Allowed with latency concern |
| ETWS, CMAS, PWS | Applicable RRC state(s) | Timeliness of delivery requirement may not be met (e.g., when the eDRX cycle length is longer than 4s, or longer than the SI modification period). |  |  |  |

Table 1: 3GPP requirements of power saving features and life-saving services in different RRC states