3GPP TSG-RAN2#129bis 	R2-250xxxx

Wuhan, China, 7th Apr – 11th Apr, 2025

Agenda item:		8.12.3 (NR_MIMO_Ph5-Core)

Source:	LG Electronics Inc.

Title: 	Discussion on UEI beam reporting impact

Document for:	Discussion and Decision

1.	Introduction

In the last meeting, RAN2 made a working assumption for UEI beam reporting as follows.

Working assumption: RAN2 assume the event evaluation for UE-initiated beam reporting is captured in 38.321, where the evaluation is based on indications from measurements described in a RAN1 specification. The final decision is up to RAN1, we can revisit this if R1 has a different decision. No need to send a LS to R1 on this.

In this contribution, we present our view on the working assumption and discuss further discussion points.

2.	Discussion

In Rel-19 MIMO, RAN1 agreed to introduce event evaluation procedure for UEI beam reporting but did not determined whether to capture the event triggering procedure in RAN1 or RAN2 specification.

[RAN1#117] Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.

Due to this situation, RAN2 thought there may be not enough time to implement MAC specification if RAN1 decides to capture the event evaluation procedure in RAN2 specification, and RAN2 made WA that the event evaluation procedure for UEI beam reporting is captured in MAC specification in the last meeting.

However, after the last RAN1 meeting, RAN1 had post e-mail discussion [1] and discussed to implement draft CR of 38.214 [2] for UEI beam reporting, and the draft CR was endorsed.

In [2], for each events, the behaviour related to UEI beam reporting is clearly specified based on the counter and the time window as follows.

… if [during the eventDetectionTimeWindowLength-r19, if provided], the L1-RSRP value determined for each transmission occasion of at least one same reference signal of the reference signals configured by the newBeamResourceSet-r19 (in a same CC of CSI-ReportConfig, or provided by carrier in the CSI-ReportConfig) is at least once, or eventInstanceCount-r19 times, if provided, an eventThreshold-r19 greater than the L1-RSRP of - the reference signal in the indicated TCI state [in a same CC of CSI-ReportConfig, or the CC indicated [by a RRC parameter]] in the CSI-ReportConfig, if newBeamResourceSet-r19 is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state [in a same CC of CSI-ReportConfig, or provided [by a RRC parameter]], if newBeamResourceSet-r19 is a CSI-SSB-ResourceSet, the UE transmits PUCCH format 0 or format 1 in the PUCCH resource configured by firstPUCCHResourceConfig-UEIBR-r19 in the CSI-ReportConfig, and … For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event7’, for periodic reference signals [with the same periodicity], if [during the eventDetectionTimeWindowLength-r19, if provided], the L1-RSRP value determined for each occurrence of any of the reference signals configured by the newBeamResourceSet-r19 (in a same CC of CSI-ReportConfig, or provided by carrier in the CSI-ReportConfig) is at least eventInstanceCount-r19 times an eventThreshold-r19 greater than the L1-RSRP of - the reference signal with the valueOfQ-r19 highest L1-RSRP out of the activated TCI state reference signals in a same CC of CSI-ReportConfig, or provided by a RRC parameter in the CSI-ReportConfig, if the NZP-CSI-RS-ResourceSet is configured with repetition set to 'on', else - the SSBSS/PBCH block which is QCLed with the measured reference signal in the indicated TCI state, the UE transmits PUCCH format 0 or format 1 in the PUCCH resource configured by firstPUCCHResourceConfig-UEIBR-r19 in the CSI-ReportConfig, and … For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event1’, for periodic reference signals [with the same periodicity], if during the eventDetectionTimeWindowLength-r19, the L1-RSRP value determined for each occurrence of - the reference signal indicated by the DCI field tci-PresentInDCI if the NZP-CSI-RS-ResourceSet is configured with repetition set to 'on', else - the SSB which is QCLed with the measured reference signal, is at least eventInstanceCount-r19 times less than eventThreshold-r19, the UE transmits PUCCH format 0 or format 1 in the PUCCH resource configured by firstPUCCHResourceConfig-UEIBR-r19 in the CSI-ReportConfig, and …

Therefore, we think RAN2 does not need to make a redundant effort to capture the event evaluation procedure in MAC specification.

Proposal 1. RAN2 does not discuss to capture the event evaluation procedure in MAC specification.

Meanwhile, for the RRC configuration for UEI beam reporting, RAN1 made RRC parameter list [3] based on RAN1 discussion.

In RAN2 post e-mail discussion [4], RAN2 implemented ASN.1 based on [3], but some parameters are still FFS or not implemented.

Therefore, regarding UEI beam reporting, we think that it is better to focus on implementing ASN.1 and to wait RAN1 progress for some parameters of FFS.

Proposal 2. RAN2 focuses on the implementation of ASN.1 for RRC specification and waits RAN1 progress for some parameters of FFS.

3.	Conclusion

In this document, we analysis RAN2 impacts of UEI beam reporting, and made observations and proposals as follows.

Proposal 1. RAN2 does not discuss to capture the event evaluation procedure in MAC specification.

Proposal 2. RAN2 focuses on the implementation of ASN.1 for RRC specification and waits RAN1 progress for some parameters of FFS.

4.	Reference

- Post-120-Rel19-38.214-NR_MIMO_Ph5-UEIBM

- R1-2501669	Introduction of UE-initiated/event-driven beam management

- R1-2501645	Consolidated Rel-19 higher layers parameters list Post RAN1#120

- [Post129][207][MIMO_Ph5] Running CR for TS 38.331 (Ericsson)