3GPP TSG RAN2 Meeting #129bis                                                              R2-2502000

7th Apr. – 11th Apr. 2025, Wuhan, China

Agenda item:	8.11.2

Source:	Samsung

Title:	Random access in SBFD

Document for:	Discussion & Decision

Introduction

The Rel-19 WID Evolution of Duplex operation introduces NR specification support for gNB-side SBFD within a TDD carrier, enhancements to inter-gNB and inter-UE CLI handling, and BS RF and RRM requirements [1].

In the Rel-19 WID, it is assumed that SBFD is deployed at the gNB side and that the TDD UE operates in half-duplex. Subband full-duplex (SBFD) operation on a time-domain resource is limited to FDM in non-overlapping DL and UL subbands. SBFD enhancements for SBFD-aware UEs assume SBFD operation for NR single carrier in FR1 or FR2-1. From the perspective of the SBFD-aware UE, SBFD operation is assumed within a single configured DL and UL BWP pair with aligned center frequencies. For the semi-static indication of frequency/time locations of the SBFD subbands to SBFD-aware UEs on a legacy DL or F symbol/slot, it is assumed that one UL subband is configured for SBFD operation in an SBFD symbol.

During the previous meetings #129, RAN2 agreed as follows:

When a SBFD aware UE supporting one or both SBFD RACH configuration options accesses a cell, the UE can apply the supported SBFD RACH configuration option in the cell. When a SBFD aware UE supporting a SBFD RACH configuration option accesses a cell configured with a different SBFD RACH configuration option, the UE applies the legacy RA operation, and does not apply the SBFD RACH configuration. For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS. If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold. FFS whether RO type selection is performed before or after the RA type selection. FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

In this contribution, we would like to discuss the detailed random access in SBFD symbols in RRC Connected and Idle/Inactive mode.

Discussion

To increase the SBFD symbol utilization, random access in SBFD symbols are proposed and studied. Random access in SBFD symbols increases the number of available UL transmission opportunities in the TDD UL-DL frame configuration period for the PRACH/preamble transmissions and reduces the random access latency.

According to RAN1 discussion so far, there could be a RACH Occasion (RO) within a SBFD symbol. And such RO may start from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots can be valid based on network configuration.

Utilizing such RO in SBFD symbols, RAN1 and RAN2 require to design/ extend the existing random access procedure such as RO configuration, Msg.1/preamble transmission, and UE power ramping behaviour.

Early Indication

RAN2 needs to discuss and decide on the necessity of an 'Early Indication' mechanism for Release 19 SBFD. Similar to previous releases of NR that supported an early indication mechanism for REDCAP, we believe Msg 3-based early indication mechanism could be considered for Release 19 SBFD, in addition to the obvious Msg 1-based early indication which is already agreed as a RAN2 understanding.

The reason behind considering an early indication mechanism is the same as for previous NR features; namely, the inability to promptly exchange or retrieve UE radio access capabilities until later during the initial access procedure once the UE identity has been verified and NAS and AS authentication have successfully completed.

The example of NR standalone mode operation for FR1 TDD 30kHz is considered in below Figure 2. Any UE in RRC_IDLE must connect to the network and first perform the NAS initial access procedure (“with stored context”) before DL/UL data scheduling.

Figure 2 NAS initial access procedure (“with stored context”) in NR SA mode

It should be considered that about 8-10 DL/UL RRC/NAS messages must be exchanged between the network and the UE during NAS initial access procedure. If an early indication type of mechanism becomes available for SBFD-aware UEs, then any PUSCH containing RRC and NAS messages during the transition from RRC_IDLE to RRC_CONNECTED mode can already be transmitted usint the SBFD UL subband following RACH Message 5 (RRC Setup Complete + NAS Registration Request) upon entering RRC_CONNECTED mode. Hence, an early indication could reduce the initial access delay significantly.

This is because SBFD UL subbands can only be configured for the UE by the RRC re-configuration msg which itself can occur only after the UE has been identified and authenticated by gNB and AMF.

As a candidate for early indication (EI) of SBFD-aware UEs, we consider both Msg1-based EI which could rely on RACH/preamble resource and Msg3-based EI that could rely on Msg.3 (or later) transmissions.

When RACH configuration option 1 is configured or indicated through SIB1, the choice of ROs used for random access attempts by SBFD-aware UEs depends on the RO selection and validation rules. If an SBFD-aware UE utilizes a legacy RO, Early Indication may require a Layer 3 (L3)-based approach.

It is also crucial to consider that feature independence must be achievable for deploying SBFD on the network side. Operators may opt to deploy SBFD solely for operations involving UEs in RRC_CONNECTED mode, meaning they may choose not to leverage the random access in SBFD resources feature. Consequently, Early Indication upon transitioning to RRC_CONNECTED mode would still necessitate reliance on an L3-based solution.

Proposal 1. RAN2 supports the Msg3-based Early Indication for Rel-19 SBFD-aware UEs.

RACH Resource Selection

Considering a User Equipment (UE) supporting the SBFD feature, it could utilize both legacy Random Access Opportunities (RO) and SBFD RO for random access. In scenarios where two ROs are configured, the UE must determine which RO to use for its random access.

To ensure that the SBFD-aware UE benefits from the additional SBFD RO, we should first establish whether such a UE should be capable of utilizing the legacy RO. When considering the relationship between the SBFD RO and the legacy RO, we might explore the possibility of introducing rules, conditions, or prioritization mechanisms among the ROs.

For the RO type selection, RAN2 have agreed as follows:

For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS. If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold.

There are remaining discussions:

Whether NW can further indicate to select SBFD RO when the measurement results are 'below' or 'above' the configured RSRP threshold, or determine to have just one direction as a default.

Note that the main objective of SBFD is to improve UL coverage, which requires SBFD RO to be performed at cell edge areas, requiring UE to select SBFD RO when the measured results are 'below' the configured threshold.

On the other hand, SBFD could also be utilized for giving UEs at cell centers with more RO probabilities and smaller latency, in which case UEs need to select SBFD RO when the measured results are 'above' the configured threshold.

To address both cases, we propose that NW transmits a separate indication (e.g., 1 bit) for UEs to select SBFD RO when measurements are 'below' or 'above' the configured threshold.

Proposal 2. NW may indicate whether the SBFD RO is selected when SSB RSRP are 'below' or 'above' the configured threshold.

Another issue concerns UE behavior when both network indications of preferred RO type and thresholds for RO type selection are not configured. In such cases, UEs have no guidance from networks to choose an RO type; therefore, we suggest allowing such UEs to select the first available RO type (legacy RO or additional RO) among all RACH resources valid to the UE.

This behavior could either serve as a default or be explicitly indicated by the network for utilization.

Proposal 3. For RO type selection upon initiation of CBRA for a SBFD-aware UE, if both network indications of preferred RO type and thresholds for RO type selection are not configured, UE selects the first available RO regardless of RO type (legacy RO or additional RO) among all RACH resources valid to the UE. FFS NW indication for this behavior.

RO Type Switching

Previously, RAN2 agreed to support a fallback mechanism from SBFD RO to legacy RO as follows:

RAN2#127bis - For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

Given the use case of SBFD aimed at enhancing UL coverage, there may arise a necessity or requirement for switching RO types from legacy RO to SBFD RO. Consequently, the remaining discussions include:

- Whether to permit the transition from legacy RO to SBFD RO.

- Whether to verify the SBFD RO type selection criterion, if it is configured.

Proposal 4. For the PRACH transmission re-attempt within one RACH procedure, after a certain (configured) number of attempts with legacy RO, the UE is permitted to switch to additional RO.

Proposal 5. If Proposal 4 is accepted, RAN2 needs to discuss on whether the UE should assess the SBFD RO type selection criterion (e.g., threshold) prior to switching.

Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Proposal 1. RAN2 supports the Msg3-based Early Indication for Rel-19 SBFD-aware UEs.

Proposal 2. NW may indicate whether the SBFD RO is selected when SSB RSRP are 'below' or 'above' the configured threshold.

Proposal 3. For RO type selection upon initiation of CBRA for a SBFD-aware UE, if both network indications of preferred RO type and thresholds for RO type selection are not configured, UE selects the first available RO regardless of RO type (legacy RO or additional RO) among all RACH resources valid to the UE. FFS NW indication for this behavior.

Proposal 4. For the PRACH transmission re-attempt within one RACH procedure, after a certain (configured) number of attempts with legacy RO, the UE is permitted to switch to additional RO.

Proposal 5. If Proposal 4 is accepted, RAN2 needs to discuss on whether the UE should assess the SBFD RO type selection criterion (e.g., threshold) prior to switching.