3GPP TSG-RAN WG2 Meeting #129bis                                 R2-2502001

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:		8.3.2.2

Source:				Samsung

Title:					Discussion on Generalization Issues for AI/ML Mobility

Document for:		Discussion & Decision

1	Introduction

In RAN2#129 meeting, the following statements are agreed.

Agreements on generalization

- Two sets of parameters (ISD, BS antenna height, BS Tx power) are used for the generalization across cell configurations study.

- FR1 is the primary focus, companies can also submit results for FR2 (however, each set of generalization results covers either FR1 or FR2).

- Agree on the two sets of configurations as in tables 2 and 3 (for FR1 and FR2).

| Parameter | Configuration #1 | Configuration #2 |
| --- | --- | --- |
| Deployment scenario | UMi | UMa |
| ISD | 200m | 500m |
| BS antenna height | 10m | 25m |
| BS Tx power | 40dBm | 44dBm |

Table 2: generalization parameters for FR1

| Parameter | Configuration #1 | Configuration #2 |
| --- | --- | --- |
| Deployment scenario | UMi | UMa |
| ISD | 200m | 500m |
| BS antenna height | 10m | 25m |
| BS Tx power | 40dBm | 44dBm |

Table 3: generalization parameters for FR2

In RAN2#128 meeting, the following statements are agreed.

Agreements on generalization

- Reuse the evaluation methodology in TR38.843 for generalization study, i.e., the generalization performance is evaluated with the following cases,

- Baseline: The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B.

- Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B.

- Generalization Case #2 (GC#2): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B.

2.	Companies can choose which case they compare with and should report it with simulation results.

3.	Generalization issues on RRM measurement prediction are prioritized.

4.  Start the study with generalization issue with RRM measurement prediction in temporal domain. Companies can choose to study frequency domain prediction cases and report what they have simulated.

5.  Study generalization over UE speeds

6.  The simulation assumption of FR1 temporal domain case B is reused for generalization study with 3 UE speeds i.e., 30Km/h, 60Km/h and 90Km/h.  FFS on combinations

7.  The simulation assumption of FR2 temporal domain case A is reused for generalization study with 3 UE speeds i.e., 60Km/h, 90Km/h and 120Km/h.  FFS on combinations

This contribution provides our simulation results on generalization issues for AI/ML mobility

2	Discussion

2.1	Generalization over speeds.

In this section, we would like to show our simulation results on generalization for RRM measurement prediction in temporal domain. We investigate how much the performance of L3 cell-level RSRP prediction is affected by different UE speeds.

2.1.1 FR2 to FR2 intra-frequency temporal domain case A (Case 4)

The following Table 1 shows our simulation assumption for the Case 4 (i.e., temporal domain prediction Case A in FR2). Please note that for fair comparison, we use the same number of training/validation samples to train each model (baseline and GC#2).

Table 1

| Report parameters |  |  |  | Samsung |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| OW |  |  |  | 800 msec (10 sampling periods) |  |  |  |
| PW |  |  |  | 400 msec (5 sampling periods) |  |  |  |
| Training/validity |  |  |  | 6000/1500 samples |  |  |  |
| Testing |  |  |  | 50000 samples* |  |  |  |
| Model complexity in model size |  |  |  | ~200 Kbyte |  |  |  |
| Computational complexity [FLOPs] |  |  |  | 20K |  |  |  |
| Model input |  |  |  | Historical L3 cell-level RSRP within OW |  |  |  |
| Model output |  |  |  | Future L3 cell-level RSRP within PW |  |  |  |
| Considered cases |  |  |  |  |  |  |  |
|  | Training @Dataset: 60km/h | Training @Dataset: 90km/h | Training @Dataset: 120km/h |  | Inference @60km/h | Inference @90km/h | Inference @120km/h |
| Baseline_60km/h | Yes |  |  |  | Yes |  |  |
| GC#2_60km/h | Yes | Yes | Yes |  | Yes |  |  |
| Baseline_90km/h |  | Yes |  |  |  | Yes |  |
| GC#2_90km/h | Yes | Yes | Yes |  |  | Yes |  |
| Baseline_120km/h |  |  | Yes |  |  |  | Yes |
| GC#2_120km/h | Yes | Yes | Yes |  |  |  | Yes |

The following Table 2 shows our simulation results for the Case 4 (i.e., temporal domain prediction Case A in FR2).

Table 2

| Case | UE speed (km/h) | Prediction accuracy (RSRP difference) |  | Degradation (= GC#2 - Baseline) |
| --- | --- | --- | --- | --- |
|  |  | Baseline | GC#2 |  |
| Case A prediction in FR2 | 60 | 1.055 dB | 1.2 dB | 0.145 dB |
|  | 90 | 1.199 dB | 1.364 dB | 0.165 dB |
|  | 120 | 1.347 dB | 1.497 dB | 0.15 dB |

In our simulation results, we observed that the performance can be slightly degraded if we use a generalized model for RSRP prediction, but the performance degradation is marginal (less than 0.2 dB). Since the channel variation caused by distance change over time is affected by not only UE speed but also UE direction, there can be a wide range of channel variations according to UE direction even in the dataset with the same UE speed. Also, in each dataset with different UE speeds, there will likely be many overlapped data from the perspective of channel variation. We think that generalized model also works well enough because of the similarity among datasets with different UE speeds. Moreover, considering that RSRP values always have some level of fluctuations naturally, the small amount of performance degradation seems acceptable.

Observation 1: As for the generalized model over speeds for RSRP prediction, the performance difference between baseline and GC#2 is marginal, even though a slight degradation is observed for Case A temporal domain prediction in FR2.

2.1.2 FR1 to FR1 intra-frequency temporal domain case B (Case 2)

The following Table 3 shows our simulation assumption for the Case 2 (i.e., temporal domain prediction Case B in FR1). Please note that for fair comparison, we use the same number of training/validation samples to train each model (baseline and GC#2).

Table 3

| Report parameters |  |  | Samsung |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reported simulation assumptions | Measurement reduction rate (R) |  | 50% |  | 66% |  | 80% |
|  | OW |  | 800 ms |  | 800 ms |  | 800 ms |
|  | PW |  | 40 ms |  | 80 ms |  | 160 ms |
|  | Filtering options (1/2/3) in [4] |  | Filtering option 3: L3 cell result at T3 is filtered based on the L1 filtered result at T3 and L3 filtered cell result at T1 (i.e. the old L3 filtered cell result from last actual measurement) |  |  |  |  |
| Data Size (Number of Samples) | Training/validity |  | 18000/1800 samples* |  |  |  |  |
|  | Testing |  | 15000 samples* |  |  |  |  |
| AI/ML model input/output | Model input |  | Historical L3 cell level RSRP in OW with the interval (= sampling period * 1/(1-R) ) |  |  |  |  |
|  | Model output |  | Future L3 cell level RSRP in PW with the interval (= sampling period) |  |  |  |  |
| Considered cases |  |  |  |  |  |  |  |
|  | Training @Dataset: 30km/h | Training @Dataset: 60km/h |  | Training @Dataset: 90km/h | Inference @30km/h | Inference @60km/h | Inference @90km/h |
| Baseline_30km/h | Yes |  |  |  | Yes |  |  |
| GC#2_30km/h | Yes | Yes |  | Yes | Yes |  |  |
| Baseline_60km/h |  | Yes |  |  |  | Yes |  |
| GC#2_60km/h | Yes | Yes |  | Yes |  | Yes |  |
| Baseline_90km/h |  |  |  | Yes |  |  | Yes |
| GC#2_90km/h | Yes | Yes |  | Yes |  |  | Yes |

The following Table 4 shows our simulation results for the Case 2 (i.e., temporal domain prediction Case B in FR1).

Table 4

| Case | UE speed (km/h) | MRRT (%) | Prediction accuracy (RSRP difference) |  | Degradation (= GC#2 - Baseline) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Baseline | GC#2 |  |
| Case B prediction in FR1 | 30 | 50 | 0.278 dB | 0.378 dB | 0.1 dB |
|  |  | 66 | 0.36 dB | 0.459 dB | 0.099 dB |
|  |  | 80 | 0.614 dB | 0.613 dB | -0.001 dB |
|  | 60 | 50 | 0.39 dB | 0.413 dB | 0.023 dB |
|  |  | 66 | 0.498 dB | 0.579 dB | 0.081 dB |
|  |  | 80 | 0.889 dB | 0.821 dB | -0.068 dB |
|  | 90 | 50 | 0.412 dB | 0.485 dB | 0.073 dB |
|  |  | 66 | 0.635 dB | 0.724 dB | 0.089 dB |
|  |  | 80 | 1.057 dB | 1.002 dB | -0.055 dB |

In our simulation results, we observed that the performance difference between baseline models and GC#2 is marginal (less than 0.1 dB). When it comes to the performance difference between baseline models and GC#2, there might be a tendency according to MRRT values or UE speeds, e.g., the performance of GC#2 gets better when we use higher value of MRRT. However, it is hard to discuss the tendency because the performance difference is so small. It is better to just conclude that the performance difference between baseline models and GC#2 is marginal for Case B temporal domain prediction in FR1.

Observation 2: As for the generalized model over speeds for RSRP prediction, the performance difference between baseline and GC#2 is marginal for Case B temporal domain prediction in FR1.

2.2	Generalization over cell configurations

In this section, we would like to show our simulation results on generalization for RRM measurement prediction in temporal domain. We investigate how much the performance of L3 cell-level RSRP prediction is affected by different cell configurations.

2.2.1 FR1 to FR1 intra-frequency temporal domain case B (Case 2)

The following Table 5 shows our configuration setup for both FR1 UMi and FR1 UMa.

Table 5

| Parameters | Value for FR1 UMi (Configuration #1) | Value for FR1 UMa (Configuration #2) |
| --- | --- | --- |
| Frequency Range | FR1@{4GHz,30KHz} as central frequency for intra-frequency scenario FR1@{2GHz, 15/30KHz} as another frequency for inter-frequency scenario |  |
| Deployment | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |  |
| Channel model | UMi With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and optional LOSsoft; without UErotation,Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage | UMa With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and optional LOSsoft; without UErotation,Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage. |
| Inter site distance | 200 m | 500 m |
| BS Tx Power | 40 dBm | 44 dBm |
| BS Antenna height | 10 m | 25 m |
| System BW | 20 MHz |  |
| UE speed | 30,60,90 km/h for study targeting measurement reduction |  |
| UE distribution | 100% outdoor |  |
| BS Antenna Configuration | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |  |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi |  |
| UE Antenna Configuration | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |  |
| UE Antenna Radiation pattern | Omni-direction |  |
| Maximum UE Tx power | 23dBm |  |
| BS receiver Noise Figure | 5dB |  |
| UE receiver Noise Figure | 9dB |  |
| UE Antenna height | 1.5m |  |
| Spatial consistency | Companies report the one of spatial consistency procedures. - Procedure A in TR38.901 - Procedure B in TR38.901 |  |

The following Table 6 shows our simulation assumption for the Case 2 (i.e., temporal domain prediction Case B in FR1). Please note that for fair comparison, we use the same number of training/validation samples to train each model (baseline and GC#2).

Table 6

| Report parameters |  | Samsung |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reported simulation assumptions | Measurement reduction rate (R) | 50% |  | 66% |  | 80% |
|  | OW | 800 ms |  | 800 ms |  | 800 ms |
|  | PW | 40 ms |  | 80 ms |  | 160 ms |
|  | Filtering options (1/2/3) in [4] | Filtering option 3: L3 cell result at T3 is filtered based on the L1 filtered result at T3 and L3 filtered cell result at T1 (i.e. the old L3 filtered cell result from last actual measurement) |  |  |  |  |
| Data Size (Number of Samples) | Training/validity | 17000/1700 samples* |  |  |  |  |
|  | Testing | 15000 samples* |  |  |  |  |
| AI/ML model input/output | Model input | Historical L3 cell level RSRP in OW with the interval (= sampling period * 1/(1-R) ) |  |  |  |  |
|  | Model output | Future L3 cell level RSRP in PW with the interval (= sampling period) |  |  |  |  |
| Considered cases |  |  |  |  |  |  |
|  | Training with UMi | Training with UMa | Inference with UMi |  | Inference with UMa |  |
| Baseline_UMi | Yes |  | Yes |  |  |  |
| GC#1_UMi |  | Yes | Yes |  |  |  |
| GC#2_UMi | Yes | Yes | Yes |  |  |  |
| Baseline_UMa |  | Yes |  |  | Yes |  |
| GC#1_UMa | Yes |  |  |  | Yes |  |
| GC#2_UMa | Yes | Yes |  |  | Yes |  |

The following Table 7 shows our simulation results for the Case 2 (i.e., temporal domain prediction Case B in FR1).

Table 7

| Case | UE speed (km/h) | Dataset for inference | MRRT (%) | Prediction accuracy (RSRP difference) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Baseline | GC#1 | GC#2 |
| Case B prediction in FR1 | 30 | UMi | 50 | 0.202 dB | 0.215 dB | 0.213 dB |
|  |  |  | 66 | 0.343 dB | 0.425 dB | 0.325 dB |
|  |  |  | 80 | 0.444 dB | 0.427 dB | 0.412 dB |
|  |  | UMa | 50 | 0.119 dB | 0.128 dB | 0.14 dB |
|  |  |  | 66 | 0.256 dB | 0.201 dB | 0.186 dB |
|  |  |  | 80 | 0.251 dB | 0.281 dB | 0.254 dB |

In our simulation results, we observed that the performance of baseline, GC#1, and GC#2, in terms of prediction accuracy seem to be similar to each other. Although the performance difference was not significant, GC#2 showed slightly better performance compared to GC#1 in most cases and GC#2 showed similar performance to baseline. In addition, it is not always guaranteed that the performance of baseline is the best over other schemes. Taking all these observations into account, it seems that it is generalizable over cell configurations with different deployment scenarios for Case B temporal domain prediction in FR1.

Observation 3: As for the generalized model over cell configurations for RSRP prediction, the performance difference between baseline model and generalized models is marginal for Case B temporal domain prediction in FR1.

Observation 4: As for the generalized model over cell configurations for RSRP prediction, GC#2 slightly improves the accuracy of the AI/ML model compared to GC#1, while offering comparable accuracy as baseline for Case B temporal domain prediction in FR1.

Proposal 1: For Case B temporal domain prediction in FR1, RAN2 to capture the observation that it is generalizable over cell configurations with different deployment scenarios.

Proposal 2: For Case B temporal domain prediction in FR1, RAN2 to capture the observation that GC#2 slightly improves the accuracy of the AI/ML model compared to GC#1, while offering comparable accuracy as baseline.

3	Conclusion

Observation 1: As for the generalized model over speeds for RSRP prediction, the performance difference between baseline and GC#2 is marginal, even though a slight degradation is observed for Case A temporal domain prediction in FR2.

Observation 2: As for the generalized model over speeds for RSRP prediction, the performance difference between baseline and GC#2 is marginal for Case B temporal domain prediction in FR1.

Observation 3: As for the generalized model over cell configurations for RSRP prediction, the performance difference between baseline model and generalized models is marginal for Case B temporal domain prediction in FR1

Observation 4: As for the generalized model over cell configurations for RSRP prediction, GC#2 slightly improves the accuracy of the AI/ML model compared to GC#1, while offering comparable accuracy as baseline for Case B temporal domain prediction in FR1.

Proposal 1: For Case B temporal domain prediction in FR1, RAN2 to capture the observation that it is generalizable over cell configurations with different deployment scenarios.

Proposal 2: For Case B temporal domain prediction in FR1, RAN2 to capture the observation that GC#2 slightly improves the accuracy of the AI/ML model compared to GC#1, while offering comparable accuracy as baseline.

4	Reference