3GPP TSG-RAN WG2 Meeting #129b	 R2-2502004

Wuhan, China, April 7-11, 2025

Agenda Item:	8.1.3

Source: 	NEC

Title:  	Discussion on NW-side data collection

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2#129 meeting, the following agreements are made:

Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not.  L1 measurement event triggered will not be supported.    FFS what to log

Low power bit indication is supported

Data availability indication is supported.  FFS when this would be triggered

As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT.

UE retains logged data during handover (HO).  FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed

UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

FFS how to handle idle/inactive and RLF cases and whether we have a unified.

In this contribution, we further discuss both logging and reporting aspects.

- 2 Discussion

- 2.1 Logging aspect

Normally when the UE has collected enough data, the NW could explicitly release the data collection configuration. Besides explicit way, we think some mechanism from logged MDT can be referred, i.e., maximum logging duration, or maximum data size required by NW could be considered to allow UE to release the configuration, in this way, NW-UE signalling interaction could be reduced, i.e., without NW additional release but both NW and UE side are aligned with the configuration. For example, the NW could configure the UE to perform data collection for a specific duration, e.g., 2 hours at night, when the duration is expired, the data collection configuration could be released by UE and NW is aware of this situation. Another example, the NW could configure the UE to perform data collection up to X MB data, and the configuration could be released once the UE has reported data up to X MB data.

Proposal-1: maximum logging duration or maximum data size can be considered to release data collection configuration, i.e., when the UE has already logged and/or reported data up to max duration/size, the UE release the configuration.

In last meeting, RAN2 agreed that UE retains logged data during handover (HO), and further consider the case of RRC state transition and RLF. In our understanding, we should keep the same framework for all cases, which means the UE should retain logged data when HO, RRC state transition, RLF happen, a unified solution can facilitate LCM procedure. For more specific reason, for RRC state transition, if logged data is not retained during transition, we believe that the UE who is only performing data logging without any other unicast service should be always kept in RRC_CONNECTED which is not friendly for power saving. The UE is supposed to enter RRC_IDLE when there is unicast service and pause logging and retain logged data and start logging when back to RRC_CONNECTED. For RLF, if logged data is not retrained, we believe that the UE who is not in a good signal quality can not perform data collection efficiently and also we can not guarantee the UE who is configured with data collection configuration always stays in a good signal quality. Therefore, the UE should retain logged data for the case of HO, RLF and RRC state transition.

Proposal-2: the UE should retain logged data for the case of HO, RLF and RRC state transition.

- 2.2 Reporting aspect

RAN2 agreed that Data availability indication is supported, here we discussion which signalling we put this indication and how to trigger it.

First of all, since this indication is just an assistance information to facilitate NW to trigger UE information procedure for collecting data and UE can proactively initiate data availability indication without waiting for NW indication. In this regard, it is promising to use UEAssistanceInformation to carry this indication. Furthermore, refer to MDT mechanism, logged data availability can also be indicated in RRC complete message, for data collection, it is promising to agree that the UE should retain logged data for the case of HO, RLF and RRC state transition, in this regard, RRC complete message is also promising to carry data availability indication. Therefore, this data availability indication can be carried in UAI and RRC complete message.

Proposal-3: Data availability indication can be carried in UAI and RRC complete message.

Second of all, we also need to FFS how to trigger this indication, in previous meeting, RAN2 discussed buffer based data availability indication, and there could be 3 solutions:

Solution-1: as long as the UE has data available, the UE may trigger data availability indication, which is no spec impact.

Solution-2: a data threshold is introduced, when the logged data size is greater than the threshold, the UE triggers data availability indication.

Solution-3: when the logged data size reaches the maximum buffer size, the UE triggers data availability indication.

For solution-1, we don’t think it can work efficiently, a big amount of availability indication may be triggered, while for solution-3, there is one drawback that the UE reports this availability indication a little bit late as the NW need some time to trigger UE information procedure for collecting data, but since now the UE buffer is full, the UE cannot continue to perform data logging at this moment. Therefore it is promising to introduce a data size threshold, when the the logged data size is greater than the threshold, the UE trigger data availability indication.

Proposal-4: Introduce a data size threshold, when the logged data size is greater than the threshold, the UE triggers data availability indication.

Now we discuss UE information procedure, in last meeting, RAN2 agreed that the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT. Considering the logged data size could be huge, a further availability indication in UEInformationResponse is promising. But there is still one drawback, in legacy, once receive UEInformationRequest from NW, the UE shall transmit UEInformationResponse including the recorded information (e.g., logged measurement) requested by NW. For data collection, the UE may have already logged a large amount of training data, one message may not be enough to carry all of training data at one time. But if NW further send the sequent multiple UEInformationRequest messages to UE, signal overhead is increased, in this sense, the UE could send multiple UEInformationResponse to respond the corresponding UEInformationRequest, and each UEInformationResponse can indicate data availability indication and remaining data size. By this way, unnecessary signal overhead can be saved when a large amount of training data need to be reported.

Proposal-5: the UE can send multiple UEInformationResponse messages to respond the same one UEInformationRequest message when large amount of training data need to be reported and UEInformationResponse could include the data availability indication and/or the remaining data size.

For which SRB is used for UEInformationResponse, currently we can have the following options:

Opt-1: SRB2

Opt-2: SRB4

Opt-3: new SRB

In our understanding, what we need to do is make sure that the SRB served for data collection is low priority. SRB4 is used for QoE which is another feature/characteristic compared with logging mechanism, this is not good for future-proof. For SRB2, currently, logged record information use this SRB2 and transmit data into gNB and forward to OAM, since we agreed that NW-side data training could happen at either gNB-side or OAM-side, this SRB2 is promising, plus, normally SRB2 is a lower priority compared with SRB1 (up to NW implementation). Having said this, if we indeed want a specific/independent SRB for data collection, e.g., for future-proof, a new SRB specific to only AI feature can also be considered so that differentiate from other logged legacy MDT/SON record information, which means the priority can be set more flexible.

Proposal-6: Consider using SRB2 or a new SRB to transmit UEInformationResponse for AI data collection.

- 3 Conclusions

Proposal-1: maximum logging duration or maximum data size can be considered to release data collection configuration, i.e., when the UE has already logged and/or reported data up to max duration/size, the UE release the configuration.

Proposal-2: the UE should retain logged data for the case of HO, RLF and RRC state transition.

Proposal-3: Data availability indication can be carried in UAI and RRC complete message.

Proposal-4: Introduce a data size threshold, when the logged data size is greater than the threshold, the UE triggers data availability indication.

Proposal-5: the UE can send multiple UEInformationResponse messages to respond the same one UEInformationRequest message when large amount of training data need to be reported and UEInformationResponse could include the data availability indication and/or the remaining data size.

Proposal-6: Consider using SRB2 or a new SRB to transmit UEInformationResponse for AI data collection.

- 4 References