3GPP TSG-RAN WG2 Meeting #129bis									R2-2502016

Wuhan, China, April 7th – 11th, 2025

Agenda item		: 8.4.4

Title				: LP-WUS operation in Connected mode

Source				: Tejas Networks Limited

Document for		: Discussion and Decision

# Introduction

The following agreements have been made so far in RAN2 for CONNECTED mode procedures for LP-WUS:

The Agreements at RAN2 #129: - For Option 1-1, UE monitors LP-WUS outside C-DRX active time at least when long DRX cycle is used. FFS whether short DRX cycle is used. - RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-1 to stop drx-onDurationTimer and drx-InactivityTimer. - RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-2 to stop the new timer and drx-InactivityTimer.

This contribution further discusses the RAN2 related issues on procedure and configuration of LP-WUS in  Connected modes.

# Discussion

## 2.1 LP-WUS monitoring during CDRX:

In previous R2 meeting, For LP-WUS operation in case of Option 1-1 was agreed. But option 1-2 still needs to be discussed.

Option 1-2: UE monitors LP-WUS outside of the active time of CDRX. When UE receives wake up indication, a new timer is started and UE monitors PDCCH and UE is considered to be in CDRX active time.

In our understanding, option 1-2 is mainly meant for sending in delay sensitive data where the UE can be indicated to receive the data instead of waiting for the next CDRX cycle. If the UE is configured with a CDRX configuration, UE follows the legacy CDRX mechanism of starting the onDurationTimer based on the CDRX cycle. If UE is configured with option1-2, UE monitors for LP-WUS only in the periods outside of the CDRX active period as determined by the legacy timers.

Proposal 1: UE starts the onDurationTimer as per legacy mechanism even if option 1-2 is configured.

Proposal 2: If option 1-2 is configured along with C-DRX, UE monitors LP-WUS only in time periods outside of the CDRX active time as determined by the legacy CDRX framework

Proposal 3: If option1-2 is configured without the CDRX configurations, UE monitors for PDCCH only based on the new timer associated with the LP-WUS.

2.2 LP-WUS for CA and DC scenarios:

In case of CA, the DRX operation is per MAC entity except for the case of dual DRX for FR1 and FR2 operations. Release 17 DCP operation does not support the use of DCP along with 2 DRX groups being configured. So its simpler to take that as base and extend to LP-WUS operation as well.

Proposal 4: LP-WUS is not used/configured along with dual DRX group for CA operation

In case of DC operations, each of the cell group will be configured with a separate LP-WUS configuration but only one cell group can use LP-WUS operation.  In order to support LP-WUS reception in 2 cell groups simultaneously, an explicitly signaled by the UE as an capability of LR to monitor multiple frequencies is needed. This will complicate the design and need to also indicate the supported band combinations. To simplify this, we propose to have separate configurations per cell group but a UE can be configured to use LP-WUS in only one cell group

Proposal 5: LP-WUS is configured per cell group for DC operation and network to ensure that at a given time configuration for only one cell group is active.

# Conclusion

Proposal 1: UE starts the onDurationTImer as per legacy mechanism even if option 1-2 is configured.

Proposal 2: UE monitors LP-WUS only in time periods outside of the CDRX active time as determined by the legacy CDRX framework

Proposal 3: If option1-2 is configured without the CDRX configurations, UE monitors for PDCCH only based on the new timer associated with the LP-WUS.

Proposal 4: LP-WUS is not used/configured along with dual DRX group for CA operation

Proposal 5: LP-WUS is configured per cell group for DC operation and network to ensure that at a given time configuration for only one cell group is active.