3GPP TSG-RAN WG2 Meeting #129bis	R2-2502033

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.6.4

Source: 	Fujitsu

Title:	Discussion on conditional Intra-CU LTM

Document for:	Discussion and Decision

- Introduction

At previous RAN2 meetings, some agreements on conditional Intra-CU LTM are reached. However, there still are some essential remaining open issues.

This contribution will discuss the following issues:

- C-LTM event evaluation

- Early TA and TAT handling

- CLTM cell switch execution when both L1 and L3 conditions are fulfilled

- Discussions

C-LTM event evaluation

RAN2 has agreed that the UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition [1]. To implement this agreement, on one hand, the interaction to start execution condition evaluation between RRC layer and MAC layer is captured in RRC running CR [2] as following:

- RRC informs lower layers to initiate the LTM cell switch conditions evaluation based on L1 measurements if l1-Condidations is included within cltm-ServingCellExecutionConditions

- Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, or upon the fulfilment of LTM cell switch conditions

  - RRC informs lower layers to initiate the LTM cell switch conditions evaluation based on L1 measurements if the LTM-Candidate IE for the selected LTM candidate configuration includes the field cltm-ExecutionConditions and if l1-Condidations is included within cltm-ExecutionConditions

According to MAC running CR [3], the MAC entity will determine if the entry condition is fulfilled for the applicable beams considering TTT when RRC informs to initiate the LTM cell switch condition evaluation.

On the other hand, the interaction to stop execution condition evaluation is captured in the RRC running CR as well:

- RRC informs lower layers to stop LTM cell switch conditions evaluation if a reconfiguration with sync is performed due to an LTM cell switch execution

- RRC informs lower layers to stop LTM cell switch conditions evaluation if cltm-ServingCellExecutionConditions is configured and if LTM conditions evaluation was done based on L1 measurements

- Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or upon the fulfilment of LTM cell switch conditions

  - RRC informs lower layers to stop LTM cell switch conditions evaluation if LTM conditions evaluation was done based on L1 measurements

- RRC informs lower layers to stop LTM cell switch conditions evaluation if LTM conditions evaluation was done based on L1 measurements upon initiating MCG failure information procedure

In our understanding, the MAC entity needs not to evaluate whether the entry condition is fulfilled if RRC layer informs to stop the LTM cell switch condition evaluation. However, this is not captured in the MAC running CR. Considering the LTM cell switch condition evaluation can be requested to be stopped by the RRC layer, the MAC entity will determine whether the entry condition is fulfilled for the applicable beams for each entry within the CLTM-ExecutionConditions requested to be initiated and not stopped by the upper layers. This should be captured in 5.y.2 of the running MAC CR.

Proposal 1: For each entry within the CLTM-ExecutionConditions requested to be initiated and not be stopped by the upper layers, the MAC entity will evaluate if the entry condition is fulfilled for the applicable beams during TTT. This should be captured in 5.y.2 of the running MAC CR.

Early TA and TAT handling

Absolute TA value and candidate configuration ID are included in the MAC CE for CLTM early TA acquisition. CLTM TAT is started/re-started when UE receives TA value for the candidate cell sent by the MAC CE from current serving cell. Regarding TA and TAT handling, Rel-18 mechanism is reused in the running MAC CR. In this mechanism, TA value is applied, and TAT associated with PTAG is started or restarted when CLTM cell switch is triggered. Additionally, TA value is stored, and TAT associated with LTM candidate cell is started or restarted when the MAC CE is received. In our understanding, these UE behaviours are simple and straightforward. So, we propose that RAN2 confirms this first.

Proposal 2: Regarding TA and TAT handling, Rel-18 mechanism is reused.

Proposal 3: TA value is stored, and TAT associated with LTM candidate cell is started or restarted when the MAC CE is received.

There could be multiple LTM candidate TAC MAC CEs receiving before CLTM cell switch execution since one configuration ID for one TA is included in in one MAC CE. So, multiple TA timers associated with LTM candidate are maintained when the CLTM cell switch is triggered. How to handle these TA timers associated with the LTM candidate has not conclusion yet. In Rel-18, partial MAC reset is not supported, i.e. all running timers (except MBS broadcast DRX timers) will be stopped and all TA timers are considered as expired. We think that the same MAC reset actions should be applied to the CLTM. If the MAC reset is requested by CLTM, all TA timers including TAT associated with LTM candidate cell are considered as expired.

Proposal 4: If the MAC reset is requested by CLTM, all running timers (except MBS broadcast DRX timers) will be stopped and all TA timers including TAT associated with LTM candidate cell are considered as expired.

CLTM cell switch execution when both L1 and L3 conditions are fulfilled

From a given source cell, network may configure either L1 execution conditions or L3 execution conditions for a CLTM candidate cell. Also, there can be the case where conditions for CLTM cell switch towards multiple candidates are fulfilled. If L1 conditions for CLTM are fulfilled, it is up to MAC entity to select one candidate beam satisfying the L1 condition and to trigger CLTM, e.g. associated LTM configuration ID is indicated to upper layers to perform LTM cell switch execution. If L3 conditions are fulfilled, the legacy prioritization applies. However, there could be one case where L1 execution condition for one CLTM candidate cell is fulfilled and L3 execution conditions for another CLTM candidate cell are fulfilled at the same time. So, it needs to be further discussed how to handle the case where both L1 and L3 conditions are fulfilled when both conditions are configured.

When L1 condition is fulfilled, the MAC entity indicates the corresponding LTM configuration ID to upper layers and waits for RRC request. So, RRC has knowledge of CLTM trigger at RRC and CLTM trigger from MAC entity and thus RRC can prioritize CLTM triggered by L1 conditions or CLTM triggered by L3 conditions. Alternatively, the prioritization is up to UE implementation. It requires RAN2 discussions.

Proposal 5: RAN2 is kindly asked to discuss whether CLTM triggered by L1 conditions or by L3 conditions should be prioritize or it is up to UE implementation when L1 execution condition for one CLTM candidate cell is fulfilled and L3 execution conditions for another CLTM candidate cell are fulfilled at the same time.

- Conclusion

This document discussed the remaining open issues for Intra-CU CLTM and made the following proposals:

C-LTM event evaluation

Proposal 1: For each entry within the CLTM-ExecutionConditions requested to be initiated and not be stopped by the upper layers, the MAC entity will evaluate if the entry condition is fulfilled for the applicable beams during TTT. This should be captured in 5.y.2 of the running MAC CR.

Early TA and TAT handling

Proposal 2: Regarding TA and TAT handling, Rel-18 mechanism is reused.

Proposal 3: TA value is stored, and TAT associated with LTM candidate cell is started or restarted when the MAC CE is received.

Proposal 4: If the MAC reset is requested by CLTM, all running timers (except MBS broadcast DRX timers) will be stopped and all TA timers including TAT associated with LTM candidate cell are considered as expired.

CLTM cell switch execution when both L1 and L3 conditions are fulfilled

Proposal 5: RAN2 is kindly asked to discuss whether CLTM triggered by L1 conditions or by L3 conditions should be prioritize or it is up to UE implementation when L1 execution condition for one CLTM candidate cell is fulfilled and L3 execution conditions for another CLTM candidate cell are fulfilled at the same time.

# Reference

- R2-2501331, “Report from session on V2X/SL, R19 NES and MOB”, Vice Chairman (Samsung), RAN2#129

- R2-2502534, “Running RRC CR for inter-CU and conditional LTM”, Ericsson, RAN2#129

- R2-2502158, “Running MAC CR for Mob Ph4”, vivo (Rapporteur), RAN2#129bis