3GPP TSG-RAN WG2 Meeting #129bis	R2-2502067

Wuhan, China, Apr. 7th – 11th, 2025

Source: 	Xiaomi

Title:  	Discussion on conditional LTM

Agenda Item:	8.6.4

Document for:	Discussion and Decision

Introduction

A Revised WID on NR mobility enhancements Phase 4 [1] has been approved. In the WID, one of the objectives is to specify support for conditional Intra-CU LTM, as shown below.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

In this contribution, we provide our understandings on conditional intra-CU LTM.

Discussion

  - The coexistence of CLTM and other mobility cases

For CLTM and LTM

In RAN2#129 meeting, it is confirmed that the LTM-Candidate IE is re-used to model an CLTM candidate cell configuration. And RAN2 has agreed “For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions.”, and the agreement implicitly indicated the coexistence of LTM and CLTM is supported in Rel-19. RAN2 can confirm it.

To avoid unnecessary signalling overhead, one LTM candidate configuration can be configured for both MAC CE tiggered LTM and CLTM. That is, for LTM candidate configured with the CLTM execution condition, the network can also trigger cell switch to the LTM candidate via LTM cell switch Command MAC CE.

Proposal 1a: RAN2 confirms the coexistence of LTM and CLTM is supported. Even if the CLTM condition is configured for the LTM candidate, the network can also trigger LTM cell switch to the LTM candidate via LTM cell switch Command MAC CE.

For CLTM and L3 mobility

In Rel-18 intra-CU LTM and Rel-19 inter-CU LTM, RAN2 has reached the following agreements for the coexistence of LTM and L3 Mobility：

For LTM and L3 HO/CHO: RAN2 #127 (Rel-19 MOB) agreement: L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM): - When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network. - The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations. - For the execution order between CHO and LTM, Rel-18 principle is applied. RAN2#128(Rel-18 MOB) agreement: Clarify in RRC that the conditional reconfiguration cannot include the ltm-config. For DAPS: RAN2#125(Rel-18 MOB): - The coexistence of LTM and DAPS HO is not supported.

For CLTM and L3 HO:

For the coexistence of CLTM and L3 HO, we can follow the Rel-19 inter-CU LTM agreement and support the coexistence of CLTM and L3 HO. When performing the L3 HO, the UE does not autonomously release CLTM configurations, unless these are explicitly released by the network and the L3 HO Command (RRCReconfiguration message) may reconfigure CLTM configurations.

Proposal 1b: The coexistence of CLTM and L3 HO is supported in Rel-19.

For CLTM and CHO:

For the coexistence of LTM and CHO, the agreement in RAN2 #127 (Rel-19 MOB) and the agreement in RAN2#128(Rel-18 MOB) are in conflict.

Observation 1: For the coexistence of LTM/CLTM and CHO, the agreement in RAN2 #127 (Rel-19 MOB) and the agreement in RAN2#128(Rel-18 MOB) are in conflict:

- The agreement in RAN2 #127 (Rel-19 MOB): The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

- The agreement in RAN2#128(Rel-18 MOB): Clarify in RRC that the conditional reconfiguration cannot include the ltm-config.

In Rel-18 and Rel-19 LTM, UE stores Rel-18 ID and Rel-19 ID associated with serving cell for L2 handling and UE based TA measurement. But in Rel-18 intra-CU LTM, the RRCReconfiguration in ConditionalReconfiguration IE cannot include the ltm-config (clarified in RAN2#128 for Rel-18 LTM). If the coexistence of ltm-config and conditionalReconfiguration is supported, when performing the CHO, the Rel-18 ID and Rel-19 ID associated with serving cell cannot be updated to associated with the triggered CHO target cell. This is not reasonable and subsequent LTM cannot work correctly.

Hence, if we follow the clarification for Rel-18 LTM that the conditional reconfiguration cannot include the ltm-config, the previous agreement in RAN2 #127 (Rel-19 MOB) for Rel-19 LTM and CHO needs to be revised. The coexistence of LTM and CHO is not supported in Rel-19. For CLTM and CHO, we can follow the Rel-19 LTM.

Proposal 1c: The coexistence of LTM and CHO is not supported in Rel-19 (which revises the associated agreement in RAN2 #127). And the coexistence of CLTM and CHO is not supported in Rel-19.

For CLTM and DAPS:

Hence, for the coexistence case of CLTM and DAPS, we can follow the Rel-18 LTM.

Proposal 1d: The coexistence of CLTM and DAPS HO is not supported.

  - CLTM TA

Issue 1: how many candidate TA timers can be maintained by UE at the same time.

RAN2 has agreed the CLTM TAT is per candidate configuration and the maximum number of CLTM candidate configurations is 8. To reduce the complexity of UE, we suggest that the maximum number of candidate TA timers can be maintained by UE at the same time is 2. As UE vendor, we don’t prefer the UE to maintain too many candidate TA timers, such as, up to 8.

In Report of post email discussion for UE capabilities [2], majority companies prefer to define per-band capability(s) for the TA timer maintenance of candidate cells. The UE capability can be used to indicate the maximum number of candidate TA timers that maintained by the UE at the same time, and the value of the new UE capability(s) is 1 or 2.

Proposal 2: To define per-band capability(s) for the TA timer maintenance of candidate cells, and the new capability(s) indicates the maximum number of candidate TA timers that maintained by the UE at the same time. The value of the UE capability(s) is 1 or 2.

An open issue for the new UE capability(s) is whether this new capability can also represent that the UE support PDCCH order TA acquisition in CLTM.

As shown in section 2.1, support the coexistence of LTM and CLTM is the common understanding. For the coexistence of LTM and CLTM configuration, if the new capability can also represent that the UE support PDCCH order TA acquisition in CLTM, and rach-EarlyTA-Measurement-r18 is indicate the UE support PDCCH order TA acquisition in LTM (MAC CE triggered LTM), when the two capabilities indicate two different values, the network might not determine how many candidate cells can be triggered to perform the PDCCH order TA acquisition before LTM execution.

Hence, a simple method is to reuse the rach-EarlyTA-Measurement-r18 only for early RACH, and define a separate UE capability for CLTM TA timer maintenance (as provided in p2). In this way, the rach-EarlyTA-Measurement-r18 can handle both LTM and CLTM to indicate the maximum number of candidate cells for TA acquisition based on PDCCH ordered CFRA procedure.

Proposal 3：Reuse the rach-EarlyTA-Measurement-r18 to indicates the maximum number of candidate cells for TA acquisition based on PDCCH ordered CFRA procedure for both LTM and CLTM.

Issue 2: how/whether to start the timeAlignmentTimer associated with PTAG upon CLTM cell switch execution to the target cell

In RAN2#129 meeting, an open issue for CLTM TA is how/whether to start the timeAlignmentTimer associated with PTAG upon CLTM cell switch execution to the target cell, some details are specified in [3], as follows:

“CLTM TAT for target cell is not stopped upon CLTM cell switch execution to the target cell.” due to the reasons: - In the current MAC specification, it is mentioned that: Thus, rapporteur suggests to discuss how/whether to start the timeAlignmentTimer associated with PTAG upon CLTM cell switch execution to the target cell during CB session: - Option 1 (revert the offline agreement): start the timeAlignmentTimer associated with PTAG as in Rel-18; - Option 2 (keep the offline agreement): discuss the options below in the MAC running CR - Option 2.1: clarify the running TAT to act as the timeAlignmentTimer associated with PTAG by a note in the MAC running CR; - Option 2.2: define that UE can perform UL tx when the TAT for the target cell is running. Besides, we need to add a note that, if SCell and SpCell are in the different TAG, SCell may not work when this TAT for target cell is running. - Option 2.3: the UE stops the CLTM timer and starts the PTAG using the remaining time of the CLTM timer.

For above options, option 1 is the simple and straightforward method and follows the legacy UE behaviours without extra spec impact.

Some companies raised concern on option 1 is that when CLTM is triggered, the remaining valid time of the candidate TA is smaller than the timeAlignmentTimer of PTAG, which may result in the uplink out of synchronization. The similar issue also exists in network triggered LTM, because the TA value in LTM CSC MAC CE is also calculated in advance based on the early TA acquisition. The difference between LTM and CLTM is the timing of TA value reception at UE, but this does not affect TA validity. For network triggered LTM, the issue can be solved by network implementation, e.g., when the LTM cell switch has been successfully completed, the target cell can send the TAC MAC CE to the UE to update the TA value of PTAG as soon as possible. And the method applies to CLTM as well. Hence, option 1 can be confirmed.

Proposal 4: (Option 1) Upon CLTM cell switch execution to the target cell, the UE shall start the timeAlignmentTimer associated with PTAG as in Rel-18.

If P4 is supported, after CLTM cell switch execution, the CLTM TA of the target cell stored in UE and maintained via CLTM TAT is useless, and the UE can release the TA of the target cell and stop associated CLTM TAT.

Proposal 5: The CLTM TA of the target cell is released at UE side upon CLTM execution.

Issue 3: whether the received CLTM TAs of other candidate cells is released or still valid at UE side upon CLTM execution.

Issue 2 has clarified how to handle the TA and TAT of the CLTM target cell. For issue 3, we can discuss how to handle the TA and TAT of other CLTM candidate cells.

After CLTM cell switch execution, the current serving cell has been changed, and the probability of the UE triggering the subsequent CLTM within a short time is very low. Therefore, the CLTM TAs of other candidate cells received in the source cell are of little practical use. And maintaining these CLTM TAs could introduce the unnecessary complexity in UE and network side: the UE must continue storing these TA values and maintaining associated CLTM TATs, and the target cell needs to know which candidate cell TAs stored in UE and their validity duration. To sum up, the received CLTM TAs of other candidate cells shall be released at UE side upon CLTM execution.

Proposal 6: The received CLTM TAs of other candidate cells is released at UE side upon CLTM execution.

Issue 4: if the candidate TAT expires before CLTM trigger, how to handle the candidate TA.

Because the candidate TA is maintained in both UE and NW side, when the candidate TAT expires before CLTM trigger, the network can know it based on the network implementation. If needed, the network can trigger TA refresh by sending another PDCCH order to UE. (It is up to network implementation.)

Proposal 7: For PDCCH-ordered early TA acquisition, if the candidate TAT expires before CLTM trigger, it is up to network implementation to trigger TA refresh by sending another PDCCH order.

  - Target cell and beam selection for L3 based CLTM execution

In the network triggered LTM, the network sends cell switch command MAC CE to trigger cell switch. The cell switch command indicates the target cell (via target configuration ID) and the target beam (via the TCI state). However, for conditional LTM, there is no LTM cell switch command MAC CE to indicate the target cell and target beam for the CLTM execution and the UE triggers conditional LTM cell switch autonomously when the CLTM condition is fulfilled.

And how to select the target cell and beam to perform CLTM need to be discussed. In RAN2#129 meeting, RAN2 has reached the following agreements on beam selection for L1 based CLTM:

3. For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell). 4. When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM. 5. For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition. 6. For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

Target cell selection for L3 based CLTM

Similar to L1 based CLTM, the target LTM configuration ID (the target cell) of L3 based CLTM is determined by UE based on the condition evaluation and the UE selects the candidate cell which fulfils the execution condition to perform CLTM. For L3 based CLTM events, the CLTM execution is triggered when at least one candidate cell fulfills the CLTM condition.

In some cases, more than one LTM candidate cell fulfills the LTM cell switch execution, how to select the target cell for CLTM execution can be discussed. In the legacy L3 CHO, if multiple NR cells are triggered in conditional reconfiguration execution, it is up to UE implementation which one to select. For L3 based CLTM, we can follow it.

Proposal 8: For L3 based CLTM, the CLTM execution is triggered when at least one candidate cell fulfills the CLTM condition. When multiple candidate cells satisfy the L3 CLTM condition, it is up to UE implementation to select a candidate cell and perform CLTM.

Target beam selection for L3 based CLTM

To support CG based RACH-less CLTM, the UE shall determine the target beam for RACH-less CLTM cell switch from beams of the selected candidate cell which fulfills the L3 CLTM condition.

In L3 RACH-less HO, a RSRP threshold (cg-RRC-RSRP-ThresholdSSB) can be configured by the NW for beam selection.

In L1 based CLTM, the target beam selection for CG based RACH-less LTM is up to UE implementation.

Both of the above methods can be considered for L3 based CLTM, RAN2 can discuss which option can be considered for L3 based CLTM.

Proposal 9: RAN2 to discuss which option can be considered on the target beam selection for L3 based RACH-less CLTM:

- Option 1: It is up to UE implementation.

- Option 2: Based on the associated beam quality, the UE to select the target beam. A RSRP threshold can be configured by the NW for the beam selection.

Conclusions

According to the analysis given above, we have the following observations and proposals:

2.1 The coexistence of CLTM and other mobility cases

Proposal 1a: RAN2 confirms the coexistence of LTM and CLTM is supported. Even if the CLTM condition is configured for the LTM candidate, the network can also trigger LTM cell switch to the LTM candidate via LTM cell switch Command MAC CE.

Proposal 1b: The coexistence of CLTM and L3 HO is supported in Rel-19.

Observation 1: For the coexistence of LTM/CLTM and CHO, the agreement in RAN2 #127 (Rel-19 MOB) and the agreement in RAN2#128(Rel-18 MOB) are in conflict:

- The agreement in RAN2 #127 (Rel-19 MOB): The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

- The agreement in RAN2#128(Rel-18 MOB): Clarify in RRC that the conditional reconfiguration cannot include the ltm-config.

Proposal 1c: The coexistence of LTM and CHO is not supported in Rel-19 (which revises the associated agreement in RAN2 #127). And the coexistence of CLTM and CHO is not supported in Rel-19.

Proposal 1d: The coexistence of CLTM and DAPS HO is not supported.

2.2	CLTM TA

Proposal 2: To define per-band capability(s) for the TA timer maintenance of candidate cells, and the new capability(s) indicates the maximum number of candidate TA timers that maintained by the UE at the same time. The value of the UE capability(s) is 1 or 2.

Proposal 3：Reuse the rach-EarlyTA-Measurement-r18 to indicates the maximum number of candidate cells for TA acquisition based on PDCCH ordered CFRA procedure for both LTM and CLTM.

Proposal 4: (Option 1) Upon CLTM cell switch execution to the target cell, the UE shall start the timeAlignmentTimer associated with PTAG as in Rel-18.

Proposal 5: The CLTM TA of the target cell is released at UE side upon CLTM execution.

Proposal 6: The received CLTM TAs of other candidate cells is released at UE side upon CLTM execution.

Proposal 7: For PDCCH-ordered early TA acquisition, if the candidate TAT expires before CLTM trigger, it is up to network implementation to trigger TA refresh by sending another PDCCH order.

2.3	Target cell and beam selection for L3 based CLTM execution

Proposal 8: For L3 based CLTM, the CLTM execution is triggered when at least one candidate cell fulfills the CLTM condition. When multiple candidate cells satisfy the L3 CLTM condition, it is up to UE implementation to select a candidate cell and perform CLTM.

Proposal 9: RAN2 to discuss which option can be considered on the target beam selection for L3 based RACH-less CLTM:

- Option 1: It is up to UE implementation.

- Option 2: Based on the associated beam quality, the UE to select the target beam. A RSRP threshold can be configured by the NW for the beam selection.

Reference

[1] RP-250339, “Revised Work Item: NR mobility enhancements Phase 4”, Apple Inc, China Telecom

[2] R2-2500xxx, “Report of [POST129][111][MOB] (CATT)”

[3] R2-2501473, “Discussion report on [AT129][113][MOB] MAC open issues (vivo)”	Vivo