3GPP TSG RAN WG2#129bis	R2-2502070

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.9.4

Source:	OPPO

Title:	Discussion on PWS for NB-IoT

Document for:	Discussion, Agreement

Introduction

In last RAN2 meetings, RAN2 discussed the support of PWS for NB-IoT, and the following agreements were made so far.

Agreements in RAN2#127bis meeting:

1.	RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS

2.	The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes.

3.	For NB-IoT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/ skip some unnecessary fields):

- SystemInformationBlockType10-NB for primary ETWS notification;

- SystemInformationBlockType11-NB for secondary ETWS notification;

- SystemInformationBlockType12-NB for CMAS notification.

4.	Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB):

- etws-Indication;

- cmas-Indication.

5.	Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (can come back to clarify further what immediately means)

6.	Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases)

Agreements in RAN2#128 meeting:

1.	We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements.

2.	We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed

3.	Add the following PWS indication in Paging-NB:

- etws-Indication;

- cmas-Indication.

Agreements in RAN2#129 meeting:

- Introduce geographic information in ETWS messages for geo-fencing in IoT NTN cells

- Introduce the Warning Area Coordinates IE for ETWS to describe an emergency area in system information.

- Introduce the Warning Area Coordinates IE to existing ETWS SIBs, i.e., SIB10-NB/11-NB, without introducing the need for segmentation of SIB10-NB

- We don’t introduce a new SIB for carrying geographical information for emergency areas

In this contribution, we further discuss open issues on support of PWS for NB-IoT NTN and present our view.

Discussion

In RAN2#128 meeting, it has been agreed to extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT, but it is still FFS whether SIB1-NB acquisition is needed. In eMTC, the concept of BCCH modification period does not apply to ETWS/CMAS notification, i.e. ETWS/CMAS notification can occur at any time. The corresponding procedure for ETWS notification and CMAS notification are captured in the spec as follow.

5.2.1.4 Indication of ETWS notification ETWS primary notification and/ or ETWS secondary notification can occur at any point in time. The Paging message is used to inform ETWS capable UEs in RRC_IDLE and UEs not in CE in RRC_CONNECTED about presence of an ETWS primary notification and/ or ETWS secondary notification. For UEs in CE supporting reception of ETWS indication in RRC_CONNECTED mode, control channels associated with the shared data channel are used to inform the UE about the presence of an ETWS primary notification and/or ETWS secondary notification. If the UE receives a Paging message or control channels associated with the shared data channel including the etws-Indication, it shall start receiving the ETWS primary notification and/ or ETWS secondary notification according to schedulingInfoList contained in SystemInformationBlockType1. If the UE receives Paging message or control channels associated with the shared data channel including the etws-Indication while it is acquiring ETWS notification(s), the UE shall continue acquiring ETWS notification(s) based on the previously acquired schedulingInfoList until it re-acquires schedulingInfoList in SystemInformationBlockType1. NOTE: The UE is not required to periodically check schedulingInfoList contained in SystemInformationBlockType1, but Paging message including the etws-Indication triggers the UE to re-acquire schedulingInfoList contained in SystemInformationBlockType1 for scheduling changes for SystemInformationBlockType10 and SystemInformationBlockType11. The UE may or may not receive a Paging message including the etws-Indication and/or systemInfoModification when ETWS is no longer scheduled. ETWS primary notification is contained in SystemInformationBlockType10 and ETWS secondary notification is contained in SystemInformationBlockType11. Segmentation can be applied for the delivery of a secondary notification. The segmentation is fixed for transmission of a given secondary notification within a cell (i.e. the same segment size for a given segment with the same messageIdentifier, serialNumber and warningMessageSegmentNumber). An ETWS secondary notification corresponds to a single CB data IE as defined according to TS 23.041 [37]. 5.2.1.5 Indication of CMAS notification CMAS notification can occur at any point in time. The Paging message is used to inform CMAS capable UEs in RRC_IDLE and UEs not in CE in RRC_CONNECTED about presence of one or more CMAS notifications. For UEs in CE supporting reception of CMAS indication in RRC_CONNECTED mode, control channels associated with the shared data channel are used to inform the UE about the presence of one or more CMAS notifications. If the UE receives a Paging message including the cmas-Indication, it shall start receiving the CMAS notifications according to schedulingInfoList contained in SystemInformationBlockType1. If the UE receives Paging message or control channels associated with the shared data channel including the cmas-Indication while it is acquiring CMAS notification(s), the UE shall continue acquiring CMAS notification(s) based on the previously acquired schedulingInfoList until it re-acquires schedulingInfoList in SystemInformationBlockType1. NOTE: The UE is not required to periodically check schedulingInfoList contained in SystemInformationBlockType1, but Paging message including the cmas-Indication triggers the UE to re-acquire schedulingInfoList contained in SystemInformationBlockType1 for scheduling changes for SystemInformationBlockType12. The UE may or may not receive a Paging message including the cmas-Indication and/or systemInfoModification when SystemInformationBlockType12 is no longer scheduled. CMAS notification is contained in SystemInformationBlockType12. A CMAS notification corresponds to a single CB data IE as defined according to TS 23.041 [37]. A CMAS notification may optionally have associated warning area coordinates. Segmentation can be applied for the delivery of a CMAS notification and, if present, the associated warning area coordinates. The segmentation is fixed for transmission of a given CMAS notification and, if present, any associated warning area coordinates within a cell (i.e. the same segment size for a given segment with the same messageIdentifier, serialNumber and warningMessageSegmentNumber). E-UTRAN does not interleave transmissions of CMAS notifications, i.e. all segments of a given CMAS notification transmission are transmitted prior to those of another CMAS notification.

As the highlighted parts above, the scheduling change of SIB10/SIB11 or SIB12 may not trigger etws-Indication or cmas-Indication and/or systemInfoModification. So when UE receives etws-Indication or cmas-Indication, the UE shall firstly re-acquire SIB1 to check whether SIB10/SIB11 or SIB12 is present rather than acquiring SIB10/SIB11 or SIB12 directly. When it comes to ETWS notification and CMAS notification for NB-IoT, we think the logic is the same.

The existing ETWS/CMAS notification RRC procedures for eMTC is reused for NB-IoT

In the current TS 36.306, the following two capabilities for support of the ETWS and CMAS are defined as follows:

- 6.2.1 ETWS It is optional for UE to support ETWS reception as specified in TS 36.331 [5]. - 6.2. CMAS It is optional for UE to support CMAS reception as specified in TS 36.331 [5]. It is optional for a CMAS-capable UE to support Geofencing information (warningAreaCoordinates-r15) as specified in TS 36.331 [5].

In R19, the same capabilities should be introduced for NB-IoT UE.

Introduce capabilities of support of ETWS and CMS services for the NB-IoT UE.

- Conclusion

Based on the discussion we propose the following:

- The existing ETWS/CMAS notification RRC procedures for eMTC is reused for NB-IoT

Introduce capabilities of support of ETWS and CMS services for the NB-IoT UE.