3GPP TSG-RAN WG2 Meeting #129bis	R2-2502094

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	 8.6.3

Source:	 Huawei, HiSilicon

Title:	 Discussion on L1 event-triggered measurement reporting

Document for:  Discussion and Decision

1	Introduction

LTM (L1/L2 Triggered Mobility) was introduced in Rel-18. The network receives L1 measurement report to determine the LTM target cell and target beam. Three types of report are defined, namely, periodic, aperiodic and semi-persistent L1 report.

For R19 mobility enhancements, one of the goals outlined in the WID is to define the required improvements for enabling event-triggered L1 measurements.

RP-242356, Revised Work Item: NR mobility enhancements Phase 4 - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1]

In this contribution, we provide a comprehensive analysis of event-triggered L1 measurement reports, covering the event evaluation and reporting procedures for the key aspects.

2	Discussion

2.1	Event evaluation

2.1.1	LTM Candidate cell

RAN2#127-bis meeting Agreements on L1 event triggered MR 8. For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed.

According to the agreement in RAN2#127-bis, the structure of the Rel-18 LTM-CSI-ReportConfig should be reused as much as possible. In this legacy structure, there is a parameter called spCellInclusion, which can only be configured in some specific scenarios. For example, during a subsequent LTM cell switch, if the serving cell is configured as one of the candidate cells and spCellInclusion is set to true, the measurement results for the serving cell should be included in the L1 measurement report. Thus, the configuration of the serving cell as one of the SpCells in the LTM candidate configuration can also be applied in the Rel-19 LTM-CSI-ReportConfig.

spCellInclusion This field indicates whether the UE shall include a L1 measurement report associated to the current SpCell. This field can only be configured if the current SpCell is configured as an SpCell of an LTM candidate configuration and the LTM-CSI-ResourceConfig IE associated to the LTM-CSI-ReportConfig IE includes resources for the current SpCell.

Proposal 1: Follow the R18 configuration to include the serving cell configured as an SpCell in the LTM candidate configuration for subsequent LTM operations.

Additionally, if the serving cell is configured as an SpCell in the LTM candidate configuration, the LTM-CSI-SSB-ResourceSet will include SSBs/CSI-RSs from both the serving cell and the other candidate cells. Since all the candidate beams, including those from the serving cell and candidate cells, are combined into one LTM resource set, the UE needs to perform event evaluation for all these beams in the LTM-CSI-SSB-ResourceSet. Especially for EventLTM3, EventLTM4, and EventLTM5, the UE may compare the current beam with the beams from the serving cell. Furthermore, even if the serving cell’s beams satisfy the event conditions, they could also trigger the event reporting procedure for those LTM events. Apparently, these scenarios should be avoided. Therefore, the beams of the serving cell should not undergo the event evaluation procedure. This action can be easily achieved with the current LTM resource set structure, as the UE can distinguish the serving cell's beams in the RRC configuration using the ltm-CandidateIdList.

Proposal 2: If the serving cell is configured as an LTM candidate cell in the LTM candidate configuration, for this candidate cell, there is no need to perform event evaluation for EventLTM3, EventLTM4, and EventLTM5.

2.2	Event reporting procedures

2.2.1	Report contents

2.2.1.1	Entering/Leaving event condition

RAN2#129 meeting Agreements on L1 event-triggered MR: 10. Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling. 12. R18 differential RSRP encoding is used as baseline.

Based on the agreements made in the previous meeting, it is supported that the MR MAC CE explicitly indicate which beam(s) meet the event condition with TTT. There are two potential options to achieve this.

The first option is to use the 'satisfied quantity' to represent the number of beams that meet the event condition. If the 'satisfied quantity' is K, it means that the first K beams meet the event condition. However, these first K beams may not be the ones with the highest RSRP, which could conflict with the agreement that the MR MAC CE should be sorted by differential RSRP values.

The second option is to use a 1 bit to indicate whether the reported SSBRI/CRI satisfies the event condition. This option is more compatible with differential RSRP reporting. Therefore, it would be more appropriate to use a 1 bit to indicate whether the reported SSBRI/CRI satisfies the event condition in the MR MAC CE.

Proposal 3: Use 1 bit to indicate whether the reported SSBRI/CRI satisfies the event condition in the MR MAC CE.

RAN2#129 meeting Agreements on L1 event-triggered MR: 11. Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam.

According to the previous agreements, it has been supported that the leaving event condition should be included in the MR MAC CE. Explicitly indicating the leaving condition will help the network identify the beam/cell that satisfies the leaving condition, without having to track historical measurement reports from the UE to deduce which one meets the leaving condition. This will reduce network overhead.

Observation 1: The indication of the leaving event condition in the MR MAC CE is intended to reduce network overhead.

Besides, it should be decided whether the indication of the leaving condition should be per beam or per cell. For L1 event-triggered measurement reporting, both the event evaluation procedure and the beam reporting procedure are based on beam-level analysis (per beam). Therefore, it is clear that the indication of the leaving condition should be per beam.

Proposal 4: The indication of the leaving event condition should be per beam in the MR MAC CE.

In addition, RAN2 should discuss how to indicate the beams that satisfying the leaving event condition. Similar to the inclusion of beams that satisfy the entering event condition, there is a need to differentiate the beams that satisfy the leaving event condition. One approach is to use a single bit to indicate whether the beam(s) satisfy the event condition, which is the simplest method and would require an additional bit in each SSBRI/CRI.

Proposal 5: Use a 1 bit to indicate whether the reported SSBRI/CRI satisfies the leaving event condition.

MeasResultNR field descriptions

cellsMetReportOnLeaveList This field indicates the list of cells which met the event leaving condition.

According to the legacy Rel-18 mechanism of the cellsMetReportOnLeaveList, only the cell IDs of the cells that meet the leaving condition are reported, without including their RSRP values. Since the beams that meet the leaving event condition typically have a low RSRP value, so the exact RSRP value of these beams is not important to the network. The network primarily cares about the beams with higher RSRP, which usually exceed the configured threshold for the event condition. Therefore, it is sufficient to report only the SSBRI/CRI of the beams that meet the leaving event condition, without including their RSRP values.

Proposal 6: Only the SSBRI/CRI of the beam(s) satisfying the leaving condition should be included in the MR MAC CE, and there is no need to report their RSRP values.

The indication of whether to include the cell when the leaving condition is satisfied was first introduce in TEI18 discussion as an optional feature. If the enteringLeavingReport is configured for the corresponding report, the list of cells that meet the leaving condition should be included in the measurement report.

In order to follow the legacy mechanism, we believe this feature should also be configurable for event-triggered LTM, as discussed. Therefore, we propose that...

Proposal 7: It should be configurable that whether to report the beams that satisfy the leaving event condition via RRC parameter.

The number N represents the maximum number of beams included in the measurement report. As discussed earlier, beams that satisfy the leaving event condition should not report their RSRP values, and therefore, they should not be included in the count of N. If these beams are counted towards N, and the network can configure whether to report beams that satisfy the leaving event condition, the number of such beams will affect the total count of N beams. The situation becomes more complex if the number of beams satisfying the leaving event condition exceeds N. Therefore, such beams should not be included in the total count of N in the measurement report.

Proposal 8: The beams reported for leaving condition should not be counted towards the total number of N in the measurement report.

2.2.1.2	Serving cell beams

RAN2#129 meeting Agreements on MAC issues: 11. The max number of N (included in the MR MAC CE) is the total number of all candidate beams without counting current serving beam.

It has been agreed that the current serving beam can also be reported in the MR MAC CE. In this meeting, we should discuss how to report the current serving beam with detailed signaling. From our perspective, there is no need to report the beam ID of the current serving beam, as the network already knows what is the current TCI state of the UE. In order to save the bits used in the MR MAC CE, it is sufficient to include only the RSRP value of the current serving beam in the measurement report. Since the RSRP value requires only 7 bits, it is possible to use just one octet for reporting the RSRP value of the current serving beam, with one reserved bit.

Proposal 9: The current beam should be included in the MR MAC CE along with its RSRP, without beam ID.

In legacy R18 LTM, the serving cell can be added into a candidate configuration for subsequent cell switch. In such cases, the network can configure the UE to measure the beams of the serving cell and include them in the event-triggered L1 measurement report. This approach aligns with the existing R18 LTM configuration, thus should be reused. The maximum number of beams that can be reported per cell is 4, hence, up to four beams from the serving cell should be included in the report.

Proposal 10: When current serving cell is configured as one of the candidate cells, multiple serving beams can be reported in the measurement report MAC CE.

2.2.1.3	Others

RAN1#120 meeting Conclusion There is no consensus in RAN1 to support specified filtering of L1-RSRP for LTM.

In the RAN1 discussion, there is no need to introduce specified filtering for L1-RSRP in the LTM CSI measurement and reporting procedure. The responsibility for filtering the L1-RSRP measurement results should be left to UE implementation, consistent with the approach used in Rel-15 CSI reporting.

Observation 3: The filtering of L1-RSRP for LTM measurement should be determined by UE implementation.

There is also a discussion regarding which measurement results should be transmitted. One suggestion is that the UE should report the average value of the measured results during the TTT period. However, this is not feasible, as it would require additional workload for the UE and result in significant MAC specification impacts. A second approach is that the UE should report only the latest measured result from the event evaluation period, which seems more acceptable. However, this issue should be decided by RAN1, as it is out of RAN2 scope. We can raise this issue and send a LS to RAN1 for resolution.

Proposal 11: The decision on which measurement results during the event evaluation period (TTT) should be transmitted should be made by RAN1.

In addition, if the event condition is satisfied for the entire duration of the TTT, the measurement reporting procedure should begin, and the related event enters the triggering status. We need to discuss when the triggering status should be cancelled in measurement reporting procedure. If the dedicated SR of the MAC CE has been transmitted and there is no UL grant to include the MR MAC CE, the triggering status should not be cancelled, as the entire MR has not been sent. Once the MR has been transmitted, the triggering status should be cancelled to prevent the unnecessary UL grant from being requested again when sending the SR.

Proposal 12: The triggering status should be cancelled once the MR MAC CE has been transmitted.

2.2.2	Truncated MAC CE

RAN2#128 meeting Agreements on L1 event-driven MR 10. Support the truncated measurement report MAC CE.

According to the agreements supported in the previous meeting, if the UL grant is insufficient to transmit the entire MR, a truncated measurement report MAC CE can be used to carry a portion of the MR. We should also consider the status of the triggering event once the truncated MAC CE has been transmitted.

For L1 event-triggered MR, if the LTM event is satisfied, the corresponding event enters a triggering state, waiting for a UL grant to transmit the measurement report. When the UL grant is limited and an event-triggered truncated MAC CE is sent, the event's triggering state should not be cancelled. This ensures that the remaining part of the MR can be transmitted later. The triggering state should only be cancelled once the complete MR has been successfully transmitted. If a portion of the MR MAC CE has been transmitted, the triggering state for the remaining events should remain active, so the SR can be sent to request a UL grant and ensure the following transmission of the remaining parts at a later time.

Proposal 13: If only part of the MR for the corresponding ReportConfigId is transmitted in the truncated MAC CE, the triggering status of the corresponding event should not be canceled. The triggering state should only be canceled once the complete MR for the corresponding ReportConfigId has been transmitted.

Furthermore, if the UE receives the LTM cell switch command immediately after transmitting the truncated MAC CE, where the non-truncated MAC CE has not been sent, the UE will perform a MAC reset upon receiving the LTM cell switch command. Consequently, the triggering status should be cancelled at this point due to the MAC reset.

Proposal 14: The triggering status of the truncated MAC CE should also be cancelled when the MAC resets.

As the truncated MAC CE is transmitted before the non-truncated MAC CE, it is evident that more critical beam information should be included in the truncated MAC CE to ensure that the network receives urgent measurement results. If the current serving beam quality is very low, the UE may need to quickly switch to a new cell. In this case, the network may make the cell switch decision on the truncated MAC CE measurement results, rather than waiting for the non-truncated MAC CE. Therefore, RAN2 should discuss the sorting quantity for the different kinds of reported beams.

Among the all reported beams in the MR MAC CE, other beams in the measurement report are less important to the network compared to beams that meet the event condition or the current serving beam. The network is primarily concerned with whether the current serving beam quality is sufficiently poor to justify a cell switch decision, and then considered about whether the candidate beams can provide suitable alternatives for the UE to switch to. The detailed sorting quantity of these beams should be discussed further. However, RAN2 should at least agree that the current serving beam has a higher priority than other beams in the MR.

Proposal 15: In the truncated MAC CE, current beam has higher priority than the other beams.

2.3	Semi-persistent CSI-RS measurement

RAN1#120 meeting Agreement - Confirm the following working assumption made in RAN1#118bis Working Assumption(RAN1#118bis) In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 Send an LS to RAN2 to inform this agreement. Final LS in R1-2501500.

In the last RAN1 meeting, it was agreed that semi-persistent CSI-RS should be supported for candidate cell L1-RSRP measurements. To activate or deactivate the semi-persistent CSI-RS resource, a MAC CE should be utilized for transmission. The design of the MAC CE format can follow the legacy mechanism used for serving cell semi-persistent CSI-RS measurements.

Observation 4: A MAC CE is required to activate or deactivate the semi-persistent CSI-RS resource for a candidate cell, which will be specified in the RAN2 specification.

RAN2 should first discuss the contents of this MAC CE. Based on the serving cell mechanism outlined below, the MAC CE for the serving cell consists of the serving cell ID, BWP ID, semi-persistent CSI-RS resource set ID, and TCI state ID. However, the BWP ID is not applicable in the similar mechanism for the candidate cell. Therefore, for the MAC CE format specific to the candidate cell mechanism, it should at least include the candidate configuration ID, semi-persistent CSI-RS resource set ID, and corresponding TCI state IDs.

Proposal 16: Introduce a new MAC CE to activate or deactivate the semi-persistent CSI-RS resource set for candidate cells, which should at least include the candidate configuration ID, SP CSI-RS resource set ID, and corresponding TCI state IDs.

# 3	Conclusion

In this paper, we have addressed the objective regarding improvements in event-triggering reports for LTM. There are the following proposals:

Event evaluation

Proposal 1: Follow the R18 configuration to include the serving cell configured as an SpCell in the LTM candidate configuration for subsequent LTM operations.

Proposal 2: If the serving cell is configured as an LTM candidate cell in the LTM candidate configuration, for this candidate cell, there is no need to perform event evaluation for EventLTM3, EventLTM4, and EventLTM5.

Report contents

Proposal 3: Use 1 bit to indicate whether the reported SSBRI/CRI satisfies the event condition in the MR MAC CE.

Proposal 4: The indication of the leaving event condition should be per beam in the MR MAC CE.

Proposal 5: Use a 1 bit to indicate whether the reported SSBRI/CRI satisfies the leaving event condition.

Proposal 6: Only the SSBRI/CRI of the beams satisfying the leaving condition should be included in the MR MAC CE, and there is no need to report their RSRP values.

Proposal 7: It should be configurable that whether to report the beams that satisfy the leaving event condition via RRC parameter.

Proposal 8: The beams reported for leaving condition should not be counted towards the total number of N in the measurement report.

Proposal 9: The current beam should be included in the MR MAC CE along with its RSRP, without beam ID.

Proposal 10: When current serving cell is configured as one of the candidate cells, multiple serving beams can be reported in the measurement report MAC CE.

Proposal 11: The decision on which measurement results during the event evaluation period (TTT) should be transmitted should be made by RAN1.

Proposal 12: The triggering status should be cancelled once the MR MAC CE has been transmitted.

Truncated MAC CE

Proposal 13: If only part of the MR for the corresponding ReportConfigId is transmitted in the truncated MAC CE, the triggering status of the corresponding event should not be canceled. The triggering state should only be canceled once the complete MR for the corresponding ReportConfigId has been transmitted.

Proposal 14: The triggering status of the truncated MAC CE should also be cancelled when the MAC resets.

Proposal 15: In the truncated MAC CE, current beam has higher priority than the other beams.

Semi-persistent CSI-RS measurement

Proposal 16: Introduce a new MAC CE to activate or deactivate the semi-persistent CSI-RS resource set for candidate cells, which should at least include the candidate configuration ID, SP CSI-RS resource set ID, and corresponding TCI state IDs.

# 4	Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4;

- RAN2 #129 chairman notes;

- RAN1 #120 chairman notes;

- TS 3GPP 38.331 V18.4.0

- R2-2500805 Draft RRC running CR