3GPP TSG-RAN WG2 Meeting #R2-129bis		R2-2502119 Wuhan, China, Apr 07-11, 2025

Agenda Item:	8.6.4

Source: 	Rakuten

Title:  	Remaining issues of Conditional LTM

WID/SID:           NR_Mob_Ph4-Core- Release 19

Document for:	Discussion, Decision

# Introduction

In RAN #105, the Rel-19 WID on NR mobility enhancements Phase 4 was updated [1], where the objectives relevant to conditional LTM (CLTM) are described below:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In RAN2#127bis, the following progress was made:

Agreements on C-LTM

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

In RAN2#128, the following progress was made:

Agreements on C-LTM:

- The triggering condition of conditional LTM can be based on L3 measurement.

- CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

- The L1 execution condition of a candidate cell is associated to only one triggering event.

- For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

- To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

- For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

The primary motivation for CLTM is to introduce robustness to the LTM procedure where the UE may not be able to receive the LTM cell-switch command from the serving gNB-DU, which may lead to radio link failure. The conditional LTM is expected to bring the benefits of robustness while retaining the interruption time reduction benefits of LTM.

In this contribution, we discuss CLTM and the related key issues.

# Discussion

## Timing Advance maintenance during CLTM

The below agreement was made in RAN2#127bis.

8.    The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

In CLTM, the TA is received by the UE in advance to the CLTM cell switch and RAN2 agreed that the UE will maintain the TA of the candidate cells.

Since the UE maintains the TA of each CLTM candidate cell, its sensible that the UE indicates to the gNB about an expiring TA. If the candidate cell still qualifies the early TA acquisition criteria, the gNB may issue a PDCCH order.

Once the RACH-less CLTM is completed and signalled to the target gNB-DU, the UE should be able to inform all the valid and active TAs of different candidate cells to the new serving gNB. This will prevent redundant RACH-based TA acquisition at the new serving gNB-DU.

If the TAT of a CLTM candidate cell expires, the UE may request a PDCCH order from the gNB-DU to re-acquire the TA. This request could be made by sending an L1 measurement report with an indication or an UL MAC CE, subject to the candidate cell satisfying the TA acquisition requirement.

After indicating a successful RACH-less CLTM cell switch, the UE informs the target gNB-DU of all the valid and active CLTM candidate cell TAs.

## UL and DL sync for L3 measurements based CLTM

In RAN2#128, it was agreed that CLTM is supported based on L3 measurements also.

In RAN2#127bis, it was agreed that early UL and DL sync is supported for intra gNB-CU CLTM.

In L3 measurements based CLTM, the gNB-CU is responsible for triggering early UL and DL sync. This could be performed by configuring beam-level L3 measurements.

In addition, this could be executed using a gNB-CU centric approach where the gNB-CU triggers the gNB-DU to initiate the UL/DL sync procedures at the appropriate time by providing the CLTM candidate cell ID and beam ID. The candidate cell ID is required for the UE to perform RACH-based TA acquisition, and the beam ID is required to activate the TCI states in advance.

Alternatively, this could be performed using a gNB-DU centric approach where the gNB-CU just forwards the received measurements, and the gNB-DU executes the UL and DL sync as in L1 measurements based CLTM.

In the gNB-CU centric approach, RAN2 may have to let RAN3 know of the required parameters to be shared from gNB-CU to the gNB-DU.

RAN2 discusses and agrees all the parameters to be sent from gNB-CU to gNB-DU to initiate early UL and DL sync for L3 measurements based CLTM.

## RACH-based TA acquisition during CLTM

One of the important advantages of LTM in R18 is the reduction of service interruption during HO, due to RACH-less cell switch. It was agreed in one of the previous meetings to support RACH-less CLTM, and at least PDCCH ordered early TA acquisition.

A UE configured with CLTM performs a cell switch without receiving a cell switch command, when the CLTM execution condition of a candidate cell is satisfied. Therefore, the existing mechanism of TA information transfer to the UE in the MAC CE cell switch command is not feasible in CLTM and an alternate mechanism is required for delivering TA for RACH-less CLTM cell switches. TA delivery using random-access response (RAR) to the UE was was ruled out for non-conditional intra- and inter-CU LTM and left as FFS for conditional LTM.

In the case of PDCCH ordered RACH based TA acquisition, one solution is that the TA could be provided by the source gNB-DU in a MAC CE, similar to R18.

While this is some improvement compared to explicitly triggered LTM cell switch, there is still some dependency on the source gNB-DU to assist the cell switch i.e an event triggered UL MAC CE to report L1 measurements, and a DL MAC CE to provide TA, before the CLTM.

To overcome this signaling and latency with the source gNB-DU, a conditional early UL sync could be employed, where the TA value can be obtained based on a new execution condition (ahead of cell switching), therefore reducing the amount of time needed during cell switch itself. It also ensures that the RACH for TA acquisition is performed without the event triggered L1 measurement report to the source gNB-DU in UL and PDCCH order in DL. The early UL sync and the cell switch could therefore be performed completely based on execution conditions being satisfied at the UE.

For this conditional early UL sync, a dedicated RACH preamble may have to be allocated at the time of CLTM target cell preparation, which enables the target gNB-DU to identify the UE’s objective of performing RACH.

Together with this, the conditional trigger for performing RACH would need to be set such that the condition is met earlier than the condition for performing CLTM cell switch.

A conditional uplink sync could be perfectly complimented by delivering the TA directly to the UE using RAR.

RAR-based TA acquisition directly from target gNB-DU is supported for conditional LTM.

Direct RAR based delivery of TA was also discussed in Rel 18 LTM and the main drawback identified was that, depending on UE capability and cell frequency, it may lead to increased interruption time at the source cell, since the UE must monitor the PDCCH of the candidate cell and may not be able to receive data from the source while monitoring for RAR.

In addition, delivery of target gNB-DU RAR via the source gNB-DU is fully compatible with the Rel-18 early UL synchronization procedure and very minimal changes are required to support the inclusion of RAR over F1.

Therefore, to address the requirements of UEs which don’t have the capability of monitor PDCCH of the target cell without its serving cell data transmission getting impacted, we propose delivery of RAR via the serving gNB-DU. This could be delivered by having RAR as a payload in a MAC CE.

Delivery of target gNB-DU RAR to the UE via the serving gNB-DU is supported.

## Conditional TA acquisition during CLTM

In one of the previous meetings, it was agreed to support explicit triggering of early UL sync (using PDCCH order for TA acquisition) and early DL sync (Rel-18 Early candidate TCI State activation/deactivation).

As explained above, to reduce the signaling and latency overhead with the source gNB-DU, a conditional early UL sync could be configured at the UE, where the RACH based TA acquisition could be performed based on a new execution condition.

The CLTM execution condition could be a trigger to execute LTM cell switch or it could be a trigger for performing early DL/UL sync (e.g. activate TCI state on candidate cell, perform a RACH procedure for TA acquisition).

In summary, it should be possible to configure an execution condition (e.g. beam-based, cell-based threshold) to perform an action (cell switch, UL sync, DL sync).

In L3 measurements based CLTM, the decision to trigger early DL and UL sync are triggered based on L3 measurements and this involves additional signaling from gNB-CU to gNB-DU, as the early UL/DL sync is initiated by the gNB-DU. Conditional execution based early UL/DL sync avoids this overhead.

Conditional execution i.e triggered based on an execution condition, of the following is supported for both L1 measurements based CLTM and L3 measurements based CLTM

- CLTM cell switch

- Early DL sync (TCI state activation)

- Early UL sync (RACH based for TA acquisition).

## TAT for CLTM candidate cells post cell switch

 CLTM TAT for target cell is not stopped upon CLTM cell switch execution to the target cell. Rapporteur suggests to discuss how/whether to start the timeAlignmentTimer associated with PTAG upon CLTM cell switch execution to the target cell during CB session: - Option 1 (revert the offline agreement): start the timeAlignmentTimer associated with PTAG as in Rel-18; - Option 2 (keep the offline agreement): discuss the options below in the MAC running CR o Option 2.1: clarify the running TAT to act as the timeAlignmentTimer associated with PTAG by a note in the MAC running CR; o Option 2.2: define that UE can perform UL tx when the TAT for the target cell is running. Besides, we need to add a note that, if SCell and SpCell are in the different TAG, SCell may not work when this TAT for target cell is running. o Option 2.3: the UE stops the CLTM timer and starts the PTAG using the remaining time of the CLTM timer. - Will be revisited next meeting.

- The above issue was discussed in RAN2#129 and was decided to be revisited in the next meeting.

- Here, we must consider two aspects.

- In Rel 18 LTM, once the LTM cell switch command is sent to the UE, the source gNB-DU sends the list of LTM candidate cells and their respective TAT (if active) to the target gNB-DU. In Rel 19 inter-CU LTM, the same mechanism would work. Hence, during C-LTM, the source gNB-DU could keep track of the TAT (despite the UE maintaining the TAT) and forward the TAT of candidate cells that are active, to the target gNB-DU. This ensures that the target gNB-DU doesn’t have to unnecessarily start TA acquisition when the TAT is active.

- The UE may be under mobility even within its own serving cell. Hence, the argument of TA suddenly becoming invalid because of the serving cell switch is skewed. The TA comes with a validity timer and it should be valid, regardless of the CLTM cell switch

CLTM TAT for target cell is not stopped upon CLTM cell switch execution to the target cell.

CLTM TAT for other candidate cells are not stopped upon CLTM cell switch execution to a given target cell.

## CLTM cell switch notification to the gNB-CU

In Rel 18 LTM, the gNB-DU notifies the gNB-CU after the LTM cell switch command is delivered to the UE. This ensures that the gNB-CU is always aware of the UE’s current serving cell.

In CLTM, the gNB-DU doesn’t deliver a cell switch command, instead the CLTM cell switch is based on CLTM execution condition. Hence, we there is a need to address this issue using an alternative mechanism.

The UE may inform the source gNB-DU of the selected CLTM target cell right when the CLTM execution condition is satisfied, but this may not be reliable as there could be a too-late HO and the message may not successfully reach the source gNB-DU.

Hence, we propose that the UE informs the target gNB-DU of the target cell configuration index sent over RRC, after a successful CLTM cell switch. The target gNB-DU can send the same to the gNB-CU, using which the gNB-CU can identify the UE’s current serving cell.

UE informs the target gNB-DU of the target cell configuration index sent over RRC, after a successful CLTM cell switch.

## CLTM for a multiTRP UE

When a UE is configured with mTRP and with C-LTM, we have the following.

- There are at least two TRPs configured for the UE.

- The TRPs could belong to different cells, which could belong to same or different gNB-DU, but the same gNB.

- The UE can be configured with one or more C-LTM candidate cells coupled with a CLTM execution condition.

- The C-LTM candidate cells and the execution condition are TRP specific i.e candidate cells and execution condition of one TRP aren’t applicable to the other.

So, there is a possibility where only one of the TRPs satisfy the CLTM execution condition, while the other does not. There are the following feasible alternatives when only one of the TRPs satisfy the CLTM execution condition.

- The UE performs RACH-less C-LTM cell switch for only that TRP. The other TRPs are retained with their serving cell, until either of the following occurs

  - TRP execution condition(s) is satisfied, and C-LTM cell switch can be executed.

  - UE detects RLF for the TRP and the TRP is autonomously discarded by the UE. No RRC re-establishment is performed after RLF detection as there is another TRP active. TRP recovery can be attempted, if configured and feasible.

- One of the TRPs is designated as the master TRP by the gNB and the UE performs C-LTM cell switch only if the execution condition(s) of the master TRP is satisfied.

- C-LTM cell switch is performed only when both TRPs have the execution condition(s) are satisfied.

- C-LTM candidates are never prepared with mTRP (only prepared with a single TRP). If C-LTM execution condition is satisfied for one TRP at the source, the UE performs C-LTM cell switch. The network may configure mTRP again after a successful C-LTM cell switch.

RAN2 discusses and agrees a way forward CLTM handling of a mTRP UE.

# Conclusion

We have the following proposals:

Proposal 1	If the TAT of a CLTM candidate cell expires, the UE may request a PDCCH order from the gNB-DU to re-acquire the TA. This request could be made by sending an L1 measurement report with an indication or an UL MAC CE, subject to the candidate cell satisfying the TA acquisition requirement.

Error! Reference source not found.	Error! Reference source not found.

Proposal 2	After indicating a successful RACH-less CLTM cell switch, the UE informs the target gNB-DU of all the valid and active CLTM candidate cell TAs.

Proposal 3	RAN2 discusses and agrees all the parameters to be sent from gNB-CU to gNB-DU to initiate early UL and DL sync for L3 measurements based CLTM.

Proposal 4	RAR-based TA acquisition directly from target gNB-DU is supported for conditional LTM.RAR-based TA acquisition directly from target gNB-DU is supported for conditional LTM.

Proposal 5	Delivery of target gNB-DU RAR to the UE via the serving gNB-DU is supported.

Proposal 6	Conditional execution i.e triggered based on an execution condition, of the following is supported for both L1 measurements based CLTM and L3 measurements based CLTM

Proposal 7	CLTM TAT for target cell is not stopped upon CLTM cell switch execution to the target cell.

Proposal 8	CLTM TAT for other candidate cells are not stopped upon CLTM cell switch execution to a given target cell.

Proposal 9	UE informs the target gNB-DU of the target cell configuration index sent over RRC, after a successful CLTM cell switch.

Proposal 10	RAN2 discusses and agrees a way forward CLTM handling of a mTRP UE.

# References

RP-242356,   “Revised Work Item: NR mobility enhancements Phase 4”

RAN2 Chairman Notes R2_127bis_ChairNotes

RAN2 Chairman Notes R2_128_ChairNotes