# 3GPP TSG RAN WG2 Meeting #129b      	                           R2-2502125
Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.1.2.3

Source:	Apple

Title:	Remaining issues on LCM procedure of AI/ML based positioning

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

The revised Rel-19 WID of AI/ML for air interface (WID RP-243244) was agreed in RAN#106 [1]. The WI objective on general LCM framework, including applicable functionalities, is led by RAN2 which is copied below:

The baseline of LCM procedure for AI positioning case 1 was agreed in RAN2#127b [2]. Then, some further agreements were made in RAN#128 [3]. We identify the following unaddressed FFSs:

- FFS 5: Order of inference config and applicability reporting

- FFS 6: Need of functionality activation

Meanwhile, RAN2#129 [4] made below agreements based on post-meeting email discussion summary [5]:

Agreements

1:	Introduce AI/ML positioning Case 1 as a new positioning method.

2:	Existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing and requesting the results of the UE sided model inference operation. The detail stage 3 message extention can be disucssed while drafting the stage 3 CR.

3:	FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed.  FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation,

4: 	The content of error cause is discussed while drafting stage3 CRs.

5: 	As a baseline, UE receives the needed assistance data for calculating UE location for AI/ML in step3 (ProvideAssistanceData) and UE receives the instruction to perform the inference in step 5 (RequestLocationInformation). The content of Assistance Data and the content of request location information is based upon RAN1 parameter list.

6: 	UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality.   FFS if any additional LMF control is needed.

As can been seen, the following FFSs need to be addressed:

- FFS 1: Whether support on-demand request for NW-side additional condition

- FFS 2: Whether support reporting UE-side additional condition

- FFS 3: Whether support autonomous fallback configuration

- FFS 4: Whether any additional LMF control is needed for applicability reporting

In this contribution, we discuss these remaining issues on the LCM procedure of AI/ML based positioning.

# 2 Discussion

According to latest RAN2 agreement, the baseline LCM procedure can be illustrated in Figure. 1. We highlight the FFSs which are discussed in this contribution.

Figure.1: Baseline LCM procedure of UE-sided model for AI/ML positioning Case 1

## 2.1 FFS 1: On-demand request for NW-side additional condition

This is one issue discussed in post-meeting email discussion [5], but no conclusion is made.

First, on-demand request for DL-PRS assistance data is already supported from Rel-17. Please note that RAN1#119 agreed all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE [6]:

Agreement

For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following:

- Alternative 1. Info #7 is provided implicitly via associated ID.

  - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference.

- Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly.

  - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference.

- Alternative 3.  Info #7 is not be provided from LMF to UE.

  - If info #7 is not provided, UE may assume info #7 is consistent between training and inference.

- Alternative 4. Info #7 is provided explicitly from LMF to UE.

| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |
| --- | --- |

Observation 1: On-demand request for DL-PRS assistance data is already supported from Rel-17.

Observation 2: RAN1 agreed all assistance data of legacy UE-based DL-TDOA (other than info 7) are reused.

Then, we think RAN2 can reuse the legacy on-demand request for DL-PRS assistance data. We don’t see need to support on-demand request for other NW-side additional conditions

Proposal 1: On NW-side additional condition, support the legacy on-demand request for DL-PRS assistance data. No need to support on-demand request for other NW-side additional condition.

## 2.2 FFS 2: Whether support reporting UE-side additional condition

This is one issue discussed in post-meeting email discussion [5], but no conclusion is made.

In RAN2#127 [7], it was agreed that the UE determines applicable functionalities based on NW-side additional condition, UE-side additional condition and model availability in AI/ML based beam management:

Agreements on procedures - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

We understand that this agreement means the applicability reporting already takes UE-side additional condition into consideration, and UE-side additional condition doesn’t need to be explicitly reported to the NW. The logic behind this agreement is because the UE-side additional condition includes UE’s internal information (e.g. battery and memory), which can’t be exposed to the NW.

Observation 3: In AI/ML based beam management, the applicability reporting already takes UE-side additional condition into consideration, and UE-side additional condition is not explicitly reported to the NW because it includes UE’s internal information (e.g. battery and memory), which can’t be exposed to the NW.

We think the same principle can be reused to AI/ML based positioning. Thus, we propose:

Proposal 2: Same as AI/ML based BM, the UE decides the applicable functionalities based on NW-side additional conditions, UE-side additional conditions and model availability in device. UE-side additional condition is not explicitly reported to the NW.

## 2.3 FFS 3: Whether support autonomous fallback configuration

This FFS was captured in agreement of RAN2#129 [3]:

3:	FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed.  FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation,

As background information, the existing LPP spec allows multiple positioning methods to be configured in RequestLocationInformation with a SEQUENCE structure (it is specified with terminology “Hybrid positioning”) [8]:

RequestLocationInformation-r9-IEs ::= SEQUENCE {

commonIEsRequestLocationInformation

CommonIEsRequestLocationInformation	OPTIONAL,	-- Need ON

a-gnss-RequestLocationInformation	A-GNSS-RequestLocationInformation	OPTIONAL,	-- Need ON

otdoa-RequestLocationInformation	OTDOA-RequestLocationInformation	OPTIONAL,	-- Need ON

ecid-RequestLocationInformation		ECID-RequestLocationInformation		OPTIONAL,	-- Need ON

epdu-RequestLocationInformation		EPDU-Sequence						OPTIONAL,	-- Need ON

...,

[[

sensor-RequestLocationInformation-r13

Sensor-RequestLocationInformation-r13

OPTIONAL,	-- Need ON

tbs-RequestLocationInformation-r13	TBS-RequestLocationInformation-r13	OPTIONAL,	-- Need ON

wlan-RequestLocationInformation-r13	WLAN-RequestLocationInformation-r13	OPTIONAL,	-- Need ON

bt-RequestLocationInformation-r13	BT-RequestLocationInformation-r13	OPTIONAL	-- Need ON

]],

[[	nr-ECID-RequestLocationInformation-r16

NR-ECID-RequestLocationInformation-r16

OPTIONAL,	-- Need ON

nr-Multi-RTT-RequestLocationInformation-r16

NR-Multi-RTT-RequestLocationInformation-r16

OPTIONAL,	-- Need ON

nr-DL-AoD-RequestLocationInformation-r16

NR-DL-AoD-RequestLocationInformation-r16

OPTIONAL,	-- Need ON

nr-DL-TDOA-RequestLocationInformation-r16

NR-DL-TDOA-RequestLocationInformation-r16

OPTIONAL	-- Need ON

]]

}

And it is specified that LMF configures the multiple positioning methods in accordance with UE capability, and the UE may combine outcome of different positioning methods or only use one positioning method to determine a single location estimation in TS 38.305 [8].

Section 5.4.4 of TS 38.305:

For positioning of a target UE, the LMF decides on the position methods to be used, based on factors that may include the LCS Client type, the required QoS, UE positioning capabilities, gNB positioning capabilities and ng-eNB positioning capabilities. The LMF then invokes these positioning methods in the UE, serving gNB and/or serving ng-eNB. The positioning methods may yield a location estimate for UE-based position methods and/or positioning measurements for UE-assisted and network-based position methods. The LMF may combine all the received results and determine a single location estimate for the target UE (hybrid positioning). Additional information like accuracy of the location estimate and velocity may also be determined.

Observation 4: The legacy LPP spec allows multiple positioning methods to be configured in RequestLocationInformation (named as “hybrid positioning”). It is specified that the UE may combine outcome of different positioning methods or only use one positioning method to determine a single location estimation.

Then, back to the FFS on autonomous fallback:

- We think hybrid positioning should still be allowed in AI/ML based positioning (i.e. both AI/ML positioning and non-AI/ML positioning can be configured as multiple positioning methods in RequestLocationInformation). And the legacy specification on UE behavior in hybrid positioning can be reused, e.g. the UE may combine outcome of both AI/ML and non-AI/ML positioning methods or only use non-AI/ML positioning method to determine a single location estimation. We don’t think further enhancement on AI/ML based positioning is needed.

- As we already have the terminology “hybrid positioning” in LPP spec, it is not necessary to introduce other terminology like “autonomous switching between AI/ML and non-AI/ML methods”

- The main benefit of autonomous fallback is just saving latency of one LPP message. However, Rel-19 AI/ML positioning doesn’t have requirement for latency reduction. So, we are not convinced its necessity.

Observation 5: The main benefit of fallback configuration is just saving latency of one LPP message. However, Rel-19 AI/ML positioning doesn’t have requirement for latency reduction.

Thus, we propose:

Proposal 3a: RAN2 confirm that hybrid positioning is supported in AI/ML based positioning (i.e. both AI/ML positioning and non-AI/ML positioning can be configured as multiple positioning methods in RequestLocationInformation). And the legacy specification on UE behavior in hybrid positioning is reused.

Proposal 3b: As LPP spec already specify the terminology “hybrid positioning”, no need to introduce other terminology like “autonomous switching between AI/ML and non-AI/ML methods” or “autonomous fallback”.

## 2.4 FFS 4: Whether any additional LMF control is needed

This FFS was captured in agreement of RAN2#129 [3]:

6: 	UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality.   FFS if any additional LMF control is needed.

In legacy unsolicited information transfer, legacy LPP procedure doesn’t need LMF control whether the UE can report. And please note that existing TS 38.305 has clearly captured such principle [8]:

7.1.2.5	Sequence of procedures

LPP procedures are not required to occur in any fixed order, in order to provide greater flexibility in positioning. Thus, a UE may request assistance data at any time in order to comply with a previous request for location measurements from the LMF; an LMF may instigate more than one request for location information (e.g., measurements or a location estimate) in case location results from a previous request were not adequate for the requested QoS; and the target device may transfer capability information to the server at any time if not already performed.

Despite the flexibility allowed by LPP, it is expected that procedures will normally occur in the following order:

1. Capability Transfer;

2. Assistance Data Transfer;

3. Location Information Transfer (measurements and/or location estimate).

Observation 6: Legacy LPP unsolicited information transfer procedure doesn’t need LMF control whether the UE can report. And TS 38.305 clearly captures that the target device may transfer capability information to the server at any time.

We see no reason to challenge this principle of legacy LPP in AI/ML based positioning. Thus, we propose:

Proposal 4: As legacy LPP, no need to introduce LMF control on the UE sending unsolicited LPP provide capabilities in step 4 for applicable functionality reporting.

## 2.4 FFS 5: order of inference config and applicability reporting

This FFS is captured in RAN2#127 post-meeting email discussion summary [9]:

Proposal 7: For positioning Case 1, UE can report a change in applicable functionality by sending applicable functionalities upon change via LPP signaling (unsolicited information transfer). UE can also provide applicable functionality reporting as a response to LMF request (solicited information transfer). FFS on the order of inference configuration and applicable functionality reporting, FFS on functionality activation.

Please note that existing TS 38.305 has captured below principle for LPP procedures [8]:

7.1.2.5	Sequence of procedures

LPP procedures are not required to occur in any fixed order, in order to provide greater flexibility in positioning. Thus, a UE may request assistance data at any time in order to comply with a previous request for location measurements from the LMF; an LMF may instigate more than one request for location information (e.g., measurements or a location estimate) in case location results from a previous request were not adequate for the requested QoS; and the target device may transfer capability information to the server at any time if not already performed.

Despite the flexibility allowed by LPP, it is expected that procedures will normally occur in the following order:

1. Capability Transfer;

2. Assistance Data Transfer;

3. Location Information Transfer (measurements and/or location estimate).

We see no reason to challenge above principle of LPP in AI/ML based positioning. Thus, we propose:

Proposal 5: Following existing principle on sequence order of LPP procedures, no need to specify fixed step order between inference configuration and applicable functionality reporting.

## 2.5 FFS 6: need of functionality activation

The existing LPP Location Information Transfer procedure can be found in Section 5.3.1 of 37.355 [10]:

### 5.3.1 Location Information Transfer procedure The Location Information Transfer procedure is shown in Figure 5.3.1-1. Figure 5.3.1-1: LPP Location Information transfer procedure 1. The server sends a RequestLocationInformation message to the target to request location information, indicating the type of location information needed and potentially the associated QoS. 2. The target sends a ProvideLocationInformation message to the server to transfer location information. The location information transferred should match or be a subset of the location information requested in step 1 unless the server explicitly allows additional location information. If step 3 does not occur, this message shall set the endTransaction IE to TRUE. 3. If requested in step 1, the target sends additional ProvideLocationInformation messages to the server to transfer location information. The location information transferred should match or be a subset of the location information requested in step 1 unless the server explicitly allows additional location information. The last message shall include the endTransaction IE set to TRUE.

As we can see, the UE immediately sends response via ProvideLocationInformation to LMF upon reception of RequestLocationInformation. Thus, it can be regarded that the functionality is activated when the LPP message RequestLocationInformation (i.e. configuration) is received, i.e. no extra activation procedure after reception of configuration.

Observation 7: In existing LPP Location Information Transfer procedure, the UE activates the response procedure when LPP message RequestLocationInformation is received, i.e. no extra activation procedure after reception of configuration.

We see no reason to introduce a new activation procedure on top of the existing procedure for AI/ML based positioning. Thus, we propose:

Proposal 6: For UE-side model of AI/ML positioning Case 1, the functionality is activated upon reception of LPP message RequestLocationInformation as legacy LPP Location Information Transfer procedure, i.e. no need to introduce extra procedure of functionality activation.

# 3 Conclusion

In this contribution, we discuss remaining issues on LCM procedure of AI/ML based positioning. Our observations are:

Observation 1: On-demand request for DL-PRS assistance data is already supported from Rel-17.

Observation 2: RAN1 agreed all assistance data of legacy UE-based DL-TDOA (other than info 7) are reused.

Observation 3: In AI/ML based beam management, the applicability reporting already takes UE-side additional condition into consideration, and UE-side additional condition is not explicitly reported to the NW because it includes UE’s internal information (e.g. battery and memory), which can’t be exposed to the NW.

Observation 4: The main benefit of fallback configuration is just saving latency of one LPP message. However, Rel-19 AI/ML positioning doesn’t have requirement for latency reduction.

Observation 5: The legacy LPP spec allows multiple positioning methods to be configured in RequestLocationInformation (named as “hybrid positioning”). It is specified that the UE may combine outcome of different positioning methods or only use one positioning method to determine a single location estimation.

Observation 6: Legacy LPP unsolicited information transfer procedure doesn’t need LMF control whether the UE can report. And TS 38.305 clearly captures that the target device may transfer capability information to the server at any time.

Observation 7: In existing LPP Location Information Transfer procedure, the UE activates the response procedure when LPP message RequestLocationInformation is received, i.e. no extra activation procedure after reception of configuration.

Based on observations, our proposals can be found below:

Proposal 1: On NW-side additional condition, support the legacy on-demand request for DL-PRS assistance data. No need to support on-demand request for other NW-side additional condition.

Proposal 2: Same as AI/ML based BM, the UE decides the applicable functionalities based on NW-side additional conditions, UE-side additional conditions and model availability in device. UE-side additional condition is not explicitly reported to the NW.

Proposal 3a: RAN2 confirm that hybrid positioning is supported in AI/ML based positioning (i.e. both AI/ML positioning and non-AI/ML positioning can be configured as multiple positioning methods in RequestLocationInformation). And the legacy specification on UE behavior in hybrid positioning is reused.

Proposal 3b: As LPP spec already specify the terminology “hybrid positioning”, no need to introduce other terminology like “autonomous switching between AI/ML and non-AI/ML methods” or “autonomous fallback”.

Proposal 4: As legacy LPP, no need to introduce LMF control on the UE sending unsolicited LPP provide capabilities in step 4 for applicable functionality reporting.

Proposal 5: Following existing principle on sequence order of LPP procedures, no need to specify fixed step order between inference configuration and applicable functionality reporting.

Proposal 6: For UE-side model of AI/ML positioning Case 1, the functionality is activated upon reception of LPP message RequestLocationInformation as legacy LPP Location Information Transfer procedure, i.e. no need to introduce extra procedure of functionality activation.

# 4 References

[1] RP-243244, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

[2] RAN2#127b, Chair Notes.

[3] RAN2#128, Chair Notes.

[4] RAN2#129, Chair Notes.

[5] R2-2500968, Report of [POST128][026][AIML] LCM Procedure for Positioning Case1 (Ericsson), Ericsson. d

[6] RAN1#119, Chair Notes.

[7] RAN2#127, Chair Notes.

[8] 3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[9] R2-2406381, Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)_Phase 2, Intel Corporation.

[10] 3GPP TS 37.355: "LTE Positioning Protocol (LPP)".