3GPP TSG-RAN WG2 Meeting #129bis                                                               R2-2502143

Wuhan, China, 7th - 11th April 2025

Source:	CATT

Title:	LP-WUS in RRC_IDLE/INACTIVE

Agenda Item:	8.4.2

Document for:	Discussion and Decision

- Introduction

In this contribution, further considerations are provided on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE, including:

- LP-WUS with eDRX;

- Remaining issues of UE_ID based LP-WUS Subgrouping;

- How to determine the type of LP-WUS subgrouping;

- Serving cell quality by MR/LR is cell selection value or measured value.

- Discussion

  - LP-WUS with eDRX

PEI is supported to be operated with eDRX together. If the eDRX cycle is longer than 1024 radio frames, the UE monitors PEI with the associated POs that are within a periodic PTW. If the eDRX cycle is no longer than 1024 radio frames, the UE monitors PEI with associated POs within the configured eDRX cycle that is same as the UE behaviors without eDRX cycle. The specification impact has only one, i.e. UE_ID based subgrouping for PEI is determined by 5G-S-TMSI mod 32768 when eDRX is applied. Similar as PEI, the similar specification impact is introduced if LP-WUS and eDRX are operated together. Considering the limited specification impact, we suggest LP-WUS is supported to be operated with eDRX together.

Proposal 1: LP-WUS is supported to be operated with eDRX simultaneously.

  - Remaining issues of UE_ID based Subgrouping for LP-WUS

In RAN2#129 meeting, the formula of UE_ID based subgrouping for LP-WUS was agreed.

For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e., SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where - UE_ID is related to 5G-S-TMSI, - N is the number of total paging frames in one DRX cycle, - Ns is the number of the PO for a PF, - Np is the number of subgroupNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1, - subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

Similar as UE_ID based subgrouping for PEI, UE_ID in the above formula is 5G-S-TMSI mod X, X is FFS.

In RAN1#120, the following was agreed:

Agreement Confirm the following working assumption with the modification: Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 31

With the agreement, the maximum value of subgroupsNumForUEID, i.e. the subgroup number for UE_ID based subgrouping for LP-WUS, is 31. And the maximum number of subgroups for UE_ID based PEI subgrouping, i.e. Np in the above agreement, is 8. For DRX, the maximum number of Ns is 4 while the maximum number of N is 256. Then for DRX with LP-WUS, UE_ID in the formula of UE_ID based subgrouping for LP-WUS should be at least 18bits, as shown in Figure 1. Otherwise, some values for LP-WUS subgrouping maybe never used.

For eDRX, the maximum number of Ns is 4 while the maximum number of N is 1024. Then for eDRX with LP-WUS, UE_ID in the formula of UE_ID based LP-WUS subgrouping should be at least 20bits, as shown in Figure 1.

Figure 1 Example of bits within UE_ID for UE_ID based LP-WUS subgrouping

Corresponding, for UE-ID based LP-WUS subgrouping, UE_ID = 5G-S-TMSI mod X, where X is 218 for DRX case and 220 for eDRX case if supported.

Proposal 2: For UE_ID based subgrouping for LP-WUS, UE_ID = 5G-S-TMSI mod X, X needs to be 218 for DRX case and 220 for eDRX case if supported.

The size of 5G-S-TMSI is 48bit. Hence, there is no issue for the UE to calculate UE-ID based LP-WUS subgrouping even if X =220. However, for RAN paging, RAN node is provided with an extended IE of 16 LSBs, not the entire UE ID, as shown below [1].

#### 9.3.3.52 Extended UE Identity Index Value This IE is used by the NG-RAN node to calculate the Paging Frame and Paging Occasion as specified in TS 36.304 [29], the Paging Frame and Paging Occasion for eDRX and the UE_ID based subgroup ID as specified in TS 38.304 [12]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Extended UE Identity Index Value; M; BIT STRING (SIZE(16)).

With proposal 2, the UE-ID in RAN node for RAN-originated paging, is not enough for UE-ID based LP-WUS subgrouping. There will be misalignment between the results of UE-ID based LP-WUS subgrouping in UE side and RAN node side for RAN-originated paging. Hence, the extended IE for RAN-originated paging in RAN node needs to be extended.

Observation 1: With the agreed formula of UE_ID based subgrouping for LP-WUS, the Extended UE Identity Index Value for RAN-originated paging in RAN node needs to be extended to at least 18bits for DRX case and at least 20bits if eDRX with LP-WUS is supported.

RAN2 need to send LS to RAN3 (CC SA2/CT1) to ask their concern on the agreed formula of UE_ID based subgrouping for LP-WUS.

Proposal 3: Send LS to RAN3 (CC SA2/CT1) to ask their concern on the agreed formula of UE_ID based subgrouping for LP-WUS.

  - Determination on the type of LP-WUS subgrouping

At the beginning of LP-WUS WI discussion, it was agreed:

=> The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups.

But there is no clear agreement to confirm how the UE to determine the type of subgrouping for LP-WUS, CN assigned subgrouping for LP-WUS or UE_ID based subgrouping for LP-WUS.

For PEI, the following description is captured in TS 38.304[2].

7.3.0 General //skip UE's subgroup can be either assigned by CN as specified in clause 7.3.1 or formed based on UE_ID as specified in clause 7.3.2: - If subgroupsNumForUEID is absent in subgroupConfig, the subgroup ID based on CN assigned subgrouping as specified in clause 7.3.1, if available for the UE, is used in the cell. - If both subgroupsNumPerPO and subgroupsNumForUEID are configured, and subgroupsNumForUEID has the same value as subgroupsNumPerPO, the subgroup ID based on UE_ID based subgrouping as specified in clause 7.3.2 is used in the cell. - If both subgroupsNumPerPO and subgroupsNumForUEID are configured, and subgroupsNumForUEID < subgroupsNumPerPO: - The subgroup ID based on CN assigned subgrouping as specified in clause 7.3.1, if available for the UE, is used in the cell; - Otherwise, the subgroup ID based on UE_ID based subgrouping as specified in clause 7.3.2 is used in the cell. If a UE has no CN assigned subgroup ID or does not support CN assigned subgrouping, and there is no configuration for subgroupsNumForUEID, the UE monitors the associated PO according to clause 7.1. //skip 7.3.2 UE_ID based subgrouping //skip If the UE is not configured with a CN assigned subgroup ID, or if the UE configured with a CN assigned subgroup ID is in a cell supporting only UE_ID based subgrouping, the subgroup ID of the UE is determined by the formula below: //skip

We propose to confirm the principle determining CN assigned subgrouping or UE_ID based subgrouping for PEI for a UE is reused for LP-WUS subgrouping.

Proposal 4: Confirm the principle that determines CN assigned subgrouping or UE_ID based subgrouping for PEI is reused for LP-WUS subgrouping.

  - Serving cell quality by MR/LR

For the metrics for entry condition and exit condition of LP-WUS monitoring, the following agreements were achieved in RAN2#127:

- The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. - The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

But according to existing relaxed measurement criteria, Srxlev and Squal are used [2], whereas Srxlev is cell selection RX level value of the serving cell, not the measured cell RX level value, i.e. Qrxlevmeas, and Squal cell RX is cell selection quality value of the serving cell, not the measured cell quality value, i.e., Qqualmeas. The corresponding definition in TS 38.304 is shown below:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset )– Pcompensation - Qoffsettemp Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp. where: Srxlev; Cell selection RX level value (dB). Squal; Cell selection quality value (dB). Qoffsettemp; Offset temporarily applied to a cell as specified in TS 38.331 [3] (dB). Qrxlevmeas; Measured cell RX level value (RSRP). Qqualmeas; Measured cell quality value (RSRQ). Qrxlevmin; Minimum required RX level in the cell (dBm). If the UE supports SUL frequency for this cell, Qrxlevmin is obtained from q-RxLevMinSUL, if present, in SIB1, SIB2 and SIB4, additionally, if QrxlevminoffsetcellSUL is present in SIB3 and SIB4 for the concerned cell, this cell specific offset is added to the corresponding Qrxlevmin to achieve the required minimum RX level in the concerned cell; else Qrxlevmin is obtained from q-RxLevMin in SIB1, SIB2 and SIB4, additionally, if Qrxlevminoffsetcell is present in SIB3 and SIB4 for the concerned cell, this cell specific offset is added to the corresponding Qrxlevmin to achieve the required minimum RX level in the concerned cell.. Qqualmin; Minimum required quality level in the cell (dB). Additionally, if Qqualminoffsetcell is signalled for the concerned cell, this cell specific offset is added to achieve the required minimum quality level in the concerned cell.. Qrxlevminoffset; Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN, as specified in TS 23.122 [9].. Qqualminoffset; Offset to the signalled Qqualmin taken into account in the Squal evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN, as specified in TS 23.122 [9].. Pcompensation; For FR1, if the UE supports the additionalPmax in the NR-NS-PmaxList, if present, in SIB1, SIB2 and SIB4: max(PEMAX1 –PPowerClass, 0) – (min(PEMAX2, PPowerClass) – min(PEMAX1, PPowerClass)) (dB); else: max(PEMAX1 –PPowerClass, 0) (dB) For FR2, Pcompensation is set to 0. For IAB-MT, Pcompensation is set to 0.. PEMAX1, PEMAX2; Maximum TX power level of a UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in TS 38.101 [15]. If UE supports SUL frequency for this cell, PEMAX1 and PEMAX2 are obtained from the p-Max for SUL in SIB1 and NR-NS-PmaxList for SUL respectively in SIB1, SIB2 and SIB4 as specified in TS 38.331 [3], else PEMAX1 and PEMAX2 are obtained from the p-Max and NR-NS-PmaxList respectively in SIB1, SIB2 and SIB4 for normal UL as specified in TS 38.331 [3].. PPowerClass; Maximum RF output power of the UE (dBm) according to the UE power class as defined in TS 38.101-1 [15]..

Hence, we need to clarify serving cell quality by MR for conditions of LP-WUS monitoring is to reuse cell selection value or measured value.

In order to align with existing relaxed measurement criteria, we slightly prefer to reuse cell selection value for serving cell quality by MR for conditions of LP-WUS monitoring. And this can be applied to serving cell quality by MR for conditions of measurement relaxation and serving cell measurement offloading for LP-WUS UEs as well.

Proposal 5: Cell selection value, i.e., existing Srxlev/ Squal in TS 38.304, is used for serving cell quality by MR for conditions of LP-WUS monitoring, measurement relaxation and serving cell measurement offloading for LP-WUS UEs.

Serving cell quality by LR is used in conditions in LP-WUS. We need to clarify if it is cell selection value or measured value. Currently there is no existing definition of cell selection value for serving cell quality by LR. And serving cell quality by LR is used for conditions of LP-WUS monitoring, measurement relaxation and serving cell measurement offloading for LP-WUS UEs, not for cell selection. We don’t need to definition a new formula for cell selection value of serving cell quality by LR.

Proposal 6: Measured value is used for serving cell quality by LR for conditions of LP-WUS monitoring, measurement relaxation and serving cell measurement offloading for LP-WUS UEs.

- Conclusion

According to the analysis in section 2, we propose:

LP-WUS with eDRX:

Proposal 1: LP-WUS is supported to be operated with eDRX simultaneously.

Remaining issues of UE_ID based Subgrouping for LP-WUS:

Observation 1: With the agreed formula of UE_ID based subgrouping for LP-WUS, the Extended UE Identity Index Value for RAN-originated paging in RAN node needs to be extended to at least 18bits for DRX case and at least 20bits if eDRX with LP-WUS is supported.

Proposal 2: For UE_ID based subgrouping for LP-WUS, UE_ID = 5G-S-TMSI mod X, X needs to be 218 for DRX case and 220 for eDRX case if supported.

Proposal 3: Send LS to RAN3 (CC SA2/CT1) to ask their concern on the agreed formula of UE_ID based subgrouping for LP-WUS.

Determination on the type of LP-WUS subgrouping

Proposal 4: Confirm the principle that determines CN assigned subgrouping or UE_ID based subgrouping for PEI is reused for LP-WUS subgrouping.

Serving cell quality by MR/LR

Proposal 5: Cell selection value, i.e., existing Srxlev/ Squal in TS 38.304, is used for serving cell quality by MR for conditions of LP-WUS monitoring, measurement relaxation and serving cell measurement offloading for LP-WUS UEs.

Proposal 6: Measured value is used for serving cell quality by LR for conditions of LP-WUS monitoring, measurement relaxation and serving cell measurement offloading for LP-WUS UEs.

- Reference

- 3GPP TS 38.413 V18.5.0, NG-RAN, NG Application Protocol (NGAP).

- 3GPP TS 38.304 V18.4.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state.