3GPP TSG-RAN WG2 Meeting #129bis                                                               R2-2502147

Wuhan, China, 7th - 11th April 2025

Source:	CATT

Title:	Discussion on UE-initiated Beam Reporting and CSI Enhancement

Agenda Item:	8.12.3

Document for:	Discussion and Decision

- 1. Introduction

For Rel-19 MIMO, during the RAN2#129 meeting, the following agreements are made for UE-initiated Beam Reporting [1].

Working assumption: RAN2 assume the event evaluation for UE-initiated beam reporting is captured in 38.321, where the evaluation is based on indications from measurements described in a RAN1 specification. The final decision is up to RAN1, we can revisit this if R1 has a different decision. No need to send a LS to R1 on this.

In this contribution, we will further discuss the detail spec impacts for UE-initiated beam reporting and CSI enhancement.

- 2. UE-initiated beam reporting

RAN1#117 and RAN1#120 had made following progress on the UE-initiated beam reporting and there are two FFS on whether the above is captured in RAN1 or RAN2 specification.

Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification. Agreement Regarding triggering event determination for Event 2, at least Candidate #2 is supported for resetting the counting. - Candidate#1: RS reconfiguration/update or MAC-CE signaling (if supported) for new beam is received; - FFS: whether to reset the counting for all new beams. - FFS: whether to maintain the counting whose new beam is NOT updated. - Candidate#2: [The measured current beam based on] indicated TCI state is updated; - In such case, the UE need to reset the counting for all new beams. - Candidate#3: UEI beam report is transmitted; - FFS: Only reset the counting of new beams fulfilling triggering condition and reported by the UEI beam report - Candidate#4: NW response (e.g., DCI in step-2 of Mode-A) is detected. - Candidate#5: The time window expires - Candidate#6: The threshold for event evaluation is re-configured by RRC signaling - (FFS) Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured. - FFS: RRC parameter(s) - FFS: Other candidates Note: Whether this proposal is captured in RAN1 or RAN2 is a separate discussion point.

In the legacy BFR procedure, UE reports the L1 measurement results in MAC CE and the counting is performed in L2 within the time window. Thus, the legacy BFR procedure is captured in RAN2 spec. However, for UEIBM, the RS for current beam is implicitly derived from indicated TCI state and the report is via UCI. The measurement and reporting are performed in RAN1 only. Therefore, the time window and counting for UE-initiated beam reporting should be done in RAN1 and such procedure should be captured in RAN1 spec.

Proposal 1: Regarding the event evaluation for UE-initiated beam reporting, suggest capture the timing window and counting for UE-initiated beam reporting in RAN1 spec.

During RAN2#129 meeting, RAN2 made some discussion on whether the UE initiated beam reporting is handled in MAC or PHY layer. The working assumption has been made that RAN2 can discuss how to capture this in MAC spec unless RAN1 make a final decision. From our perspective, if the event evaluation for UE-initiated beam reporting is captured in 38.321, only the time window and counting for Event-2 can be captured based on above RAN1 agreements. Moreover, the time window and counting for Event-2 should be captured separately compared with LTM event- triggered L1 measurement reporting, since UE-initiated beam reporting and LTM event- triggered L1 measurement reporting are designed independently with different event evaluation and trigger conditions.

Proposal 2: If the time window and counting for UE-initiated beam reporting Event-2 is captured in MAC, RAN2 confirms that UE-initiated beam reporting does not share common framework of LTM event-triggered L1 measurement.

- 3. CSI enhancement

RAN1 sent an LS[2] with a revised list of RRC parameters during RAN1#119 meeting. Among these RRC parameters, there are some parameters which need to be discussed in RAN2. During the email discuss [Post129][208], Rapporteur discussed most of parameters and summary companies view and proposal in [3]. But one left parameter should be discussed as following.

| Sub-feature group | RAN2 Parent IE | Parameter name in the spec | New or existing? | Value range | Comment |
| --- | --- | --- | --- | --- | --- |
| CSI-CRI | CSI-ReportConfig | reportQuantity-r19 | New | ‘cri-RI-PMI-CQI’, ‘cri-RI-LI-PMI-CQI’ | It is up to RAN2 whether the legacy reportQuantity is reused for this or not. If reused, this parameter is not needed |

Regarding the above RRC parameter reportQuantity-r19, we prefer to reuse the legacy parameter reportQuantity. The legacy parameter reportQuantity is as following.

reportQuantity                          CHOICE {

none                                    NULL,

cri-RI-PMI-CQI                          NULL,

cri-RI-i1                               NULL,

cri-RI-i1-CQI                           SEQUENCE {

pdsch-BundleSizeForCSI                  ENUMERATED {n2, n4}                              OPTIONAL    -- Need S

},

cri-RI-CQI                              NULL,

cri-RSRP                                NULL,

ssb-Index-RSRP                          NULL,

cri-RI-LI-PMI-CQI                       NULL

},

reportFreqConfiguration                 SEQUENCE {

Since the value range of legacy reportQuantity also includes ‘cri-RI-PMI-CQI’ and ‘cri-RI-LI-PMI-CQI’ as above, the legacy reportQuantity can be reused for Rel-19. Thus, the parameter reportQuantity-r19 is not needed.

Proposal 3: The legacy parameter reportQuantity can be reused for Rel-19 MIMO. The parameter reportQuantity-r19 in the list of RRC parameters is not needed.

During the post email discussion on running CR for TS 38.331, many issues were discussed and addressed in [4]. From our perspective, one issue on IE linkedCJTCReport should be further discussed as following.

In current running CR[4], IE linkedCJTCReport is under the CSI-ReportCJTC-r19 as following.

CSI-ReportConfig ::=                SEQUENCE {

reportConfigId                          CSI-ReportConfigId,

carrier                                 ServCellIndex                   OPTIONAL,   -- Need S

resourcesForChannelMeasurement          CSI-ResourceConfigId,

csi-IM-ResourcesForInterference         CSI-ResourceConfigId            OPTIONAL,   -- Need R

nzp-CSI-RS-ResourcesForInterference     CSI-ResourceConfigId            OPTIONAL,   -- Need R

Omit…

CSI-ReportCJTC-r19 ::=              SEQUENCE {

valueOfAD-r19               ENUMERATED {dot5, one}             OPTIONAL,    -- Need R

valueOfMD-r19               ENUMERATED {n32, n64, n128, n256}  OPTIONAL,    -- Need R

valueOfAFO-r19              ENUMERATED {zeroDot1, zeroDo       OPTIONAL,    -- Need R

valueOfMFO-r19              ENUMERATED {n16, n32, n256 }       OPTIONAL,    -- Need R

valueOfMPhi-r19             ENUMERATED {n16, n32}              OPTIONAL,    -- Need R

linkedCJTCReport-r19        CSI-ReportConfigID                 OPTIONAL,    -- Need R

subbandSizeCJTC-19       ENUMERATED {n1, n2, n4, n8, n16, wideband} OPTIONAL     -- Need R

}

CSI-ReportCJTC field descriptions

linkedCJTCReport Linked CJTC report when codebook type is set to typeII-CJT-r18.

According to RAN1 agreement, one CJTC report can link with different CJT report. Above current RRC structure can’t achieve this feature. Thus, we suggest configure linkedCJTCReport outside of CSI-ReportCJTC-r19, i.e., under CSI-ReportConfig, so that the linkedCJTCReport can configure the ID of CJTC report for each CJT report.

Proposal 4: Configure linkedCJTCReport outside of CSI-ReportCJTC-r19, i.e., under CSI-ReportConfig.

- 4. Conclusion

This paper discusses the detail spec impacts for UE-initiated beam reporting and CSI enhancement. The contribution of this paper is summarized as follows.

Proposal 1: Regarding the event evaluation for UE-initiated beam reporting, suggest capture the timing window and counting for UE-initiated beam reporting in RAN1 spec.

Proposal 2: If the time window and counting for UE-initiated beam reporting Event-2 is captured in MAC, RAN2 confirms that UE-initiated beam reporting does not share common framework of LTM event-triggered L1 measurement.

Proposal 3: The legacy parameter reportQuantity can be reused for Rel-19 MIMO. The parameter reportQuantity-r19 in the list of RRC parameters is not needed.

Proposal 4: Configure linkedCJTCReport outside of CSI-ReportCJTC-r19, i.e., under CSI-ReportConfig.

- 5. Reference

- R2-2501332	, Rel-18 MIMO, Rel-19 MIMO, LPWUS, and SBFD	Vice Chairman (CATT) report

- R2-2500008, LS to RAN2 on RRC and MAC impacts for Rel-19 NR MIMO Ph5 (R1-2410758; contact: Samsung)

- R2-2502546, Report of [Post129][208][ MIMO_Ph5], Erisson

- R2-2502545, Running CR for MIMO Phase 5, Ericsson